3GPP TSG-SA5 Meeting #125AH 	S5-194468

# Sapporo, Japan, 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0117 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | generate JSON definition for NR NRM based on new style guideline |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Align with JSON definition style guide and stage 2 definition |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change JSON definition for NR NRM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The JSON definition is difficult to be implemented by operators and vendors |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, D.4.2, D.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Include nrNrm.json |  |  |  |  |  |  |  |  |

Start of 1st modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

[x]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

End of 1st modification

Start of 2nd modification

Annex D (normative):
JSON definitions for NR NRM

# D.1	General

This annex contains the JSON definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM information model definitions specified in clause 4.

# D.2	Architectural features

# D.3	Mapping

# Refer to TS 32.160 [x]

# D.4	Solution Set (SS) definitions

## D.4.1	JSON definition structure

JSON is used as resource representations format carried in the HTTP request and HTTP response message bodies. The properties (key-value pairs) on an object are defined using the properties keyword.

## D.4.2	Graphical representation

None

## D.4.3	JSON schema "nrNrm.json"

{

"openapi": "3.0.1",

"info": {

"title": "3GPP NR NRM",

"version": "15.3.0",

"description": "OAS 3.0.1 specification compatible schema for 3GPP NR NRM"

},

"paths": {},

"components": {

"schemas": {

"GnbId": {

"type": "string"

},

"GnbIdLength": {

"type": "integer",

"minimum": 22,

"maximum": 32

},

"GnbName": {

"type": "string",

"maxLength": 150

},

"GnbDuId": {

"type": "number",

"minimum": 0,

"maximum": 68719476735

},

"NCi": {

"type": "object",

"properties": {

"plmnId": {

"$ref": "#/components/schemas/PlmnId"

},

"nCI": {

"$ref": "#/components/schemas/NrCellId"

}

}

},

"SnssaiList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/Snssai"

}

},

"RrmPolicy": {

"type": "string"

},

"NrPci": {

"type": "integer",

"maximum": 503

},

"NrTac": {

"type": "integer",

"maximum": 16777215

},

"NrCellId": {

"type": "integer",

"maximum": 68719476735

},

"Sst": {

"type": "integer",

"maximum": 255

},

"Snssai": {

"type": "object",

"properties": {

"sst": {

"$ref": "#/components/schemas/Sst"

},

"sd": {

"type": "string"

}

}

},

"CellState": {

"enum": [

"IDLE",

"INACTIVE",

"ACTIVE"

]

},

"CyclicPrefix": {

"enum": [

"15",

"30",

"60",

"120"

]

},

"TxDirection": {

"enum": [

"DL",

"UL",

"DL and UL"

]

},

"BwpContext": {

"enum": [

"DL",

"UL",

"SUL"

]

},

"IsInitialBwp": {

"enum": [

"INITIAL",

"OTHER",

"SUL"

]

},

"QuotaType": {

"enum": [

"STRICT",

"FLOAT"

]

},

"RrmPolicyRatio2": {

"type": "object",

"properties": {

"groupId": {

"type": "integer"

},

"sNSSAIList": {

"$ref": "#/components/schemas/SnssaiList"

},

"quotaType": {

"$ref": "#/components/schemas/QuotaType"

},

"rRMPolicyMaxRation": {

"type": "integer"

},

"rRMPolicyMarginMaxRation": {

"type": "integer"

},

"rRMPolicyMinRation": {

"type": "integer"

},

"rRMPolicyMarginMinRation": {

"type": "integer"

}

}

},

"Mnc": {

"type": "string",

"pattern": "[0-9]{3}|[0-9]{2}"

},

"PlmnId": {

"type": "object",

"properties": {

"mcc": {

"$ref": "genericNrm.json#/components/schemas/Mcc"

},

"mnc": {

"$ref": "#/components/schemas/Mnc"

}

}

},

"PlmnIdList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/PlmnId"

}

},

"LocalAddress": {

"type": "object",

"properties": {

"ipv4Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv4Addr"

},

"ipv6Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv6Addr"

},

"vlanId": {

"type": "integer",

"minimum": 0,

"maximum": 4096

},

"port": {

"type": "integer",

"minimum": 0,

"maximum": 65535

}

}

},

"RemoteAddress": {

"type": "object",

"properties": {

"ipv4Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv4Addr"

},

"ipv6Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv6Addr"

}

}

},

"GnbDuFunction": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbDuId": {

"$ref": "#/components/schemas/GnbDuId"

},

"gnbDuName": {

"$ref": "#/components/schemas/GnbName"

},

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

}

}

}

]

},

"MeasurementControl": {

"$ref": "genericNrm.json#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "genericNrm.json#/components/schemas/VsDataContainer-Multiple"

},

"EP_F1C": {

"$ref": "#/components/schemas/EP_F1C"

},

"EP_F1U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1U"

}

},

"NrCellDu": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NrCellDu"

}

},

"NrSectorCarrier": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NrSectorCarrier"

}

},

"Bwp": {

"type": "array",

"items": {

"$ref": "#/components/schemas/Bwp"

}

}

}

},

"GnbCuCpFunction": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

},

"gnbCuName": {

"$ref": "#/components/schemas/GnbName"

},

"plmnIdList": {

"$ref": "#/components/schemas/PlmnIdList"

}

}

}

]

},

"MeasurementControl": {

"$ref": "genericNrm.json#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "genericNrm.json#/components/schemas/VsDataContainer-Multiple"

},

"EP_F1C": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1C"

}

},

"EP_E1": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_E1"

}

},

"NrCellCu": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NrCellCu"

}

}

}

},

"GnbCuUpFunction": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

},

"plmnIdList": {

"$ref": "#/components/schemas/PlmnIdList"

}

}

}

]

},

"MeasurementControl": {

"$ref": "genericNrm.json#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "genericNrm.json#/components/schemas/VsDataContainer-Multiple"

},

"EP_E1": {

"$ref": "#/components/schemas/EP_E1"

},

"EP_F1U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1U"

}

},

"EP_Core": {

"type": "array",

"items": {

"oneOf": [

{

"$ref": "#/components/schemas/EP_NgU"

},

{

"$ref": "#/components/schemas/EP_XnU"

}

]

}

},

"EP_gNB": {

"type": "array",

"items": {

"oneOf": [

{

"$ref": "#/components/schemas/EP_X2U"

},

{

"$ref": "#/components/schemas/EP_S1U"

}

]

}

}

}

},

"NrCellCu": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"nCi": {

"$ref": "#/components/schemas/NCi"

},

"plmnIdList": {

"$ref": "#/components/schemas/PlmnIdList"

},

"snssaiList": {

"$ref": "#/components/schemas/SnssaiList"

},

"rrmPolicyType": {

"type": "integer"

},

"rrmPolicyNSSIId": {

"$ref": "genericNrm.json#/components/schemas/Dn"

},

"rrmPolicyRatio": {

"type": "integer"

},

"rrmPolicy": {

"$ref": "#/components/schemas/RrmPolicy"

},

"rrmPolicyRatio2": {

"$ref": "#/components/schemas/RrmPolicyRatio2"

}

}

}

]

},

"MeasurementControl": {

"$ref": "genericNrm.json#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "genericNrm.json#/components/schemas/VsDataContainer-Multiple"

}

}

},

"NrCellDu": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"administrativeState": {

"$ref": "genericNrm.json#/components/schemas/AdministrativeState"

},

"operationalState": {

"$ref": "genericNrm.json#/components/schemas/OperationalState"

},

"cellLocalId": {

"type": "integer"

},

"cellState": {

"$ref": "#/components/schemas/CellState"

},

"nCi": {

"$ref": "#/components/schemas/NCi"

},

"plmnIdList": {

"$ref": "#/components/schemas/PlmnIdList"

},

"snssaiList": {

"$ref": "#/components/schemas/SnssaiList"

},

"nrPci": {

"$ref": "#/components/schemas/NrPci"

},

"nrTac": {

"$ref": "#/components/schemas/NrTac"

},

"arfcnDL": {

"type": "integer"

},

"arfcnUL": {

"type": "integer"

},

"arfcnSUL": {

"type": "integer"

},

"bSChannelBwDL": {

"type": "integer"

},

"bSChannelBwUL": {

"type": "integer"

},

"bSChannelBwSUL": {

"type": "integer"

},

"nRSectorCarrier": {

"type": "string"

},

"bwp": {

"type": "string"

}

}

}

]

},

"MeasurementControl": {

"$ref": "genericNrm.json#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "genericNrm.json#/components/schemas/VsDataContainer-Multiple"

}

}

},

"NrSectorCarrier": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"txDirection": {

"$ref": "#/components/schemas/TxDirection"

},

"configuredMaxTxPower": {

"type": "integer"

},

"arfcnDL": {

"type": "integer"

},

"arfcnUL": {

"type": "integer"

},

"bSChannelBwDL": {

"type": "integer"

},

"bSChannelBwUL": {

"type": "integer"

},

"sectorEquipmentFunction": {

"type": "string"

}

}

}

]

},

"MeasurementControl": {

"$ref": "genericNrm.json#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "genericNrm.json#/components/schemas/VsDataContainer-Multiple"

}

}

},

"Bwp": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"bwpContext": {

"$ref": "#/components/schemas/BwpContext"

},

"isInitialBwp": {

"$ref": "#/components/schemas/IsInitialBwp"

},

"subCarrierSpacing": {

"type": "integer"

},

"cyclicPrefix": {

"$ref": "#/components/schemas/CyclicPrefix"

},

"startRB": {

"type": "integer"

},

"numberOfRBs": {

"type": "integer"

}

}

}

]

},

"MeasurementControl": {

"$ref": "genericNrm.json#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "genericNrm.json#/components/schemas/VsDataContainer-Multiple"

}

}

},

"ExternalGnbDuFunction": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

}

}

}

]

},

"EP_F1C": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1C"

}

},

"EP_F1U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1U"

}

}

}

},

"ExternalGnbCuCpFunction": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

}

}

}

]

},

"ExternalNrCellCu": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalNrCellCu"

}

},

"EP_F1C": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1C"

}

},

"EP_E1": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_E1"

}

},

"EP_XnC": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_XnC"

}

}

}

},

"ExternalGnbCuUpFunction": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

}

}

}

]

},

"EP_E1": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_E1"

}

},

"EP_F1U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1U"

}

},

"EP_XnU": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_XnU"

}

}

}

},

"ExternalAmfFunction": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

"EP_NgC": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_NgC"

}

}

}

},

"ExternalUpfFunction": {

"required": [

"id"

],

"type": "object",

"properties": {

"id": {

"type": "string"

},

"attributes": {

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

"EP_X2U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_X2U"

}

}

}

},

"ExternalNrCellCu": {

"required": [

"id"

],

"type": "object",

"properties": {

"id": {

"type": "string"

},

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"cellLocalId": {

"type": "integer"

},

"nrPci": {

"$ref": "#/components/schemas/NrPci"

},

"arfcnDL": {

"type": "integer"

}

}

}

]

}

}

},

"ManagedElement-Single": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"$ref": "genericNrm.json#/components/schemas/ManagedElement-Attributes"

},

"GnbDuFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/GnbDuFunction"

}

},

"GnbCuCpFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/GnbCuCpFunction"

}

},

"GnbCuUpFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/GnbCuUpFunction"

}

},

"MeasurementControl": {

"$ref": "genericNrm.json#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "genericNrm.json#/components/schemas/VsDataContainer-Multiple"

}

}

},

"ManagedElement-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ManagedElement-Single"

}

},

"SubNetwork-Single": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"$ref": "genericNrm.json#/components/schemas/SubNetwork-Attributes"

},

"ManagementNode": {

"$ref": "genericNrm.json#/components/schemas/ManagementNode-Multiple"

},

"MeContext": {

"$ref": "genericNrm.json#/components/schemas/MeContext-Multiple"

},

"SubNetwork": {

"$ref": "#/components/schemas/SubNetwork-Multiple"

},

"ManagedElement": {

"$ref": "#/components/schemas/ManagedElement-Multiple"

},

"MeasurementControl": {

"$ref": "genericNrm.json#/components/schemas/MeasurementControl-Multiple"

},

"VsDataContainer": {

"$ref": "genericNrm.json#/components/schemas/VsDataContainer-Multiple"

},

"ExternalGnbCuCpFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalGnbCuCpFunction"

}

}

}

},

"SubNetwork-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SubNetwork-Single"

}

},

"EP_RP": {

"type": "object",

"required": [

"id"

],

"properties": {

"id": {

"type": "string"

},

"attributes": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/components/schemas/LocalAddress"

},

"remoteAddress": {

"$ref": "#/components/schemas/RemoteAddress"

}

}

}

}

},

"EP_E1": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_XnC": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_XnU": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_NgC": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_NgU": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_F1C": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_F1U": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_S1U": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_X2C": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_X2U": {

"$ref": "#/components/schemas/EP_RP"

}

}

}

}

End of 2nd modification