3GPP TSG-SA5 Meeting #127	S5-196259

# Sophia-Antipolis, France, 14th Oct 2019 - 18th Oct 2019                    Revision of S5-196xxx

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.274 | CR | 0071 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Multiple Unit Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SMSCH |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is possible for CHF to grant quota for multiple SMSs in ECUR mode, however the "Multiple Unit Information" Information Element is described as not applicable in Charging Date response. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the "Multiple Unit Information" description for applicability in ECUR. Clarify the "Multiple Unit Usage" when multiple SMSs are reported in ECUR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ECUR mode is not completely specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2a.1.2.1, 6.2a.1.2.2, 6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 6.2a.1.2.1	Charging Data Request message

Table 6.2a.1.2.1.1 illustrates the basic structure of a Charging Data Request message as used for SMS converged charging.

# Table 6.2a.1.2.1.1: Charging Data Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57] |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| NF Functionality | M | Described in TS 32.290 [57] |
| NF Name | OC | Described in TS 32.290 [57] |
| NF Address | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Notify URI | OC | This field is not applicable. |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the SMS specific triggers described in clause 5.x |
| Multiple Unit Usage | OC | This field is present when the number of units is beyond one (i.e. more than one SMS) |
| Rating Group | M | Described in TS 32.290 [57] |
| Requested Unit | OC | Described in TS 32.290 [57] |
| Used Unit Container | OC | This field holds SMS charging information when more than one SMS. It may have multiple occurrences. |
| SMS Charging Information | OM | This field holds the SMS specific information described in clause 6.5.2. |
| SMS Charging Information | OM | This field holds the SMS specific information described in clause 6.5.2. |

Next change

##### 6.2a.1.2.2	Charging Data Response message

Table 6.2a.1.2.2.1 illustrates the basic structure of a Charging Data Response message as used for SMS converged charging.

# Table 6.2a.1.2.2.1: Charging Data Response Message Contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | OC | Described in TS 32.290 [57] |
| Result code | OC | Described in TS 32.290 [57] |
| Failed parameter | OC | Described in TS 32.290 [57] |
| Failure Handling | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is not applicable. |
| Multiple Unit information | OC | This field is applicable for ECUR. |
| Result Code | OC | Described in TS 32.290 [57] |
| Rating Group | OM | Described in TS 32.290 [57] |
| Granted Unit | OC | Described in TS 32.290 [57] |
| Validity Time | OC | Described in TS 32.290 [57] |

Next change

### 6.5.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by SMS Node for 5G SMS converged charging.

The Operation Types are listed in the following order: I (Initial)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as ITE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.5.3.1 defines the basic structure of the supported fields in the Charging Data Request message for SMS converged charging.

# Table 6.5.3.1: Supported fields in Charging Data Request message

| Information Element | Node Type | SMSF |
| --- | --- | --- |
|  | Supported Operation Types | ITE |
| Session Identifier |  | ITE |
| Subscriber Identifier |  | ITE |
| NF Consumer Identification |  | ITE |
| Invocation Timestamp |  | ITE |
| Invocation Sequence Number |  | ITE |
| Notify URI |  | - |
| Triggers |  | ITE |
| Multiple Unit Usage |  | ITE |
| Rating Group |  | IT- |
| Requested Unit |  | I-- |
| Used Unit Container |  | IT- |
| SMS Charging Information |  |  |
| Originator Info |  | ITE |
| Recipient Info |  | ITE |
| User Equipment Info |  | ITE |
| User Location Info |  | ITE |
| UE Time Zone |  | ITE |
| RAT Type |  | ITE |
| SMSC Address |  | ITE |
| SM Data Coding Scheme |  | ITE |
| SM Message Type |  | ITE |
| SM Reply Path Requested |  | ITE |
| SM User Data Header |  | ITE |
| SM Status |  | ITE |
| SM Discharge Time |  | ITE |
| Number of Messages Sent |  | ITE |
| SM Service Type |  | ITE |
| SM Sequence Number |  | ITE |
| SMS result |  | ITE |
| Submission Time |  | ITE |
| SMPriority |  | ITE |
| Message Reference |  | ITE |
| Message Size |  | ITE |
| Message Class |  | ITE |
| Delivery Report Requested |  | ITE |

Table 6.5.3.2 defines the basic structure of the supported fields in the Charging Data Response message for SMS converged charging.

# Table 6.5.3.2: Supported fields in Charging Data Response message

| Information Element | Node Type | SMSF |
| --- | --- | --- |
|  | Supported Operation Types | ITE |
| Session Identifier |  | ITE |
| Invocation Timestamp |  | ITE |
| Invocation Result |  | ITE |
| Invocation Sequence Number |  | ITE |
| Session Failover |  | I-- |
| Triggers |  | - |
| Multiple Unit information |  | I-E |
| Result Code |  | I-- |
| Rating Group |  | I-- |
| Granted Unit |  | I-- |
| Validity Time |  | I-- |

End of changes