3GPP TSG-SA5 Meeting #127	S5-196389

# Sophia-Antipolis, France, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0034 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add usecase and requirements for NF KPI job control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecommunications, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA-OAM_KPIRPT |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the objective of the new work item KPI reporting, it mentioned that “enhance (with maximum reuse) the performance assurance MnS to support KPI collection and reporting”. In the current specfication TS 28.550, it is lack of the use cases and requirements for KPI job control. So it is proposed to add new KPI job control related use cases and requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add usecase and requirements for NF KPI job control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no requirements and corresponding use cases for NF KPI job control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X (new), 5.1.1.X.1 (new), 5.1.1.X.2 (new), 5.1.1.X.3 (new), 5.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.1	NF PM services

5.1.1.X	NF KPI job control service

5.1.1.X.1	Creation of KPI job for NF(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to create a KPI job for collecting the KPIs of NF(s). |  |
| Actors and Roles | An authorized consumer of NF KPI job control service. |  |
| Telecom resources | NF(s); NF KPI job control service producer. NF measurement job control service producer |  |
| Assumptions | N/A |  |
| Pre-conditions | - The NF(s) have been deployed. - The NF KPI job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to create KPI job for collecting the KPIs of NF(s). |  |
| Step 1 (M) | The authorized consumer requests the NF KPI job control service producer to create KPI job to calculate and collect the KPIs of NF(s). The request needs to indicate that the KPIs needs to be reported by performance data file or by performance data streaming. |  |
| Step 2 (M) | The NF KPI job control service producer identifies the performance measurement types used for the NF KPI(s) and checks whether the corresponding performance measurement types can be collected by the existing measurement job(s) for the NF(s). If new measurement job(s) for the NF(s) are required, the NF KPI job control service producer consumes the NF measurement job control service to create the new measurement job(s) for the NF(s) (according to the use case "Creation of measurement job for NF(s)" as described in clause 5.1.1.1.1). The NF KPI job control service producer requests the NF(s) to calculate and collect the KPIs, per the received KPI job creation request. | Creation of measurement job for NF(s) |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The KPI job for NF(s) has been created, and the NF KPI job control service producer generates the KPIs for the NF KPI job. |  |
| Traceability | REQ- KJCS _NF-FUN-1, REQ- KJCS _NF-FUN-2, REQ- KJCS _NF-FUN-3, REQ- KJCS _NF-FUN-4 and REQ- KJCS _NF-FUN-7 |  |

5.1.1.X.2	Termination of KPI job for NF(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request the NF KPI job control service producer to terminate a NF KPI job. |  |
| Actors and Roles | An authorized consumer of NF KPI job control service. |  |
| Telecom resources | NF(s) NF KPI job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NF KPI job has been created. |  |
| Begins when | The authorized consumer does not need the NF KPI job that is calculating or collecting the KPIs of NF(s). |  |
| Step 1 (M) | The authorized consumer requests the NF KPI job control service producer to terminate a KPI job that is calculating or collecting the KPIs of NF(s). |  |
| Step 2 (M) | The NF KPI job control service producer terminates the KPI job. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NF KPI job is terminated |  |
| Traceability | REQ- KJCS _NF-FUN-5 |  |

5.1.1.X.3	Query of KPI jobs for NF(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to query the ongoing NF KPI jobs (i.e. the NF KPI jobs that have been created by the subject consumer and not terminated). |  |
| Actors and Roles | An authorized consumer of NF KPI job control service. |  |
| Telecom resources | NF(s) NF KPI job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NF KPI job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to query the ongoing NF KPI jobs. |  |
| Step 1 (M) | The authorized consumer queries the information about the ongoing NF KPI jobs from the NF KPI job control service producer. |  |
| Step 2 (M) | The NF KPI job control service producer provides the information about the ongoing NF KPI jobs to the consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The information about the ongoing NF KPI jobs are available to the consumer. |  |
| Traceability | REQ- KJCS _NF-FUN-6 |  |

Next change

5.2	Requirements

5.2.X	Requirements for NF KPI job control service

REQ-KJCS_NF-FUN-1	The management service producer responsible for NF KPI job control shall have the capability allowing its authorized consumer to request creation of a KPI job to collect the KPIs of NF(s).

REQ- KJCS _NF-FUN-2	The management service producer responsible for NF KPI job control shall have the capability allowing its authorized consumer to indicate the reporting method (i.e. by performance data file or by performance data streaming) for the KPIs when requesting to create a KPI job for NF(s).

REQ- KJCS _NF-FUN-3	The management service producer responsible for NF KPI job control shall have the capability to fulfill the consumer's request to create a KPI job for NF(s).

REQ- KJCS _NF-FUN-4	The management service producer responsible for NF KPI job control shall have the capability to generate the KPIs of NF(s) according to the KPI job.

REQ- KJCS _NF-FUN-5	The management service producer responsible for NF KPI job control shall have the capability to fulfill the request from its authorized consumer to terminate a NF KPI job.

REQ- KJCS _NF-FUN-6	The management service producer responsible for NF KPI job control shall have the capability allowing its authorized consumer to query the information about the ongoing NF KPI jobs.

REQ- KJCS _NF-FUN-7	The management service producer responsible for NF KPI job control may reject a NF KPI job creation request.

End of changes