3GPP TSG-SA5 Meeting #127 	S5-196518

# Sophia Antipolis, France, 14 – 18 October 2019

# Source:	3GPP RAN2

# Title:	Reply LS on radio resource management policy

# Document for:	Information, Discussion

Agenda Item: 6.4.7

3GPP TSG-RAN WG2 Meeting #107	R2-1911791

Prague,  Czech Republic, 26 – 30 August 2019

Title:	Reply LS on radio resource management policy

Response to:	R2-1908671/S5-194485

Release:	Release 16

Work Item:	eNRM

Source:	TSG RAN WG2

To:	TSG RAN WG3

Cc:	TSG SA WG5

Contact Person:

#### Name:	Malgorzata Tomala

E-mail Address:	malgorzata.tomala@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 thank SA5 for their LS on radio resource management policy.

Generally, the presented RRM policy concepts for the 2-split and 3-split scenario are difficult to be advised from RAN2 (specifications) viewpoint. While the modelling of information should not limit implementation options for performing resource control, it needs to be aligned with standardized configuration requirements/capabilities that defined in RAN3 specifications.

When it comes to the used by SA5 terminology, RAN2 would like to draw the attention to two terms, which use in TS28.541 may potentially lead to misinterpretations:

- “NRCellCU” – according to the NG-RAN architecture description in TS 38.300 valid for non-split deployment scenario, there is no gNB-CU node. Therefore, referencing to gNB-CU (NRCellCU) for a non-split deployment, which technically does not have it, may limit some implementation options

- “PDCP resources” - in terms of functional and protocol stacks in RAN the PDCP has special meaning. With bearer concept, PDCP terminating point may vary for different bearer types, even though within the same gNB architecture type.  “PDCP” is not the only protocol hosted at CU-UP. Therefore, RAN2 suggest considering use of generic term e.g. “users resources” or “bearer resources” instead of “PDCP resources”.

2. Actions:

To SA5 group.

ACTION: 	RAN2 respectfully asks SA5 to take the above into account.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#107-Bis			14 – 18 October 2019		Chongqing

3GPP TSG RAN WG3#106		18 – 22 November, 2019	Reno (NV), USA