3GPP TSG-SA5 Meeting #128 	S5-197552

# Zhuhai,China, 18-22 November 2019

# Source:	Huawei

# Title:	Update the requirements for ANR management in NG-RAN

# Document for:	Approval

Agenda Item:	6.4.13

1	Decision/action requested

The group is asked to discuss and approve the proposals.

2	References

[1]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks v0.0.0".

[2]             3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage 2".

3	Rationale

ANR function resides in gNB is described in TS 38.300 [2] which requires O&M to configure three attributes of the NCR, including no remove, no HO and no Xn. To support the ANR function management, the requirements of controlling no remove attribute of the neighbour cell relation should be included.

As defined in TS 38.300 [2], an NG-RAN node is either a gNB or an ng-eNB in NG-RAN architecture. The gNBs and ng-eNBs are interconnected with each other by means of the Xn interface. The establishment of an X2 connection to a neighbour en-gNB is not existed.

4	Detailed proposal

1st modified section

6.1.1.3	ANR management in NG-RAN

The business level requirements in section 5.1.1 are decomposed into the following specification level requirements, applicable for NG-RAN:

REQ-NR-ANR-FUN-01	MnS producer of provisioning shall support a capability allowing an authorized consumer to request that HO be allowed from source cell to target cell.

REQ-NR-ANR-FUN-02	MnS producer of provisioning shall support a capability allowing an authorized consumer to request that HO be prohibited from source cell to target cell.

REQ-NR-ANR-FUN-03	MnS producer of provisioning shall support a capability allowing an authorized consumer to request establishment of an Xn connection to the neighbour gNB, or an Xn connection to the neighbour ng-eNB.

REQ-NR-ANR-FUN-04	MnS producer of provisioning shall support a capability allowing an authorized consumer to request that an existing Xn connection to a neighbour gNB, or an Xn connection to a neighbour ng-eNB to be released, and that the establishment of such a connection is prohibited.

REQ-NR-ANR-FUN-05	MnS producer of provisioning shall support a capability allowing an authorized consumer to request that a NCR is allowed to be removed.

REQ-NR-ANR-FUN-06	MnS producer of provisioning shall support a capability allowing an authorized consumer to request that a NCR is not allowed to be removed.

REQ-NR-ANR-FUN-07	MnS producer of provisioning shall support a capability allowing an authorized consumer to disable or enable the ANR function in one or more gNBs.

End of modified section