3GPP TSG-SA5 Meeting #128 	S5-197626

# Zhuhai,China, 18-22 November 2019							  	   				    Revision of S5-197404

# Source:	China Mobile

# Title:	Add Automatic CSI creation use case

# Document for:	Approval

Agenda Item:	6.5.2

1	Decision/action requested

Please approve the proposed changes.

Add an Automatic CSI creation use case, in view of the Study on Self-Organizing Networks (SON) for 5G, TR 28.861

2	References

[1]		3GPP TR 28.861 V0.6.0 (2019-07) Study on the Self-Organizing Networks (SON) for 5G networks

[2]		3GPP TR 28.805 V16.0.0 (2019-09) Telecommunication management; Study on management aspects of communication services

3	Detailed proposal

Modified Section

# 5	Use cases

## 5.N	Automatic CSI creation

### 5.N.1	Goal

The goal is to enable operators to properly design and initiate a new communication service. Automatic CSI creation is able to digest the SLA into service profile so that corresponding network functionalities can use the service profile.

### 5.N.2	Pre-conditions

A set of E2E services is provided by the network to certain group (category) of the UEs.

The following functions and services are deployed and active:

- NG-RAN, 5GC, CSI(s), NSI(s),

### 5.N.3	Description

The service producer of automatic CSI creation receives the SLA requirements from an authorized consumer.

The service producer of automatic CSI creation analyses SLA requirements to determine the resource parameters configuration needed to support the requirements.

The service producer of automatic CSI creation needs to analysis between the input SLA with the capability of existing network resource during feasibility check, and provide the possible negotiate the output service profile.

### 5.N.4	Post-conditions

The CSI is created.

End of Modified Sections