3GPP TSG-SA5 Meeting #129e 	S5-201246

# e-meeting, 24 February – 4 March 2020

# Source:	Ericsson

# Title:	Addition of CSIF and CHF flows

# Document for:	Approval

Agenda Item:	7.4.3

# 1	Decision/action requested

Adding the following changes to TS 28.201

# 2	References

[1]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI).

# 3	Rationale

Addition of flows between CSIF and CHF based on TS 32.290 [1], for the interaction with NWDAF this interaction is still FFS.

# 4	Detailed proposal

First change

#### 5.2.2.2	Analytics and performance charging from CSIF

##### 5.2.2.2.1	General

The clause below describes analytics and performance charging from CSIF scenarios.

##### 5.2.2.2.2	General analytics and performance charging – PEC

The following figure 5.2.2.2.2 describes analytics and performance charging:

Figure 5.2.2.2.2: Analytics and performance reporting

1)	Collection of resource usage information: Collection of information related to the network slice analytics and performance is done by the CSIF.

2)	Trigger condition met: the CSIF determines based on the collected information that a report of charging information to the CHF is needed.

3)	Charging Data Request [Event]: The CSIF generates charging data related to the collected information and sends the charging data request for the CHF to process the store related charging data for CDR generation purpose.

4)	 Create CDR: the CHF stores received information and creates a CDR related to the event.

5)	Charging Data Response [Event]: The CHF informs the CSIF on the result of the request.

Second change

#### 5.2.2.3	Network data analytics subscription from NWDAF

##### 5.2.2.3.1	General

The clause below describes network data analytics subscription scenarios.

Editor’s note: The need for specific flows or if the flows in TS 23.288 can be referenced is FFS.

End of changes