| 3GPP TS 28.313 V0.3.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Self-Organizing Networks (SON) for 5G networks (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Concepts and background	9

4.1	SON concepts	9

4.1.1	Overview	9

4.1.2	Centralized SON	10

4.1.2.1	Introduction	10

4.1.2.2	Cross Domain-Centralized SON	11

4.1.2.3	Domain-Centralized SON	11

4.1.3	Distributed SON	11

4.1.4	Hybrid SON	12

4.2	  Self-establishment of new RAN NE in network	13

4.2.1	Introduction	13

4.2.2	Network configuration data handling	13

4.2.3	Plug and connect to management system	13

4.2.4	Self-configuration	13

5	Business level requirements	13

5.1	Requirements	13

5.1.1	Distributed SON management	13

5.1.2	Centralized SON	14

5.2	Actor roles	14

5.3	Telecommunication resources	14

5.4	High-level use cases	14

5.4.1	Distributed SON management	14

5.4.2	Centralized SON	14

6	Specification level requirements	14

6.1	Requirements	14

6.1.1	Distributed SON management	14

6.1.1.1	RACH Optimization (Random Access Optimisation)	14

6.1.1.2	MRO (Mobility Robustness Optimisation)	14

6.1.1.3	ANR management in NG-RAN	15

6.1.1.4	PCI configuration and re-configuration	15

6.1.2	Centralized SON	15

6.1.2,1	PCI configuration	15

6.1.2.2	Requirements for RAN NE plug and connect to management system	15

6.1.2.3	  Requirements for self-configuration of a new RAN NE	16

6.2	Actor roles	16

6.3	Telecommunication resources	16

6.4	Use cases	16

6.4.1	Distributed SON management	16

6.4.1.1	RACH Optimization (Random Access Optimisation)	16

6.4.1.2	MRO (Mobility Robustness Optimisation)	17

6.4.1.3	ANR management	17

6.4.1.3.1	Starting the ANR function	17

6.4.1.3.2	Stopping the ANR function	18

6.4.1.3.3	Sending notification of added or deleted NCR	18

6.4.1.3.4	Handover Whitelisting	19

6.4.1.3.5	Handover Blacklisting	19

6.4.1.3.6	Prohibiting X2 or Xn connection to a peer node (X2/Xn blacklisting)	20

6.4.1.3.7	Prohibiting handover over X2 or Xn (X2/Xn handover blacklisting)	20

6.4.1.4	PCI configuration	21

6.4.1.4.1	Initial PCI configuration	21

6.4.1.x.5	PCI re-configuration	21

6.4.2	Centralized SON	22

6.4.2.1	PCI configuration	22

6.4.2.1.1	Initial PCI configuration	22

6.4.2.1.2	PCI re-configuration	22

6.4.2.2	Use case for establishment of a new RAN NE in network	23

6.4.2.2.1	Use case for RAN NE plug and connect to management system	23

6.4.2.2.2	Use case for self-configuration of a new RAN NE	25

7	Management services for SON	26

7.1	Management services for D-SON management	26

7.1.1	RACH Optimization (Random Access Optimisation)	26

7.1.1.1	MnS component type A	26

7.1.2.1	MnS Component Type B definition	26

7.1.2.1.1	Targets information	26

7.1.2.1.2	Control information	27

7.1.2.1.3	Parameters to be updated	27

7.1.1.3	MnS Component Type C definition	27

7.1.1.3.1	Performance measurements	27

7.1.2	MRO (Mobility Robustness Optimisation)	27

7.1.2.1	MnS component type A	27

7.1.2.2	MnS Component Type B definition	28

7.1.2.2.1	Targets information	28

7.1.2.2.2	Control information	28

7.1.2.2.3	Parameters to be updated	28

7.1.2.3	MnS Component Type C definition	28

7.1.2.3.1	Performance measurements	28

7.1.3	PCI configuration	29

7.1.3.1	MnS component type A	29

7.1.3.2	MnS Component Type B definition	30

7.1.3.2.1	Control information	30

7.1.3.2.2	Parameters to be updated	30

7.1.3.3	MnS Component Type C definition	30

7.1.3.3.1	Alarm information	30

7.1.4	ANR management	30

7.2	Management services for C-SON	31

7.2.1	PCI configuration	31

7.2.1.1	MnS component type A	31

7.2.1.2	MnS Component Type B definition	31

7.2.1.2.1	Control information	31

7.2.1.3	MnS Component Type C definition	31

7.2.1.3.1	Alarm information	31

7.2.1.3.2	Performance measurements	32

8	SON procedures	32

8.1	Introduction	32

8.2	Distributed SON management	32

8.2.1	RACH Optimization (Random Access Optimisation)	32

8.2.2	MRO (Mobility Robustness Optimisation)	33

8.2.3	PCI configuration	34

8.2.3.1	Initial PCI configuration	34

8.2.3.2	PCI re-configuration	35

8.3	Centralized SON	36

8.3.1	PCI configuration	36

8.3.1.1	Initial PCI configuration	36

8.3.1.2	PCI re-configuration	37

8.3.2	Procedures for establishment of a new RAN NE in network	38

8.3.2.1	Procedures for RAN NE plug and connect to management system	38

8.3.2.2	Procedures for self-configuration management	39

Annex <A> (informative): PlantUML source code	40

A.1 Procedures for establishment of a new RAN NE in network	40

A.1.1	Procedure for plug and connect to management system	40

A.1.2	Procedure for self-configuration management	40

Annex <X> (informative): Change history	42

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.313:	Management and orchestration; Self-Organizing Networks (SON) for 5G networks.

TS 28.552:	Management and orchestration; 5G performance measurements.

TS 28.544:	Management and orchestration; Self-Organizing Networks (SON) Network Resource Model (NRM) for 5G networks.

# 1	Scope

The present document specifies the concepts, use cases, requirements, and procedures for the SON functions in 5GS.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication Management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS 28.532: "Management and orchestration; Generic management services”

[4]	3GPP TS 38.321 “NR; Medium Access Control (MAC) protocol specification”.

[5]	3GPP TS 28.552 “Management and orchestration; 5G performance measurements”

[6] 	3GPP TS 32.511 " Telecommunication management; Automatic Neighbour Relation (ANR) management; Concepts and requirements"

[7]	3GPP TS 38.300 "NR; Overall description; Stage-2"

[8]	3GPP TS 36.300 "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2"

[9]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[10]	3GPP TS 28.545 "Management and orchestration; Fault Supervision (FS)”

[11]	3GPP TS 28.531 "Management and orchestration; Provisioning"

[12]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[13]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Centralized SON (C-SON): SON solution where SON algorithms are executed in the 3GPP management system. Centralized SON has two variants:

- Cross Domain-Centralized SON: SON solution where SON algorithms are executed in the 3GPP Cross Domain layer.

- Domain-Centralized SON: SON solution where SON algorithms are executed in the Domain layer.

Distributed SON (D-SON): SON solution where SON algorithms are executed in the Network Function layer.

Hybrid SON: SON solution where SON algorithms are executed at two or more of the following layers: NF layer or Domain layer or 3GPP Cross Domain layer.

Searchlist: list of frequencies and supporting information to be used for neighbour cell measurements, which contains entries for NG-RAN and supported IRATs.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ANR	Automatic Neighbour Relation

NCR	Neighbour Cell Relation

NG-RAN	Next Generation Radio Access Network

# 4	Concepts and background

## 4.1	SON concepts

### 4.1.1	Overview

Based on the location of the SON algorithm, SON is categorized into four different solutions that are possible for implementing various SON use cases, the solution is selected depending on the needs of the SON use cases.

a)	Centralized SON:

1)	Cross Domain-Centralized SON

2)	Domain-Centralized SON

b)	Distributed SON

c)	Hybrid SON.

The SON algorithm is not standardized by 3GPP.

The following figure illustrates the overview of SON Framework.

Figure 4.1.1-1 Overview of SON Framework

### 4.1.2	Centralized SON

#### 4.1.2.1	Introduction

Centralized SON (C-SON) means that the SON algorithm executes in the 3GPP management system. The centralized SON concept has been defined for LTE in TS 32.500 [2].

Figure 4.1.2-1 shows a generic C-SON process, where the SON algorithms execute in the 3GPP management system interact with network functions in RAN and/or CN to implement SON functions. The SON algorithm may consist of the following functionalities:

-	Monitoring: monitor the network(s) by collecting management data, including the data provided by MDAS.

-	Analysis: analyse the management data to determine if there are issues in the network(s) that need to be resolved.

-	Decision: makes the decision on the SON actions to resolve the issues.

-	Execution: execute the SON actions.

-	Evaluation: evaluate whether the issues have been solved by analysing the management data

NOTE 1: The management data as well as the exact algorithm may vary for different SON cases. For example, for automated creation of NSI, NSSI and/or NFs related cases, the management data may be SLA requirements received from service management layer; for automated optimization related cases, the management data may be performance measurements of the networks; for automated healing related cases, the management data may be alarm information of the networks. The management data and the algorithm are to be specified case by case.

Figure 4.1.2-1 C-SON process

#### 4.1.2.2	Cross Domain-Centralized SON

The Cross Domain-Centralized SON means that the SON algorithm executes in the 3GPP Cross Domain layer.

For Cross Domain-Centralized SON, the MnF(s) in 3GPP Cross Domain layer monitors the networks via management data, analyses the management data, makes decisions on the SON actions, and executes the SON actions.

#### 4.1.2.3	Domain-Centralized SON

The Domain-Centralized SON means the SON algorithm executes in the Domain layer.

For Domain-Centralized SON, the MnF(s) in Domain layer monitors the networks via management data, analyses the management data, makes decisions on the SON actions, and executes the SON actions.

The MnF(s) in 3GPP Cross Domain is responsible for management and control of the Domain-Centralized SON function. The management and control may include switching on/off a Domain-Centralized SON function, making policies for a Domain-Centralized SON function, and/or evaluating the performance of a Domain-Centralized SON function.

### 4.1.3	Distributed SON

The distributed SON (D-SON) means that the SON algorithm is located in the NFs. The distributed SON concept has been defined for LTE in TS 32.500 [2].

For D-SON, the NFs monitors the network events, analyses the network data, makes decisions on the SON actions and executes the SON actions in the network nodes.

Figure 4.1.3-1 shows that the 3GPP management system (MnF in Domain or 3GPP Cross Domain) is responsible for the following functions:

a) D-SON management function:

1) Switch on/off a D-SON function,

2) Provide policies, targets, and supplementary information (e.g., the range attributes) for a D-SON function.

b) D-SON evaluation function: evaluate whether the issues have been resolved, and may apply D-SON management actions.

Editor’s Note: the reference to corresponding SON concept to RAN specification will be added later.

Figure 4.1.3-1 D-SON process

### 4.1.4	Hybrid SON

The hybrid SON (H-SON) means the SON algorithm are executed at two or more of the following levels: NF layer, Domain layer or 3GPP Cross Domain layer. The hybrid SON concept has been defined for LTE in TS 32.500 [2].

The 3GPP management system (i.e. MnF(s) in Domain or 3GPP Cross Domain) and NFs work together, in a coordinated manner, to build up a complete SON algorithm. The decisions on SON actions may be made by 3GPP management system and/or NFs, depending on the specific cases.

Figure 4.1.4-1 shows that the C-SON component is monitoring and executes SON actions.

Figure 4.1.4-1 H-SON process

## 4.2			Self-establishment of new RAN NE in network

### 4.2.1	Introduction

Self-establishment of new RAN NE in network describes the procedure of a new NG-RAN NE can automatically establish when it is powered up and connect to the IP network in multi-vendor scenario, which includes:

-	Network Configuration data handling

-	Plug and connect to management system

-	Self-Configuration

### 4.2.2	Network configuration data handling

Network configuration data handling makes the network configuration data available to the management system support self-configuration process, which may include network configuration data preparation, network configuration data transfer and network configuration data validation. This happens except all of the network configuration data for NE can be generated by the management system supporting self-configuration process.

Network configuration data preparation: This makes the network configuration data ready in operator’s network management system who provides the network configuration data. How to prepare the network configuration data in operator’s network management system is out of scope of this specification.

Network configuration data transfer: This transfers the Network configuration data from network configuration data Provider to the network configuration data Consumer.

Network configuration data validation: This validates the syntax and semantics of network configuration data. It takes place in the network configuration data Consumer.

### 4.2.3	Plug and connect to management system

Plug and connect to management system connects the deployed NE to its management system providing support for self-configuration process as automatically as possible. The following functionality is included: initial IP Autoconfiguration service, Certificate enrolment, establish secure connection, establish connect to management system providing support for self-configuration process.

### 4.2.4	Self-configuration

Self-configuration puts the NE into a state to be ready to carry traffic in an automated manner. Self-configuration includes following functionality: create self-configuration task, monitor self-configuration process, generate configuration data if needed, download and activate software, download and active configuration data, perform self-test and update network resource model, etc.

# 5	Business level requirements

## 5.1	Requirements

### 5.1.1	Distributed SON management

REQ-NR-ANR-CON-01	For NG-RAN ANR management, the initial status of a newly created NCR by the ANR function to a NG-RAN cell shall be such that HO is allowed, X2 or Xn connection setup is allowed, and the NCR is allowed to be removed by the ANR function in the gNB.

REQ-NR-ANR-CON-02	NG-RAN ANR management shall support NCRs from NG-RAN to NG-RAN and from NG-RAN to E-UTRAN.

REQ-NR-ANR-CON-03	For E-UTRAN ANR management, the initial status of a newly created NCR by ANR function to a NG-RAN cell shall be such that HO is allowed, X2 connection setup is allowed, and the NCR is allowed to be removed by the ANR function in the eNB.

REQ-NR-ANR-CON-04	E-UTRAN ANR management shall support NCRs from E-UTRAN to NG-RAN, in addition to support to other technologies, described in TS 32.511 [6] clause 5.1.

### 5.1.2	Centralized SON

## 5.2	Actor roles

## 5.3	Telecommunication resources

## 5.4	High-level use cases

### 5.4.1	Distributed SON management

### 5.4.2	Centralized SON

# 6	Specification level requirements

## 6.1	Requirements

### 6.1.1	Distributed SON management

#### 6.1.1.1	RACH Optimization (Random Access Optimisation)

REQ-RACH-FUN-1 MnS producer of D-SON management should have a capability allowing the authorized consumer to set and update the targets for RACH optimization function.

REQ-RACH-FUN-2 MnS producer of D-SON management should have a capability allowing an authorized consumer to enable or disable the RACH optimization function.

REQ-RACH-FUN-3 MnS producer of D-SON management should have a capability allowing the authorized consumer to collect performance measurements that are used to evaluate the RACH performance.

#### 6.1.1.2	MRO (Mobility Robustness Optimisation)

REQ-MRO-FUN-1 The MnS producer of D-SON management should have a capability allowing the MnS consumer to set the targets for MRO function.

REQ-MRO-FUN-2 The MnS producer of D-SON management should have a capability allowing the MnS consumer to collect the handover related performance measurements that are used to evaluate the MRO performance.

REQ-MRO-FUN-3 MnS producer of D-SON management should have a capability allowing the MnS consumer to enable or disable the MRO function.

#### 6.1.1.3	ANR management in NG-RAN

The business level requirements in section 5.1.1 are decomposed into the following specification level requirements, applicable for NG-RAN:

REQ-NR-ANR-FUN-01	Producer of provisioning MnS shall support a capability allowing an authorized consumer to request establishment of an Xn connection to the neighbour gNB, or an Xn connection to the neighbour ng-eNB.

REQ-NR-ANR-FUN-02	Producer of provisioning MnS shall support a capability allowing an authorized consumer to request that an existing Xn connection to a neighbour gNB, or an Xn connection to a neighbour ng-eNB to be released, and that the establishment of such a connection is prohibited.

REQ-NR-ANR-FUN-03	Producer of provisioning MnS shall support a capability allowing an authorized consumer to request that a NCR is allowed to be removed.

REQ-NR-ANR-FUN-04	Producer of provisioning MnS shall support a capability allowing an authorized consumer to request that a NCR is not allowed to be removed.

REQ-NR-ANR-FUN-05	Producer of provisioning MnS shall support a capability allowing an authorized consumer to disable or enable the ANR function in one or more gNBs.

#### 6.1.1.4	PCI configuration and re-configuration

REQ-DPCI-CONFIG-FUN-1 producer of provisioning MnS should have a capability allowing an authorized consumer to set or update the list(s) of PCI value(s) for NR cell(s).

REQ-DPCI-CONFIG-FUN-2 producer of provisioning MnS should have a capability allowing an authorized consumer to enable or disable the PCI configuration function.

REQ-DPCI-CONFIG-FUN-3 producer of provisioning MnS should have a capability to notify the authorized consumer with the PCI value(s) being selected for NR cell(s).

REQ-DPCI-CONFIG-FUN-4 producer of fault supervision MnS should have a capability to notify the authorized consumer about the detection or resolution of PCI collision or PCI confusion problems for NR cells.

REQ-DPCI-CONFIG-FUN-5 producer of provisioning MnS should have a capability allowing an authorized consumer to configure or re-configure the PCI list at the PCI configuration function.

### 6.1.2	Centralized SON

#### 6.1.2,1	PCI configuration

REQ- CPCI-CONFIG-FUN-1 producer of provisioning MnS should have a capability allowing an authorized consumer to configure or re-configure the PCI value(s) for NR cell(s).

REQ- CPCI-CONFIG-FUN-2 producer of provisioning MnS should have a capability to notify the authorized consumer with the PCI value(s) being assigned to NR cell(s).

REQ-CPCI-CONFIG-FUN-3 producer of fault supervision MnS should have a capability to notify the authorized consumer about the detection or resolution of PCI collision or PCI confusion problems for NR cells.

#### 6.1.2.2	Requirements for RAN NE plug and connect to management system

REQ-PnC-CON-1 NE shall be able to get its own IP addresses and MnF IP address without manual configuration during plug and play for a NE connection to the network.

#### 6.1.2.3			Requirements for self-configuration of a new RAN NE

REQ-SCM-CON-1 The MnS for self-configuration management shall have the capability allowing MnS consumer request MnS producer to create, query and delete Self-configuration management profile.

REQ-SCM-CON-2 The MnS for Self-configuration management shall have the capability allowing MnS consumer obtain the progress of self-configuration process form MnS producer.

## 6.2	Actor roles

## 6.3	Telecommunication resources

## 6.4	Use cases

### 6.4.1	Distributed SON management

#### 6.4.1.1	RACH Optimization (Random Access Optimisation)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically configure the RACH parameters in a cell in order to achieve the optimal network performance by reducing the network access time, and minimize the failures. |  |
| Actors and Roles | An authorized consumer of the MnS producer of D-SON management. |  |
| Telecom resources | - gNB; - The MnS producer of D-SON management |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - MnS producer of D-SON management is in operation. |  |
| Begins when | The MnS producer of D-SON management has been deployed. |  |
| Step 1 (M) | The MnS producer of D-SON management sets the targets for the RACH optimization function. |  |
| Step 2 (M) | The MnS producer of D-SON management enables the RACH optimization function. |  |
| Step 3 (M) | The MnS producer of D-SON management collects the RACH related measurements, and analyze them to evaluate the RACH performance. |  |
| Step 4 (O) | If the MnS consumer of D-SON determines that the RACH performance does not meet the target, it updates the targets for RACH optimization function; |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The RACH performance has been optimized. |  |
| Traceability | REQ-RACH-FUN-1, REQ-RACH-FUN-2, REQ-RACH-FUN-3, REQ-RACH-FUN-4 |  |

#### 6.4.1.2	MRO (Mobility Robustness Optimisation)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically configure the handover parameters in cells in order to improve the handover performance. |  |
| Actors and Roles | An authorized consumer of the MnS producer of D-SON management. |  |
| Telecom resources | - gNB; - The MnS producer of D-SON management |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - MnS producer of D-SON management is in operation. |  |
| Begins when | The MnS producer of D-SON management has been deployed. |  |
| Step 1 (M) | The MnS producer of D-SON management sets the targets for the MRO function. |  |
| Step 2 (M) | The MnS producer of D-SON management enables the MRO function. |  |
| Step 3 (M) | The MRO function detects handover issues (e.g. too late HO, too early HO and HO to a wrong cell) in intra-RAT or inter-RAT mobility by analyzing reports from UEs and network side information, and acts to mitigate the HO issues by adjusting HO related parameters. |  |
| Step 4 (M) | The MnS producer of D-SON management collects MRO related measurements, and analyzes them to evaluate the MRO performance. |  |
| Step 5 (M) | The MnS producer of D-SON management performs the following action, if the MRO performance does not meet the target: 1. Update the targets for MRO function. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The MRO performance has been optimized. |  |
| Traceability | REQ-MRO-FUN-1, REQ- MR-FUN-2, REQ-MRO-FUN-3 |  |

#### 6.4.1.3	ANR management

##### 6.4.1.3.1	Starting the ANR function

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | The goal is to make the ANR function in the gNB is enabled. |  |
| Actors and Roles | A MnS consumer of the MnS of D-SON management |  |
| Telecom resources | The MnS producer of D-SON management gNB |  |
| Assumptions |  |  |
| Pre conditions | The ANR function is not active. The gNB may have NCRs. The NCRs may be configured by a MnS consumer or may have been added by the ANR function if the ANR function has been active previously. |  |
| Begins when | The Use Case begins when the MnS consumer decides to enable the ANR function in a gNB. |  |
| Step 1 (M) | The MnS consumer enables the ANR function in the gNB. |  |
| Ends when | Ends when all steps identified above are completed or when an exception occurs |  |
| Exceptions | One of the steps identified above fails and retry is unsuccessful. |  |
| Post Conditions | The ANR function in gNB is successfully enabled by the MnS consumer, or if unsuccessful, still disabled. |  |
| Traceability | REQ-NR-ANR-FUN-0h |  |

##### 6.4.1.3.2	Stopping the ANR function

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | The goal is to make the ANR function in the gNB is disabled. |  |
| Actors and Roles | A MnS consumer of the MnS of D-SON management |  |
| Telecom resources | The MnS producer of D-SON management gNB |  |
| Assumptions |  |  |
| Pre conditions | The ANR function is active |  |
| Begins when | The Use Case begins when the MnS consumer decides to disable the ANR function in a gNB. |  |
| Step 1 (M) | The MnS consumer disables the ANR function in the gNB. |  |
| Ends when | Ends when all steps identified above are completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails and retry is unsuccessful. |  |
| Post Conditions | The ANR function in gNB is successfully disabled by the MnS consumer, or if unsuccessful, still enabled. All existing NCRs, whether created by ANR or otherwise are unaltered. |  |
| Traceability | REQ-NR-ANR-FUN-0h |  |

##### 6.4.1.3.3	Sending notification of added or deleted NCR

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | The goal is for the MnS producer to send a notification of added or deleted NCR to the MnS consumer. |  |
| Actors and Roles | A MnS consumer of the MnS of D-SON management. |  |
| Telecom resources | The MnS producer of D-SON management. gNB |  |
| Assumptions |  |  |
| Pre conditions | The ANR function is active |  |
| Begins when | An NCR is added or deleted. This could be the result of either the ANR function’s action, or the creation of the deletion of an NCR by a MnS consumer. |  |
| Step 1 (M) | The MnS producer sends a notification to the MnS consumer. |  |
| Ends when | Ends when all steps identified above are completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails and retry is unsuccessful. |  |
| Post Conditions | The MnS consumer is aware of the creation or deletion of the NCR. |  |
| Traceability | REQ-NR-ANR-FUN-0m |  |

##### 6.4.1.3.4	Handover Whitelisting

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | The goal is to make an NCR present in the NCRT, useful for handovers. |  |
| Actors and Roles | A MnS consumer of the MnS of D-SON management |  |
| Telecom resources | The MnS producer of D-SON management gNB |  |
| Assumptions |  |  |
| Pre conditions | The ANR function is active. |  |
| Begins when | The Use Case begins when the MnS consumer decides to whitelist an NCR. |  |
| Step 1 (O) | The MnS consumer creates the NCR This step is executed if it the wanted NCR not already present in the NCRT. |  |
| Step 2 (M) | The MnS consumer marks the NCR so that handovers are allowed, and so that the ANR function is not allowed to remove the NCR. |  |
| Ends when | Ends when all steps identified above are completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails and retry is unsuccessful. |  |
| Post Conditions | The wanted NCR is present in the NCRT. It is protected from being removed by the ANR function. |  |
| Traceability | REQ-NR-ANR-FUN-0c, REQ-NR-ANR-FUN-0i |  |

##### 6.4.1.3.5	Handover Blacklisting

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | The goal is to make an NCR is present in the NCRT and made unavailable for handovers. |  |
| Actors and Roles | A MnS consumer of the MnS of D-SON management |  |
| Telecom resources | The MnS producer of D-SON management gNB |  |
| Assumptions |  |  |
| Pre conditions | The ANR function is active. |  |
| Begins when | The Use Case begins when the MnS consumer decides to blacklist an NCR. |  |
| Step 1 (O) | The MnS consumer creates the NCR. This step is executed if it the wanted NCR not already present in the NCRT. |  |
| Step 2 (M) | The MnS consumer marks the NCR so that handovers are prohibited, and so that the ANR function is not allowed to remove the NCR. |  |
| Ends when | Ends when all steps identified above are completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails and retry is unsuccessful. |  |
| Post Conditions | The wanted NCR is present in the NCRT. It is protected from being removed by the ANR function. |  |
| Traceability | REQ-NR-ANR-FUN-0d, REQ-NR-ANR-FUN-0i |  |

##### 6.4.1.3.6	Prohibiting X2 or Xn connection to a peer node (X2/Xn blacklisting)

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | The goal is to prohibit a gNB from setting up an X2 or Xn connection to a peer gNB or eNB. If such a connection existed, it is brought down. |  |
| Actors and Roles | A MnS consumer of the MnS of D-SON management |  |
| Telecom resources | The MnS producer of D-SON management gNB |  |
| Assumptions |  |  |
| Pre conditions | The ANR function is active. |  |
| Begins when | The Use Case begins when the MnS consumer decides to prohibit the setting up of X2 or Xn connections to a peer node. |  |
| Step 1 (M) | The MnS consumer configures the MnS producer with the peer node into the list of nodes for which X2 or Xn connections are prohibited. |  |
| Ends when | Ends when all steps identified above are completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails and retry is unsuccessful. |  |
| Post Conditions | The peer node is in the black-list. If an X2 or Xn connection was present to the peer node, it is brought down. |  |
| Traceability | REQ-NR-ANR-FUN-0g |  |

##### 6.4.1.3.7	Prohibiting handover over X2 or Xn (X2/Xn handover blacklisting)

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | The goal is to prohibit a gNB from using an X2 or Xn connection to a peer gNB or eNB for handover. |  |
| Actors and Roles | A MnS consumer of the MnS of D-SON management. |  |
| Telecom resources | The MnS producer of D-SON management gNB |  |
| Assumptions |  |  |
| Pre conditions | The ANR function is active. |  |
| Begins when | The Use Case begins when the MnS consumer decides to prohibit using the X2 or Xn connection to a peer node for handover. |  |
| Step 1 (M) | The MnS consumer configures the MnS producer to mark the NCR to the peer node so that handovers over the X2 or Xn connection are prohibited. |  |
| Ends when | Ends when all steps identified above are completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails and retry is unsuccessful. |  |
| Post Conditions | The gNB is prohibited from using the using the X2 or Xn connection to the peer node for handovers. |  |
| Traceability | REQ-NR-ANR-FUN-0o |  |

#### 6.4.1.4	PCI configuration

##### 6.4.1.4.1	Initial PCI configuration

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically configure the PCIs for NR cells that have not been assigned with PCIs. |  |
| Actors and Roles | D-SON management function to support initial PCI configuration. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - No PCI values have been assigned to NR cells. |  |
| Begins when | The D-SON management function decided to configure the PCI list for NR cell(s). |  |
| Step 1 (M) | The D-SON management function requests the producer of provisioning MnS to configure the PCI list at the PCI configuration function. |  |
| Step 2 (M) | The D-SON management function requests the producer of provisioning MnS to enable the PCI configuration function at NR cell(s). |  |
| Step 3 (M) | The PCI configuration function selects PCI value(s) from the list of PCI values provided by the producer of provisioning MnS. |  |
| Step 4 (M) | The producer of provisioning MnS notifies the consumer with the PCI value(s) being assigned for the NR cell(s). |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The PCI value of a NR cell has been selected. |  |
| Traceability | REQ-DPCI-CONFIG-FUN-1, REQ-DPCI-CONFIG-FUN-2, REQ-DPCI-CONFIG-FUN-3, REQ-DPCI-CONFIG-FUN-5 |  |

##### 6.4.1.x.5	PCI re-configuration

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically re-configure the PCIs of NR, due to the PCI collision or PCI confusion problems. |  |
| Actors and Roles | D-SON management function to support PCI re-configuration. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS - The producer of fault supervision MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cell(s) have being assigned with PCI value(s). - The PCI configuration function is in operation, and enabled. |  |
| Begins when | PCI configuration function has detected the PCI problem of PCI collision or PCI confusion for NR cell(s). |  |
| Step 1 (M) | The D-SON management function receives an alarm from the producer of fault supervision MnS indicating the PCI collision or PCI confusion problems for an NR cell(s). |  |
| Step 2 (M) | The D-SON management function requests the producer of provisioning MnS to re-configure the PCI list at the PCI configuration function. |  |
| Step 3 (M) | The PCI configuration function selects PCI value(s) from the PCI list. |  |
| Step 4 (M) | The producer of provisioning MnS notifies the consumer with the PCI value(s) being assigned for the NR cell(s). |  |
| Step 5 (M) | The D-SON management function receives a clear alarm notification from the producer of fault supervision MnS. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The PCI collision or PCI confusion have been resolved. |  |
| Traceability | REQ-DPCI-CONFIG-FUN-3, REQ-DPCI-CONFIG-FUN-4, REQ-DPCI-CONFIG-FUN-5 |  |

### 6.4.2	Centralized SON

#### 6.4.2.1	PCI configuration

##### 6.4.2.1.1	Initial PCI configuration

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically configure the PCIs of NR cell(s) that have not been assigned with PCI value(s). |  |
| Actors and Roles | C-SON function to support PCI configuration. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells have not been assigned with PCI values yet. - The C-SON has been enabled. |  |
| Begins when | The C-SON function decides to configure PCI values for NR cell(s). |  |
| Step 1 (M) | The C-SON function determines the PCI value(s) for the NR cell(s) that have no collision or confusion with its neighbours. |  |
| Step 2 (M) | The C-SON function requests the producer of provisioning MnS to configure the PCI value(s) at the NR cell(s). |  |
| Step 3 (M) | The producer of provisioning MnS notifies the consumer with the PCI value(s) being assigned for the NR cell(s). |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The PCI value of a NR cell has been selected. |  |
| Traceability | REQ-CPCI-CONFIG-FUN-1, REQ-CPCI-CONFIG-FUN-2 |  |

##### 6.4.2.1.2	PCI re-configuration

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically re-configure the PCIs of NR cells, due to the PCI collision or PCI confusion problems. |  |
| Actors and Roles | C-SON to support PCI re-configuration. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS - The producer of fault supervision MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - The C-SON function has been in operation, and enabled. |  |
| Begins when | The C-SON function requests the producer of provisioning MnS to collect the PCI related measurements reported by NG-RAN. |  |
| Step 1 (M) | The C-SON function analyzes the PCI related information and detects that NR cells have experienced PCI conflict or confusion issues. |  |
| Step 2 (M) | The producer of fault supervision MnS notifies the C-SON function about the PCI collision or PCI confusion problems for NR cell(s). |  |
| Step 3 (M) | The C-SON function determines the new PCI value(s), and requests the producer of provisioning MnS to re-configure the PCI value for the NR cell(s) experienced PCI conflict or confusion issues. |  |
| Step 4 (M) | The producer of provisioning MnS notifies the C-SON function about the resolution of PCI collision or PCI confusion problems for NR cell(s). |  |
| Step 4 (M) | The producer of fault supervision MnS notifies the consumer-SON function about the resolution of PCI collision or PCI confusion problems for NR cell(s). |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The PCI value of a NR cell has been selected. |  |
| Traceability | REQ-CPCI-CONFIG-FUN-1, REQ-CPCI-CONFIG-FUN-2, REQ-CPCI-CONFIG-FUN-3 |  |

#### 6.4.2.2	Use case for establishment of a new RAN NE in network

##### 6.4.2.2.1	Use case for RAN NE plug and connect to management system

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | After NE installation, connect the NE to its MnF providing support for self-configuration process, and to the External Network(s) as automatically as possible. The NE described in this use case can be gNB in non-split scenario and gNB-DU in split scenario. Editor’s Note: Whether the NE described in this use case can be virtuaized or other type of RAN NE (e.g. GNB-CU) is FFS. |  |
| Actors and Roles | NE |  |
| Telecom resources | IP networks: Non-Secure Operator Network, External Network, and its elements like DHCP server optionally DNS, CA/RA servers, Security Gateway(s) (each protecting one or more Secure Operator Networks), Secure Operator Network(s) including MnF providing support for self-configuration process. |  |
| Assumptions | There is a functional power supply for the NE. There may be one or more IP Autoconfiguration Services like DHCP and Router Advertisements and zero or more DNS servers. |  |
| Pre conditions | The NE is installed. IP connectivity exists between the involved telecom resources. The involved telecom resources are functional. The relevant information is stored and available: - Vendor Certificate at the NE - Operator Certificate at the CA/RA - For the External Network or Non-Secure Operator Network: - (Outer) IP autoconfiguration information at the IP Autoconfiguration Service - FQDN of the initial OAM SeGW at the NE and/or FQDN or IP address of the initial OAM SeGW at the IP Autoconfiguration Service - FQDN of the CA/RA servers at the NE and/or FQDN or IP address of the CA/RA servers at the IP Autoconfiguration Service - If FQDNs need to be resolved, corresponding IP address(es) at the DNS server(s) - For the Secure Operator Network: - (Inner) IP autoconfiguration information at the IP Autoconfiguration Service or at the initial OAM SeGW - FQDN or IP address of the initial MnF at the NE and/or DHCP Server of the Secure Operator Network. - If FQDNs need to be resolved, corresponding IP address(es) at the DNS server(s) - Configuration and software for the NE at the MnF(s) |  |
| Begins when | The NE is installed. |  |
| Step 1 (M) | If a VLAN ID is available the NE uses it. Otherwise the NE uses the native VLAN where PnP traffic is sent and received untagged |  |
| Step 2 (M) | The NE acquires its IP address through stateful or stateless IP autoconfiguration. This may provide 0 or more DNS server addresses. |  |
| Step 3 (M) | The NE acquires the IP address of the CA/RA server. The FQDN of the CA/RA server may be pre-configured in the NE or the FQDN or IP address of the CA/RA server may be provided by the IP Autoconfiguration Service. FQDNs are resolved through the DNS if necessary. Information provided by the IP Autoconfiguration Services shall supersede those pre-configured at the NE. |  |
| Step 4 (M) | The NE performs Certificate Enrolment. |  |
| Step 5 (M) | The NE acquires the IP address of the OAM SeGW. The FQDN of the OAM SeGW may be pre-configured in the NE or the FQDN or the IP address of the OAM SeGW may be provided by the IP Autoconfiguration Service. FQDNs are resolved through the DNS if necessary. |  |
| Step 6 (M) | The NE establishes a secure connection (tunnel) to the Security Gateway given by Step 5. The NE receives its (inner) IP autoconfiguration information (which may be the same as the outer IP address obtained in step2) and optionally the address of one or more DNS servers within the Secure Operator Network from the Configuration Parameters of IKEv2 during tunnel establishment. |  |
| Step 7 (M) | The NE acquires the IP address of the correct MnF by either, issuing a DHCP request including the NE’s vendor information, resolving FQDNs via DNS if necessary, or by having a pre-configured FQDN (including the NE’s vendor information) resolved via DNS. |  |
| Step 8 (M) | The NE establishes a connection to the provided MnF and acquires its configuration and software if any. The configuration may contain an address to another MnF that this specific node shall use as MnF. The configuration may contain an address to another SeGW that should be used before connecting to the MnF. The MnF may then - release the connection to the current MnF and OAM SeGW and then restart (returning to step 1), - release the connection to the current MnF and OAM SeGW and then return to step 6, - release the connection to the current MnF and then repeat step 8, or - continue with step 9. |  |
| Step 9 (M) | The NE establishes a connection to the External Network(s) using the transport (VLAN ID, IP addresses) and security parameters provided in step 8. |  |
| Ends when | Ends when all mandatory steps identified above are successfully completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post Conditions | One or more secure connections exist between the NE and the MnF and the External Network(s). Via the connection to the MnF the NE can receive further instructions to become operational and carry user traffic, e.g. the administrativeState is set to “unlocked”. | Clause 6.4.2.2.2 Use case for self-configuration of a new RAN NE |
| Traceability | All requirements of clause 6.1.2.2 |  |

##### 6.4.2.2.2	Use case for self-configuration of a new RAN NE

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal (*) | After installation, put in an automated manner the NE into a state to be ready to carry traffic. |  |
| Actors and Roles (*) | MnF providing support for self-configuration process act as MnS Producer for Self-configuration management |  |
| Telecom resources | NE MnS Consumer of Self-configuration management |  |
| Assumptions | IP network connectivity exists between the NE and the MnF(s) providing support for the self-configuration process. |  |
| Pre conditions | The NE is installed and connected to an IP network. | Clause 6.4.X.2 Use case for Plug and connect to management system |
| Begins when | The field personnel start the self-configuration process. It is also possible that the process is triggered automatically after the completion of an NE self-test or receiving the self-configuration management profile creation request from MnS Consumer for self-configuration management. |  |
| Step 1 (O) | MnF providing support for self-configuration process may notify MnS Consumer of self-configuration management about the start of the self configuration process. |  |
| Step 1 (*) (M¦O) | The order of the bullet points in the list below does not imply any statements on the order of execution. - An NE IP address is allocated to the new NE. - Basic information about the transport network (e. g. gateways) environment is provided to the NE. With this information the NE is able to exchange IP packets with other internet hosts. - The NE provides information about its type, hardware and other relevant data about itself to the MnF(s) providing support for the self-configuration process. - The address(es) of the MnF(s) providing support for the self-configuration process (e.g. MnF for software download, MnF for configuration data download) is provided to the NE. The address is equal to an IP address and a port number, or a DNS name and port number, or an URI. The address(es) of the MnF(s) providing support for normal OAM functions after completion of the self-configuration process are provided to the NE. The address is equal to an IP address and a port number, or a DNS name and port number, or an URI. - The NE connects to the MnF providing support for the software download. - The decision which software or software packages have to be downloaded to the NE is taken. - The software is downloaded into the NE. - The NE connects to the MnF providing support for the configuration data download. - The configuration data for the NE is made available by either preparing it or making prepared configuration data available. - The configuration data is downloaded into the NE. - Dependent External nodes are updated with new configuration data as well (if required). - The NE connects to the MnF providing support for normal OAM functions after completion of the self-configuration process. - The inventory system in the MnF is informed that a new NE is in the field. - The NE performs a self-test. Self-tests of different types can run at different places within the self-configuration procedure. - The operator is informed about the progess of the self-configuration process and important events occuring during the self-configuration process. - The network resource models are updated during and after the self-configuration process. - SW is installed, i.e. prepared in such a way, that the NE is ready to use it. NE is allowed to use the SW. |  |
| Step 3 (O) | MnF providing support for self-configuration process may notify MnS Consumer of Self-configuration management about the progress of the self configuration during self-configuration process. |  |
| Ends when (*) | Ends when all steps identified above are successfully completed or when an exception occurs. |  |
| Exceptions |  |  |
| Post Conditions | The NE is ready to carry traffic. |  |
| Traceability (*) | All requirements of clause 6.1.2.3 |  |

# 7	Management services for SON

Editor’s note: this clause contains the management services for SON and definition of information, such as targets, control, parameters, and PM measurements that are needed to support SON functions.

## 7.1	Management services for D-SON management

### 7.1.1	RACH Optimization (Random Access Optimisation)

#### 7.1.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.1.2.1	MnS Component Type B definition

##### 7.1.2.1.1	Targets information

The targets of RACH optimization are shown in Table 7.1.2.1.1-1.

Table 7.1.2.1.1-1.  RACH optimization targets

| Targets | Definition | Legal Values |
| --- | --- | --- |
| UE access delay probability | The probability distribution of UE access delay that is used to minimize the access delays for the UEs under the coverage of popular SSBs. | CDF of access delay |

Editor's Note:	The definition of targets to support the following targets needs clarification from RAN3.

-  Minimize the delays for the UEs to request the other SIs

-  Minimize the imbalance of UEs access delays on uplink (UL) and supplementary uplink (SUL) channel

-  Minimize the beam failure recovery delays for the UEs in RRC_Connected.

-  Minimize the failed/unnecessary RACH attempts on RACH resource before success.

##### 7.1.2.1.2	Control information

The parameter is used to control the RACH optimization function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| RACH optimization control | This attribute allows authorized consumer to enable/disable the RACH optimization functionality. See attribute rachOptimizationControl in TS 28.541 [13]. | Boolean On, off |

##### 7.1.2.1.3	Parameters to be updated

##### 7.1.1.3	MnS Component Type C definition

##### 7.1.1.3.1	Performance measurements

Performance measurements related to the RACH optimization are captured in Table 7.1.1.3.1-1:

Table 7.1.1.3.1-1.  RACH optimization related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Distribution of RACH preambles sent | Distribution of the number of preambles UEs sent to achieve synchronization, where the number of preambles sent corresponds to PREAMBLE_TRANSMISSION_COUNTER (see clause 5.1.1 in TS 38.321 [4]) in UE. | UE access delay probability |
| Distribution of UEs access delay | Distribution of the time needed for UEs to successfully attach to the network. | UE access delay probability |

### 7.1.2	MRO (Mobility Robustness Optimisation)

#### 7.1.2.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.1.2.2	MnS Component Type B definition

##### 7.1.2.2.1	Targets information

The targets of MRO are shown in the Table 7.1.2.2.1-1.

Table 7.1.2.2.1-1.  MRO targets

| Target Name | Definition | Legal Values |
| --- | --- | --- |
| Total handover failure rate | (the number of failure events related to handover) / (the total number of handover events) | [0..100] in unit percentage |
| Total intra-RAT handover failure rate | (the number of failure events related to intra-RAT handover) / (the total number of handover events) | [0..100] in unit percentage |
| Total inter-RAT handover failure rate | (the number of failure events related to inter-RAT handover) / (the total number of handover events) | [0..100] in unit percentage |

##### 7.1.2.2.2	Control information

The parameter is used to control the MRO function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| MRO function control | This attribute allows the operator to enable/disable the MRO functionality. See attribute mroControl in TS 28.541 [13]. | Boolean On, off |

##### 7.1.2.2.3	Parameters to be updated

##### 7.1.2.3	MnS Component Type C definition

##### 7.1.2.3.1	Performance measurements

Performance measurements related MRO are captured in Table 7.1.2.3.1.-1:

Table 7.1.2.3.1-1.  MRO related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Number of handover events | Includes all successful and unsuccessful handover events (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of handover failures | Includes unsuccessful handover events with failure causes (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of intra-RAT handover events | Includes all successful and unsuccessful intra-RAT handover eventsl | Total intra-RAT handover failure rate |
| Number of intra-RAT handover failures | Includes unsuccessful intra-RAT handover events with failure causes. | Total intra-RAT handover failure rate |
| Number of inter-RAT handover events | Includes all successful and unsuccessful inter-RAT handover events. | Total inter-RAT handover failure rate |
| Number of inter-RAT handover failures | Includes unsuccessful inter-RAT handover events with failure causes. | Total inter-RAT handover failure rate |
| Number of inter-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed for a long period of time in the cell. | Imbalance of UE access delay on UL and SUL probability |
| Number of intra-RAT too late handover failures | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure. | Total intra-RAT handover failure rate |
| Number of intra-RAT handover failures to wrong cell | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure. | Total intra-RAT handover failure rate |
| Number of inter-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed in an E-UTRAN cell which connects with 5GC for a long period of time. | Total intra-RAT handover failure rate |
| Number of inter-RAT too late handover failures | Deteccted when an RLF occurs shortly after a successful handover from an E-UTRAN cell which connects with EPC to a target cell in a E-UTRAN cell which connects with 5GC. | Total inter-RAT handover failure rate |
| Number of unnecessary handover to another RAT | Detected when a UE is handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient for the service used by the UE. | Total inter-RAT handover failure rate |
| Number of inter-RAT handover ping pong | Detected when an UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE. | Total inter-RAT handover failure rate |

### 7.1.3	PCI configuration

#### 7.1.3.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.2 of TS 28.532 [3]: - notifyNewAlarm - notifyClearedAlarm | It is supported by Fault Supervision MnS for NF, as defined in TS 28.545 [10]. |

#### 7.1.3.2	MnS Component Type B definition

##### 7.1.3.2.1	Control information

The parameter is used to control the D-SON PCI configuration function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| PCI configuration control | This attribute allows authorized consumer to enable/disable the D-SON PCI configuration functionality. | enable, disable |

Editor’s Note: need to consider ways (e.g. add a new attribute or modify an existing attribute in 28.541) to support PCI configuration control.

##### 7.1.3.2.2	Parameters to be updated

The table below lists the parameter related to the D-SON PCI configuration function.

| Parameters | Definition | Legal Values |
| --- | --- | --- |
| PCI list | The list of PCI values to be used by D-SON PCI configuration function to assign the PCI for NR cells. | List of integers |

Editor’s Note: need to consider ways (e.g. add a new attribute or modify an existing attribute in 28.541) to support PCI list.

#### 7.1.3.3	MnS Component Type C definition

##### 7.1.3.3.1	Alarm information

The table below lists the alarms related to D-SON PCI configuration,

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI collision alarm | The collision alarm is used to indicate two neighbouring cells of a serving cell are using the same PCIs. |  |
| PCI Confusion alarm | The confusion alarm is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. |  |

Editor’s Note: need to consider ways (e.g. add new alarms or modify existing alarms in TS 28.545 or TS 32.111-2) to support PCI collision and confusion.

### 7.1.4	ANR management

Stage 2 for ANR management is located in TS 28.541 [13], clauses 4.3.2.2, 4.3.2.3, 4.3.32.2 and 4.3.32.3.

Stage 3 for ANR management is located in TS 28.541 [13], clauses C.4.3, D.4.3, and E.5.

## 7.2	Management services for C-SON

### 7.2.1	PCI configuration

#### 7.2.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 11.1.1 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.2.1 of TS 28.532 [3]: - notifyNewAlarm - notifyClearedAlarm | It is supported by Fault Supervision MnS for NF, as defined in TS 28.545 [10]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.2.1.2	MnS Component Type B definition

##### 7.2.1.2.1	Control information

The parameter is used to control the C-SON PCI configuration function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| PCI configuration control | This attribute allows authorized consumer to enable/disable the C-SON PCI configuration functionality. | disable, enable |

Editor’s Note: need to consider ways (e.g. add a new attribute or modify an existing attribute in 28.541) to support PCI configuration control.

#### 7.2.1.3	MnS Component Type C definition

##### 7.2.1.3.1	Alarm information

The table below lists the alarms related to PCI configuration are generated from the NR cells,

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI collision alarm | The collision alarm is used to indicate two neighbouring cells of a serving cell are using the same PCIs. |  |
| PCI Confusion alarm | The confusion alarm is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. |  |

Editor’s Note: need to consider ways (e.g. add a new attribute or modify an existing attribute in 28.541) to support PCI list.

##### 7.2.1.3.2	Performance measurements

Performance measurements related to the PCI configuration are collected from the NR cells.

Table 7.2.1.3.2-1.  PCI related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI of candidate cells | The measurement contains cumulative counter with subcounters that is identified by the PCI value(s) of the candidate cells, and is derived from MeasResultListNR (see clause 6.3.2 in TS 38.331 [9]) where it contains PCI in PhysCellId, and RSRP/RSRQ in MeasQuantityResults of candidate cells. It is generated when the RSRP received from the candidate cells exceeds certain thresholds. |  |

Editor’s Note: Performance measurement needs to be defined in the TS 28.552 [5].

# 8	SON procedures

## 8.1	Introduction

The procedures listed in clause 8 are some of all the possibilities, and are not exhaustive.

## 8.2	Distributed SON management

### 8.2.1	RACH Optimization (Random Access Optimisation)

Figure 8.2.1-1 depicts a procedure that describes how D-SON management function can manage the RACH optimization (D-SON) function. It is assumed that the D-SON management function has consumed the performance assurance management service to collect RACH optimisation related measurements.

Figure 8.2.1-1: RACH Optimization procedure

1. The D-SON management function consumes the provisioning MnS with modifyMOIAttributes operation (see clause 5.1.3 in TS 28.532 [3]) to configure the targets for RACH optimization function.

1.a The provisioning MnS sets the targets for RACH optimization (D-SON) function (NOTE).

2. The D-SON management function consumes the provisioning MnS with modifyMOIAttributes operation to enable the RACH optimization function for a given NR cell.

3.a The provisioning MnS enables the RACH optimization (D-SON) function (NOTE).

3. The RACH optimization (D-SON) function receives the RACH information report from UE(s), and analyses them to determine the actions to optimize the RACH performance if the performance does not meet the targets by updating the RACH parameters.

4. The D-SON management function collects the RACH related performance measurements.

5. The D-SON management function analyses the measurements to evaluate the RACH performance,

6.  The D-SON management function consumes the provisioning MnS with modifyMOIAttributes operation to update the targets of the RACH optimization function, when the RACH optimization performance does not meet the targets:

6.a The provisioning MnS updates the targets for RACH optimization function (NOTE).

NOTE: The interface between provisioning MnS and RACH optimization is not subject to standardization.

### 8.2.2	MRO (Mobility Robustness Optimisation)

Figure 8.2.2-1 depicts a procedure that describes how D-SON management function can manage the MRO function. It is assumed that the D-SON management function has consumed the performance assurance MnS to create PM jobs to collect handover related measurements.

Figure 8.2.2-1: MRO procedure

1. The D-SON management function consumes the provisioning MnS with modifyMOIAttributes operation (see clause 5.1.3 in TS 28.532 [3]) to configure targets for the MRO function.

1.a The provisioning MnS sets the targets for MRO function (NOTE).

2. The D-SON management function consumes the NF provisioning MnS with modifyMOIAttributes operation to enable the MRO function for a given NR cell.

3.a The provisioning MnS enables the MRO function (NOTE).

3. The MRO function receives MRO information reports from UE(s), and analyses them to determine the actions to optimize the MRO performance.  fI the performance does not meet the targets, it updates the handover parameters.

4. The D-SON management function collects MRO related performance measurements.

5. The D-SON management function analyses the measurements to evaluate the MRO performance,

6.  The D-SON management function consumes the provisioning MnS with modifyMOIAttributes operation to update the targets of the MRO function if the MRO performance does not meet the targets.

6.a The provisioning MnS updates the targets for MRO function (NOTE).

NOTE: The interface between provisioning MnS and MRO function is not subject to standardization.

### 8.2.3	PCI configuration

#### 8.2.3.1	Initial PCI configuration

Figure 8.2.3.1-1 depicts a procedure that describes how D-SON management function can manage the PCI configuration (D-SON) function to assign the PCI values to NR cells the first time.

Figure 8.2.3.1-1: Initial PCI configuration procedure

1. The D-SON management function consumes the MnS of NF provisioning with modifyMOIAttributes operation to configure the PCI list for NR cell(s).

1.a The MnS of provisioning sets the PCI list at the PCI configuration (D-SON) function (NOTE)

2. The D-SON management function consumes the MnS of NF provisioning with modifyMOIAttributes operation to enable the PCI configuration function for NR cell(s).

2.a The MnS of provisioning enables the PCI configuration (D-SON) function (NOTE).

3. The PCI configuration (D-SON) function selects PCI value(s) from the PCI list.

4. The PCI configuration (D-SON) function reports the PCI value(s) being assigned to the MnS of NF provisioning.

5. The MnS of NF provisioning sends a notification notifyMOIAttributeValueChange to D-SON management function to indicate the PCI value(s) being assigned to NR cell(s).

NOTE: The interface between MnS of NF provisioning and PCI configuration (D-SON) function is not subject to standardization.

#### 8.2.3.2	PCI re-configuration

Figure 8.2.3.2-1 depicts a procedure that describes how D-SONmanagement function can re-configure the PCI list for NR cell(s) when PCI collision or PCI confusion issues were detected.

Figure 8.2.3.2-1: PCI re-configuration procedure

1. The PCI configuration (D-SON) function detects and reports the PCI collision or PCI confusion problems for NR cell(s) to MnS of fault supervision (NOTE).

2. The producer of fault supervision MnS sends a notification notifyNewAlarm to D-SON management function to report the PCI collision or PCI confusion problems detected on NR cell(s).

3. The D-SON management function consumes the MnS of NF provisioning with modifyMOIAttributes operation to re-configure the PCI list for NR cell(s).

3.a The MnS of NF provisioning re-configures the PCI list for NR cell(s) (NOTE).

4. The PCI configuration (D-SON) function selects PCI value(s) from the updated PCI list.

5. The PCI configuration (D-SON) function reports the PCI value(s) being assigned to the MnS of NF provisioning.

6. The MnS of NF provisioning sends a notification notifyMOIAttributeValueChange to D-SON management function to indicate the PCI value(s) being assigned to NR cell(s).

7. The PCI configuration (D-SON) function notifies MnS of fault supervision that the PCI collision or PCI confusion problems have been respoved (NOTE).

8. The producer of fault supervision MnS sends a notification notifyClearedAlarm to D-SON management function to report the PCI collision or PCI confusion problems being resolved.

NOTE: The interface between MnS of NF provisioning and PCI configuration (D-SON) function is not subject to standardization.

## 8.3	Centralized SON

### 8.3.1	PCI configuration

#### 8.3.1.1	Initial PCI configuration

Figure 8.3.1.1-1 depicts a procedure that describes how C-SON can assign the PCI values to NR cells the first time.

Figure 8.3.1.1-1: Initial PCI configuration procedure

1. The C-SON determines the PCI value(s) for NR cell(s).

2. The C-SON consumes the MnS of NF provisioning with modifyMOIAttributes operation to configure the PCI value(s) for NR cell(s).

2.a The MnS of provisioning sets the PCI value(s) for NR cell(s) (NOTE)

3. The producer of provisioning MnS sends a notification notifyMOIAttributeValueChange to C-SON function to indicate the PCI value(s) being assigned to NR cell(s).

NOTE: The interface between MnS of provisioning and PCI configuration (D-SON) function is not subject to standardization.

#### 8.3.1.2	PCI re-configuration

Figure 8.3.1.2-1 depicts a procedure that describes how C-SON function can re-configure the PCI list for NR cell(s) when PCI collision or PCI confusion issues were detected. It is assumed that the C-SON function has consumed the MnS of performance assurance to create PM jobs to collect PCI related measurements.

Figure 8.3.1.2-1: PCI re-configuration procedure

1. The C-SON function collects PCI related performance measurements that are derived from MeasResultListNR (see clause 6.3.2 in TS 38.331 [9]) from producer of performance assurance MnS.

2. The C-SON function analyzes the NRM data and PCI related measurements to detect the PCI collision or PCI confusion problems for NR cell(s).

3. The C-SON function determines the new PCI value(s) for NR cell(s).

4. The C-SON function consumes the MnS of NF provisioning with modifyMOIAttributes operation to re-configure the PCI list for NR cell(s).

4.a The MnS of NF provisioning set the PCI value(s) for NR cell(s).

5. The producer of provisioning MnS sends a notification notifyMOIAttributeValueChange to C-SON function to indicate the PCI value(s) being assigned to NR cell(s).

### 8.3.2	Procedures for establishment of a new RAN NE in network

#### 8.3.2.1	Procedures for RAN NE plug and connect to management system

The Figure 8.3.2.1-1 illustrates the procedure for plug and connect to management system. The NE described in this procedure can be gNB in non-split scenario and gNB-DU in split scenario.

Editor’s Note 1: Whether the NE described in this procedure can be virtualized or other type of RAN NE (e.g. gNB-CU) is FFS.

Figure 8.3.2.1-1 Procedures for plug and connect to management system

1, If a VLAN ID is available, the NE uses it. Otherwise the NE uses the native VLAN where PnP traffic is sent and received untagged.

2. NE invokes the “Initial IP Autoconfiguration” procedure and acquires its IP address through stateful or stateless IP Autoconfiguration. There may be additional information provided to the NE.

Editor’s Note: the detailed “Initial IP Autoconfiguration” procedure to be added later.

3. NE invokes the "Certificate Enrolment" procedure.

Editor’s Note: the detailed “Certificate Enrolment” procedure to be added later.

4. NE invokes the "Establishing Secure Connection" procedure and connects to the OAM SeGW.

Editor’s Note: the detailed “Establishing Secure Connection” procedure to be added later.

5. NE invokes the "Establishing Connection to MnF" procedure.

Editor’s Note: the detailed “Establishing Secure Connection” procedure to be added later.

#### 8.3.2.2	Procedures for self-configuration management

The Figure 8.3.2.2-1 illustrates the procedure for start self-configuration management

Figure 8.3.2.2-1 Procedures for plug and connect to management system

1. MnS consumer of self-configuration management sends createScManagementProfile request for NE(s) of a certain type to MnS producer of self-configuration management. NE information, stop point information and step information may be included in the request.

2. MnS producer of self-configuration management creates ScManagementProfile instance for NE(s) specified in the received request.

3. MnS producer of self-configuration management sends the create ScManagementProfile response to MnS producer of self-configuration management.

4. For each NE (specified in the created ScManagementProfile) starting its self-configuration process, MnS producer of self-configuration management sends NotifyScProcessCreation notification to MnS consumer of self-configuration management.

5. When arrival at a stop point (e.g. stop point waiting for the network configuration data) or step described in corresponding ScManagementProfile, MnS producer of self-configuration management sends NotifyProcessStage notification to MnS consumer of self-configuration management.

6. If arrival at a stop point in step 5), MnS consumer of self-configuration management sends ResumeScProcess request to MnS producer of self-configuration management. If the self-configuration process is suspended at a stop point and is waiting for the network configuration data, the request include network configuration data or information indicating location of network configuration data.

7. When the self-configuration process is terminated, the MnS producer of self-configuration management sends NotifyScProcessDeletion notification to MnS consumer of self-configuration management.

Annex <A> (informative): PlantUML source code

## A.1 Procedures for establishment of a new RAN NE in network

### A.1.1	Procedure for plug and connect to management system

The following PlantUML source code is used to describe the procedure for multi-vendor plug and connect to management system, as depicted by Figure 8.2.X.2-1:

@startuml

title " Plug and connect to management system"

actor NE

participant "IP autoconfiugration server" as IP_Server

participant "public DNS server" as P_DNS_Server

participant “CA/RA” as CA_RA

participant SeGW

participant "secure DNS server" as S_DNS_Server

participant "secure DHCP server" as S_DHCP_Server

participant MnF

alt VLAN ID is available

NE->NE: 1a.use available VLAN Id

Else

NE->NE: 1b.use native VLAN Id

End

Ref over NE, IP_Server: 2. Initial IP Autoconfiguration

Ref over NE, IP_Server,CA_RA: 3. Certificate Enrolment

Ref over NE, IP_Server,CA_RA,SeGW: 4. Establishing Secure Connection

Ref over NE, IP_Server,CA_RA,SeGW,MnF: 5 Establishing Connection to MnF

skinparam sequenceActorBackgroundColor #FFFFFF

skinparam sequenceParticipantBackgroundColor #FFFFFF

skinparam noteBackgroundColor #FFFFFF

autonumber "#'.'"

skinparam monochrome true

skinparam shadowing false

@enduml

### A.1.2	Procedure for self-configuration management

The following PlantUML source code is used to describe the procedure for self-configuration management, as depicted by Figure 8.2.X.3-1:

@startuml

title " Procedures for self-configuration management "

actor “MnS Consumer of \n self-configuration management” as SC

participant “MnS Producer of \n self-configuration management” as SP

SC -> SP: 1. createScManagementProfile request

SP -> SP: 2. Create ScManagementProfile

SP -> SC: 3. createScManagementProfile response

loop  [Corresponding NE start its self-configuration process]

opt

SP -> SC: 4. NotifyScProcessCreation

end

|||

loop stop point or step is arrived

SP -> SC: 5. NotifyScProcessStage

opt if the stop point is arrived

SC -> SP: 6. ResumeScProcess

end

end

|||

SP->SC: 7.NotifyScProcessDeletion

end

skinparam sequenceActorBackgroundColor #FFFFFF

skinparam sequenceParticipantBackgroundColor #FFFFFF

skinparam noteBackgroundColor #FFFFFF

autonumber "#'.'"

skinparam monochrome true

skinparam shadowing false

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 |  |  |  |  |  | TS 28.313 Skeleton – before SA5#127 | 0.0.0 |

| 2019-10 | SA5#127 | S5-196672 S5-196674 S5-196675 S5-196696 S5-196697 S5-196698 S5-196857 S5-196747 S5-196858 S5-196802 S5-196860 S5-196861 S5-196862 S5-196863 S5-196864 S5-196865 S5-196865 S5-196803 |  |  |  | Update according to the meeting agreement in SA5#127: S5-196672 pCR 28.313 skeleton S5-196674 pCR 28.313 introduction and scope S5-196675 Add SON Concept S5-196696 pCR 28.313 RACH optimization use case S5-196697 pCR 28.313 RACH optimization procedure S5-196698 pCR 28.313 RACH optimization information S5-196857 pCR 28.313 MRO use case S5-196747 pCR 28.313 MRO procedure S5-196858 pCR 28.313 MRO information S5-196802 pCr 28.313 ANR Business level requirements S5-196860 pCR 28.313 Add ANR Use Case: Start S5-196861 pCR 28.313 Add ANR Use Case: Stop S5-196862 pCR 28.313 Add ANR Use Case: Notification S5-196863 pCR 28.313 Add ANR Use Case: Handover whitelisting S5-196864 pCR 28.313 Add ANR Use Case: Handover blacklisting S5-196865 pCR 28.313 Add ANR Use Case: Prohibit X2/Xn S5-196865 pCR 28.313 Add ANR Use Case: Prohibit X2/Xn handover S5-196803 pCR 28.313 ANR Specification level requirements for NG-RAN | 0.1.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2019-11 | SA5#128 | S5-197790 S5-197791 S5-197792 S5-197793 S5-197835 S5-197836 S5-197551 S5-197586 S5-197818 S5-197819 S5-197796 |  |  |  | Update according to the meeting agreement in SA5#128: S5-197790 pCR 28.313 D-SON PCI configuration use case S5-197791 pCR 28.313 C-SON PCI configuration use case S5-197792 pCR 28.313 D-SON PCI configuration procedure S5-197793 pCR 28.313 C-SON PCI configuration procedure S5-197835 pCR 28.313 D-SON PCI configuration information S5-197836 pCR 28.313 C-SON PCI configuration information S5-197551 Add usecase and requirements for establishment of new NE in network S5-197586 Add procedures for establishment of a new NE in network S5-197818 Update the requirements for ANR management in NG-RAN S5-197819 Add solutions for ANR management S5-197796 pCR 28.313 Add Concept for Establishment of new NE in network | 0.2.0 |
| 2020-03 | SA5#129e | S5-201583 S5-201584 S5-201376 |  |  |  | Update according to the meeting agreement in SA5#128: S5-201583 pCR 28.313 changes to MRO S5-201584 pCR 28.313 changes to RACH optimization S5-201376 pCR 28.313 Replace duplicated ANR management Stage 2 with reference | 0.3.0 |