3GPP TSG-SA5 Meeting #130e	S5-202089

20-28 April 2020, E-meeting

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0050 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add service components for configurable performance measurement control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-ADPM5G |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The NRMs for configurable performance measurement control has been defined in TS 28.622, however the service components for configurable performance measurement control are missing. Without the service components, it is unclear that how the management service for configurable performance measurement control can be consumed. The service components for performance data streaming were implemented incorrectly from the agreed CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added service components for configurable performance measurement control; Corrected the service components for performance data streaming. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The management service for configurable performance measurement control may be not implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3, 7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 7.3	Performance data streaming services

The components of performance data streaming services for NFs, NSSIs, NSIs and networks/sub-networks are listed in table 7.3-1.

Table 7.3-1: Components of performance data streaming services

| Management service | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Performance data streaming service for NFs | establishStreamingConnection; terminateStreamingConnection; reportStreamData; getStreamInfo; addStreamInfo; updateStreamInfo; deleteStreamInfo. | IOCs for 5G NFs, as defined in TS 28.541 [3] | Performance measurements for 5G NFs, as defined in TS 28.552 [2]. |
| Performance data streaming service for NSSIs | establishStreamingConnection; terminateStreamingConnection; reportStreamData; getStreamInfo; addStreamInfo; updateStreamInfo; deleteStreamInfo. | IOC(s) for NSSI, as defined in TS 28.541 [3]. | Performance measurements for NSSI, as defined in TS 28.552 [2]. |
| Performance data streaming service for NSIs | establishStreamingConnection; terminateStreamingConnection; reportStreamData; getStreamInfo; addStreamInfo; updateStreamInfo; deleteStreamInfo. | IOC(s) for NSI, as defined in TS 28.541 [3]. | Performance measurements for NSI, as defined in TS 28.552 [2]. |
| Performance data streaming service for sub-networks | establishStreamingConnection; terminateStreamingConnection; reportStreamData; getStreamInfo; addStreamInfo; updateStreamInfo; deleteStreamInfo. | IOC(s) for sub-network, as defined in TS 28.622 [5]. | Performance measurements for sub-network, as defined in TS 28.552 [2]. |

Next Modified Sections

## 7.x	Management service for configurable performance measurement control

The components of MnS for configurable performance measurement control, i.e.., the NRM fragment-based performance measurement control service, are listed in table 7.1-1.

Table 7.1-1: Components of MnS for configurable performance measurement control

| Management service | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Configurable performance measurement control for NFs | createMOI operation (see TS 28.532 [7]) | MeasurementControl IOC, MeasurementReader IOC, and Measurement dataType contained by or associated to ManagedElement IOC or ManagedFunction IOC, see TS 28.622 [5]. | Performance measurements for 5G NFs, as defined in TS 28.552 [2]. |
|  | getMOIAttributes operation (see TS 28.532 [7]) |  |  |
|  | modifyMOIAttributes operation (see TS 28.532 [7]) |  |  |
|  | deleteMOI operation (see TS 28.532 [7]) |  |  |
| Configurable performance measurement control for NSSIs | createMOI operation (see TS 28.532 [7]) | MeasurementControl IOC, MeasurementReader IOC, and Measurement dataType contained by or associated to NetworkSliceSubnet IOC, see TS 28.622 [5]. and TS 28.541 [3]. | Performance measurements for NSSI, as defined in TS 28.552 [2]. |
|  | getMOIAttributes operation (see TS 28.532 [7]) |  |  |
|  | modifyMOIAttributes operation (see TS 28.532 [7]) |  |  |
|  | deleteMOI operation (see TS 28.532 [7]) |  |  |
| Configurable performance measurement control for NSIs | createMOI operation (see TS 28.532 [7]) | MeasurementControl IOC, MeasurementReader IOC, and Measurement dataType contained by or associated to NetworkSlice IOC, see TS 28.622 [5] and TS 28.541 [3]. | Performance measurements for NSI, as defined in TS 28.552 [2]. |
|  | getMOIAttributes operation (see TS 28.532 [7]) |  |  |
|  | modifyMOIAttributes operation (see TS 28.532 [7]) |  |  |
|  | deleteMOI operation (see TS 28.532 [7]) |  |  |
| Configurable performance measurement control for Sub-networks | createMOI operation (see TS 28.532 [7]) | MeasurementControl IOC, MeasurementReader IOC, and Measurement dataType contained by or associated to SubNetwork IOC, see TS 28.622 [5] and TS 28.541 [3]. | Performance measurements for sub-network, as defined in TS 28.552 [2]. |
|  | getMOIAttributes operation (see TS 28.532 [7]) |  |  |
|  | modifyMOIAttributes operation (see TS 28.532 [7]) |  |  |
|  | deleteMOI operation (see TS 28.532 [7]) |  |  |

End of Modified Sections