3GPP TSG-SA5 Meeting #130e	S5-202279

# e-meeting, 20 – 28 April 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.530 | CR | 0025 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleanup Network Slice related definitions in OAM space |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Intel, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | network slice and network slice instance concept and usage in SA5 were not correctly reflected in their definitions in clause 3.1 of this specification. In addition, network slice instance and corresponding acronym “NSI” in SA5 have different meaning than the same term used by SA2, that is causing confusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct network slice definition in SA5 to reflect its concept in SA5. In SA5, network slice is object offered to network slice customer by network slice provider. It’s service view of logical network which exposed specific network capabilities and network characteristics. It is delivered to customer to support certain communication service, thus serving certain business purpose - Replace network slice instance with NetworkSlice instance which represents Managed Object Instance of NetworkSlice IOC. "network slice instance” used in existing SA5 specifications representing SA5::network slice will be replaced with network slice, "network slice instance” used in existing SA5 specifications representing Managed Object Instance (MOI) of SA5::NetworkSlice IOC will be replaced with NetworkSlice instance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect definition of network slice and network slice instance caused. caused conceptual issues inside and outside 3GPP, and let existing specification not implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modification

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

network slice: a logical network that provides specific network capabilities and network characteristics,  supporting various service properties for network slice customers.

Note: NetworkSlice Information Object Class (IOC) is used to model network slice.

Note: Represent Network Slice defined in TS 23.501 with added service properties.

NetworkSlice instance: A Managed Object Instance of NetworkSlice IOC

network slice subnet: a representation of a set of Network Functions and the required resources (e.g. compute, storage and networking resources) supporting network slice.

Note: NetworkSliceSubnet IOC is used to model network slice subnet which may include core network only, access network only or end to end network slice. Network Slice instance defined in 3gpp TS 23.501 can be represented by network slice subnet.

NetworkSliceSubnet instance: A Managed Object Instance of NetworkSliceSubnet IOC

Note: NetworkSlice instance represents service view of a root NetworkSliceSubnet instance  with associated Service Level Specification.

Service Level Specification: a set of service level requirements associated with a Service Level Agreement to be satisfied by a network slice instance

Note: 	Network Slice Subnet Information Object Class describes the structure (i.e. contained components and connectivity between them) and configuration of a network slice subnet, as well as network capability.

End of 1st modification