3GPP TSG-SA5 Meeting #131e 	S5-203204

# e-meeting 25 May- 04June 2020

# Source:	Samsung, Huawei, Ericsson

# Title:	Generic SLS Assurance Procedure

# Document for:	Approval

Agenda Item:	6.4.7

# 1	Decision/action requested

The group is asked to discuss and approve this document.

# 2	References

[1]		3GPP TS 28.535: "Management and orchestration; Management Services for Communication Service Assurance, requirements"

[x]		3GPP TS 28.550: "Management and orchestration; Performance assurance"

# 3	Rationale

Use cases related with SLA assurance have been captured in TS 28.535[1], also the NRM related with assurance control loop are agreed in last meeting.

The contribution provides a generic SLS assurance procedure.

# 4	Detailed proposal

It is proposed to add the following text to the draft TS.

1st Change Start

# 4 	Communication service assurance service

## 4.1	Stage 2

Editor’s Note: the substructure is for FFS.

4.1.X		Procedures

4.1.X.1	SLS Assurance Procedure

- AssuranceControlLoop_consumer derives AssuranceControlLoopGoal from the ServiceProfile or SliceProfile.

- AssuranceControlLoop_consumer provides the AssuranceControlLoopGoal to Entities_Participating_in_loop by utilizing the provision management services defined in as defined in clause 11.1.1.3, TS 28.532[7].

Note: In case the Entities_Participating_in_loop represents CrossDomain_Entities_Participating_in_loop, the AssuranceControlLoopGoal is the attribute(s) of the ServiceProfile. In case the Entities_Participating_in_loop represents Domain_Entities_Participating_in_loop, the AssuranceControlLoopGoal is the attribute(s) of the SliceProfile.

- Entities_Participating_in_loop subscribes the related performance data (e.g., the packet delay related measurements), fault data, QoE data (e.g., buffer level) and MDT data from respective sources by utilizing the Operation establishStreamingConnection as defined in clause 6.2.1, TS 28.550[x].

- Entities_Participating_in_loop, optionally, subscribes the related analytical data from MDAS or network functions, e.g., NWDAF. In case of NWDAF as a provider, Nnwdaf_EventsSubscription Service as defined in clause 4.2 is used.

- Entities_Participating_in_loop collects the related performance, fault, QoE and MDT data (e.g., the packet delay related measurements), fault data, QoE data (e.g., buffer level) and MDT data from respective sources by utilizing the Operation establishStreamingConnection as defined in clause 6.2.1, TS 28.550[x].

- Entities_Participating_in_loop, optionally, collects the related analytical data from MDAS or network functions, e.g., NWDAF. In case of NWDAF as a provider, Nnwdaf_EventsSubscription Service as defined in clause 4.2, TS 29.520[y] is used.

- Entities_Participating_in_loop assesses if the AssuranceControlLoopGoal has been fulfilled.

- Entities_Participating_in_loop assesses if and which action to take in case the AssuranceControlLoopGoal has not been fulfilled.

- As per the mitigation action (e.g., scale out) resources are changed, the generic provisioning management service as defined in clause 11.1, TS 28.532[x] is utilized for the same.

- Action completed

Note: The Entities_Participating_in_ loop continues to monitor and analyse the performance and perform the adjustment until the attribute(s) of SliceProfile is assured.

- AssuranceControlLoop_consumer receives the confirmation of assurance fulfilment from Entities_Participating_in_loop by utilizing the provision management services defined in as defined in clause 11.1.1.3, TS 28.532[7].

1st Change end