3GPP TSG-SA5 Meeting #132e 	S5-204275rev1

# e-meeting 17th 28th August 2020

# Source:	Huawei

# Title:	Add concept of NPN

# Document for:	Approval

Agenda Item:	6.5.4

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	TS 28.557 Management of non-public networks; Stage 1 and stage 2 v0.0.0

[2]	TR 28.807 Study on management aspects of non-public networks v16.0.0

3	Rationale

It is proposed to add concept in draft TS 28.557 [1] based on the corresponding content in TR 28.807 [2].

4	Detailed proposal

This document proposes the following changes in TS 28.557 [1].

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[U]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[X]	3GPP TS 28.530: "Concepts, use cases and requirements".

Next Change

4	Concepts and overview

4.1	Overview

A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 23.501 [U]. In contrast to public networks that offer mobile network services to the general public, non-public networks are intended for the sole use of a private entity such as a college or an enterprise. Non-public networks may be deployed on the entity's defined premises such as a campus or a factory to provide coverage within a specific geographic area.

An NPN may be deployed as:

-	a Stand-alone Non-Public Network (SNPN), i.e. operated by an NPN operator and not relying on network functions provided by a PLMN; or

-	a Public network integrated NPN (PNI-NPN), i.e. a non-public network deployed with the support of a PLMN.

End of change