3GPP TSG-SA5 Meeting #132e 	S5-2044436rev1

# e-meeting 17th-28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 0015 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the URI structure definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1.URI structure for resources not representing managed object instances is defined as follows: {scheme}://{authority}/{root}/{MnSName}/{MnSVersion}/{MnSResourcePath} The following abbreviated URI structure is given for the sake of brevity, where the URI template {MnSRoot} is introduced that includes the scheme component, the two slash characters ("//") and the authority component. {MnSRoot}/{MnSName}/{MnSVersion}/{MnSResourcePath} Compared the full URI structure and the abbreviated URI, the {root} component is missing in the abbreviated URI. 2. The following description for {scheme}, only “http” and “https” are allowed, however, in TS 28.532, the allowed value for {protocal} (same as {scheme}) can be “http”, “https” and “wss”. {scheme} Scheme component "http" or "https" In addition to these changes the concept of MnsRoot is introduced in clause 4.4.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description for {MnSRoot} - Add allowed value “wss” for {scheme} component. - Add concept of MnsRoot to clause 4.4.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The abbreviated URI structure is not same as full URI structure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2, 4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 4.4.2	URI structure for resources representing managed object instances

URIs identifying resources representing managed object instances shall follow a structure given by

{scheme}://{URI-DN-prefix}/{root}/{MnSName}/{MnSVersion}/{URI-LDN}

with:

{scheme}		Scheme component "http" or "https"

{URI-DN-prefix}		Authority component (host identifier and optional TCP port), the host name is constructed from the DN prefix as defined in clause 4.2.3.

{root}		Part of the path component, allows specifying optional path segments for structuring the resource hierarchy on a HTTP server.

{MnSName}		Part of the path component, allows specifying an optional MnS name in a single path segment.

{MnSVersion}		Part of the path component, allows specifying an optional MnS version in a single path segment.

{URI-LDN}		Part of the path component, constructed from the LDN as defined in clause 4.2.3, one or more path segments.

As seen above, to construct the URI from a DN, it is necessary to map the "DNPrefixPlusRDNSeparator" as defined in clause 7.3 of [3], the “LocalDN” as defined in clause 7.3 of [3], and to add the additional optional path segments "/{root}/{MnSName}/{MnSVersion}".

To allow for a predictive mapping from a URI to the original DN it is necessary to specify "/{MnSName}/{MnSVersion}" in such a way that the beginning of the "LocalDN" can be unambigously identified.

Note it may be required when specifying a MnS to clearly identify the last RDN of "{URI-LDN}" and to use the following instead of "{URI-LDN}"

{URI-LDN-first-part}/{RDN}

or

{URI-LDN-first-part}/{className}={id}.

For the sake of brevity, "MnSRoot" is introduced that includes the "{scheme}" part, the two slash characters ("//"), the "{authority}" part, a single slash character ("/") and the "{root}" part. When using "{MnSRoot}" the abbreviated URI structure is given by

{MnSRoot}/{MnSName}/{MnSVersion}/{URI-LDN}

or

{MnSRoot}/{MnSName}/{MnSVersion}/{URI-LDN-first-part}/{className}={id}

It is recommended to use this abbreviated version of the URI structure when defining Management Services.

Next Change

### 4.4.3	URI structure for resources not representing managed object instances

URIs identifying other resources shall follow a structure given by

{scheme}://{authority}/{root}/{MnSName}/{MnSVersion}/{MnSResourcePath}

with:

{scheme}		Scheme component "http" or "https" or "wss"

{authority}	Authority component (host identifier and optional TCP port)

{root}		Part of the path component, allows specifying optional path segments for structuring the resource hierarchy on a HTTP server.

{MnSName}		Part of the path component, specifies the mandatory MnS name in a single path segment.

{MnSVersion}		Part of the path component, specifies the mandatory MnS version in a single path segment.

{MnSResourcePath}		Part of the path component, one or more path segments, specifies a resource of the MnS

For the sake of brevity, {MnSRoot} is introduced that includes the "{scheme}" part, the two slash characters ("//"), the "{authority}" part, a single slash character ("/") and the "{root}" part. When using "{MnSRoot}" the abbreviated URI structure is given by

{MnSRoot}/{MnSName}/{MnSVersion}/{MnSResourcePath}

It is recommended to use this abbreviated version of the URI structure when defining Management Services.

End of Change