3GPP TSG-SA5 Meeting #133e 	S5-205203rev1

# e-meeting 12th - 21st October 2020

# Source:	Huawei, Telefónica S.A.

# Title:	Add generic management aspects

# Document for:	Approval

Agenda Item:	6.4.1

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	TS 28.557 Management of non-public networks; Stage 1 and stage 2 v0.1.0

3	Rationale

It is proposed to add generic management aspects in draft TS 28.557 [1].

4	Detailed proposal

This document proposes the following changes in TS 28.557 [1].

1st Change

4.3	NPN management aspects

4.3.1	Specific management aspects

Comparing with traditional management of PLMN, management of NPN has the following specific aspects:

- Assurance for diversified SLA requirements: The diversified SLA requirements from different kinds of vertical industries need to be guaranteed, e.g. manufacturing industry and medical care need ultra-reliable low-latency wireless connectivity and indoor, outdoor or hybrid coverage NPN deployments.

- Support of different O&M models: an O&M model allows specifying who is responsible for managing what part of the network. The various NPN scenarios, with a number of vertical use cases and a plenty of deployment variants, in some cases may lead to the definition of different O&M models.

- Management capability exposure: this expresses the ability of an NPN-SP to expose some management capabilities, such as performance and KPIs monitoring and provisioning management capabilities, to the corresponding NPN-SC. The NPN-SP makes the selected NPN management capabilities available through well-defined APIs to allow the NPN-SC to consume these capabilities, as well as extending them with their own operation and maintenance systems, if needed. NPN-SC may provide their business objectives by intents and policies management to NPN-SP and no need to focus on detailed configuration parameters of NPNs.

4.3.2	UE related management aspects

4.3.3	NG-RAN related management aspects

4.3.4	5GC related management aspects

End of change