3GPP TSG-SA5 Meeting #135e	S5-211128

January 25 – Feburary 3, 2021, e-Meeting	s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add LBO use cases, requirements, and related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSON_5G |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LBO use cases, requirements, and related information are needed to complete the Rel. 17 eSON_5G WI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add LBO use cases, requirements, and related information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel. 17 eSON_5G WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.x(new), 6.1.1.x (new), 6.4.2.x (new), 6.1.2.x (new), 7.1.x(new), 7.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is input to the Rel-17 28.313 draft CR for eSON_5G |  |  |  |  |  |  |  |  |

First Modified Sections

## 6.4	Use cases

### 6.4.1	Distributed SON management

#### 6.4.1.x	LBO (Load Balancing Optimisation)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically distribute user traffic among neighboring cells to ensure the radio resources are efficiently used, while providing quality end-user experience and performance. |  |
| Actors and Roles | D-SON management function to support LBO function. |  |
| Telecom resources | - The producer of provisioning MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - D-LBO is in operation. |  |
| Begins when | The D-SON management decides to enable D-LBO function. |  |
| Step 1 (M) | The D-SON management function requests the producer of provisioning MnS to set the handover and/or reselection parameters ranges (see clause 15.5.1.4 in TS 38.300 [7]), and to enable the D-LBO function. |  |
| Step 3 (M) | The LBO function uses the reported load information over the X2, Xn, F1 and E1 interfaces from neignboring cells to perform load balancing by means of cell reselection/handover parameters re-configuration and handover actions (e.g. offloading UEs to neighboring cells) (see clause 15.5 in TS 38.300 [7]), and may send a notification to the D-LBO management function when the LBO action has been performed. |  |
| Step 4 (M) | The D-SON management function collects LBO related measurements. |  |
| Step 5 (M) | The D-SON management function analyses the measurements to evaluate the LBO performape, and may request the producer of provisioning MnS to update the ranges for handover parameters. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The LBO performance has been optimized. |  |
| Traceability | REQ-DLBO-FUN-1, REQ-DLBO-FUN-2, REQ-DLBO-FUN-3 |  |

Next Modified Sections

## 6.1	Requirements

### 6.1.1	Distributed SON management

#### 6.1.1.2	LBO (Load Balancing Optimisation)

REQ-DLBO-FUN-1 Provisioning MnS for D-LBO function should have a capability allowing an authorized consumer to set or update the HO offset ranges, and control parameters for LBO function.

REQ-DLBO-FUN-2 Performance assurance MnS for D-LBO function should have a capability allowing the authorized consumer to collect the LBO related performance measurements that are used to evaluate the LBO performance.

REQ-DLBO-FUN-3 Provisioning MnS for D-LBO function should have a capability to notify the authorized consumer about the LBO actions being performed.

Next Modified Sections

### 6.4.2	Centralized SON

#### 6.4.2.x	LBO (Load Balancing Optimisation)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically distribute user traffic among neighboring cells to ensure the radio resources are efficiently used, while providing quality end-user experience and performance. |  |
| Actors and Roles | C-SON function to support LBO. |  |
| Telecom resources | - The producer of provisioning MnS |  |
| Assumptions | Both Domain Centralized SON and Cross-Domain Centralized SON are supported. |  |
| Pre-conditions | - The C-SON has been enabled. |  |
| Begins when | The C-SON function decides to enable C-LBO function. |  |
| Step 1 (M) | The C-SON function collects LBO load measurements by consuming the MnS of performance assurance. |  |
| Step 2 (M) | The C-SON function analyses measurements to determine the actions to optimize the traffic load distributions among neighboring cells that include consuming the MnS of provisioning to update the ranges for handover parameters. |  |
| Step 3 (M) | The C-SON function collects LBO related measurements, and analyses them to evaluate the LBO performance, and may request the producer of provisioning MnS to update the ranges for handover parameters. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The LBO performance has been optimized. |  |
| Traceability | REQ-CLBO-FUN-1, REQ-CLBO-FUN-2 |  |

Next Modified Sections

### 6.1.2	Centralized SON

#### 6.1.2.x	LBO (Load Balancing Optimisation)

REQ-CLBO-FUN-1 Provisioning MnS for C-LBO function should have a capability allowing an authorized consumer to set or update the HO offset ranges for LBO function.

REQ-CLBO-FUN-2 Performance assurance MnS for C-LBO function should have a capability allowing the authorized consumer to collect the LBO load and LBO related performance measurements.

Next Modified Sections

# 7	Management services for SON

## 7.1	Management services for D-SON management

### 7.1.x	LBO (Load Balancing Optimisation)

#### 7.1.x.1	MnS component type A

Table 7.1.x.1-1: D-LBO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 5 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - notifyMOIAttributeValueChanges operation - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in TS 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - establishStreamingConnection operation - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in TS 28.550 [12]. |

#### 7.1.x.2	MnS Component Type B definition

##### 7.1.x.2.1	Control information

The parameter is used to control the LBO function.

Table 7.1.x.2.1-1: D-LBO control information

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| D-LBO function control | This attribute allows the operator to enable/disable the LBO functionality. | Boolean On, off |

##### 7.1.x.2.2	Parameters to be updated

#### 7.1.x.3	MnS Component Type C definition

##### 7.1.x.3.1	Performance measurements

Performance measurements related LBO are captured in Table 7.1.x.3.1.-1:

Table 7.1.x.3.1-1. D-LBO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| DL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink (see clause 5.1.1.2.1 in TS 28.552 [5]). |  |
| UL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink (see clause 5.1.1.2.2 in TS 28.552 [5]). |  |
| Distribution of DL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the downlink (see clause 5.1.1.2.3 in TS 28.552 [5]). |  |
| Distribution of UL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the uplink (see clause 5.1.1.2.4 in TS 28.552 [5]). |  |
| DL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in downlink for data traffic (see clause 5.1.1.2.5 in TS 28.552 [5]). |  |
| UL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in uplink for data traffic (see clause 5.1.1.2.7 in TS 28.552 [5]). |  |
| Mean number of RRC Connections | This measurement provides the mean number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.1 in TS 28.552 [5]). |  |
| Max number of RRC Connections | This measurement provides the maximum number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.2 in TS 28.552 [5]). |  |
| Mean number of stored inactive RRC Connections | This measurement provides the mean number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |
| Max number of stored inactive RRC Connections | This measurement provides the maximum number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |

Next Modified Sections

## 7.2	Management services for C-SON

### 7.2.x	LBO (Load Balancing Optimisation)

#### 7.2.x.1	MnS component type A

Table 7.2.x.1-1: C-LBO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation --- modifyMOIAttributes operation - - deleteMOI operation - - notifyMOIAttributeValueChanges operation - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - establishStreamingConnection operation - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.2.x.2	MnS Component Type B definition

##### 7.2.x.2.1	Control information

The parameter is used to control the LBO function.

Table 7.2.x.2.1-1: C-LBO control information

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| C-LBO function control | This attribute allows the operator to enable/disable the LBO functionality. | Boolean On, off |

##### 7.2.x.2.2	Parameters to be updated

#### 7.2.x.3	MnS Component Type C definition

##### 7.2.x.3.1	Performance measurements

Table 7.2.x.3.1-1. lists the performance measurements that are used to monitor the load of NR cells (see clause 15.5.1.2 in TS 38.300 [7]).

Table 7.2.x.3.1-1. C-LBO load performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| DL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink (see clause 5.1.1.2.1 in TS 28.552 [5]). |  |
| UL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink (see clause 5.1.1.2.2 in TS 28.552 [5]). |  |
| Distribution of DL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the downlink (see clause 5.1.1.2.3 in TS 28.552 [5]). |  |
| Distribution of UL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the uplink (see clause 5.1.1.2.4 in TS 28.552 [5]). |  |
| DL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in downlink for data traffic (see clause 5.1.1.2.5 in TS 28.552 [5]). |  |
| UL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in uplink for data traffic (see clause 5.1.1.2.7 in TS 28.552 [5]). |  |
| Mean number of RRC Connections | This measurement provides the mean number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.1 in TS 28.552 [5]). |  |
| Max number of RRC Connections | This measurement provides the maximum number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.2 in TS 28.552 [5]). |  |
| Mean number of stored inactive RRC Connections | This measurement provides the mean number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |
| Max number of stored inactive RRC Connections | This measurement provides the maximum number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |

Table 7.2.x.3.1-2 lists the performance measurements used to monitor the LBO performance:

Table 7.2.x.3.1-2. C-LBO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Attempted RRC connection establishments | Includes the number of RRC connection establishment attempts (see clause 5.1.1.15.1 in TS 28.552 [5]). |  |
| Successful RRC connection establishments | Includes the number of successful RRC establishments (see clause 5.1.1.15.2 in TS 28.552 [5]). |  |
| Number of RRC connection re-establishment attempts | Includes the number of RRC connection re-establishment attempts (see clauses 5.1.1.17.1 in TS 28.552 [5]). |  |
| Successful RRC connection re-establishment | Includes the number of successful RRC connection re-establishment (see clauses 5.1.1.17.2 and 5.1.1.17.3 in TS 28.552 [5]). |  |
| Number of RRC connection resuming attempts | Includes Number of RRC connection resuming attempts (see clause 5.1.1.18.1 in TS 28.552 [5]). |  |
| Successful RRC connection resuming | Includes the number of successful RRC connection resuming (see clause 5.1.1.18.2 in TS 28.552 [5]). |  |

End of Modified Sections