3GPP TSG-SA5 Meeting #135e	S5-211128

January 25 – Feburary 3, 2021, e-Meeting	s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add LBO use cases, requirements, and related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSON_5G |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LBO use cases, requirements, and related information are needed to complete the Rel. 17 eSON_5G WI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add LBO use cases, requirements, and related information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel. 17 eSON_5G WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.x(new), 6.1.1.x (new), 6.4.2.x (new), 6.1.2.x (new), 7.1.x(new), 7.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is input to the Rel-17 28.313 draft CR for eSON_5G |  |  |  |  |  |  |  |  |

First Modified Sections

## 6.4	Use cases

### 6.4.1	Distributed SON management

#### 6.4.1.x	LBO (Load Balancing Optimisation)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically distribute user traffic among neighboring cells to ensure the radio resources are efficiently used, while providing quality end-user experience and performance. |  |
| Actors and Roles | D-SON management function to support LBO function. |  |
| Telecom resources | - The producer of provisioning MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - D-LBO is in operation. |  |
| Begins when | The D-SON management function decides to enable D-LBO function. |  |
| Step 1 (M) | The D-SON management function requests the producer of provisioning MnS to set the handover and/or reselection parameters ranges (see clause 15.5.1.4 in TS 38.300 [7]), and to enable the D-LBO function. |  |
| Step 3 (M) | The D-LBO function perform load balancing as describe in clause 15.5 in TS 38.300 [7])” and may notify D-LBO management function when the LBO action has been performed. |  |
| Step 4 (M) | The D-SON management function collects LBO related measurements. |  |
| Step 5 (M) | The D-SON management function analyses the measurements to evaluate the LBO performape, and may request the producer of provisioning MnS to update the ranges for handover parameters. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The LBO performance has been optimized. |  |
| Traceability | REQ-DLBO-FUN-1, REQ-DLBO-FUN-2, REQ-DLBO-FUN-3 |  |

Next Modified Sections

## 6.1	Requirements

### 6.1.1	Distributed SON management

#### 6.1.1.2	LBO (Load Balancing Optimisation)

REQ-DLBO-FUN-1 Provisioning MnS for D-LBO function should have a capability allowing an authorized consumer to set or update the HO offset ranges, and control parameters for LBO function.

REQ-DLBO-FUN-2 Performance assurance MnS for D-LBO function should have a capability allowing the authorized consumer to collect the LBO related performance measurements that are used to evaluate the LBO performance.

REQ-DLBO-FUN-3 Provisioning MnS for D-LBO function should have a capability to notify the authorized consumer about the LBO actions being performed.

Next Modified Sections

### 6.4.2	Centralized SON

#### 6.4.2.x	LBO (Load Balancing Optimisation)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically distribute user traffic among neighboring cells to ensure the radio resources are efficiently used, while providing quality end-user experience and performance. |  |
| Actors and Roles | C-LBO function to support LBO. |  |
| Telecom resources | - The producer of provisioning MnS |  |
| Assumptions | Both Domain Centralized SON and Cross-Domain Centralized SON are supported. |  |
| Pre-conditions | - The C-LBO has been enabled. |  |
| Begins when | The C-LBO function decides to enable C-LBO function. |  |
| Step 1 (M) | The C-LBOfunction collects LBO load measurements by consuming the MnS of performance assurance. |  |
| Step 2 (M) | The C-LBOfunction analyses measurements to determine the actions to optimize the traffic load distributions among neighboring cells that include consuming the MnS of provisioning to update the ranges for handover parameters. |  |
| Step 3 (M) | The C-LBOfunction collects LBO related measurements, and analyses them to evaluate the LBO performance, and may request the producer of provisioning MnS to update the ranges for handover parameters. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The LBO performance has been optimized. |  |
| Traceability | REQ-CLBO-FUN-1, REQ-CLBO-FUN-2 |  |

Next Modified Sections

### 6.1.2	Centralized SON

#### 6.1.2.x	LBO (Load Balancing Optimisation)

REQ-CLBO-FUN-1 Provisioning MnS for C-LBO function should have a capability allowing an authorized consumer to set or update the HO offset ranges for LBO function.

REQ-CLBO-FUN-2 Performance assurance MnS for C-LBO function should have a capability allowing the authorized consumer to collect the LBO load and LBO related performance measurements.

Next Modified Sections

# 7	Management services for SON

## 7.1	Management services for D-SON management

### 7.1.x	LBO (Load Balancing Optimisation)

#### 7.1.x.1	MnS component type A

Table 7.1.x.1-1: D-LBO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 5 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - notifyMOIAttributeValueChanges - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in TS 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - establishStreamingConnection operation - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in TS 28.550 [12]. |

#### 7.1.x.2	MnS Component Type B definition

##### 7.1.x.2.1	Control information

The parameter is used to control the LBO function.

Table 7.1.x.2.1-1: D-LBO control information

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| D-LBO function control | This attribute allows the operator to enable/disable the LBO functionality. | Boolean On, off |

##### 7.1.x.2.2	Parameters to be updated

#### 7.1.x.3	MnS Component Type C definition

##### 7.1.x.3.1	Performance measurements

Performance measurements related LBO are captured in Table 7.1.x.3.1.-1:

Table 7.1.x.3.1-1. D-LBO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| DL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink (see clause 5.1.1.2.1 in TS 28.552 [5]). |  |
| UL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink (see clause 5.1.1.2.2 in TS 28.552 [5]). |  |
| Distribution of DL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the downlink (see clause 5.1.1.2.3 in TS 28.552 [5]). |  |
| Distribution of UL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the uplink (see clause 5.1.1.2.4 in TS 28.552 [5]). |  |
| DL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in downlink for data traffic (see clause 5.1.1.2.5 in TS 28.552 [5]). |  |
| UL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in uplink for data traffic (see clause 5.1.1.2.7 in TS 28.552 [5]). |  |
| Mean number of RRC Connections | This measurement provides the mean number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.1 in TS 28.552 [5]). |  |
| Max number of RRC Connections | This measurement provides the maximum number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.2 in TS 28.552 [5]). |  |
| Mean number of stored inactive RRC Connections | This measurement provides the mean number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |
| Max number of stored inactive RRC Connections | This measurement provides the maximum number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |

Next Modified Sections

## 7.2	Management services for C-SON

### 7.2.x	LBO (Load Balancing Optimisation)

#### 7.2.x.1	MnS component type A

Table 7.2.x.1-1: C-LBO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation --- modifyMOIAttributes operation - - deleteMOI operation - - notifyMOIAttributeValueChanges - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - establishStreamingConnection operation - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.2.x.2	MnS Component Type B definition

##### 7.2.x.2.1	Control information

The parameter is used to control the LBO function.

Table 7.2.x.2.1-1: C-LBO control information

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| C-LBO function control | This attribute allows the operator to enable/disable the LBO functionality. | Boolean On, off |

##### 7.2.x.2.2	Parameters to be updated

#### 7.2.x.3	MnS Component Type C definition

##### 7.2.x.3.1	Performance measurements

Table 7.2.x.3.1-1. lists the performance measurements that are used to monitor the load of NR cells (see clause 15.5.1.2 in TS 38.300 [7]).

Table 7.2.x.3.1-1. C-LBO load performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| DL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink (see clause 5.1.1.2.1 in TS 28.552 [5]). |  |
| UL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink (see clause 5.1.1.2.2 in TS 28.552 [5]). |  |
| Distribution of DL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the downlink (see clause 5.1.1.2.3 in TS 28.552 [5]). |  |
| Distribution of UL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the uplink (see clause 5.1.1.2.4 in TS 28.552 [5]). |  |
| DL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in downlink for data traffic (see clause 5.1.1.2.5 in TS 28.552 [5]). |  |
| UL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in uplink for data traffic (see clause 5.1.1.2.7 in TS 28.552 [5]). |  |
| Mean number of RRC Connections | This measurement provides the mean number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.1 in TS 28.552 [5]). |  |
| Max number of RRC Connections | This measurement provides the maximum number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.2 in TS 28.552 [5]). |  |
| Mean number of stored inactive RRC Connections | This measurement provides the mean number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |
| Max number of stored inactive RRC Connections | This measurement provides the maximum number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |

Table 7.2.x.3.1-2 lists the performance measurements used to monitor the LBO performance:

Table 7.2.x.3.1-2. C-LBO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Attempted RRC connection establishments | Includes the number of RRC connection establishment attempts (see clause 5.1.1.15.1 in TS 28.552 [5]). |  |
| Successful RRC connection establishments | Includes the number of successful RRC establishments (see clause 5.1.1.15.2 in TS 28.552 [5]). |  |
| Number of RRC connection re-establishment attempts | Includes the number of RRC connection re-establishment attempts (see clauses 5.1.1.17.1 in TS 28.552 [5]). |  |
| Successful RRC connection re-establishment | Includes the number of successful RRC connection re-establishment (see clauses 5.1.1.17.2 and 5.1.1.17.3 in TS 28.552 [5]). |  |
| Number of RRC connection resuming attempts | Includes Number of RRC connection resuming attempts (see clause 5.1.1.18.1 in TS 28.552 [5]). |  |
| Successful RRC connection resuming | Includes the number of successful RRC connection resuming (see clause 5.1.1.18.2 in TS 28.552 [5]). |  |

End of Modified Sections