3GPP TSG SA WG5 Meeting 135-e		S5-211261

# electronic meeting, online, 25 January - 3 February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0412 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of converged charging trigger tables using IMS node |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S5 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The clause 5 is about charging principles and “service based charging” is not a charging principle but converged charging is, in comparison with the other clauses in 5. Solving the editor’s note on wherever to have tables per IMS node |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changing the heading of clause 5.4 to converged charging scenarios and updating tables to only show IMS node and not individual node types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The editor’s note cannot be resolved and the clauses in 5 is misaligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4, 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.4	IMS converged charging scenarios

Second change

### 5.4.3	Applicable triggers for IMS charging

The IMS nodes for which SIP method a charging Data Request is sent shall be operator configurable. The tables below describe all possible Charging Data Requests that might be sent from IMS nodes.

The generation of a Charging Data Request message by the IMS nodes in response to a particular "triggering SIP method" shall be operator configurable (enable or disable).

Table 5.4.3.1: Default Trigger conditions in IMS Node

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| SIP INVITE | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial] |
| SIP NOTIFY | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Initial] IEC: Charging Data Request [Event] |
| SIP MESSAGE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REGISTER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP SUBSCRIBE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REFER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP PUBLISH | - | Immediate | Not Applicable | Not Applicable |  |
| Change of charging conditions |  |  |  |  | SCUR: Charging Data Request [Update] |
| SIP RE-INVITE or UPDATE (e.g. change in media components, terminating identity change) | - | Immediate | No | No |  |
| SIP 2xx acknowledging a SIP INVITE, RE-INVITE or UPDATE (e.g. change in media components) | - | Immediate | No | No |  |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body | - | Immediate | No | No |  |
| SIP 4xx, 5xx or 6xx response, indicating an unsuccessful SIP RE-INVITE or UPDATE | - | Immediate | No | No |  |
| Any other SIP message during a SIP session, that allows the SIP session to continue | - | Immediate | Yes | No |  |
| CHF limit |  |  |  |  |  |
| Expiry of time limit | - | Immediate | No | Yes |  |
| Expiry of limit of number of charging condition changes | - | Immediate | No | Yes |  |
| Quota management |  |  |  |  |  |
| Time threshold reached | - | Immediate | No | Yes |  |
| Time quota exhausted | - | Immediate | No | Yes |  |
| Unit quota exhausted | - | Immediate | No | Yes |  |
| Expiry of quota validity time | - | Immediate | No | Yes |  |
| Expiry of quota holding time | - | Immediate | No | Yes |  |
| Re-authorization request by CHF | - | Immediate | No | Yes |  |
| Other |  |  |  |  |  |
| Management intervention | - | Immediate | No | No |  |
| SIP BYE message | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] |
| SIP 2xx acknowledging a SIP BYE | - | Immediate | Not Applicable | Not Applicable |  |
| Aborting a SIP session set-up procedure, using an internal trigger or a SIP CANCEL | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 3xx final or redirection response | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 2xx acknowledging non-session related SIP messages | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| Deregistration | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 4xx, 5xx or 6xx final response, indicating an unsuccessful procedure | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] PEC: Charging Data Request [Event] |

For converged charging, the following details of chargeable events and corresponding actions in the IMS nodes are defined in Table 5.4.3.2:

Table 5.4.3.2: Chargeable events and their related actions in IMS nodes

| Chargeable event | Conditions | IMS node action |
| --- | --- | --- |
| SIP INVITE |  | SCUR: Charging Data Request [Initial] with a possible request quota for later use |
| SIP NOTIFY |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP MESSAGE |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP REGISTER |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP SUBSCRIBE |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP REFER |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP PUBLISH |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP RE-INVITE or UPDATE (e.g. change in media components, terminating identity change) |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP 2xx acknowledging a SIP INVITE, RE-INVITE or UPDATE (e.g. change in media components) |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP 4xx, 5xx or 6xx response, indicating an unsuccessful SIP RE-INVITE or UPDATE |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| Any other SIP message during a SIP session, that allows the SIP session to continue |  | Start new counts with time stamps for all active service data flows. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | SCUR: Charging Data Request [Update] with a possible request quota. |
| Expiry of time limit |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of limit of number of charging condition changes |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Time threshold reached |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Time quota exhausted |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Unit quota exhausted |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of quota validity time |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of quota holding time |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Re-authorization request by CHF |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Management intervention |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| SIP BYE message |  | SCUR:Charging Data Request [Termination] Close the counts with time stamps |
| SIP 2xx acknowledging a SIP BYE |  | SCUR:Charging Data Request [Termination] Close the counts with time stamps |
| SIP 2xx acknowledging non-session related SIP messages |  | ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| Aborting a SIP session set-up procedure, using an internal trigger or a SIP CANCEL |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| Deregistration |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| SIP 3xx final or redirection response |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| SIP 4xx, 5xx or 6xx final response, indicating an unsuccessful procedure |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps PEC: Charging Data Request [Event] |

Editor’note: The applicable triggers in the tables is ffs.

End of changes