3GPP TSG-SA5 Meeting #139-e	S5-215351

# Online, , 11th Oct 2021 - 20th Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0107 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.622 Add missing definitions of common data types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Common data types are specified in stage 3, but not at stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reflect the common data types defined at stage 3 into stage 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The common data types cannot be used at stage 2, In addition there would be an inconsistency between stage 2 and stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Note: This CR is submitted to SA5#139 for comments. Target is to approve a complete set of common definitions at SA5#140.

First modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[4]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and Definitions".

[5]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification"

[6]	3GPP TS 32.532: " Telecommunication management; Software Management Integration Reference Point (IRP); Information Service (IS) "

[7]	ITU-T Recommendation X.710 (1991): "Common Management Information Service Definition for CCITT Applications".

[8]	TS 32.107: "Telecommunication management; Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM)"

[9]	TS 28.620: "Telecommunication management; Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM) Umbrella Information Model (UIM)"

[10]	TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model Repertoire"

[11]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[12]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM Information Service (IS)".

[13]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[14]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[15]	ETSI GS NFV 003 V1.1.1: "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[16] 	ETSI GS NFV-IFA 008 v2.1.1: "Network Functions Virtualisation (NFV); Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[17]	ETSI GS NFV-IFA 015 v2.1.2: "Network Functions Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[18]	ETSI ES 202 336-12 V1.1.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[19]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[20]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[21]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS) ".

[22]	3GPP TS 23.501: "System Architecture for the 5G System".

[23]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[24]	IETF RFC 791: "Internet Protocol".

[25]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[26]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[27]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[28]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[29]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[30]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[31]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[g]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[f]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[a]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[b]	IETF RFC 1166: "Internet Numbers".

[d]	IETF RFC 5952: "Internet Protocol, Version 6 (IPv6)".

[c]	IETF RFC 5952: "A Recommendation for IPv6 Address Text Representation".

[e]	IETF RFC 1034: "Domain Names – Concepts and Facilities".

Next modification

# 3a	Common data type definitions

## 3a.1	General data types

### 3a.1.1	Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| DateTime | String | Type of date and time in a format defined in IETF RFC3339 [a] |
| Latitude | Float | Latitude based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. Minimum: -90.0000, Maximum: +90.0000 |
| Longitude | Float | Longitude based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. Maximum: -180.0000, Minimum: +180.0000 |

### 3a.1.2	Enumerations

### 3a.1.3	Structured data types

#### 3a.2.3.1	GeoLocation

| Attribute name | S | Data type | Description |
| --- | --- | --- | --- |
| latitude | M | Latitude | Latitude as defined in clause 3a.1.1 |
| longitude | M | Longitude | Longitude as defined in clause 3a.1.1 |

#### 3a.2.3.2	ConvexGeoPolygon

| Data type | Description |
| --- | --- |
| array(GeoLocation) | Geographical area described as a convex polygon. The points of the polygon are GeoLocation values. |

## 3a.2	SBMA specific data types

### 3a.2.1	Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| Dn | String | Type of a Distinguished Name (3GPP TS 32.300 [13]) |
| DnList | array(Dn) | Type with multiple Distinguished Names |
| NotificationId | Integer | Type of a notification identifier |
| SystemDn | Dn | Type of a system DN |
| Filter | String | Type of a filter. The filter format is protocol specific. |

### 3a.2.2	Enumerations

#### 3a.2.2.1	AdministrativeState

| Enumeration value | Description |
| --- | --- |
| LOCKED |  |
| UNLOCKED |  |

#### 3a.2.2.2	OperationalState

| Enumeration value | Description |
| --- | --- |
| ENABLED |  |
| DISABLED |  |

#### 3a.2.2.3	UsageState

| Enumeration value | Description |
| --- | --- |
| IDLE |  |
| ACTIVE |  |
| BUSY |  |

### 3a.2.3	Structured data types

#### 3a.2.3.1	NotificationHeader

| Attribute name | S | Data type | Description |
| --- | --- | --- | --- |

## 3a.3	Internet specific data types

### 3a.3.1	Simple data types

| Type name |  |  |  | Type definition | Description |
| --- | --- | --- | --- | --- | --- |
| DomainName |  |  |  | String | Domain Name in textual dotted notation as defined in clause 3.5 of IETF RFC 1034 [e]. The domain name should be fully qualified. |
| Fqdn |  | Editor's note: This is ffs if the term Fqdn should be replaced by DomainName. |
| Ipv4Addr |  | String | Type of an IPv4 address in dotted decimal text representation format of four decimal integers, ranging from 0 to 255 each, as defined in IETF RFC 1166 [b]. Pattern: '^(([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])\.){3}([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])$' |  |  |
| Ipv6Addr |  | String | Type of an Ipv6 address (IETF RFC 4260 [d]) in the canonical text representation format defined in clause 4 of IETF RFC 5952 [c]. Pattern: '^((:¦(0?¦([1-9a-f][0-9a-f]{0,3}))):)((0?¦([1-9a-f][0-9a-f]{0,3})):){0,6}(:¦(0?¦([1-9a-f][0-9a-f]{0,3})))$' and Pattern: '^((([^:]+:){7}([^:]+))¦((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$' |  |  |
| IpAddr |  | CHOICE(Ipv4Addr, Ipv6Addr) | Type of an IP address, either an ipv4 address or an ipv6 address. |  |  |
| Host |  | CHOICE(Ipv4Addr, Ipv6Addr, DomainName) | Editor's note: This is ffs since host names have a stricter format than domain names. |  |  |
| Port |  |  |  | Integer | Port number, as defined e.g. in clause 3.2.3 of IETF RFC 3986 [a] Minimum: 0, Maximum: 65535 |
| Uri |  | String | Type of a URI as defined in in IETF RFC 3986 [a] |  |  |

## 3a.4	Mobile network specific data types

### 3a.4.1	Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| Mcc | String | Type of a Mobile Country Code, consisting of three decimal digits, as defined in clause 2.2 of TS 23.003 [5] Pattern: '^[0-9]{3}$' |
| Mnc | String | Type of a Mobile Network Code, consisting of two or three decimal digits, as defined in clause 2.2 of TS 23.003 [5] Pattern: '^[0-9]{2,3}$' |
| Nid | String | Network Identifier, which together with a PLMN ID is used to identify an SNPN (see 3GPP TS 23.003 [5] and 3GPP TS 23.501 [22] clause 5.30.2.1). Pattern: '^[A-Fa-f0-9]{11}$' |
| Tac | String | 3-octet string identifying a tracking area code as specified in clause 9.3.3.10 of 3GPP TS 38.413 [f], in hexadecimal representation. Pattern: '(^[A-Fa-f0-9]{4}$)¦(^[A-Fa-f0-9]{6}$)' |

### 3a.4.2	Enumerations

None.

### 3a.4.3	Structured data types

### 3a.4.3.1	PlmnId

Type of a Public Land Mobile Network (PLMN) identifier (PLMN-Id), as defined in clause 12.1 and clause 2.2 of TS 23.003 [5].

| Attribute name | S | Data type | Description |
| --- | --- | --- | --- |
| mcc | M | Mcc | Mobile Country Code (MCC) |
| mnc | M | Mnc | Mobile Network Code (MNC) |

## 3a.5	4G network specific data types

### 3a.5.1	Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| EutraCellId | String | 28-bit string identifying an E-UTRA Cell Identity as specified in clause 9.2.1.38 of 3GPP TS 36.413 [g], in hexadecimal representation. Pattern: '^[A-Fa-f0-9]{7}$' |

### 3a.5.2	Enumerations

### 3a.5.3	Structured data types

## 3a.6	5G network specific data types

### 3a.6.1	Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| NrCellId | String | 36-bit string identifying an NR Cell Identity as specified in clause 9.3.1.7 of 3GPP TS 38.413 [f], in hexadecimal representation. Pattern: '^[A-Fa-f0-9]{9}$' |

### 3a.6.2	Enumerations

### 3a.6.3	Structured data types

End of modifications