3GPP TSG SA WG5 Meeting 139-e		S5-215274

# electronic meeting, online, 11- 20 October 2021

# Source:	Nokia

# Title:	Add MDA Reporting Control

# Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

# 3	Rationale

This contribution introduces an MDA reporting control description, use case and requirements that enables the MDA MnS consumer to request analytics indicating certain reporting control attributes to the MDA MnS producer.

This MDA reporting control can be applicable to all analytics use cases described in [1], which request an MDA report.

# 4	Detailed proposal

1st Modified Section

## X.Y	MDA Reporting Control

### X.Y.1	Description

The MDA reporting control allows any authorized MDA MnS consumer to requestMDA.

### X.Y.2	Use Case

The MDA MnS conusmer can request the MDA MnS producer to provide MDA reports for a specified type of analytics, which corresponds to a MDA capability, i.e., MDA use case. The MDA MnS conusmer may introduce reporting attributes related to the MDA report with respect to the geographic location and/or the target objects, e.g., managed elements, time schedule, i.e., time conditions related to the prearation of MDA results, and potential filter conditions to be met before receiving a MDA report, e.g. threshold crossing or a specified time schedule. The MDA MnS conusmer may control the reporting attributes and has the capability to modify them.

The MDA MnS conusmer can request a MDA report that contains numeric output results, e.g. average, Cumulative Distribution Function (CDF), nomal distribution, etc., recommendation options, e.g. potential handover cells, or root case analysis, e.g. alarm prediciton.

The MDA MnS consumer can be informed with an acknowledgment that includes the MDA report name if the request was successful. If the request was not successful the consumer is informed about potential errors indicating the reasons. The MDA MnS consumer can also deactivate the MDA reporting control request once it is no longer needed.

### X.Y.3	Requirements for Requesting MDA

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MDA_MDA-REQ-1 | The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to request MDA reporting, while indicating its preferences on the MDA type. | All use cases |
| REQ-MDA_MDA-REQ-2 | The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to request MDA reporting, while indicating its preferences on the reporting time schedule. | All use cases |
| REQ-MDA_MDA-REQ-3 | The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to request MDA reporting, while indicating its preferences on geographic location and/or the target objects. | All use cases |
| REQ-MDA_MDA-REQ-4 | The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to request MDA reporting, while indicating its preferences on the time schedule related to MDA results. | All use cases |
| REQ-MDA_MDA-REQ-5 | The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to modify MDA reporting, while indicating its new preferences for any of the attributes related to the requested MDA report. | All use cases |
| REQ-MDA_MDA-REQ-6 | The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to specify filter conditions based on threshold crossing for MDA reporting. | All use cases |
| REQ-MDA_MDA-REQ-7 | The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to express that the requested MDA reporting is no longer needed. | All use cases |
| REQ-MDA_MDA-REQ-8 | The MDA MnS producer shall inform any authorized MDA MnS consumer whether the MDA reporting request was successfully processed and any errors that may have occurred. | All use cases |

End of Modified Sections