# 3GPP TSG SA WG5 Meeting #139e	S5-215565

# Online, , 11 Oct 2021- 20 Oct 2021

# Source:	Samsung

# Title:	pCR 28.538 EASFunction definition

# Document for:	Approval

Agenda Item:	6.4.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution defined the EASFunction IOC.

# 4	Detailed proposal

First modification

## 6.3	Class definition

6.3.1		EASFunction

6.3.1.1 		Definition

This IOC represent the properties of a EAS in a 3GPP network. For more information about EAS, see 3GPP TS 23.558.

6.3.1.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| eASIdentifier | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| eASRequirementsRef | M | T | T | F | T |

#### 6.3.1.3	Attribute constraints

#### 6.3.1.4	Notifications

TBD.

6.3.2		EASRequirements

6.3.2.1 		Definition

This represent the requirements needed to deploy EAS(s)..

6.3.2.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| requiredEASservingLocation | M | T | F | F | T |

#### Editor’s Note:  The definition of IOCs is not complete. It is expected additional attributes, as needed.

#### 6.3.2.3	Attribute constraints

#### 6.3.2.4	Notifications

TBD.

6.3.3		ServingLocation <<datatype>>

6.3.3.1 		Definition

This datatype represent the location which is to be served by the node.

6.3.3.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| geographicalLocation | CM | T | F | F | T |
| tAI | CM | T | T | F | T |

#### 6.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| geographicalLocation Support Qualifier | Condition: If the serving location is described with Geographical Service Area [2]. |
| tAI Support Qualifier | Condition: If the serving location is described with Topological Service Area [2]. |

#### 6.3.3.4	Notifications

TBD.

6.3.4		GeoLoc <<datatype>>

6.3.4.1 		Definition

This datatype represent the geographical location.

6.3.4.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| lat | M | T | T | F | T |
| long | M | T | T | F | T |
| civicAddress | O | T | T | F | T |

#### 6.3.4.3	Attribute constraints

None

#### 6.3.4.4	Notifications

TBD

## 6.4	Attribute definition

6.4.1		Attribute Properties

Editors Note: The definition of attributes are not complete, and are subject to changes.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| eASIdentifier | It identifies the EAS, see 3GPP TS 23.558. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eASREquirementsRef | This is the DN of EASRequirements. allowedValues: Not applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| requiredEASservingLocation | It defines the location where the EAS service should be available. | type: ServingLocation multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| geographicalLocation | This refers to the Geographical Service Area, see 3GPP TS 23.558. | type: GeoLoc multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| lat | This defines the single latitude coordinate. | type: Float multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| long | This defines the single longtitude coordinate. | type: Float multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| civicAddress | This defines the location in terms of a civic address | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| tAI | Indicates the TAI (see subclause 4.3.49 in TS 28.541 [3]), including pLMNId ID and nRTAC. | type: TAI multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |

End of First modification

Third modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[2]	3GPP TS 23.558: " Architecture for enabling Edge Applications ".

[3]	3GPP TS 28.541: "5G Network Resource Model (NRM);".

End of third modification