3GPP TSG-SA5 Meeting #141-e 	S5-221756

# e-meeting, 17 - 26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0047 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MRO additions for CHO and DAPS handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_HOO |  |  |  |  |  | Date: |  |  | 2022-01-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Make sure Stage 1, Stage 2 and Stage 3 are in sync. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove parts of Stage 1 that is not implemented in Stage 2 and 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Discrepancy between Stagen 1 and Stage 2,3 may lead to confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.X, 6.1.1.X (first occurrance), 6.1.1.X (second occurrence), 6.4.1.X, (first occurrance), 6.4.1.X (second occurrence). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ANR	Automatic Neighbour Relation

CHO	Conditional Handover

DAPS	Dual Active Protocol Stack

NCR	Neighbour Cell Relation

NG-RAN	Next Generation Radio Access Network

Next change

#### 6.1.1.X	CHO management

REQ-DCHO-FUN-1	The producer of NF provisioning MnS should have the capability allowing an authorized consumer to enable or disable Conditional Handover for a gNB.

REQ-DCHO-FUN-2	The producer of NF performance assurance MnS should have the capability to produce measurements related to CHO.

Next change

#### 6.1.1.X	DAPS handover management

REQ-DDAPSHO-FUN-1	The producer of NF provisioning MnS should have the capability allowing an authorized consumer to enable or disable DAPS handover for a gNB.

REQ-DDAPSHO-FUN-2	The producer of NF performance assurance MnS should have the capability to produce measurements related to DAPS handover.

Next change

#### 6.4.1.X	CHO (Conditional Handover)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable CHO. |  |
| Actors and Roles | D-SON management function to support the CHO function. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS. |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - CHO is not in operation for a gNB. |  |
| Begins when | The D-SON management function intends to enable CHO for a gNB. |  |
| Step 1 (M) | The D-SON management function requests the producer of provisioning MnS to enable CHO for a gNB. |  |
| Step 2 (M) | The D-SON management function collects CHO related measurements and analyses them to evaluate the CHO performance. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | CHO is in operation from for a gNB. |  |
| Traceability | REQ-DCHO-FUN-1, REQ-DCHO-FUN-2 |  |

Next change

#### 6.4.1.X	DAPS HO (Dual Active Protocol Stack Handover)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable DAPS HO. |  |
| Actors and Roles | D-SON management function to support the DAPS HO function. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS. |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - DAPS HO is not in operation for a gNB. |  |
| Begins when | The D-SON management function intends to enable DAPS HO for a gNB. |  |
| Step 1 (M) | The D-SON management function requests the producer of provisioning MnS to enable DAPS HO from a source cell to a target cell. |  |
| Step 2 (M) | The D-SON management function collects DAPS HO related measurements and analyses them to evaluate the DAPS HO performance. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | DAPS HO is in operation for a gNB. |  |
| Traceability | REQ-DDAPSHO-FUN-1, REQ-DDAPSHO-FUN-2 |  |

Next change

### 7.1.x	MRO for Conditional Handover (CHO)

#### 7.1.x.1	MnS component type A

MRO for CHO re-uses the component A for MRO, see clause 7.1.2.1.

#### 7.1.x.2	MnS Component Type B definition

##### 7.1.x.2.1	Control information

These parameters are used to control the CHO function.

Table 7.1.x.2.1-1: MRO fro CHO control

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| CHO function control | This attribute allows the operator to enable/disable the CHO functionality. See attribute choControl in TS 28.541 [13]. | Boolean On, off |

##### 7.1.x.2.2	Parameters to be updated

MRO for CHO re-uses the same parameters to be updated as MRO, see clause 7.1.2.2.3.

#### 7.1.x.3	MnS Component Type C definition

##### 7.1.x.3.1	Performance measurements

Performance measurements related to MRO for CHO are captured in Table 7.1.x.3.1.-1:

Table 7.1.x.3.1-1. MRO for CHO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Number of requested conditional handover preparations | Counts the number of successful and unsuccessful inter-gNB conditional handover preparations sent (see TS 28.552 clause 5.1.1.6.x.1) |  |
| Number of successful conditional handover preparations | Counts the number of unsuccessful inter-gNB conditional handover preparations sent (see TS 28.552 clause 5.1.1.6.x.2) |  |
| Number of failed conditional handover preparations | Counts the number of unsuccessful inter-gNB conditional handover preparations sent (see TS 28.552 clause 5.1.1.6.x.3) |  |
| Number of requested conditional handover resource allocations | Counts the number of successful and unsuccessful inter-gNB conditional handover preparations (see TS 28.552 clause 5.1.1.6.x.4) |  |
| Number of successful conditional handover resource allocations | Counts the number of successful inter-gNB conditional handover preparations (see TS 28.552 clause 5.1.1.6.x.5) |  |
| Number of failed conditional handover resource allocations | Counts the number of unsuccessful inter-gNB conditional handover preparations (see TS 28.552 clause 5.1.1.6.x.6) |  |
| Number of configured conditional handover candidates | Counts the number of outgoing inter-gNB conditional handover candidates requested (see TS 28.552 clause 5.1.1.6.x.7) |  |
| Number of UEs configured with conditional handover. | Counts the number of UEs that has been configured with inter-gNB conditional handover (see TS 28.552 clause 5.1.1.6.x.8) |  |
| Number of successful conditional handover executions | Counts the number of successful inter-gNB conditional handover executions received (see TS 28.552 clause 5.1.1.6.x.9) |  |
| Number of failed conditional handover executions | Counts the the number of failed inter-gNB conditional handover executions received (see TS 28.552 clause 5.1.1.6.x.10) |  |
| Mean Time of requested conditional handover executions | Counts the mean time of inter-gNB conditional handover executions (see TS 28.552 clause 5.1.1.6.x.11) |  |
| Max Time of requested conditional handover executions | Counts the max time of inter-gNB conditional handover executions (see TS 28.552 clause 5.1.1.6.x.12) |  |
| Number of configured conditional handover candidates | Counts the number of outgoing intra-gNB conditional handover candidates requested (see TS 28.552 clause 5.1.1.6.y.1) |  |
| Number of UEs configured with conditional handover | Countes the the number of UEs that has been configured with conditional handover (see TS 28.552 clause 5.1.1.6.y.2) |  |
| Number of successful conditional handover executions | Counts the number of successful intra-gNB conditional handover executions received (see TS 28.552 clause 5.1.1.6.y.3) |  |
| Number of requested conditional handover preparations | Counts the number of outgoing intra-gNB conditional handover preparations requested, for a split gNB deployment (see TS 28.552 clause 5.1.3.7.1.a) |  |
| Number of successful conditional handover preparations | Countes the number of successful intra-gNB conditional handover preparations, for a split gNB deployment (see TS 28.552 clause 5.1.3.7.1.b) |  |

Next change

### 7.1.z	MRO for DAPS handover

#### 7.1.z.1	MnS component type A

MRO for DAPS handover re-uses the component A for MRO, see clause 7.1.2.1.

#### 7.1.z.2	MnS Component Type B definition

##### 7.1.z.2.1	Control information

The parameter is used to control the DAPS handover function.

Table 7.1.z.2.1-1: MRO for DAPS handover control

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| DAPS HO function control | This attribute allows the operator to enable/disable the DAPS HO functionality. See attribute dapsHoControl in TS 28.541 [13]. | Boolean On, off |

##### 7.1.z.2.2	Parameters to be updated

MRO for DAPS handover re-uses the same parameters to be updated as MRO, see clause 7.1.2.2.3.

#### 7.1.z.3	MnS Component Type C definition

##### 7.1.z.3.1	Performance measurements

Performance measurements related to MRO for DAPS handover are captured in Table 7.1.z.3.1.-1:

Table 7.1.z.3.1-1. MRO for DAPS handover related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Number of requested DAPS handover preparations | Counts the number of successful and unsuccessful inter-gNB DAPS handover preparations sent (see TS 28.552 clause 5.1.1.6.x.1) |  |
| Number of successful DAPS handover preparations | Counts the number of unsuccessful inter-gNB DAPS handover preparations sent (see TS 28.552 clause 5.1.1.6.x.2) |  |
| Number of failed DAPS handover preparations | Counts the number of unsuccessful inter-gNB DAPS handover preparations sent (see TS 28.552 clause 5.1.1.6.x.3) |  |
| Number of requested DAPS handover resource allocations | Counts the number of successful and unsuccessful inter-gNB DAPS handover preparations (see TS 28.552 clause 5.1.1.6.x.4) |  |
| Number of successful DAPS handover resource allocations | Counts the number of successful inter-gNB DAPS handover preparations (see TS 28.552 clause 5.1.1.6.x.5) |  |
| Number of failed DAPS handover resource allocations | Counts the number of unsuccessful inter-gNB DAPS handover preparations (see TS 28.552 clause 5.1.1.6.x.6) |  |
| Number of requested DAPS handover executions | Counts the number of outgoing inter-gNB DAPS handover candidates requested (see TS 28.552 clause 5.1.1.6.x.7) |  |
| Number of successful DAPS handover executions | Counts the number of successful inter-gNB DAPS handover executions received (see TS 28.552 clause 5.1.1.6.x.8) |  |
| Number of failed DAPS handover executions | Counts the the number of failed inter-gNB DAPS handover executions received (see TS 28.552 clause 5.1.1.6.x.9) |  |
| Number of DAPS handover requested | Counts the number of outgoing intra-gNB DAPS handovers requested (see TS 28.552 clause 5.1.1.6.y.1) |  |
| Number of successful DAPS handovers | Counts the number of successful intra-gNB DAPS handovers (see TS 28.552 clause 5.1.1.6.y.2) |  |
| Number of requested DAPS handover preparations | Counts the number of outgoing intra-gNB DAPS handover preparations requested, for a split gNB deployment (see TS 28.552 clause 5.1.3.7.1.a) |  |
| Number of successful DAPS handover preparations | Countes the number of successful intra-gNB DAPS handover preparations, for a split gNB deployment (see TS 28.552 clause 5.1.3.7.1.b) |  |

End of changes