3GPP TSG-SA5 Meeting #141-e 	S5-221158

# e-meeting, 17 -26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.662 | CR | 0012 | rev | 2 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-15 CR TS 28.662 Update Generic RAN NRM to be applicable for SBMA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2021-12-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The current description in scope and several contents limit the Generic RAN NRM,making it only applicable for the deployments using IRP framework. However, the Generic RAN NRM should also be applicable for deployments using SBMA. - There are some description related to NR is not aligned |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the Scope and several content to be applicable for deployments using SBMA. - Align the description related to NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The specification is not applicable for deployments using SBMA - Some misalignment exists |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2 ,4.1, 4.2.1, 4.2.2, 4.3.1.3, 4.3.2.3, 4.3.3.3,4.3.5.2,4.3.6,4.3.9,4.4.1,4.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-216618 is the revision of S5-216213 - S5-221158 is the revision of S5-216618 |  |  |  |  |  |  |  |  |

1st Change

1	Scope

The present document specifies the Generic Radio Access Network (RAN) network resource model (NRM) that can be communicated between an IRPAgent and an IRPManager in the deployment scenarios using IRP framework as defined in TS 32.102 [3], or between an MnS consumer and MnS producer in deployment scenarios using the Service Based Management Architecture (SBMA) as defined in TS 28.533 [X], for telecommunication network management purposes, including management of converged networks.

This document specifies the semantics and behaviour of information object class attributes and relations visible across the reference point in a protocol and technology neutral way.  It does not define their syntax and encoding.

In order to access the information defined by this NRM, an Interface IRP such as the "Basic CM IRP"(3GPP TS 32.602 [5] and TS 32.606 [Z]) or an MnS such as "Generic Provisioning MnS" (3GPP TS 28.532 [Y]) is needed. However, which Interface IRP is applicable is outside the scope of the present document.

Next Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[5]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP) ; Information Service (IS)".

[6]	Void.

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E_UTRA); Base Station (BS) radio transmission and reception".

[8]	Void.

[9]	Void.

[10]	3GPP TS 28.661: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[11]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[12]	3GPP TS 28.652: "Telecommunication management; Universal Terrestrial Radio Access Network (UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[13]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	3GPP TS 28.655:"Telecommunication management; GSM/EDGE Radio Access Network (GERAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[15]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[17]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM Information Service (IS)".

[18]	3GPP TS 25.106: "Technical Specification Group Radio Access Network; UTRA repeater radio transmission and reception".

[19]	3GPP TS 45.005: "Radio transmission and reception".

[20]	3GPP TS 45.010: "Radio subsystem synchronization".

[21]	3GPP TS 25.104: "Base Station (BS) radio transmission and reception (FDD)".

[22]	3GPP TS 25.105: "Base Station (BS) radio transmission and reception (TDD)".

[23]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[24]	3GPP TS 28.541: "NR and NG-RAN Network Resource Model (NRM) stage 2 and stage 3".

[25] 	3GPP TS 28.652: "UTRAN Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[26]	3GPP TS 37.466: "Iuant Interface: Application Part".

[X]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[Y]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[Z]	3GPP TS 32.606: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP); Solution Set (SS) definitions".

Next Change

4.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.622 [15], IOC, ManagedFunction | ManagedFunction |
| 3GPP TS 28.652 [12], IOC, UtranGenericCell | UtranGenericCell |
| 3GPP TS 28.658 [13], IOC, EUtranGenericCell | EUtranGenericCell |
| 3GPP TS 28.655 [14], IOC, GSMCell | GSMCell |
| 3GPP TS 28.541 [24], IOC, NRSectorCarrier | NRSectorCarrier |
| 3GPP TS 28.541 [24], IOC, NRCellDU | NRCellDU |
| 3GPP TS 28.658 [13], IOC, ENBFunction | ENBFunction |
| 3GPP TS 28.652 [25], IOC, NodeBFunction | NodeBFunction |
| 3GPP TS 28.655 [14], IOC, BssFunction | BssFunction |
| 3GPP TS 28.541 [24], IOC, GNBDUFunction | GNBDUFunction |

Next Change

4.2	Class diagrams

4.2.1	Relationships

This subclause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this IRP or MnS. This subclause provides the overview of the relationships of relevant classes in UML. Subsequent subclauses provide more detailed specification of various aspects of these classes.

Figure 4.2.1.1: UTRAN/E-UTRAN/NR/GERAN sharing

Figure 4.2.1.3: CommonBsFunction NRM fragment

Figure 4.2.1.4: RepeaterFunction NRM fragment

Figure 4.2.1.5: RepeaterFunction related VsDataContainer Containment/Naming and Association diagram

4.2.2	Inheritance

This subclause depicts the inheritance relationships.

Figure 4.2.2.1:  Inheritance diagram

Next Change

4.3	Class definitions

4.3.1	SectorEquipmentFunction

4.3.1.1	Definition

This IOC represents a set of cells within a geographical area that has common functions relating to AntennaFunction, TMAFunction and supporting equipment, such as power amplifier.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

4.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fqBand | CM | T | F | F | T |
| eUTRANFqBands | CM | T | F | F | T |
| nRFqBands | CM | T | F | F | T |
| uTRANFDDFqBands | CM | T | F | F | T |
| uTRANTDDFqBands | CM | T | F | F | T |
| confOutputPower | O | T | T | F | F |
| Attribute related to role |  |  |  |  |  |
| theTMAList | CM | T | F | F | T |
| theAntennaList | CM | T | F | F | T |
| theCellList | CM | T | F | F | T |
| theNRSectorCarrierList | CM | T | F | F | T |

4.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| fqBand CM Support Qualifier | Condition: EUTRAN is supported, and only one EUTRAN frequency band is supported, and eUTRANFqBands is not used. |
| eUTRANFqBands CM Support Qualifier | Condition: EUTRAN is supported, and fqBand is not used. |
| nRFqBands CM Support Qualifier | Condition: NR is supported. |
| uTRANFDDFqBands CM Support Qualifier | Condition: UTRAN FDD is supported. |
| uTRANTDDFqBands CM Support Qualifier | Condition: UTRAN TDD is supported. |
| theTMAList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and AntennaFunction is absent AND is supporting the UTRAN/E-UTRAN/NR sharing/non-sharing case OR is supporting the GERAN sharing case. In such case, at least one TMAFunction is present. |
| theAntennaList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and TMAFunction is absent AND is supporting the UTRAN/E-UTRAN/NR sharing/non-sharing OR is supporting GERAN sharing case. In such case, at least one AntennaFunction is present. |
| theCellList CM Support Qualifier | Condition: Supporting UTRAN/E-UTRAN sharing (and non-sharing) cases. In such case, at least one UtranGenericCell/EUtranGenericCell is present. Condition: Supporting the GERAN sharing case. In such case, at least one GSMCellPart is present. |
| theNRSectorCarrierList CM Support Qualifier | Condition: Supporting NR sharing and non-sharing cases. In such cases, at least one NRSectorCarrier is present. |

4.3.1.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

4.3.2	AntennaFunction

4.3.2.1	Definition

This IOC represents an array of radiating elements that may be tilted to adjust the RF coverage of a cell(s).

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

4.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | IsNotifyable |
| --- | --- | --- | --- | --- | --- |
| retTiltValue | O | T | T | F | T |
| bearing | O | T | T | F | T |
| retGroupName | O | T | T | F | T |
| height | O | T | T | F | T |
| maxAzimuthValue | O | T | T | F | T |
| minAzimuthValue | O | T | T | F | T |
| horizBeamwidth | O | T | T | F | T |
| vertBeamwidth | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| theCellList | CM | T | F | F | T |

4.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| theCellList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and ProxyCell is absent; and AND is supporting the UTRAN/E-UTRAN sharing/non-sharing case OR is supporting the GERAN sharing case |

4.3.2.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

4.3.3	TMAFunction

4.3.3.1	Definition

This IOC represents a Tower Mounted Amplifier or a number of TMA subunits within one TMA, each separately addressable by a specific index at the application layer.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

4.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| tmaSubunitNumber | M | T | T | F | T |
| tmaStateFlag | M | T | O | F | T |
| tmaFunctionFlag | M | T | T | F | T |
| tmaMinGain | M | T | F | F | T |
| tmaMaxGain | M | T | F | F | T |
| tmaResolution | M | T | F | F | T |
| tmaGainFigure | M | T | O | F | T |
| tmaNumberOfSubunits | M | T | F | F | T |
| tmaBaseStationId | CO | T | CO | F | T |
| tmaSectorId | CO | T | CO | F | T |
| tmaAntennaBearing | CO | T | CO | F | T |
| tmaInstalledMechanicalTilt | CO | T | CO | F | T |
| tmaSubunitType | CO | T | CO | F | T |
| tmaSubunitRxFrequencyBand | CO | T | CO | F | T |
| tmaSubunitTxFrequencyBand | CO | T | CO | F | T |
| tmaGainResolution | CO | T | CO | F | T |
| Attribute related to role |  |  |  |  |  |
| theCellList | CM | T | F | F | T |

4.3.3.3	Attribute Constraints

| Name | Definition |
| --- | --- |
| theCellList CM Support Qualifier | Condition: Association between SectorEquipmentFunction and ProxyCellList is absent ; AND is supporting the UTRAN/E-UTRAN sharing/non-sharing case OR is supporting the GERAN sharing case. |

| Name | Definition |
| --- | --- |
| The CO support qualifier of the attributes tmaBaseStationId through tmaGainResolution | Condition: The TMA subunit supports the read operation in 3GPP TS 37.466 [26] |
| The CO write qualifier of the attributes tmaBaseStationId through tmaGainResolution | Condition: The TMA subunit supports the write operation in 3GPP TS 37.466 [26] |

4.3.3.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

4.3.4	GSMCellPart

4.3.4.1	Definition

A GSM cell can consist of a number of carriers. These carriers can be configured in a number of ways, for example, the carriers can have different propagation properties which are sent with different antenna tilt, with different RF power, different radio band and even possibly different antenna.

The various GSMCellPart instances capture different radio propagation properties allowing different frequency planning schemes, e.g. some GSMCellPart instances can use frequency groups planned for tighter frequency reuse.

Hence, a GSM cell can, and in some cases must, be distributed on more than one SectorEquipmentFunction.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

4.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | IsNotifyable |
| --- | --- | --- | --- | --- | --- |
| aRFCN | M | T | T | F | T |
| tsc | M | T | T | F | T |
| aTA | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| theSectorEquipment | M | T | F | F | T |

4.3.4.3	Attribute constraints

None

4.3.4.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

4.3.5	CommonBsFunction

4.3.5.1	Definition

This IOC represents common aspects of Base Station (BS) functionality shared by several radio access technologies.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

4.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sharedTechnologies | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| theProxyBsList | M | T | F | F | T |

4.3.5.3	Attribute constraints

None

4.3.5.4	Notifications

There is no notification defined.

4.3.6	Void

4.3.7	RepeaterFunction

4.3.7.1	Definition

This IOC represents the management aspect of a repeater. For the information on repeater see 3GPP TS 25.106 [18].

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.661 [10] | REQ-GRAN_NRM-CON-003 |  |

4.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| priority | M | T | T | F | T |
| latitude | M | T | F | F | F |
| longitude | M | T | F | F | F |
| ctrlConnMode | M | T | T | F | T |
| environmentInfo | M | T | - | F | - |
| powerSwitch | M | T | T | F | T |
| ulAttenuation | M | T | T | F | T |
| dlAttenuation | M | T | T | F | T |
| firmwareVer | M | T | F | F | F |
| repeaterType | M | T | F | F | F |
| Attribute related to role |  |  |  |  |  |
| externalUTRANCell | M | T | F | F | T |

4.3.7.3	Attribute constraints

None.

4.3.7.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

4.3.8	ProxyCell <<ProxyClass>>

4.3.8.1	Definition

This IOC represents GSMCellPart, UTranGenericCell and EUtranGenericCell.

4.3.8.3	Attribute constraints

See respective IOCs

4.3.8.4	Notifications

See respective IOCs.

4.3.9	ProxyBsFunction <<ProxyClass>>

4.3. 9.1	Definition

This IOC represents GNBDUFunction, ENBFunction, NodeBFunction and BssFunction.

4.3.9.3	Attribute constraints

See respective IOCs.

4.3.9.4	Notifications

See respective IOCs.

Next Change

4.4	Attribute definitions

4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aRFCN | This attribute (Absolute Radio Frequency Channel Number) defines a pair of Radio Frequency (RF) channel frequencies for uplink and downlink use. See 3GPP TS 45.005 [19] clause 2 for the ARFCN for GSM. ARFCN are based on a 200 kHz channel raster. allowedValues: See 3GPP TS 45.005 [19] clause 2 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| aTA | This attribute (allowed Timing Advance) defines the signal sent by the BTS to the MS which the MS uses to advance its timings of transmissions to the BTS so as to compensate for propagation delay. allowedValues: See 3GPP TS 45.010 [20] | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| bearing | The bearing in degrees that the antenna is pointing in. Antenna bearing" in Ref. 3GPP TS 25.463 [8]. allowedValues: See "Antenna bearing" in 3GPP TS 25.463 [8]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confOutputPower | It defines the allowed total power to use for all cells together in this sector. It may be set by the operator and/or limited by HW limitation or licensed power, e.g.: 20, 40, 60, 80,120 watts allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ctrlConnMode | Remote communication mode used by a repeater to send and receive control message, such as GSM SMS, WCDMA SMS, Circle Switch Data-CSD, Package Switch Dat-IP, Serial port. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dlAttenuation | Downlink signal attenuation of the device to change downlink gain. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| environmentInfo | The repeater device is located either in the building or out of the building. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| eUTRANFqBands | This is the list of LTE frequency bands supported by the hardware associated with the SectorEquipmentFunction. The earfcnDl and earfcnUl or earfcn of LTE cells associated with the SectorEquipmentFunction must be assigned with value within one of the specified eUTRANFqBands values. allowedValues: A list of frequency bands expressed as strings. Valid frequency band values are specified in sub-clause 5.7.3 in 36.104 [7]. For HW not supporting LTE frequency bands, the list shall be empty. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| firmwareVer | Version of the device firmware. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| fqBand | This is the LTE frequency band supported by the hardware associated with the SectorEquipmentFunction. The earfcnDl and earfcnUl of cells associated with the SectorEquipmentFunction must be assigned with value within this fqBand value. allowedValues: See clause 5 Table 5.5-1 “E-UTRA frequency band” of 3GPP TS 36.104 [7]. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| nRFqBands | This is the list of NR frequency bands supported by the hardware associated with the SectorEquipmentFunction. The arfcnDl and arfcnUl of the NRSectorCarrier must be assigned with value within one of the specified nRFqBands values – if the attributes on NRSectorCarriers are set. The arfcnDl and arfcnUl of the NRCellDU associated with the NRSectorCarrier must be assigned with value within one of the specified nRFqBands values – if there is a NRCellDU associated with the NRSectorCarrier. allowedValues: A list of frequency bands expressed as strings. Valid frequency band values are specified in sub-clause 5.4.2 in 3GPP TS 38.104 [23]. For HW not supporting NR frequency bands, the list shall be empty. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| height | The height of an antenna above sea level. Note: The value of this attribute has no operational impact on the network, e.g. the NE behavior is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 37.466 [26]. An integral value representing a number of meters in 0.1 meter increments. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| horizBeamwidth | The 3 dB power beamwidth of the antenna pattern in the horizontal plane. A value of 360 indicates an omni-directional antenna. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS37.466 [26]. A single integral value corresponding to an angle in degrees between 0 and 360. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| latitude | The latitude of the antenna location based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. allowedValues: -90.0000 to +90.0000 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| longitude | The longitude of the antenna location based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. allowedValues: -180.0000 to +180.0000 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| maxAzimuthValue | The maximum amount of change of azimuth the RET system can support. This is the change in degrees clockwise from bearing. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 37.466 [26]. A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| minAzimuthValue | The minimum amount of change of azimuth the RET system can support. This is the change in degrees counter-clockwise from bearing. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [9]. A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority | The priority of a repeater decided by an operator. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| powerSwitch | Power switch of device which has two status: ON/OFF. allowedValues: ON, OFF | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| relatedAntennaList | This attribute contains the DNs of one or more AntennaFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: N/A isUnique: T defaultValue: None isNullable: True |
| relatedSectorEquipment | This attribute contains the DN of one SectorEquipmentFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| relatedTMAList | This attribute contains the DNs of one or more TmaFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: N/A isUnique: T defaultValue: None isNullable: True |
| repeaterType | The repeater type defined by operator, such as wide band, frequency selective, indoor and fiber optic. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| retGroupName | The group name is a textual, alpha-numeric string to define a logical grouping of antennas which may be in different cells. This attribute permits the definition of a logical grouping of the antennas. This may be defined either at installation time, or by management activity to provisioning the group name via the Itf-N. allowedValues: N/A (String size is bounded to 80 characters.) | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| retTiltValue | The electrical tilt setting of the antenna, "Tilt value" in Ref. 3GPP TS 37.466 [26]. allowedValues: See "Tilt value" in Ref. 3GPP TS 37.466 [26]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sharedTechnologies | This attribute defines the radio access technologies sharing the common functionalities of a Base Station (BS). allowedValues: GSM, UMTS, LTE, NR. | type:ENUM multiplicity: 1..4 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaAntennaBearing | A data field defined in Table B.3 of 3GPP TS 37.466 [26]. See definition in 3GPP TS 37.466 [26]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaBaseStationId | A data field defined in Table B.3 of 3GPP TS 37.466 [26] allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaFunctionFlag | Defined in 3GPP TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaGainFigure | Defined in 3GPP TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaGainResolution | A data field defined in Table B.3 of 3GPP TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaInstalledMechanicalTilt | A data field defined in Table B.3 of 3GPP TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaMaxGain | Defined in 3GPP TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaMinGain | Defined in 3GPP TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaNumberOfSubunits | Defined in 3GPP TS 37.466 [26] allowedValues: -- | Defined in 3GPP TS 37.466 [26] type: -- multiplicity: -- isOrdered: -- isUnique: -- defaultValue: -- isNullable: -- |
| tmaResolution | Defined in 3GPP TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaSectorId | A data field defined in Table B.3 of 3GPP TS 37.466 [26] allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaStateFlag | Defined in 3GPP TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaSubunitNumber | Defined in 3GPP TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaSubunitRxFrequencyBand | A data field defined in Table B.3 of 3GPP TS 37.466 [26] allowedValues: See 3GPP TS 37.466 [26]. | type: Integer multiplicity: 2 isOrdered: True isUnique: True defaultValue: None isNullable: False |
| tmaSubunitType | A data field defined in Table B.3 of 3GPP TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tmaSubunitTxFrequencyBand | A data field defined in Table B.3 of 3GPP TS 37.466 [26] allowedValues: See 3GPP TS 37.466 [26]. | type: Integer multiplicity: 2 isOrdered: True isUnique: True defaultValue: None isNullable: False |
| tsc | This attribute has the same definition as the one used in GsmCell IOC. The presence of GSMCellPart means the tsc attribute in GsmCell IOC instance is irrelevant (not applicable). allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ulAttenuation | Uplink signal attenuation of the device to change uplink gain. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uTRANFDDFqBands | This is the list of UTRAN FDD frequency bands supported by the hardware associated with the SectorEquipmentFunction. The arfcnDl and arfcnUl of UTRAN FDD cells associated with the SectorEquipmentFunction must be assigned with value within one of the specified uTRANFDDFqBands values. allowedValues: A list of frequency bands expressed as strings. Valid frequency band values are specified in sub-clause 5.2 of 3GPP TS 25.104 [21]. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| uTRANTDDFqBands | This is the list of UTRAN TDD frequency bands supported by the hardware associated with the SectorEquipmentFunction. The earfcn of UTRAN TDD cells associated with the SectorEquipmentFunction must be assigned with value within one of the specified uTRANTDDFqBands values. allowedValues: A list of frequency bands expressed as strings. Valid frequency band values are specified in sub-clause 5.2 of 3GPP TS 25.105 [22]. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| vertBeamwidth | The 3 dB power beamwidth of the antenna pattern in the vertical plane. The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. This attribute is not supported over the Iuant interface according to Ref. 3GPP TS 37.466 [26]. allowedValues: A single integral value corresponding to an angle in degrees between 0 and 180. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| Attribute related to role |  |  |
| externalUTRANCell | This role (when present) represents repeaterFunction capability to identify one ExternalUtranCell. When present, it shall contain one ExternalUtranCell DN. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| theAntennaList | This attribute contains the DNs of one or more AntennaFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theCellList | This attribute contains the DNs of EUtranGenericCell or UtranGenericCell if associations between them exist. This attribute contains the DNs of GSMCellPart if association between them exist. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theProxyBsList | A CommonBsFunction instance serves a number of ProxyBsFunction instances. This CommonBsFunction role-attribute contains a list of DNs of ENBFunction (3GPP TS 28.658 [13]), NodeBFunction (3GPP TS 28.652 [12]), BssFunction (3GPP TS 28.655 [14]) and GNBDUFUnction (3GPP TS 28.541[24]) that it serves. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theTMAList | This attribute contains the DNs of one or more TMAFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |

4.4.2	Constraints

None

Next Change

4.5	Common Notifications

4.5.1	Alarm notifications

4.5.1.1	Alarm notifications used in deployments using IRP framework

This subclause presents a list of notifications, defined in 3GPP TS 32.111-2 [11], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in 3GPP TS 32.302 [16], would capture the DN of an instance of an IOC defined in this IRP specification.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |

4.5.1.2	Alarm notifications used in deployments using SBMA

This clause presents a list of notifications, defined in TS 28.532 [Y], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | M | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

4.5.2	Configuration notifications

4.5.2.1	Configuration notifications used in deployments using IRP framework

This subclause presents a list of notifications, defined in 3GPP TS 32.662 [17], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in 3GPP TS 32.302[16], would capture the DN of an instance of an IOC defined in this IRP specification.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

4.5.2.2	Configuration notifications used in deployments using SBMA

This clause presents a list of notifications, defined in TS 28.532 [Y], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

End of Change