3GPP TSG-SA5 Meeting #141-e 	S5-221713

# e-meeting, 17 -26 January 2022

# Source:	Huawei, Alibaba

# Title:	Skeleton restructuring proposal

# Document for:	Approval

Agenda Item:	6.5.2

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	TS 28.824 Study on network slice management capability exposure v0.4.0

[2]	S5-221089 pCR 28.824 Skeleton restructuring proposal

3	Rationale

Based on the SA5#141e meeting discussion about the skeleton proposal in S5-221089 [2], to reach an agreed skeleton before the next SA5 meeting, it is proposed to change the 4.2. Key issues to 4.2 Issues in draft TS 28.824 [1] to avoid confusion with the existing use cases clauses.

4	Detailed proposal

This document proposes the following changes in TR 28.824 [1].

1st Change

4.2	Issues

4.2.1	Issue #1: Types of NSCs

The problem of network slice capability exposure is mostly relevant for B2B/B2B2C market, where Network Slice as a Service (NSaaS) [1] model applies. In this regard, different types of NSCs can be found.

- Baseline vertical customer: it corresponds to a NSC which is only interested in monitoring the network slice, to verify it behaves as expected, according to the SLA. This NSC does typically have no telco experience, and is associated to a network slice that is entirely deployed on a PLMN. The capabilities offered to a baseline vertical customer includes the ability for this NSC to receive information on subscribed items, including network slice status (e.g. active, inactive) and subscribed management data (e.g., KPIs, events/logs, trace data, etc.). The profile of this NSC type is a ‘passive NSC’.

- Advanced vertical customer: it corresponds to an NSC which requests (to the NSP) a dedicated network slice for the provision of PNI-NPN services. In this scenario, a portion of the network slice is deployed within the NSC premises (e.g., RAN, UPF) and the other portion (e.g., 5GC control plane functions) is hosted by one or more PLMN nodes. Unlike the baseline vertical customer, this new NSC does typically have (yet limited) telco knowledge, and wants to retain certain control over the allocated network slice. The capabilities offered to an advanced vertical customer might include (i) monitoring capabilities, i.e. the same capabilities offered to a baseline vertical customer and (ii) device configuration capabilities, i.e. provision of parameters for battery, mobility and communication patterns associated to the device, (iii) edge discovery/selection, e.g. in case the vertical wants to deploy workloads on the telco edge cloud.

- Hyperscaler: it corresponds to a NSC which requests (to the NSP) a dedicated network slice to provide a service-tailored connectivity pipe to a NSC’s customer. With some enterprises (i.e., NSC’s customer) starting to migrate workloads towards hyperscaler nodes, it is necessary for the hyperscaler (i.e., NSC) to provide SLA guarantees to these enterprises, especially for critical processes/services. However, the hyperscaler does not have network resources between its cloud nodes and customer premises, and therefore has to ask the mobile network operator (i.e., NSP) to set up a network slice between these endpoints. The capabilities offered to a hyperscaler might include (i) monitoring capabilities, i.e., the same capabilities offered to a baseline vertical customer; (ii) quality on demand, i.e. dynamic QoS and bandwidth management; (iii) policy control.

- Mobile (Virtual) Network Operator.

NOTE 1: In all the above cases, the NSP role is assumed to be played by an MNO.

NOTE 2: The capabilities mentioned above are neither exhaustive nor accurate, but examples to motivate the need for considering different NSC types.

The NSC types within the scope of FS_NSCE should be use case driven.

End of change