3GPP TSG-SA5 Meeting #143e	S5-223367

e-meeting 9th - 17th May 2022

# Source:	China Unicom

# Title:	Add candidates for study of feature management

# Document for:	Approval

Agenda Item:	6.5.10 Study on Management Aspects of URLLC

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.832 v0.1.0: “Management Aspects of URLLC”

3	Rationale

It was approval in SP-220146 to study the management aspect of URLLC and one of the objectives is to investigate  the potential configuration management requirements of URLLC features in RAN network. In order to achieve the objective mentioned above accurately and efficiently, based on the result of features classification we select some important features to form a set of candidates to be studied. And some references are also added.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains; Stage 1”

[3]	3GPP TS 22.261: “Service requirements for the 5G system; Stage 1”

[4]	3GPP TS 38.211: “NR; Physical channels and modulation”

[5]	3GPP TS 38.212: “NR; Multiplexing and channel coding”

[6]	3GPP TS 38.213: “NR; Physical layer procedures for control”

[7]	3GPP TS 38.214: “NR; Physical layer procedures for data”

[8]	3GPP TS 38.300: “NR; NR and NG-RAN Overall Description; Stage2”

[9]	3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification”

[10]	3GPP TS 38.323: “NR; Packet Data Convergence Protocol (PDCP) specification”

[11]	3GPP TR 38.824: “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”

2nd Change

5.1	Key Issue #1: Classification of URLLC related RAN features from management perspective

5.1.1	Description

URLLC is a set of service scenarios which require low-latency and high reliabile communications. Specific SLAs are defined for each scenario correspondingly. Different scenarios have different requirements, some of which focus on latency (e.g. Motion control) and some focus reliability (e.g. Discrete automation).

In order to satisfy the requirements of URLLC, many features have been defined by 3GPP RAN side to decrease latency and increase reliability to guarantee the SLAs of different URLLC service scenarios. These features with different functions and different effectives are distributed in different specifications, which bring complexity to the invocation and management when deploying URLLC service. Features related to URLLC need to be classified from the perspective of management, so that different features can be invoked according to different SLAs, and the management of URLLC-related features can be achieved.

5.1.2	Potential solutions

5.1.2.1	Classification of URLLC related RAN features

This document sorts out the features related to URLLC defined in RAN and classifies them from a management perspective. According to the characteristics of URLLC service, features are classified into the following two categories based on their effects : low latency and ultra reliability. Features belonging to low-latency category are mainly used to reduce data transmission delay, reliability features are mainly used to improve the reliability of transmission. Among the features belonging to low-latency category, some of them reduce the transmission latency from the effective mechanism and other features improve service priority to reduce URLLC latency in multi-service scenario.

| Feature | Category | Reference |
| --- | --- | --- |
| Mini-slot transmission | Low latency | TS 38.214 |
| Numerology/SCS | Low latency | TS 38.211 |
| UL configured grant | Low latency | TS 38.214 |
| DL SPS | Low latency | TS 38.213 |
| PDCCH monitoring | Low latency | TS 38.213 |
| Logical channel priority | Low latency | TS 38.321 |
| Short PUCCH | Low latency | TS 38.211 |
| UE processing capability#2 | Low latency | TS 38.214 |
| Span based PDCCH monitoring | Low latency | TS 38.212/TS 38.213 |
| UL configured grant enhancements | Low latency | TS 38.214 |
| DL SPS enhancements | Low latency | TS 38.213 |
| Sub-slot level HARQ-ACK (UCI enhancements) | Low latency | TS 38.213 |
| Two HAQR-ACK codebooks (UCI enhancements) | Low latency | TS 38.213 |
| Low SE MCS/CQI table | Ultra reliability | TS 38.214 |
| PDSCH repetitions | Ultra reliability | TS 38.214 |
| PUSCH repetitions | Ultra reliability | TS 38.214 |
| PUCCH repetitions | Ultra reliability | TS 38.213 |
| PDCCH aggregation level 16 | Ultra reliability | TS 38.213 |
| PDCP duplication | Ultra reliability | TS 38.323 |
| PUSCH repetitions enhancements | Ultra reliability | TS 38.214 |
| DCI format 0_2 and DCI format 1_2 | Ultra reliability | TS 38.213 |
| DL Multi-TRP for URLLC data channel repetitions | Ultra reliability | TS 38.213 |
| PDCP duplication enhancements | Ultra reliability | TS 38.323 |
| DL Preemption Indication (PI) | Low latency | TS 38.213 |
| Code Block Group (CBG) | Low latency | TS 38.214 |
| Inter UE: UL Cancellation Indication(CI) | Low latency | TS 38.213 |
| Inter UE: Power Boosting | Low latency | TS 38.213 |
| Intra UE: UL Prioritization | Low latency | TS 38.213 |

5.1.2.2	Candidates of features to be studied in management

There are nearly 30 features classified in the clause 5.1.2.1. The mechanisms of features aboves are various and the protocol layers where they effect on are different.   Different features have different effects and they also show different performance in improving the charateristics of URLLC service. The amout of RAN features realted to URLLC service is large. In order to ensure the accuracy and efficiency of the subsequent study, it is necessary to select some important features as candidates for the study of management. The potential results could be configuration parameters, feature switchs, performance measurements, etc. Based on the conclusions made in TR 38.824 and TS 38.300 together with the machenisms of features, the following features are selected as candidates for management investigation.

PDCCH enhancements

- UE processing capability#2

- Span based PDCCH monitoring

- DCI format 0_2 and DCI format 1_2

UCI enhancements

- Sub-slot level HARQ-ACK

- Two HAQR-ACK codebooks

PUSCH enhancements

- PUSCH repetitons/PUSCH repetitons enhancedment

- Mini-slot

PDSCH enhancements

- PDSCH repetitions

- Mini-slot

Inter-UE multiplexing

- UL Cancellation Indication

- UL Power Boosting

- DL Preemption Indication

Scheduling

- UL configured grant transmission

- DL SPS

- Low SE MCS/CQI table

Layer 2 enhancements

- Logical Channel Priority Restrictions

- PDCP Duplication

End of changes