3GPP TSG-SA5 Meeting #143-e 	S5-223111

# e-meeting, 9 - 17 May 2022

# Source:	Ericsson

# Title:	Solution for roaming charging profile

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Correcting solution 1.4 and adding a new solution where the roaming charging profile is provisioned in VPLMN.

# 4	Detailed proposal

First change

#### 7.1.4.4	Solution #1.4: Roaming Charging Profile exchanged between HPLMN and VPLMN

##### 7.1.4.4.1	General

A possible solution for REQ-CH_VMNO-05, Roaming Charging Profile negotiation enhancement for 5G data connectivity charging provided to the home MNO by the visited MNO.

##### 7.1.4.4.2	Reference architecture

The reference architecture would be the same as in clause 4.2.2 of TS 32.255 [4].

##### 7.1.4.4.3	Message flows

The message flows about roaming charging profile negotiation for 5G data connectivity charging are present in the figure 5.2.2.12.2.1, PDU session establishment, of TS 32.255 [4].

The “Roaming charging profile” may only be set or changed at PDU session establishment or PDU session transfer from a different VPLMN. It may at this point be set, changed, applied, and transferred in the following order:

1.	Default set by V-SMF and transferred to V-CHF

2.	Changed by V-CHF and transferred to V-SMF

3.	Transferred from V-SMF to H-SMF

4.	Transferred from H-SMF to H-CHF

5.	Changed by H-CHF and transferred to H-SMF

6.	Applied in H-SMF and transferred to V-SMF

7.	Applied in V-SMF and transferred to V-CHF

In case of roaming local breakout scenario, the H-SMF will be replaced by V-SMF, and would look something like:

1.	Default set by V-SMF and transferred to V-CHF

2.	Changed by V-CHF and transferred to V-SMF

3.	Transferred from V-SMF to H-CHF

4.	Changed by H-CHF and transferred to V-SMF

5.	Applied in V-SMF and transferred to V-CHF

Support for “Roaming changing profile” exchange is done by transferring it i.e., an NF may only change the “Roaming charging profile” if it has received it.

Second change

#### 7.1.4.x	Solution #1.x: Roaming Charging Profile provisioned in VPLMN

##### 7.1.4.x.1	General

A possible solution for REQ-CH_VMNO-05, Roaming Charging Profile negotiation enhancement for 5G data connectivity charging provided to the home MNO by the visited MNO.

##### 7.1.4.x.2	Reference architecture

The reference architecture would be the same as in clause 4.2.2 of TS 32.255 [4].

##### 7.1.4.x.3	Message flows

The negotiation of how the roaming charging profile should be configured is done as part of the roaming agreement between the VPLMN and HPLMN and can be pre-provisioned in the CHF or V-SMF and used based on the HPLMN of the UE.

End of changes