3GPP TSG-SA5 Meeting #143-e 	S5-223121

# e-meeting, 9 - 17 May 2022

# Source:	Ericsson

# Title:	Solution for issue 2d service to use between CHFs

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Two new solutions for issue 2d service to use between CHFs: reusing Nchf_ConvergedCharging service API between CHFs and new Nchf service API between CHFs.

# 4	Detailed proposal

First change

#### 7.1.4.x	Solution #2.x: Reusing Nchf_ConvergedCharging service API between CHFs

##### 7.1.4.x.1	General

A possible solution for key issue #2d, service based interface to use between visited CHF and home CHF, would be to reuse the Nchf_ConvergedCharging service API. This would mean that the V-CHF would proxy the request from the AMF, SMF or SMSF to the H-CHF, and the same with the response. The V-CHF could do some changes to message like: filter (e.g. trigger), enrich, or convert (e.g., rating groups).

##### 7.1.4.x.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

#### 7.1.4.y	Solution #2.y: New Nchf service API between CHFs

##### 7.1.4.y.1	General

A possible solution for key issue #2d, service based interface to use between visited CHF and home CHF, would be to create a new service API. This would mean that the V-CHF would translate the request from the AMF, SMF or SMSF to the new service API towards the H-CHF. This would mean that the message could look completely different and even the triggers in the V-CHF for sending the message towards the H-CHF wouldn’t have to be related to the request from AMF, SMF or SMSF.

##### 7.1.4.y.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

End of changes