3GPP TSG-SA5 Meeting #143-e 	S5-223365

# e-meeting, 9 - 17 May 2022

# Source:	China Unicom

# Title:	Add potential solution for performance measurements without PLMN ID at NRcellCU

# Document for:	Approval

Agenda Item:	6.5.13

1	1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.835 v0.1.0: “Management Aspects of 5G MOCN Network Sharing Phase2”

3	Rationale

It was approved to add potential solution for performance measurements without PLMN ID at NRcellCU. This contribution proposes to add OperatorCU and NROperatorCU IOCs to support performance measurements without PLMN ID for 5G MOCN network sharing. Some related modifies also are proposed in the existing NR NRM.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

### 5.Y.2	Potential solution

- The following enhanced NR NRM solution is proposed:

Introduce new OperatorCU <<IOC>> name-contained by GNBCUCPFunction<<IOC >> to represent operator specific gNB level information (including operator specific gNB Id, gNBIdlength, pLMNId), and new NROperatorCellCU<<IOC>> name-contained by OperatorCU<<IOC>> to represent operator specific cell level information (including operator specific CelllocalId, PLMNInfoList). Which means individual OperatorCU instance and NROperatorCellCU needs to be created and configured for each POP.

Figure 5.Y.1 NR NRM enhancement to suport NG-RAN sharing MOCN network sharing

The OperatorCU <<IOC>> contains attributes to support operator specific gNB-CU level information (including gNB Id, gNBIdLength, PLMNId) to support 5G Multi-Operator Core Network (5G MOCN) network sharing. Individual instance of OperatorCU <<IOC>> should be created and configured for each POP. When configured the attributes override those in parent GNBCUCPFunction instance.

The OperatorCU<<IOC>> is only used to support MOCN. If the MOCN is not supported, this IOC is not used.

The OperatorCU<<IOC>> includes attributes inherited from TOP IOC  and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| pLMNId | M | T | T | F | T |

NROperatorCellCU<<IOC>>

The NROperatorCellCU <<IOC>> contains attributes to support operator specific cell level information (including cellLocalId, pLMNInfoList) to support 5G Multi-Operator Core Network (5G MOCN) network sharing. Individual instance of NROperatorCellCU <<IOC>> should be created and configured for each POP. When configured the attributes override those in associated NRCellCU instance.

The NROperatorCellCU <<IOC>> is only used to support MOCN. If the MOCN is not supported, this IOC is not used.

The NROperatorCellCU <<IOC>> includes attributes inherited from TOP IOC  and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| pLMNInfoList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRCellCURef | M | T | T | F | T |

Some updates are needed for NRCellCU IOC, NRCellRelation IOC and ExternalNRCellCU IOC in the existing NR NRM.

- The Support Qualifier for attribute "cellLocalId " and "pLMNInfoList" in existing NRCellCU IOC needs to be changed to "CM", the condition is " If MOCN Network Sharing is configured the value is not used ".

- The define of exisiting NRCellRelation IOC needs to be added: In the MOCN networking sharing scenario, the NRCellCU instance of the target cell and source cell need to be changed an NROperatorCU instance.

- The define of exisiting ExternalNRCellCU IOC needs to be added: In the MOCN networking sharing scenario, ExternalNRCellCU IOC represents the properties of an NROperatorCellCU controlled by another Management Service Provider.

- Operator specific measurements can be collected and reported within OperatorCU<<IOC>> and NROperatorCellCU<<IOC>>, Operator agnostic measurements can be collected and reported within GNBCUCPFunction<<IOC>> and NRCellCU<<IOC>>.

- The following NR NRM fragments need to be added.

Figure 5.X.2 NRM for Eps to support <<IOC>>OperatorCU for NG-RAN MOCN network sharing

For the scenario of individual interfaces for NG-RAN MOCN network sharing, EP_XnC, EP_E1, EP_F1C, EF_NgC and EP-X2C name contained by OperatorCU can be used to represent individual interfaces for each POP.

End of Changes