3GPP TSG-SA5 Meeting #143-e 	S5-223384

e-meeting, 9 - 17 May 2022

# Source: 	Huawei

# Title:	pCR TR 28.908 Add AI/ML training workflow and deployment scenarios for AI-ML management

# Document for:	Approval

Agenda Item:	6.5.5.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

References

[1]		SP-211443 New Study on AI/ ML management

[2] 		3GPP TR 37.817 Study on enhancement for Data Collection for NR and EN-DC.

[3] 		3GPP TR 28.813 Study on new aspects of Energy Efficiency (EE) for 5G.

[4] 		3GPP TR 28.908 Study on Artificial Intelligence / Machine Learning (AI/ML) management

3	Rationale

The approved new SI [1] proposed to study the AI/ML management capabilities and management services to support/coordinate AI/ML in 5GS (3GPP management system, 5GC and NG-RAN). The AI/ML management workflow will helpful to illustrate the AI/ML management related functions and potential relationship.

In addition, one of the objective in SI [1] is investigation of deployment scenarios where the solutions are needed for AI/ML model training and each of the AI/ML model management capability. As discussed in RAN TR 38.817[2], OAM may need to be involved to support AI/ML capability in RAN. And as discussed in TR 28.813 [3], to provide centralized ES for RAN domain area, 3GPP management system need AI technology, so as to provide more efficient ES for RAN domain area while keeping basic KPIs stable for SLA assurance. Therefore, this contribution proposes to add AI/ML training deployment scenarios for AI-ML management.

This contribution proposes to add AI/ML management workflow and AI/ML training deployment scenarios.

4	Detailed proposal

It is proposed to add the following chapter in TR 28.908 [2].

1st modified section

X	Concepts and overview

X.Y	AI/ML management workflow

Figure X.Y-1 illustrates the generic workflow for AI/ML management, which involves AI/ML training function, AI/ML management function, and AI/ML inference function.

Figure X.Y-1: AI/ML management workflow

The AI/ML management related functions and potential relationship are described as following.

- AI/ML training function: AI/ML training function provides the capability that performs the AI/ML model training, validation, and testing. The AI/ML training includes initial model training and retraining. After the training is finished, the trained AI/ML model can be deployed/updated for inference.

- AI/ML inference function: AI/ML inference function provides the capability that generates the expected inference output based on the input inference data and a trained AI/ML model.

- AI/ML management function: AI/ML management function provides the capability of managing the entire process of using the AI/ML model, which includes:

- Model training control: the AI/ML management function can set the model retraining/updating period or trigger the model retraining/updating based on the performance evaluation results contained in the model performance feedback. For example, if the model performance decreases, the AI/ML performance management function can request the AI/ML training management function to start retraining.

- Model activation/deactivation/termination: the AI/ML management function can activate/deactivate/terminate the AI/ML model in the AI/ML inference function to control the inference capability of the AI/ML model;

X.Z	AI/ML deployment scenarios with management

This clause provides the deployment scenarios for AI/ML management, which addresses the above AI/ML related functions including AI/ML training function, AI/ML inference function and AI/ML management function.

As shown in the following figure, AI/ML cross domain training function, AI/ML cross domain inference function and AI/ML management function resides on 3GPP cross domain management layer coordinates with CN domain and RAN domain. Where AI/ML domain training function and AI/ML domain inference function resides on 3GPP domain management layer, AI/ML RAN training function, AI/ML RAN inference function, AI/ML CN training function and AI/ML CN inference function resides on Network function layer.

Figure X.Z-1: AI/ML Deployment scenario

The above figure shows the overview of AI/ML related functions. Depending on different scenarios, different logical functions may be used.

Example Deployment scenario1: In this deployment scenario which providing 3GPP domain management layer intelligence, the AI/ML Management function function is located in 3GPP cross domain layer to provide management function. The AI/ML domain training function and AI/ML domain inference function is located in 3GPP domain layer.

Figure X.Z-2: Example Deployment scenario1

Example Deployment scenario2: In this deployment scenario which to support the intelligence in RAN, the AI/ML Management function function is located in 3GPP cross domain layer to provide management function. The AI/ML domain training function is located in 3GPP domain layer to provide the AI/ML training function to network function layer. The AI/ML RAN inference function is located in Network function layer to provide inference function.

Figure X.Z-3: Example Deployment scenario2

End of modified section