3GPP TSG-SA5 Meeting #143-e 	S5-223423

# e-meeting, 9 - 17 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0072 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add granularity period examples |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | 2022-04-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel- |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The granularity period defines the measurement data production rate and is specified in 28.550. The examples provided with the description are 5 minutes, 15 minutes, 30 minutes, 1 hour, 12 hours or 24 hours or other values (see Note 1 below). The note describes that the supported rate depends on the capacitiy of the producer. It does not explicitly say that second range production rates can also be supported. The given examples do not reflect the current state of technology as other ranges 1, 2, 5, 10 and 60 seconds are also supported and should be added as examples. The granularity period is defined as Integer however no where it is stated that the granularity period should be expressed in seconds. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add second range examples to table 6.1.1.2 and clarify that granularityPeriod is expressed in seconds |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Developers may conclude that the standard does not support second range granularity periods and seek alternative more costly solutions |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on stage 3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 6.1	Measurement job control related operations

### 6.1.1	Operation createMeasurementJob (M)

#### 6.1.1.1	Definition

This operation supports the authorized consumer to request the measurement job control related service producer to create a measurement job.

One measurement job can collect the value of one or multiple measurement types. The measurement types are the performance measurements and assurance data defined in TS 28.552 [2].

When a measurement type is collected by one measurement job for a given instance (e.g., an NF instance), another measurement job creation request which wants to collect the same measurement type for the same instance with different granularity period may be rejected. This behaviour shall be consistent for a given implementation by a specific management service producer.

There are two different methods for the performance data to be reported:

-	Performance data file method: In this method the performance data is accumulated for certain time before it is reported; the data will be delivered as a file.

-	Performance data streaming method: In this method, the performance data streaming producer, when the performance data are ready, sends the performance data to the consumer (i.e., stream target). The volume of the performance data reported by streaming is expected to be small, and the Granularity period of the performance data stream needs to be configurable and is expected to be short.

#### 6.1.1.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| iOCName | M | The IOC name defined of the NRMs (e.g., as defined in TS 28.541 [3]), or the class name defined locally in the performance measurements specifications (e.g., TS 28.552 [2]). | It specifies one object class name. The consumer requests to collect one or more measurement type(s) of the instances of this class. |
| iOCInstanceList | M | List of DN | It specifies the list of DNs of object instances whose measurements of the corresponding type(s) are to be collected. An empty list means that for all instances (including the object instances existing at the time of measurement job creation, and the instances added later) known by the management service producer the measurements will be collected. |
| measurementCategoryList | M | List of measurement type names (see TS 28.552 [2]). | It specifies the measurement type(s) to be measured. The elements of the measurementCategoryList shall be one of the following forms: - The form "family.measurementName.subcounter" can be used in order to retrieve a specified subcounter of a measurement type. - The form "family.measurementName" can be used in order to retrieve a specific measurement type. In case the measurement type includes subcounters, all subcounters will be retrieved. - The form "family" can be used in order to retrieve all measurement types in this family. |
| reportingMethod | M | The reporting method of the collected performance data. | It specifies the method for the collected performance data to be reported. One of the following methods can be selected: - by performance data file - by performance data streaming (optional). |
| granularityPeriod | M | The period between generation of two successive measurements. | The management service producer will produce the value of the measurements at the end of each granularityPeriod. If the reportingMethod is performance data file reporting: - The value of granularityPeriod can be 1, 2, 5, 10 and 60 seconds, 2 minutes, 5 minutes, 15 minutes, 30 minutes, 1 hour, 12 hours or 24 hours or other values (see Note 1 below). The value of granularityPeriod is an integer value in seconds If the reportingMethod is performance data streaming: - The value of granularityPeriod is an integer value in seconds (see Note 1 below). |
| reportingPeriod | M | The period between two successive performance data reporting. | Applicable when the reportingMethod is performance data file reporting. The performance data report(s) are produced when the reporting period arrives. The reportingPeriod shall be one or multiple of granularityPeriod. The measurement value of each granularityPeriod will be made available to the performance data reporting related service producer, who will prepare the performance data file(s) for each reportingPeriod. If the consumer has subscribed to the notifyFileReady and notifyFilePreparationError notifications from the performance data reporting related service producer, the consumer will receive the notifications about the result of the performance data file preparation from that producer with the interval as defined by reportPeriod; |
| startTime | O | It specifies the begin time from which the measurement job will be active. | All values that indicate valid timestamp. Default value is "start now". If startTime is in the past, the current time will be used and the job will start immediately. When a measurement job becomes active, it does not mean that the measurement job immediately starts generation of the measurements of the given type(s). The consumer can set the detailed time frame (e.g. dailySchedule or weeklySchedule) by schedule parameter for a measurement job to generate the measurements. If there is no time frame scheduled, the measurement job immediately starts generation of the measurements when it becomes active. |
| stopTime | O | It specifies the end time after which the measurement job will be stopped. | The value indicates valid timestamp and shall be later than startTime and current time. This attribute may carry the value "indefinitely". Default value is to run indefinitely. |
| schedule | O | It specifies the detailed time frames (within the startTime and stopTime) during which the measurement job is active and monitors the measurement type(s). | Its value is only one of the following, dailyScheduling or weeklyScheduling. The legal values for them refer to ITU-T Recommendation X.721 [4]. The legal values for them are as follows. dailyScheduling: {{ intervalStart {hour 0, minute 0}, intervalEnd {hour 23, minute 59}}} weeklyScheduling: {{ daysOfWeek '1111111'B, intervalsOfDay dailyScheduling}} Default value is "daily". |
| streamTarget | M | It specifies the target of performance data streams carrying the performance data stream unit(s). | Applicable when the reportingMethod is performance data streaming. |
| priority | O | It specifies the priority of measurement job | Its value should be one of the following: Low, Medium, High Default value is “Medium” |
| reliability | O | It specifies the reliability of measurement job | Its value is vendor specific. See NOTE 2. |
| Note 1: The granularityPeriod defines the measurement data production rate. The supported rates are dependent on the capacity of the producer involved (e.g. the processing power of the producer, number of measurements being measured by the producer at the time, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported rates can only reflect the negotiated agreement between producer and the consumer involved. NOTE 2: meaning of “reliability” is not defined in the present document. |  |  |  |

#### 6.1.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| jobId | M | It identifies the measurement job instance (and distinguishes it from all other ongoing and stopped measurement job instances that have been created for the subject consumer). | Unique identifier of the measurement job from all the ongoing and stopped Measurement jobs that have been created for the subject consumer. |
| unsupportedList | M | List of < iOC instance, measurement type name, reason > | To create a measurement job, best-effort is required. The parameter of 'unsupportedList' has to be returned if status = PartialSuccess. The reason can be any of: - Measurement type name is unknown. - Measurement type name is invalid. - Measurement type name is not supported in the specific implementation. - Measurement type name is already monitored for the IOC instance with a different granularityPeriod. - The related IOC instance is unknown (e.g. it does not exist at the time of this operation invocation). - Insufficient capacity to monitor the related IOC instance(s). |
| status | M | ENUM (Success, Failure, PartialSuccess) | An operation may fail because of a specified or unspecified reason. |

#### 6.1.1.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| invalidStartTime | Condition: startTime is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| invalidStopTime | Condition: stopTime is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| invalidSchedule | Condition: schedule is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| invalidReportingMethod | Condition: reportingMethod is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| invalidGranularityPeriod | Condition: granularityPeriod is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| invalidReportingPeriod | Condition: reportingPeriod is invalid. Returned Information: Name of the exception; status is set to 'Failure'. |
| highWorkLoad | Condition: no sufficient capacity Returned Information: Name of the exception and the detailed reason which is one of: CpuBusy; DiskShortage, LowMemory, maxJobReached, otherReason; status is set to 'Failure'. |
| noValidMeasurementType | Condition: all measurement type names are invalid (i.e. none of the measurement type names are valid). Returned information: output parameter status is set to 'Failure'. |
| invalidPriority | Condition: priority is invalid. Returned Information: Name of the exception; status is set to ‘Failure'. |
| invalidReliability | Condition: reliability is invalid. Returned Information: Name of the exception; status is set to ‘Failure'. |

End of change