3GPP TSG-SA5 Meeting #144-e 	S5-224108

# e-meeting, 27 June – 1 July 2022

# Source:	Huawei

# Title:	Add use case for NF creation as cloud native VNF

# Document for:	Approval

Agenda Item:	6.8.5.1

1	Decision/action requested

For approval

2	References

[1]	3GPP TR 28.834 V0.1.0 Study on Management of Cloud Native Virtualized Network Functions

3	Rationale

TS 28.526 describes the VNF instantiation procedure based on release 3 ETSI NFV IFA specifications (e.g. IFA013 and IFA011), which do not support cloud native VNF lifecycle management. This contribution adds a new use case to describe the issue in the current SA5 spec when creation of NF as a cloud-native VNF.

4	Detailed proposal

In 1st change, it is proposed to add related references in clause 2 of TR 28.834 V0.1.0 [1].

In 2nd change, it is proposed to add a new use case (Use cases# X: NF creation as a cloud native VNF) in new clause 5.x of TR 28.834 V0.1.0 [1], which includes description and requirements.

1st change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[y]	3GPP TS 28.526: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions".

[z]	ETSI GS NFV-IFA013 V3.1.2 (2018-08) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[xx]	ETSI GS NFV-IFA013 V4.3.1 (2022-06) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[yy]	ETSI GS NFV-IFA011 V4.3.1 (2022-06) "Network Function Virtualization (NFV); Management and Orchestration; VNF Descriptor and Packaging Specification".

2nd change

5.X	Use cases# X: NF creation as a cloud native VNF

5.X.1	Description

TS 28.531 [x] describes the procedure of NF instance creation with interaction with ETSI NFV MANO, when a NF instance to be created contains virtualized part, the NFMS_P (Network Function Management Service Provider) invokes VNF lifecycle management with requirements for VNF instance as descried in clause 4.2.2.2 in TS 28.526 [y].

Clause 4.2.2.2 in TS 28.526 [y] describes the VNF instantiation as part of NS update initiated through Os-Ma-nfvo interface based on release 3 ETSI NFV IFA013 [z], which does not support cloud native VNF lifecycle management.

The Cloud-Native VNFs and Container Infrastructure management feature is supported in release 4 specifications of ETSI NFV

The major enhancement in release 4 specification of ETSI NFV-IFA013 [xx] to support cloud native VNF lifecycle management is the following:

1, it supports to use VNFD (and VNF package) which based on release 4 specification of ETSI NFV-IFA011 [yy] that supports modeling of VNF based on OS contains.

2, it supports managing container cluster network which will be used for establishing connection with VNF.

3, after a cloud native VNF has been created, the virtualized resouces which related to containers can be queried by OSS (i.e. 3GPP management system).

. In order to support lifecycle management of cloud native VNF, 3GPP management system shall use the release 4 specifications as defined by ETSI NFV to interact with ETSI NFV MANO (e.g., ETSI GS NFV-IFA013  V4.3.1 [xx]).

5.X.2		Requirements

REQ-NF-Creation-1: The 3GPP management system shall have a capability to interact with ETSI NFV MANO for NF creation as a cloud native VNF.

End of changes