3GPP TSG-SA5 Meeting #146 	S5-227022

Toulouse, France, 14 -18 November 2022

# Source:	Huawei

# Title:	pCR 28.865 Update solultion of service assurance for PLC control

# Document for:	Approval

Agenda Item:	6.9.5.4

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-211442: "New SID on deterministic communication service assurance"

[2]	S5-225676: "draft TR 28.865 Study on deterministic communication service assurance"; v0.3.1

[3]	3GPP TR 38.890:" study on NR QoE management and optimizations for diverse services "; v17.0.0

3	Rationale

This tdoc addresses the solution update of service assurance for PLC control.

It is proposed to add solution update of service assurance for PLC control in draft TR 28.865.

4	Detailed proposal

This document proposes the following changes in TR 28.865.

1st Change

5	Issues and potential solutions

Editor's note: this clause will contain the issues and potential solutions for deterministic communication service assurance. Relation and potential enhancements to eCOSLA will also be studied for the related issues.

5.Z	Issue #3: Service assurance for PLC control

Editor's note: this clause will contain the description and potential solutions for the service assurance of PLC control, e.g. service requirements, related measurements and service performance analysis. PLC control is one of the deterministic communication services in cyber-physical control scenarios described in TS 22.104.

5.Z.1	Description

Editor’s note: This clause provides a description of the issue#3.

5.Z.2	Potential solutions

5.Z.2.a	Potential solution #1: PLC control

5.Z.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution for issue#3 at a high-level.

In the deterministic communication service requirement analysis phase, network requirements (data rate, delay, and reliability) are analyzed. Network deployment requirements of different areas need to be considered for network preparation. Deterministic communication services have high SLS requirements. For example, the planning of PLC control services focus on E2E latency, interactive latency, PLC control period, burst packet loss ratio etc. It demands on high delay stability and periodic deterministic of small data packets. The network requirements need to be derived based on the application characteristics of specific services. For example, network preparation information, such as network capacity, coverage, reliability, and data rate estimation, needs to be output based on the three-layer service requirement model of the PLC control service.

5.Z.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made for issue#3.

1. Data collection:

Collect service experience data and network performance data of PLC control services based on the three-layer service requirement modeling. For example, the following data may collected: latency and throughput related performance data defined in TS 28.552 and TS 28.554. Abnormal information such as large round-trip delay, large interactive delay, large jitter, low data rate distribution, high packet loss rate, and service quality deterioration etc.

2. Analytics and demarcation

Service and network analysis of deterministic communication services includes monitoring, poor-QoE identification, demarcation, and root cause analysis. Identify abnormal issues, determine the impact scope, and restore services.

Monitoring and the poor-QoE identification requires two steps:

- Builds the three-layer service requirement model based on service characteristics, including poor experience information.

- Collects experience, service quality, and network-related indicators and abnormal incidents to identify poor-QoE users and services.

Fault demarcation is used to generate service interruption/ poor experience incident when a service anomaly occurs. The E2E network KPI, alarm information, terminal exceptions are correlated and analzed. Demarcation results for video monitoring services are provided accordingly.

3. Optimization and verification

Optimization:

For PLC control services, delay-related optimization are mainly considered. To optimize delay-related problems, it is needed to analyze the average delay, delay jitter, and delay reliability. It can analyzed the delay-related factors of the air interface, such as the uplink and downlink slot assignment proportion, air interface resource capacity, and scheduling policy.

Verification:

According to the service assurance objective of deterministic communication, that is, the quality of experience indicator of each service meets the deterministic requirement after optimization, it is further verified that the network capability meets the deterministic requirement. Select indicators that reflect PLC control service experience, such as PLC control period, survival time, interactive delay, interruption etc, or indicators that reflect network performance, such as packet loss rate, rate, and delay.

To support the analysis and optimization of PLC control services, some data collection requirements are as in the following table:

| Number | measurements | Data collection points |
| --- | --- | --- |
| 1 | RTT: the average RTT in minute’s level based on the time of the Job, Ack_Data, and Ack messages of the S7Comm protocol. | UE PDCP or UPF |
| 2 | PLC response rate: the ratio of Ack_Data to the number of Job messages in minute’s level. | UE PDCP or UPF |

To support the analysis and optimization of PLC control services, some data collection requirements are as in the following table:

| Number | measurements | Data collection points |
| --- | --- | --- |
| 1 | PLC client interruption: This event is triggered when the job request from the PLC controller was not received within a detection period. | client, reported to the UE |
| 2 | PLC server interruption: This event is triggered when a Job request was sent from the PLC control end but an Ack_Data response was not received from the PLC execution end within a detection period. | Server, reported to the AF |
| 3 | High packet loss rate on the server: This event is triggered when the packet loss rate of the Ack_Data response corresponding to a job request is greater than the packet loss rate threshold within a detection period. | Server, reported to the AF |
| 4 | Excessive RTT: This event is triggered when the number of RTTs greater than the threshold within a detection period is greater than the threshold. | UE PDCP or UPF |

Note: UE data collection could be implemented through existing MDT or QoE data report or the related enhancements. It may impact RAN2 and RAN3 specifications and cooperations may be needed.

Some similar performance measurements are currently available in TS 28.552, e.g. packet drop rate, average packet delay, IP latency measurements etc. They are defined as average value in PDU session granularity. For deterministic communication services, some more stringent measurements method and thresholds constraints need to be defined. Because the stability and deadline of latency are defined as data packet level from end to end perspective according to TS 22.104. They should be based on data packet granularity instead of average value for the PDU session level. For some of the measurements, application layer information may be needed, some use cases have been studied in RAN3 and can be referrenced. How to evaluate and mitigate the abnormal issues which have not met the deterministic communication service requirements should be further studied.

5.Z.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work for issue#3.

End of change