3GPP TSG-SA5 Meeting #147 	S5-232637

Athens, Greece, 27th February - 3rd March 2023

# Source:	ZTE Corporation

# Title:	pCR TR 28.835 Add recommendation for KI#2

# Document for:	Approval

Agenda Item:	6.8.6

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.835 v0.5.0: “Management Aspects of 5G MOCN Network Sharing Phase2”

3	Rationale

This contribution proposes to add conclusion and recommendation for Performance measurements without PLMN ID at NRCellCU.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

## 6.2	Issue #2:   Performance measurements without PLMN ID at NRCellCU

The study has identified the issue and solutions for NRCellCU to collect performance measurements without PLMN ID. There are two solutions to address key issue#2 in the present TR. These solutions propose the principle that the new or existing IOCs are able to support the collection of these common and operator specific measurements.

-	Solution 1 proposes that new IOCs (i.e. OperatorCU, NROperatorCellCU) could be added to the NR NRM to support NG-RAN sharing MOCN network sharing.  The OperatorCU and NROperatorCellCU to represent the operator specific part can collect PLMN related measurements. The NRCellCU to represent the common part of the CU can collect these common measurements (i.e. RRC.ConnEstabAtt and RRC.ReEstabAtt).

-	Solution 2 proposes that NRCellDU is added as the measurement object class to the attempt and failure measurements of RRC connection related measurements.

The comparison of solutions is shown in the following table.

Table 6.2-1: Comparison of solutions of Issue#2

| Dimensions | Solution 1 | Solution 2 |
| --- | --- | --- |
| The impacts of technical specifications | TS 28.541 Add two new IOCs Higher impact on the NR NRM to support NG-RAN sharing MOCN network sharing. | TS 28.552 Higher impact on the measurement object class of existing performance measurements (non-PLMN measurements) |
| ROI | Complex for collecting non-PLMN measurements, causing big migration of configurations | Simple solution for collecting non-PLMN measurements |

For key issue #2, it is recommended that solution 1 and solution 2 can be used as informative Implementer's Guide.

End of Change