# 3GPP TSG-SA5 Meeting #148e	S5-233234rev5

# Electronic meeting, Online, 17 -25 April 2023

# Source:	Samsung, EUTC, EDF, Deutsche Telekom, BMWK, NOVAMINT

# Title:	Rel-18 pCR 28.829 – Clean up

# Document for:	Approval

Agenda Item:	6.9.3.0

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1]	3GPP TR 22.867: "Study on 5G smart energy and infrastructure

[2]            3GPP TR 28.829: "Study on Network and Service Operations for Energy Utilities"

[3]            S5-216428 : New SID on Network and Service Operations for Energy Utilities

# 3	Rationale

changes in rev2

- 6.3.5 section break was not clear [Rapporteur]

- additional text in definitions [for a comment by Huawei]

- clean up the text around the new figure 6.9.2.2 [Nokia, Huawei]

- remove the introduction [Mirko]

- moved 6.1, 6.2, 6.3, 6.4, 6.6 and 6.7 into a new annex C, now C.2, C.3, C.4, C.5, C.6 and C.7 respectivelyl [Nokia]

- renumbered clauses 6.5 to 6.1, 6.8 to 6.2 and 6.8 to 6.3. Fixed references in the TR to align with these new clause numbers.

- added a table for information elements and

- removed capital letters in clause titles

changes in rev3

- adjusted the wording of the TR to use the term 'the present document' instead of this TR, this study, this specification, this report... [Mirko]

- added references and further information to the tables in 6.5.3 to answer Nokia's comment in S5-233366: Proposal #2 states that KPIs need a point of reference where the data can be observed (response: the base station), management object (existing with reference, or to be defined)

- added the missing clauses in the complete clean up of TR 28.829 in the pCR below that were not in rev2. Changes include:

- checked and corrected all problems with references (all now are needed, none are duplicated, all are implemented properly)

- corrected references to the clauses moved to Annex C throughout the TR

- removed all mention of requirements in 6.4 (now C.5) which are not fulfilled by solutions in the TR nor mentioned in the conclusions.

rev4

- updated and improved content in the new tables in 6.1.3

rev5

- another careful pass through the TR to remove incorrect formatting

- corrected incorrect section numbering and references (after TR restructuring with annex C)

- added a missing row to a table in 6.1

- added Mirko's propose changes

- scope rewording

- clean up of references

- clean up of definitions

- clean up of overview/general

- correction of text refering to the present specification and to others corrected throughout

- moved table footers to table headers consistently

This document cleans up TR 28.829. In addition to some editorial clean up, the following remaining editor's notes are resolved.

| Clause | Editor's Note | Reason for removal | Changes |
| --- | --- | --- | --- |
| 3.1 | site operator: In some network sharing scenarios, for some particularly base station(s) and/or cell site(s), there exist business entities who operate this infrastructure on behalf of MNOs. Editor's Note: This term is to be clarified. | There was no clarification proposed to be added. If some clarification is identified at SA5 148e, it will be added. | EN is removed. |
| 6.9.2 | Use case service flow Preconditions: There is a feeder, whose operation requires smart energy services. The smart energy services are available through DSO equipment RTUs. These RTUs are connected with a router, a CPE in the DSO site, that supports a mobile telecommunications interface, a UE. The DSO can obtain information from each UE in the DSO network. The DSO-MS is aware of the Base station ID of the serving base station for each UE. Editor's Note: Additional clarification of the preceding two paragraphs will be added. | The two paragraphs lack of clarity is resolved. | The two paragraphs are reworded and a figure is added. The EN is removed. |
| 6.9.3 | PR 6.9.3-1. The 3GPP management system should expose management services, subject to operator policy and other conditions (see NOTE 4), to enable the DSO to provide the site operator with information concerning the expected restoration time of its distribution services for site operator for effected sites. Editor's Note: Further detail may be needed for the preceding requirement. | No additional information is needed. If some additional information is identified during SA5 148e, it will be added. | EN is removed. |
| 7.3 | ## 7.3 Key Issue #3: Energy utility and telecommunication coordinated recovery of energy service Editor's Note: Key issue #3 corresponds to a solution provided in S5-23zzzz | The corresponding solution was not agreed at SA5 147, but is propopsed to be added in SA5 148e (please see S5-233zzz). | EN is removed. |
| 7.3.1.3 | The functions described here correspond to the requirements in the use case 6.9 "Business use case: Rapid Intervention for Outages without Redundant Topology". Editor's Note: A new use case is proposed separately. This new use case includes the requirements referred to below. | The EN is not needed because the corresponding text is already in TR 28.829, 1.0.0, clause 6.9. | EN is removed. |

# 4	Detailed proposal

It is proposed to make the following changes in TR 28.829, 1.0.0.

Start of Changes

Next Change

# 1	Scope

The present document  considers service requirements introduced in TS 22.104 [2] and TS 22.261 [3] related to telecom management.  The present document identifies use cases and requirements for exposing capabilities of the 3GPP management system to external energy utility service providers. The present document further considers how management capabilities or what information can be provided to mobile network operators by the external energy utility service providers.  Both energy utility use cases and requirements are considered. TR 28.824 [4] can be considered for the technical investigation.

Next Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	3GPP TS 22.261: "Service requirements for the 5G system".

[4]	3GPP TR 28.824: " Study on network slice management capability exposure"

[5]	IT Process Wiki – The ITIL Wiki:. https://wiki.en.it-processmaps.com/index.php/ITIL_Service_Operation  Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 08.12.21.

[6]	3GPP TR 22.867: "Study on 5G smart energy and infrastructure"

[7]	Connected Nations 2020, UK Report, Ofcom. https://www.ofcom.org.uk/__data/assets/pdf_file/0024/209373/connected-nations-2020.pdf Access 20.4.22.

[8]	Telecom Services Security Incidents 2019 Annual Analysis Report, ENISA European Agency for Cybersecurity, July 23, 2020. https://www.enisa.europa.eu/publications/annual-report-telecom-security-incidents-2019

NOTE: 	This publication is intended for information purposes only and is accessible free of charge. Reproduction is authorised provided the source is acknowledged. Access 20.4.22.

[9]	DIRECTIVE (EU) 2019/ 944 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL - of 5 June 2019 - on common rules for the internal market for electricity and amending Directive 2012/ 27/ EU (europa.eu)
https://eur-lex.europa.eu/legal-content/EN/TXT/PDF/?uri=CELEX:32019L0944&from=EN

[10]	IEC TC 57 https://www.iec.ch/ords/f?p=103:7:511571509228708::::FSP_ORG_ID,FSP_LANG_ID:1273,25

[11]	3GPP TS 32.130: " Telecommunication management;Network sharing; Concepts and requirements".

[12]                       3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[13]                       3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[14]	3GPP TS 22.104, " Service requirements for cyber-physical control applications in vertical domains ".

[15]	3GPP TS 32.404, " Performance Management (PM); Performance measurements; Definitions and template".

[18]	CAMARA: Telco Global API Alliance <https://www.gsma.com/futurenetworks/ip_services/understanding-5g/camara-telco-global-api-alliance/>, accessed 16.02.23.

[m]	3GPP TS 28.530, "Management and orchestration; Concepts, use cases and requirements".

Next Change

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Distribution System Operator: a natural or legal person who is responsible for operating, ensuring the maintenance of and, if necessary, developing the distribution system in a given area and, where applicable, its interconnections with other systems, and for ensuring the long-term ability of the system to meet reasonable demands for the distribution of electricity.

NOTE 1:	Seeee Article 2, definitions in DIRECTIVE (EU) 2019/ 944 [9].

SCADA: Supervisory Control and Data Acquisition.

NOTE 2:	The management system of power grid is standardized by IEC TC 57, see Dashboard, scope [10].

Distribution Automation: A family of technologies, systems and processes (including sensors, actuators, processors, communication networks, switches, etc.) that enable the remote, real-time monitoring, operation, and optimization of utility distribution systems on the field.

Remote Terminal Unit: a host in a customer network operated entirely out of the scope of 3GPP standardization.

Uninterruptable Power Supply: an independent source of energy that, for a limited time duration, can sustain operations normally despite an interruption of energy distribution services.

Customer Premises Equipment: a component of communications infrastructure that is installed in the facility owned and operated by a customer.

Energy Supply ID: This is the point where the energy supply terminates in the operator site and has a unique ID that is known by both the MNO and DSO.

site operator: A business entity who operate infrastructure on behalf of MNOs, e.g. in some network sharing scenarios, for base station(s) and/or cell site(s).

NOTE 3:	This entity supports telecommunications operations and management, e.g. in network sharing scenarios. The site operator is a Master Operator (MOP) as defined in 32.130. [11]

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DSO	Distribution System Operator

SCADA	Supervisory Control And Data Acquisition

DA	Distribution Automation

CPE	Customer Premises Equipment

RTU	Remote Terminal Unit

UPS	Uninterruptable Power Supply

# 4	Overview

# 4.1	General

The delivery of energy has to occur with extreme levels of availability. Reliability is crucial in this domain, for regulatory, business and public health and safety requirements. To achieve this reliability, a range of ‘smart energy services’ are employed by the energy system. These services, largely standardized by IEEE and IEC, require communication. As greater degrees of efficieny, resiliency, responsiveness and other capabilities are sought in the generation and delivery of energy, more and more communications services are required by the energy sector.

There are many options for delivery of communication services today – power line communications, fiber optics, fixed networks, microwave transmission, satellite communications and mobile telecommunications. The appropriate means, or rather mix of communications services, depends on several factors including the location, the possibility of leveraging existing assets (e.g. power lines,) and the ‘total cost of operations,’ and the properties of the communication service.

To the extent that the 3GPP system can provide services that meet the needs of the energy sector, telecommunications will be an increasingly important part of the technical ecosystem by which energy is delivered.

The stage 1 feature ‘Smart Energy Infrastructure’ includes service requirements that will be considered further, in detail, in the present document. In particular, TR 22.867 [6] identified requirements for specific standardized capabilities that allow a utility operator to obtain information from an operator’s network, and to share information with the operator. This information all serves to improve the realized availability of energy system services.

The energy utility service provider needs information regarding outages and performance degradation of the communication system, as it may be possible for the energy utility service provider to reactively or even proactively establish and use an alternative means of communication. Changes in the configuration of the network may also impact the energy utility service provider. Finally, the energy utility service provider can share information with the MNO in order to facilitate rapid diagnosis and recovery from performance problems and energy supply interruptions.

Another reason that energy utilities are an important sector for 3GPP is that telecommunications network operations themselves require energy. The relationship is bi-directional: MNOs require energy services, and energy utilities require communication services. This demands a particular risk management be undertaken by both systems, especially in the event of an energy outage. The present document will consider how energy utility service providers and MNOs can exchange information in a standardized format related to an energy service interruption and how to resolve such energy service interruptions efficiently.

# 5	Concepts

## 5.1	Energy Service is Critical for Telecommunications Service Availability

It is important to note that energy outages are one of the principal factors that in telecommunication service availability failures.

Ofcom analyzed service outages between September 2019 and August 2020 in the UK and found that power cuts were the #2 most common causes of telecommunications failure incidents during that period. [7]

ENISA analyzed European data and found that in 2019, the second most common cause of telecommunications service failure were power cuts. " Power cuts are the second most common detailed cause: Overall, independent from the underlying root cause, power cuts are either a primary or a secondary cause in over a fifth of the major incidents." [8]

Governments in each country settle requirements for reporting of failure of power grid. In some cases, the the consumer can claim economic compensations due to caused damage.

As a common principle in all countries worldwide, a state autority is responsible to report failures on a yearly basis in the power grid but deviations do exist.

The telecom network can be impacted by external factors, specifically by the power grid.  If there is a failure in the grid, then the faulty part needs to be disconnected to avoid risk for fire or human injuries due to short circuit. Further, the telecom network requires electricity and has a limited capacity to operate without energy from the power grid.

The telecom network can be impacted due to internal power system failure at the site. This can be a short circuit or damaged batteries.  This is part of telecom operator’s incident management or fault management system and out of scope of this study.

The startup of electrical power grid can be cumbersome. As common a top-down approach is performed in centralized energy distribution system. However, in future a bottom-up approach may be the best alternative, since some of the micro grids may still work to the extent that they can operate autonomously. Detailed assessment of micro-grids is outside the scope of this study.

Each country sets their own requirements and regulations for the energy sector and telecom sector operating in that country. It is common for there to be the following regulation:

-	Telecom act is controlling the telecom regulation. This is setting requirement for how the telecom service in the country is handling. This can include reliable operation.

-	Energy act is controlling the energy regulation

Disaster act is giving guidance when major problem is impacting the countries.Network Operators do have backup system installed at site since they are operating a mobile network 24/7. There can be external power failure and then the backup system can have internal problem or limited capacity.

Next Change 6 was unchanged, but most of 6.x clauses were moved to the new annex C these changes are captured at the end of this pCR

# 6	Business use cases and potential requirements

Next Change

## 6.1	Business use case: MNO exposes network performance monitoring

### 6.1.1	Description

Motivation

In C.2, it was explained that DSOs require extremely high availability for communication to provide distribution automation and SCADA services to prevent energy service outages. Unlike the use case C.5 MNO exposes Network Service Alarm, this use case focusses on proactive measures, based on information obtained through reports. The motivation for this is described in C.2.1.

The sooner and in sufficient detail that the DSO obtains information regarding communication service deterioration, the better. This deterioration may be considered a 'problem.' (The term 'problem' is used in the sense described in Annex A, a reduction in service metrics. This is distinguished from an 'incident' in which required service levels cannot be maintained.)

As described in C.2, a DSO can determine levels of service over time by means of their own infrastructure. The DSO has many routers in their network. These provide networking within substation networks and have wireless access interfaces to connect the substation network over a wide area. These routers perform periodic monitoring operations, e.g. sending ICMP echo (ping) messages to ascertain latency and reachability. In addition, the UE has access to radio and cellular information - signal strength, serviced Cell ID, radio technology. These measurements are captured on the UE and obtained 'over the top' by the DSO using their own management system over time. The acquired data are assessed to discern trends that, historically considered, indicate that an incident is likely. This monitoring occurs at a coarse granularity (e.g. one measurement per minute.) There are two shortcomings to this approach that this use case seeks to overcome:

1. 	The information is based on measurements of single nodes only, not the overall network. The DSO knows (a) the location of each of their devices and the serving Cell ID. (b) The DSO has several devices (in the same cell) and can by means of correlation of data received by devices identify possible problems that affect the entire cell. (c) the nature of the deterioration of performance remains ambiguous - is it an issue in the DSO's own network (essentially a managed set of VLANs and substation networks), or is it a problem in the MNO's network?

2. 	The granularity of the measurement is coarse, the bandwidth requirements to control and collect the data significant compared to the service data traffic (when there is no need for more than routine monitoring and management of the energy system) and the measurements are distributed - requiring connectivity to all UEs. To the extent the performance deteriorates, so too does the access of the DSO to the UEs that provide measurements. So, as an incident approaches, just when more information granularity is needed, it becomes increasingly difficult to acquire data.

These two problems have one clear solution: centralized information obtained from the MNO instead of decentralized information acquisition. The exposed information from the MNO will correspond to the network performance absolutely, it need not be approximated. It will assist in determining the cause of performance problems - if the MNO does not report the problem but it is detected in the DSO network, this indicates that the problem is in the DSO network. If the MNO reports performance indicating a problem, then the DSO can focus on this rather than on investigating the root cause of the problem in their own network. The centralized measurements will be more efficient, can be of finer granularity, will be available even if the network performance seriously declines.

The acquired data is used, as described in C.2, to determine when to initiate back up communication capabilities. These take some time (e.g. 2 minutes) to activate. Accurate, timely, sufficiently granular information can lead to better historical information, which correlates diverse behaviors of the network including service performance incidents. This can lead to improved understanding of service, such as periodic changes in service levels and on the other hand developments that have historically been associated with service level incidents.

In the latter, rare case, the DSO may elect to take proactive decisions. These proactive decisions will improve communication service availability. It is essential to maintain the best possible communication service availability to avoid even brief intervals of lack of availability of communication. If there is a specific need to monitor and manage the network during one of these intervals, it will not be possible to do so (i.e. by means of Distribution Automation or SCADA smart energy services), and this could lead to damage or an outage affecting energy service customers.

Background

See use case C.2.

### 6.1.2	Details

Use Case Actors

DSO network operations center engineer:	The DSO network operations center engineer is responsible for deploying monitoring and control mechanisms in the network. The DSO network operations center engineer determines how to control and configure the network for resiliency, e.g. when and how to switch between different accesses to maximize availability.

DSO electrical system operations center engineer: This actor is responsible for maintaining availability, efficiency and safety of the energy system.

Use case

Service flow:

1. The energy system is monitored and managed by the DSO electrical system operations center engineer who relies upon a functioning network to communicate with electrical utility equipment in substation networks.

2. There are a set of substation networks with routers that include a UE for wide area communications. The DSO network operations center engineer is responsible for this network. The DSO network operations center engineer employs monitoring mechanisms to observe the performance of the telecommunications network over time, to capture historic data and continually watch for performance degradation indicating risk of an incident, as described in C.2.1. These monitoring mechanisms are exposed by the MNO, so that authorized third parties (including the DSO) are able to receive performance monitoring information.

NOTE: 	The reports are provided at the network level, based on the overall statistical performance of the network. The reports described here do not correspond to the performance of individual UEs or sessions.

3. The DSO network operations center engineer, using the exposed monitoring mechanism interface, can request which metrics, under which conditions, will be provided. The interface provides a means by which the reports can be configured, including frequency and parameters to report.

Table 6.1.2-1: Report Parameters

| Parameter | Configuration | Expected Behavior | Notes |
| --- | --- | --- | --- |
| latency | location (cell ID), frequency of reporting, granularity of reporting. | The report is provided by network exposure by the MNO. | The time of the measurement is crucial, so this information is included in the report delivered to the DSO. |
| throughput | location (cell ID), frequency of reporting, granularity of reporting. | The report is provided by network exposure by the MNO. | As above. |
| packet loss | location (cell ID), frequency of reporting, granularity of reporting. | The report is provided by network exposure by the MNO. | As above. |
| availability | location (cell ID) | The report is provided by the network exposure by the MNO. | As above. |

The information that are needed by the DSO will change over time, thus it is important that the DSO can configure the monitoring process, especially in terms of granularity.

4. The reports are delivered according to the above configuration.

Service flow result:

When a problem develops, the DSO network operations center engineer (or an automated management function) may trigger a fail-over to a backup network communications facility. This proactive and timely intervention can reduce or even eliminate the occurrence of incidents. Even if the activation of a back up communication facility occurs too late to entirely eliminate interval in which the DSO substation network components are unreachable or only reachable with inadequate quality of service, this interval will be significantly reduced.

### 6.1.3	Potential Requirements

PR 6.1.3-1. The 3GPP management system shall, according to mobile network operator policy, regulatory requirements and contractual obligations, expose standardized interfaces to authorized third parties that provide the ability to initiate and terminate requests for monitoring including the configuration of the monitoring (e.g. monitoring interval, measurement period granularity, location of interest, etc.)

Table 6.1.3-1: Information Elements and Management Functionality Mapping (see 7.1.2)

| Purpose | Applied | Existing Management functionality or New Management Functionality |
| --- | --- | --- |
| Determining monitoring configuration (monitoring interval, measurement period granularity) | Monitoring configuration is general and applied to any monitoring done to satisfy this requirement. | Existing: ThresholdMonitor IOC, monitorGranularityPeriod attribute of the ThresholdMonitor IOC |
| Determining location of interest (Lat/long, TAC, cell ID) | Monitoring location of interest will be used to scope the object instance to be monitored | Existing: targetThresholdlocation in ThresholdMonitor IOC |

PR 6.1.3-2. The 3GPP management system shall, according to mobile network operator policy, regulatory requirements and contractual obligations, expose standardized interfaces to authorized third parties that provide a mechanism for the mobile network operator to send reports containing  required performance metrics measurements to the DSO. The measurements in these reports are provided in a form such that it will be possible to ascertain the number of measurements made as well as to calculate the standard deviation of those measurements, in order to aid in the interpretation of the reported measurement.

Table 6.1.3-2: Information Elements and Management Functionality Mapping (see 7.1.2)

| Purpose | Applied | Existing Management functionality or New Management Functionality |
| --- | --- | --- |
| Communicating Measurements | For any monitoring information provided by the MNO to the DSO. | Existing: Measurement [15] |

PR 6.1.3-3. The 3GPP management system shall support the following performance metrics to monitor information according to the associated configuration:

a) Latency between the UE and the base station, base station or network slice;

b) Throughput [an average for the base station's network traffic or network slice];

c) Packet loss [an average for the base station's network traffic or network slice];

d) Availability [an average for the cell's availability at a specific base station or at a (RAN) network level].

Table 6.1.3-2: Information Elements and Management Functionality Mapping (see 7.1.3)

| Purpose | Applied | Existing Management functionality or New Management Functionality |
| --- | --- | --- |
| Latency | Over Uu, for traffic between the UE and the base station (averaged); either for all traffic or for the network slice traffic. | Existing: As defined in 5.1.1.1.1 and 5.1.1.1.3 in TS 28.552 |
| Throughput | As above | Existing: As defined in 5.1.1.3.1 and 5.1.1.3.3 in TS 28.552 |
| Packet Loss | As above | Existing: As defined in 4.2.1.5.1 in TS 38.314 for DL and in 5.1.3.1.1 in TS 28.552 for UL. |
| Availability | Availability (average) of cells. | New: To be defined (based on a cell's availability at the base station) |

Next Change The change to 6.8 was added to S5-233234d2 to address comment

## 6.2	Business use case: Energy utility and telecommunication coordinated recovery of energy service

### 6.2.1	Description

There is clearly a mutual interest in coordination between telecommunications and energy service operations to achieve rapid recovery of energy service. This is not only true for the MNO, who benefits from the availability of supplied power, but also of the DSO who needs mobile telecommunication service to restore and maintain the operation of its grid.

Energy service interruptions may occur in many parts of the energy system, ultimately effecting the end customer. This use case does not consider every service interruption scenario. It rather concentrates on one specific level of the system. The remainder of the task of efficient recovery is out of scope of this use case. Please consider the following model:

Figure 6.2.1-1: A simple model of the energy service delivery system

We concentrate on the distribution aspects, which is depicted above as a single level in a hierarchy, but in fact may have a series of sub-stations between high voltage transmission and the final distribution to energy consumers.

Interruptions occur in the distribution system, as at some level, the medium voltage network may rquire local reconfigurations or suffer unplanned loss of distribution service (e.g. due to distribution cable damage, etc.) The resulting energy service interruptions last a variable amount of time (from minutes to hours.)

The normal situation is that a utility may need to disconnect a certain Medium Voltage feeder. As the grid does not automatically re-connect this feeder to an active one, there will be a power outage in all points of supply connected to it while the power is being restored. The procedures and times to reconnect the affected feeder would be the same if for any reason this has been caused by a planned operation, an unexpected incident, or if it is a major grid problem.

In the recovery procedures, the DSO network operations center needs to restore power in a certain order often for regulatory compliance: the order includes prioritizing the more important energy consumers (e.g., Hospitals, government sites).

This prioritization may also include major MNOs’ sites, such as base stations and core network sites to restore communication services, which is in the interest of both MNO and DSO stakeholders. This prioritized recovery of MNO sites is enabled by this use case.

It is here where a standardized mechanism connecting DSOs and MNOs would be beneficial. This use case describes the information and operations that would be required on that interface.

Part of the information may have a more static nature.

This is the point of Energy Supply ID. The DSO is aware of each point of supply (to an energy consumer), by means of the Energy Supply ID. The MNO needs to let the DSO know the relevant Energy Supply IDs, so that the utility can know where they connect to its feeders. Another critical information element is the power back up installed by the MNO in each site (including 'back up' base stations), and its expected duration. In addition, for each Energy Supply ID, base station, e.g. eNB, IDs (serving the cell IDs) served by the base station as shown in figure 6.2.1-2 below.

Figure 6.2.1-2: Local recovery scenario

NOTE: 	The representation of the ID above corresponds to the Energy Supply termination of the base station site.

The correspondence between the communication system serving the distribution substation by means of a UE (camped in the depicted cell), the Energy Supply ID of the base station and the ID of the base station enables the DSO to identify important operational aspects, such as:

(a) the impact of the distribution substation interrupting service: specifically the base station that the distribution substation relies on will have its power supply interrupted;

(b) upon recovery of the distribution substation, which substations may be high priority for resumption of service.

As implied in Figure 6.2.1-2, there is a dependency that the DSO has on the MNO in that the DSO relies upon data connectivity to smart energy equipment in the distribution system. Through the use of smart energy services, restoration of energy service is rapid and reliable. If the energy substation lacks data connectivity, it is sometimes necessary to send a technician to the site to restore service, which is time consuming.

Within the distribution substation is a router, a kind of customer premises equipment (CPE) that communicates as a router that supports a mobile interface, shown as the UE. Beyond the CPEs DSO Remote Terminal Units (RTUs) are connected. The DSO monitors and controls the RTUs as part of normal operations, especially in the event of a service outage.

Other information exposed by the new mechanism is dynamic.

The figure below depicts the situation at the time of a failure and what is needed in order to restore energy service.

Figure 6.2.1-3: Local failure event

NOTE: 	In Figure 6.2.1-3 and 6.2.1-4 below, the green cubes labeled 'ES' represent energy supply points, associated with an Energy Supply ID.

The red lines above are medium voltage lines between energy distribution subtations (DS). Note that initially Primary Substation (PS) 1 feeds DS7, DS6 and DS5, and PS2 feeds DS1, DS2, DS3 and DS4.

Between DS2 and DS3 there is a distribution failure. This has the result of causing DS3 and DS4 to no longer be able to maintain the energy distribution service to customers.

Note that the topology of energy distribution lines is redundant. In order to achieve distribution successfully through this redundant topology, energy service has to be switched on and off at corresponding distribution substations. This for example could be done between DS4 and DS5 to feed restore service to all substations. The switching is disruptive of energy service. For some time, energy supplied by the distribution substation to the energy customers is off-line. This would result in the following topology:

Figure 6.2.1-4: Local failure restored

The topology in Figure 6.2.1-4 is altered so that DS4 and DS5 are now connected, accepting the medium voltage line between DS2 and DS3 are disconnected. PS1 and PS2 remain disconnected; they represent another level of redundancy in the system.

If the process of switching on and off until a stable configuration is achieved has to be done manually the process can take considerable time (e.g. hours) as technicians has to be on-site to control the substation configuration. The technicians have to communicate, e.g. using mobile telecommunications. However, if the process takes a long time - the mobile telecommunication infrastructure, as shown by base station A and base station B, will likely have exhausted their Uninterruptable Power Supply (UPS), further complicating recovery procedures.

The alternative investigated in this use case involves taking into account the configuration (as described in Figure 6.2.1-2) and dynamic information obtained from the 3GPP network management system concerning the current capacity (expected remaining duration of service) of the UPS in each of the MNO's critical infrastructure sites.

Using smart energy services (principally Distribution Automation), it is possible to perform the 'switch on and off' procedures rapidly, within minutes, and reconfigure the network. There will still be transient electrical service outages to the customers that the distribution substations serves.

Taking into account (a) which mobile network infrastructure sites (i.e. base stations) are critical to service and which cells they serve, (b) the corresponding Energy Supply ID, (c) the distribution substation (UEs) that rely on these base stations, (d) the remaining UPS capacity available in the sites, it is possible for the DSO to plan the 'switch on / switch off process' to avoid any interruption of service. This is done by selective ordering of the switch on / switch off activity, to avoid exhausting any mobile network infrastructure UPS - especially where this base station (etc.) serves to enable communication services to a distribution substation.

Note that in figure 6.8.1-3, the cell in which each is located is served by a base station that can be affected by 'switching off.' It is also known to the DSO that e.g. DS2 is served by A and (secondarily) by base station B. Knowledge of this redundancy can also be of benefit to the planning of the DSO, to prevent in the worst case that both base stations UPS capacity is exhausted and there is no way to restore energy service to these sites.

In order to support the process described above, another example of dynamic real-time information is the ‘real’ back up power available at the base station, at any moment in time; the intention is to see how much time the DSO has to restore the power before service interruption compromises the MNO's operation.

### 6.2.2	Details

Use case actors

DSO network operations center 'management system'

The DSO network operations center management system (DSO-MS) maintains operational information to inform DSO network operations.

MNO network operations center 'management system'

The MNO network operations center management system (MNO-MS) has and can expose operational information to DSOs  concerning the network's configuration and status.

The MNO-MS is a 3GPP management system and the DSO-MS is not a 3GPP management system, however it supports mechanisms that are defined by 3GPP standards (e.g. it uses 'northbound interfaces' exposed by the 5G network management system.)

Use case service flow

Preconditions:

There is a feeder, whose operation requires smart energy services. The smart energy services are available through DSO equipment RTUs. These RTUs are connected with a router, a CPE in the DSO site, that supports a mobile telecommunications interface, a UE.

The DSO can obtain information from each UE in the DSO network. The DSO-MS is aware of the Base station ID of the serving base station for each UE.

On a regular basis, e.g. daily, the DSO-MS obtains 'static information' from the MNO-MS. The DSO-MS is aware which base stations each of the DSO's UE camp on. The DSO-MS is also aware of which base stations rely on which Distribution Substation.

Service Flow: MNO-DSO coordination to achieve rapid energy service outage recovery

Figure 6.2.2-1: Timeline for Restoration of a Distribution Substation

1.	The DSO-MS uses the standardized mechanism to identify an energy service outage at a particular Distribution Substation (or set of Distribution Substations.)

This is shown in Figure 6.2.2-1 as the time of the outage incident.

2.	The DSO-MS uses the standardized mechanism to request information from the MNO-MS, to identify the UPS capacity of the base stations in the vicinity of the outage, where the distribution substations will need to be switched on and off. This is shown in figure 6.2.2-1 as a, b, c, d which correspond to base stations A, B, C and D. 

This request may be done repeatedly, over time, so that the DSO-MS can track the status of the MNO-MS.

3.	The DSO-MS can use the standardized mechanism to inform the MNO-MS of which sites (using the Energy Supply ID known by both the MNO and the DSO). The DSO can inform the MNO in advance of a planned outage, e.g. when switching on and off will take place.

4.	The DSO will actively switch on and off the medium voltage topology at the different distribution stations seeking to establish a stable and sufficient topology. The DSO will prioritize switching involving DS1 so that the UPS of base station A is not exhausted, because 'a' will expire first as shown in Figure 6.2.2-1.

5.	A stable and sufficient medium voltage distribution topology is established without exhausting any of the UPS capacity of the base stations and other critical mobile infrastructure sites.

Post-conditions:

The DSO-MS has informed the MNO-MS before, during and after an energy service outage.

The DSO-MS has been able to obtain dynamic UPS information corresponding to mobile infrastructure sites throughout the recovery process.

Service Result:

Energy service is restored to the MNO sites and to the rest of the DSO's energy service customers efficiently, without requiring mobile manual intervention.

### 6.2.3	Potential Requirements

PR 6.2.3-1.	The 3GPP management system should support, subject to operator policy, regulatory requirements and contractual obligations, the for the DSO to obtain the following information from the site operator.

For each 'site' for which energy service is critical to the site operator:

- Energy Supply ID

- UPS Capacity of the site (at the time at which this information is obtained);

- Base Station ID (if applicable. The site may not be a base station, e.g. it could be data centre or other facility.)

PR 6.2.3-2. 	The 3GPP management system should support, subject to operator policy, regulatory requirements and contractual obligations, the capability to enable the DSO to provide the site operator with information concerning the beginning of an energy service outage and the effected sites.

PR 6.2.3-3. 	The 3GPP management system should support, subject to operator policy, regulatory requirements and contractual obligations, the capability to enable the DSO to obtain information from the site operator concerning the UPS capacity corresponding to a specific site.

PR 6.2.3-4.	 The 3GPP management system should support, subject to operator policy, regulatory requirements and contractual obligations, the capability to enable the DSO to inform the site operator concerning the end of an energy service outage and the related sites.

Next Change

## 6.3	Business use case: Rapid intervention for outages without redundant energy feeder topology

### 6.3.1	Description

This use case is a variation on 6.2 above.

In 6.8, a redundant topology offers the opportunity to adjust the switching of medium voltage lines for energy distribution. When there is a failure of one of these lines, it is possible to adjust the topology to reestablish energy supply to all distribution substations.

This possibility exists only if there is a redundant topology. There are scenarios in which there is no redundant topology, either because a substation is remotely located or because the redundant topology is sufficiently damaged that a sufficient network cannot be reestablished. This use case addresses these scenarios.

Figure 6.3.1-1: Local Failure Event of feeder of Distribution Substations without Redundant Toplogy

In figure 6.3.1-1, there is a failure in the distribution line between Distribution Substation 2 and Distribution substation 3. This will result in a failure to supply energy to ES2, and thereby base station 2.

In this use case, it is necessary to reestablish the energy supply between substation 2 and substation 3 prior to resuming service in substation 3 and 4.

Once this has been reestablished the topolology is restored as shown in Figure 6.3.1-2.

Figure 6.3.1-2: Restored Energy feeder to Distribution Substations

At the point when the distribution is again possible, it is necessary to resume service at distribution substation 3 and 4. However, as some time has elapsed, the UPS capacity of B will be exhausted. This will mean that any smart energy automated operations to distribution substation 3 and 4 will be impossible. Manual intervention to restart service will require substantially more time than automated response.

For this reason, this use case suggests a new operational capability to achieve rapid coordinated recovery. In this approach the DSO informs the site operators to reserve UPS capacity in certain sites, so that it will be possible to resume telecom operations subsequent to the resumption of energy distribution service. If energy distribution service does not resume, after the UPS capacity becomes exhausted, telecom operations will become impossible.

### 6.3.2	Details

Use case actors

DSO network operations center 'management system'

The DSO network operations center management system (DSO-MS) maintains operational information used for DSO network operations. The DSO-MS supports interfaces defined in this use case. All other aspects of the DSO-MS are out of scope of 3GPP specification. The DSO-MS is a consumer of the 3GPP management system.

Site Operator network operations center 'management system'

The site operator network operations center management system (SiteOp-MS) has and can expose operational information to DSOs  concerning the network's configuration and status. The SiteOp-MS as discussed in this use case can be considered a producer of management interfaces consumed by the DSO-MS. The SiteOp-MS is effectively a standardized subset of interfaces and semantics of the 3GPP management system.The SiteOp-MS is a 3GPP management system for 5G and the DSO-MS is not a 3GPP management system, however it supports mechanisms that are defined by 3GPP standards (e.g. it uses 'northbound interfaces' exposed by the 5G network management system.)

Use case service flow

Preconditions:

The purpose of the following description is to explain the scenario in which the energy utility operates a network by means of diverse accesses. It is important to mention that other access systems are used to access the energy utility site networks as well, but only access via the mobile telecommmunication system mis in of scope of this use case.

An energy utility maintains many energy distribution substations. Each is an energy utility infrastructure site that is responsible for distribution of energy to customer sites. This energy utiltity infrastructure site's operation requires smart energy services. The smart energy services are used to manage and control DSO equipment. This equipment is present on a local area network in the energy utility infrastructure site, which is accessed (e.g. as a VLAN) over any access. In this use case, the access that is used is 3GPP access. A UE is used effectively to carry DSO energy utility infrastructure site communication opaquely (that is, as encrypted traffic) to the Utility Service Provider Network. For background infomation on this scenario, see TR 22.867, clause 5.7. [6]

The DSO can obtain information from each UE that is used to provide access to the DSO networks. The DSO is, by means of this informtion obtained from UEs in the DSO network, aware of the Base station ID of the serving base station for each UE.

On a regular basis, e.g. daily, the DSO-MS reads information from the SiteOp-MS exposed 3GPP management system MnS Producer's exposed interfaces. The DSO-MS is aware which base stations each of the DSO's UE camp on. The DSO-MS is also aware of which base stations rely on which Distribution Substation.

Service Flow:

Figure 6.3.2-1: Timeline for Restoration of a Distribution Substation

1.	At some time (T0) there is an outage incident either a planned or unplanned incident.

2.	At some subsequent time (T1), the energy utility begins restoration of energy feeder lines or other affected infrastructure.

3.	The period of time that will elapse before the restoration of energy service from some set of distribution substations will be longer than the UPS capacity of the mobile infrastructure sites. This use case assumes that the MNO knows or can estimate the remaining time of operation after T0 given the UPS capacity of different mobile infrastructure sites, shown as a, b, c, d in Figure 6.3.2-1. That is, T2 occurs after the UPS capacity in the sites affected by the energy outage. This use case assumes that the DSO knows or can estimate time at which energy distribution service can resume, shown as T2 in figure 6.3.2-1. This may not be the exact time at which resumption of energy service can resume, which is shown as T3. T2 is an estimate when the energy feeder of while c represents the time at which energy feeder service resumes and restoration of distribution is possible.

4.	The energy feeder for one or more energy distribution substations is now complete. At this point, it will be possible to restore energy distribution. However, operations are required at the distribution substation. This can be performed by smart energy services remotely if there is network coverage. The starting time, when the MNO provides service with remaining UPS capacity, is shown in Figure 6.3.2-1 as T3. The smart energy services to restore energy distribution services to all customers, including the MNO, is shown as T4.

There are two alternatives for how the restoration can occur. Manually, as described in 5a, or with remote intervention, as described in 5b.

5a.	Without prior arrangement, there will be no UPS capacity remaining in the infrastructure that serves the distribution substations that have restored power. They will not be able to perform automated recovery, as explained in use case C.4. In this case, manual intervention is required to restore energy distribution. This will be complete after a substantial period of time (T5).

5b.	Alternatively, prior arrangement can be made so UPS capacity will remain in the infrastructure at the time it is needed to restore energy distribution service. This prior arrangement is described in the steps below, and consists of operations between the DSO-MS and MNO-MS.

This is to enable the situation that, at time (T3), the MNO is able to use remaining UPS capacity to offer telecommunication service at the time at which the DSO will perform remote operations by means of data communicadtions to restore service in the sites affected by the outage, and operates them until the outage concludes.

5.b.1.	In this use case, the DSO-MS communicates to the MNO-MS:

- the affected sites (Energy Supply IDs) by the outage

- the time X after which recovery is possible

- the time Y (that is a certain interval of time after X) that the recovery is expected to complete (a small number of minutes)

The site operator, knowing this, has the opportunity to manage the use of the UPS in the affected sites so that they do not exhaust at time (a, b, c, d, etc.). Rather, capacity sufficient for operation between time X and Y are reserved. Figure 6.3.2-2 below shows a concrete example of this interaction.

Figure 6.3.2-2: Example Restoration Scenario of a Distribution Substation

Table 6.3.2-1: Example Sequence of Restoration of a Distribution Substation

| Time | Event |
| --- | --- |
| 00:34 | A storm rages and causes power lines to collapse in the mountain province. Energy distribution service by Distribution Substation 3 is no longer possible! |
| 00:36 | The DSO informs the MNO that there has been an energy distribution incident that will affect site S. The DSO expects to repair the medium voltage line by 02:45, and if telecommunications service permits it, remote control (using Distribution Automation and SCADA controls) of all sites served by Distrution Substation 3 can be restored - by 02:50. In terms of the procedure step 5a, X=0:2:45, Y = 5 minutes. |
| 00:40 | The MNO conserves energy as appopriate to conserve 5 minutes of operating capacity of their UPS reserve. |
| 02:38 | The DSO informs the MNO that the feeder line is restored. Remote control restoration of service can begin now. This is T2 in Figure 6.3.2-1. |
| 02:43 | The MNO resumes operation at site S. The DSO is informed that the service has resumed. |
| 02:47 | The DSO completes distribution automation. Site S now has energy service. This is T4 in Figure 6.3.2-1. |

5.b.2.	The DSO, after time (T3), employs smart energy services such as distribution automation or specific SCADA operations to restore service to customers rapidly, including the site operator.

5.b.3.	The incident concludes (T4). Energy distribution service has been restored to the MNO site(s) as well as other energy service customers.

Service flow result

T4 occurs before manual uncoordinated recovery of service would be successful (T5 in Figure 6.9.2-1.) Thus, alternative 5.b is superior to 5.a for both the site operator and the DSO.

Service is restored to distribution substation 3 and 4 at T4, within minutes of the restoration of the medium voltage line between distribution substation 2 and 3 T3. This is substantially faster than service could be restored if a technician had to visit distribution substation 2 and 3 - represented on Figure 6.3.2-1 as T5. As a result, service is restored to the MNO sites affected more rapidly than in an uncoordinated incident.

Next Change

### 6.3.3	Potential requirements

This use case also has potential requirements 6.2.3-1, 6.2.3-2, 6.2.3-3 and 6.2.3-4. These requirements, if supported, enable the site operator to voluntarily discontinue service, retain UPS capacity, and wait until an opportune time to restore service. This is subject to operator policy, contractual obligations and regulatory restrictions.

For all the requirements below, supported interaction is described between the DSO and the site operator or site operator. The reason for this term 'site operator' is that in network sharing scenarios, the base station and/or cell site may be operated by a third party. In active network sharing scenarios the DSO can communicate with an site operator that operates the site, but can be distinct from the serving site operator. The interaction described is really between the DSO and the management services of the entity that operates the site essential for telecommunication service, identified by the energy supply ID.

PR 6.3.3-1. 	The 3GPP management system should expose management services, subject to operator policy and other conditions (see NOTE 4), to enable the DSO to provide the site operator with information concerning the expected restoration time of its distribution services for  site operator for effected sites.

PR 6.3.3-2. 	The 3GPP management system should support, subject to operator policy, the capability to enable the DSO (or site operator) to provide the site operator with information concerning the time when DSO is ready to remote energy distribution services can resume, and the locations where distribution is possible. The time when communication services are needed corresponds to the time when DSO has restored its energy transmission service and is ready to  remote energy distribution services can resume. The location information expressing where restoration will occur could be expressed in terms such as latitude-longitude pairs or Energy Supply ID.

PR 6.3.3-3. 	The 3GPP management system should support, subject to operator policy, the capability to enable the DSO to provide the site operator with information concerning the time duration for which DSO expects to require site operator's communication services to achieve coordinated recovery for being able to use smart energy services to restore its energy distribution services. The time that the DSO provides to the site operator can be adjusted as new information becomes available. The time estimate specifies which base stations are needed for the remote recovery operations.

PR 6.3.3-4. 	The 3GPP management system should support, subject to operator policy, the capability to enable the DSO to provide the site operator with information concerning the locations where for example DSO substations need to restore distribution services on priority. Location information expressing where restoration will occur could be expressed in terms such as latitude-longitude pairs or Energy Supply Id.

PR 6.3.3-5. 	The 3GPP management system should support, subject to operator policy and other conditions (see NOTE 3), the capability to enable the DSO to provide the site operator with information concerning the time at which MNO (or site operator) should actually able to provide communication services to achieve coordinated recovery to DSO for a particular region.

PR 6.3.3-6. 	The 3GPP management system should support, subject to operator policy, the capability to enable the DSO to provide the site operator with information concerning the time duration for which site operator should actually able to provide communication services to achieve coordinated recovery to DSO for a particular region.

Next Change

# 7	Key Issues and potential solutions

## 7.1 	Key Issue #1: MNO exposes network performance monitoring

### 7.1.1	Description

This describes the issues to be studied in context of the use case of MNO exposes Network Performance Monitoring. This corresponds to the use cases and associated requirements in 6.5 above. The requirements from C.5 are not addressed by this key issue: alarms and fault management are not considered by the solutions corresponding to this key issue.

This key issue is if the that DSO must be an MNO trusted entity to access MnSes provided by the 3GPP Management System. The existing performance assurance mechanism can be used to report network performance to DSO. However, the following need to be studied

a.	How the existing ThresholdMonitor can be used for configuring the network monitoring.

b.	How the existing NtfSubscriptionControl can be used for sending reports as notifications against monitoring.

c.	Whether the existing performance measurements [13] and KPIs [12] are enough to support the requirements (e.g. latency, throughput, packet loss, availability etc.)

### 7.1.2	Potential solutions

#### 7.1.2.1	Potential Solution #1: MNO exposes network performance monitoring

##### 7.1.2.1.1	Introduction

This solution addresses Key Issue 1 and the requirements in 6.5.

The solution assumes that DSO is an MNO trusted entity and can access MnSes provided by the 3GPP Management System. In this solution, the existing provisioning MnS and related NRM fragments are used. DSO uses ThresholdMonitor to configure the related threshold for various network performance requirements e.g latency, throughput, packet loss, cell/network availability etc. DSO also uses the NtfSubscriptionControl to subscribe for NotifyThresholdCrossing notifications. The management system monitors the network and delivers the notification when the measurement or KPI crosses the configured threshold limit.

Depending on the configuration of the monitorGranularityPeriod attribute of the ThresholdMonitor, the DSO can identify the frequency of the threshold to be checked, post which the notification will be sent.

The returned Measurements conform to the Measurement Template [15], which identifies each individual measured value distinctly. Thus, it is possible to ascertain from the report the number of samples per measurement as well as the distribution of the data (e.g. to compute the standard deviation.)

##### 7.1.2.1.2	Description

Figure 7.1.2.1.2-1: Monitoring Configuration Procedure

1.	DSO  sends createMOI request for perfMetricJob  IOC to Performance Assurance Producer.

2.	createMOI response is sent by Performance Assurance Producer to DSO.

3.	Performance Assurance Producer calculates the required measurements and KPIs as per request.

4.	Performance Assurance Producer provides the required measurements and KPIs to DSO.

5.	DSO sends createMOI request for ThresholdMonitor IOC to Threshold Monitoring Producer.

5a.	The attribute performanceMetrics,  contains the measurements defined for each of the network performance requirements as required by DSO e.g latency, throughout, packet loss, availability etc.

5b.	It  also contains an attribute containing the location (Lat/long, TAC, cellid). This is used to scope the object instance to be monitored.

6.	Threshold Monitoring Producer  creates the MOI.

7.	createMOI response is sent by Threshold Monitoring Producer to DSO.

8.	DSO sends createMOI for NtfSubscriptionControl IOC to Threshold Monitoring Producer.

8a.	The attribute notificationRecipientAddress  contains the address of the notification recipient i.e. DSO.

9.	Threshold Monitoring Producer  creates the MOI.

10.	createMOI response is sent by Threshold Monitoring Producer to DSO.

11.	Threshold Monitoring Producer checks for threshold crossing based on the current state of the related    performance measurement.

12.	Threshold Monitor Producer sends notifyThresholdCrossing notification to DSO if the performanceMetrics     value crosses the configured thresholdValue.

## 7.2 	Key Issue # 2: DSO reports an incident to the MNO

### 7.2.1	Description

In response to the use case C.7 and the potential requirements identified in C.7.3, new and existing solutions for incident reporting will be considered. These requirements are not addressed in the present document.

### 7.2.2	Potential Solutions

#### 7.2.2.1	Potential Solution # 1: TMF API 621 (Trouble Ticket)

##### 7.2.2.1.1	Introduction

It was suggested to investigate whether and how a DSO could create a Trouble Ticket to report an incident to satisfy the requirements in clause C.7. This solution will not be further developed in this version of this document.

#### 7.2.2.2	Potential Solution # 2: MnS-based standardized interface

##### 7.2.2.2.1	Introduction

It was suggested to investigate the feasibility of a MnS-based standardized interface to satisfy the requirements in clause C.7. This solution will not be pursued in this version of the present document.

## 7.3 	Key Issue #3:  Energy utility and network operator coordinated recovery of energy service

Next Change

### 7.3.1	Description

#### 7.3.1.1	General

This describes the issues to study in context of the use case of Energy utility and network operator coordinated recovery of energy service.

There is clearly a mutual interest in coordination between network operator operations and energy service operations to achieve rapid recovery of energy service. This is not only true for the MNO, who benefits from the availability of supplied power, but also of the DSO who needs mobile telecommunication service to restore and maintain the operation of its grid.

NOTE: 	Though DSOs use many forms of access (e.g. fixed access, dedicated fiber access, etc.), they increasingly rely on telecommunications services for communication access to many substations and other facilities. It therefore directly benefits the overall availability of communication service for the energy utility when the availability of the telecommunication service improves. The Energy utility provider (DSO) knows when and where outage has occured and when telecommunication services are critically important for recovery. The MNO knows their uninterruptable power supply resources and the possibility of utilizing telecommunication services to enable utility’s energy system rapid recovery via smart energy services.

Without a standardized mechanism to share all this information, the service recovery mechanism would be very inefficient and time consuming. Therefore, a standardized mechanism is needed to share all this information between DSO and MNO to enable efficient usage of smart energy services for service recovery.

#### 7.3.1.2	Coordinated recovery of energy service with redundant topology

The functions described here correspond to the requirements in the use case 6.8 "Business use case: Energy utility and telecommunication coordinated recovery of energy service".

a)	How can the DSO obtain the information from the MNO that is listed in PR 6.2.3-1?

b)	How can the DSO provide the MNO with information concerning the beginning of an energy service outage and the affected sites? [PR 6.2.3-2]

c)	How can the DSO obtain the UPS capacity related info of a specific site from the MNO? [PR 6.2.3-3]

d)	How can the DSO inform the MNO of energy service outages ending and the affected sites? [PR 6.2.3-4]

Next Change

#### 7.3.1.3	Coordinated recovery of energy service without redundent topology

The functions described here correspond to the requirements in the use case 6.9 "Business use case: Rapid intervention for outages without redundant topology".

The key issue topics described in 7.3.1.2 have to be resolved for this key issue. In addition, the following key issues also have to be addressed.

a)	How does the DSO provide the MNO with information concerning the expected time at which the DSO will be able to restore its energy services? [PR 6.3.3-1]

b)	How does the DSO provide the MNO with the  time at which it has restoted its energy transmission service? [PR 6.3.3-2]

c)	How does the DSO provide the MNO with the time duration  that the DSO expects to require to restore energy services, facilitated by telecommunication services? [PR 6.3.3-3]

d)	How does the DSO inform the MNO of the locations affected by an energy service outage (latitude-longitude pairs, energy supply IDs) where the DSO will like to restore service on priority than some other locations? [PR 6.3.3-4]

e)	How does the MNO inform the DSO of the actual time and locations (e.g. base station IDs) where the MNO is able to provide telecommunication services to enable the DSO to restore energy service? [PR 6.3.3-5]

f)	How does the MNO inform the DSO of the time duration for which the MNO is able to provide telecommunication services to enable the DSO to restore energy service? [PR 6.3.3-6]

Next Change

# 8 	Conclusion and recommendation

## 8.1         Key Issue #1: MNO exposes network performance monitoring

The solution in 7.1.2.1 provides a mechanism by which the 3GPP management system, according to mobile network operator policy, can expose standarized interfaces to DSO entity that provides the ability to initiate and terminate requests for monitoring and to receive reports containing the required performance data. This corresponds to the key issue objectives listed in 7.1.1.

The solution in 7.1.2.1 addresses only performance management use case (6.5). The purpose of the table below is to show that key issue 1 is fully addressed by the solution 7.1.2.1.

Table 8.1-1: Key Issue to Solution Mapping

| Key Issue | Solution | Remarks |
| --- | --- | --- |
| 7.1.1.a. configuring network monitoring | 7.1.2.1.2, step 5-7 |  |
| 7.1.1.b. obtain reports | 7.1.2.1.2, step 8-12 |  |
| 7.1.1.c. does PM contain enough measurements and KPIs to support the requirements? | 7.1.2.1.2, step 5a | It is reccomended that the required measurement and KPIs be defined as part of 28.552 [13] and 28.554 [12] respectively. |

The solution will entail:

1.	An update to ThresholdMonitor for an additional location based attributes to be used to scope the objectInstance.

2.	New Performance Measurements and KPI related to availability, cell in-service and out-service.

3.	The specification of the procedure and explanation of its relevance to and use by energy utilities.

The solution is feasible as it re-uses the existing mechanism and follows the model driven approach. Hence, the recommendation is to normatively define the solution. It is recommended to proceed with 1-3 listed above in normative specifications.

NOTE:	The solution enhances 3GPP management system capabilities so that DSO can use it for MNO’s network monitoring purposes. However, the access to these capabilities, when it occurrs, will be subject to proper authorization and authentication. Further, all use of these mechanisms is subject to operator policy, contractual obligations and regulatory restrictions.

## 8.2	Key Issue #3: Energy utility and telecommunication coordinated recovery of energy service

The solution in 7.3.2. provides a solution to each of the elements in the key issues 7.3.1.2 and 7.3.1.3.

The data model containing all the above elements will be defined in the normative specification phase.

The solution provides a mechanism for the energy utility (DSO) and telecommunication coordinated recovery of energy service. This involves mutual sharing of information between DSO and site operator, according to mobile network operator policy, through standarized interfaces. The information shall be at least the following information elements:

-	the time of the beginning of an energy service outage and the effected sites;

-	the UPS capacity corresponding to a specific site, end of an energy service outage and the related sites

-	DSO and site operator rapid intervention related information (transmission restore time, required intervention duration, feasible intervention duration).

For the solution, supported interaction is described between the DSO and the site operator. In some network sharing scenarios, the base station and/or cell site may be operated by this third party. In this case, the specified interaction described is really between the DSO and the site operator would be between the DSO and the site operator of the site essential for telecommunication service, identified by the energy supply ID.

The solution requires the NRM enhancements related to step 1, step 4 in clause 7.3.2.1.2.

NOTE:	The solution enhances 3GPP management system capabilities so that DSO can use it for site operator’s network monitoring purpose. However the access to these capabilities, when occurred, will be subject to proper authorization and authentication.

The specification of the procedure and explanation of its relevance to and use by energy utilities will be defined as part of the normative work.

The solution is feasible as it re-uses the existing mechanism and follows the model driven approach. It satisfies the requirements given in 6.2 and 6.3. Hence, the recommendation is to normatively define the solution.

It is recommended to specify 7.3.2 in normative specifications.

# Annex A: Service model for energy utilities and communication service providers

The purpose of this annex is to discuss a model and terminology for IT service processes that will be useful when considering the use cases for this study.

IT processes concern the delivery of IT services within a given organization. The IT processes and operations of a third party are out of scope of 3GPP. The IT processes and operations of a MNO are also out of scope of the standard, and not exposed to a third party except in very specific cases, for specific reasons.

In the present document, the interaction between the energy utility and the communication service provider at the IT service level are considered. These interactions are essential to the successful service delivery of energy services. To the extent that these interactions can be standardized, they will become more efficient to operate and require less integration effort for both the energy utility service provider and the MNO.

There are three IT Service processes that will be considered briefly in this section to provide context for the use cases that follow. [5]

Table A.1: ITIL Processes

| IT Process | ITIL Definition | Relevance to this study |
| --- | --- | --- |
| Event Management | Process Objective: To make sure CIs and services are constantly monitored, and to filter and categorize Events in order to decide on appropriate actions. An 'Event' is essentially an alert or alarm created by any IT service, CI or monitoring tool. It is further characterized, often as input to the Problem Management or Incident Management processes described below. | To address the objectives of the present document, alerts or alarms and related information will be considered, as communicated between the energy utility operator and the MNO. These may trigger further action by either the energy utility operator or MNO. While the specifics of the use of categorization of events and the details of IT Processes is out of scope of this study (and 3GPP), the utility of creating and communicating standardized events to enable such processes is in scope. |
| Problem Management | Process Objective: To manage the lifecycle of all Problems. The primary objectives of Problem Management are to prevent Incidents from happening, and to minimize the impact of incidents that cannot be prevented. Proactive Problem Management analyzes Incident Records, and uses data collected by other IT Service Management processes to identify trends or significant Problems. | Many of the use cases in the present document concern different problems. Either the energy utility operator or the MNO ascertains that a problem exists and is able to take further action to address this. The specifics of the IT processes of the energy utility operator and the MNO are out of scope of the present document. However, there are some 'Problems' that may have relevance to both operators. |
| Incident Management | Process Objective: To manage the lifecycle of all Incidents. The primary objective of Incident Management is to return the IT service to users as quickly as possible. | Some of the use cases in the present document concern Incidents, specifically – communication service outages and energy service outages. Both of these have relevance to both energy utility operators and MNOs. For these specific Incidents some operational requirements may be identified to enable all actors to eliminated the unplanned interruption in service or reduction in quality of service. |

# Annex B: Applicability statement

The present annex discusses how NSCE and layered services can complement the functionality in the conclusions of the present document.

Figure B-1: Consumer Interaction with or without a Layered Interface

In Figure B-1, above, two distinct architectures are shown.

(A) includes a well-defined and understood management system relationship as defined in 3GPP.

NOTE: Given only model (A), an implementation and deployment of NSOEU will only be feasible given two assumptions:

- Energy utility backend  IT systems from are able to consume as-is SA5 defined MnSs (i.e., they are able to interpret MnS component Type A, B and C,) and thus are able to become MnS consumers.

- As MnS consumers, the energy utility is authorized to gain access  to MnS producer capabilities, according to a previous authentication and authorization procedure. Such procedures are not in scope of this technical report.

It is noted that there are already systems that are deployed today by which energy utilities exchange information, mostly request and receive information, in a 'custom' (that is to say not a standards specified) manner. Extensive and successful experience in the past with custom exposed information through vendor extensions to RADIUS was the motivation to pursue SEI in 3GPP. This approach has two drawbacks:

-	The functionality of the interface needs to be implemented for each energy utility-operator relationship.

-	The security and policy governing this interface needs to be implemented for each energy utility-operator relationship.

The present documemnt addresses these shortcomings of a proprietary approach. The conclusions of this study address the first shortcoming but not the second.

To address a policy and security framework for exposure of management services, the mechanisms in the conclusions of the present document can rely upon a layered interface. The layered service can do so in a general way, so that capabilities that are supported for all forms of management services, including those for energy utilities and far beyond.

Additional layered services are possible beyond those for authorization, security and access policy.

For example, the CAMARA initiative of GSMA [18] aims to define, develop, integrate and validate APIs that 3rd parties can invoke. Unlike standard telco-centric APIs (e.g. 3GPP APIs),  which are mostly focused on ensuring interoperability within (multi-vendor) and between operators, CAMARA APIs will follow developer-friendly style, with semantics and consumption patterns that are tailored to the business and operational needs of 3rd parties (including especially third parties who are not in the telecommunications industry sector, specifically.) CAMARA can in future focus on defining which information will be exchanged between the network operator and third parties, including energy utilities. As stated in [18], CAMARA offers an " Abstraction from Network APIs to Service APIs is necessary: To hide telco complexity making APIs easy to consume for customers with no telco expertise (user-friendly APIs); To fulfil data privacy and regulatory requirements; To facilitate application to network integration."

In summary, without layered interface support, the solutions in the present document will conform to the model depicted in Figure B-1 (A) above. The solution  requires proprietary (closed) solutions to achieve the three objectives (hiding complexity, data privacy and regulatory requirements, application to network integration.) Standards supporting layered interfaces over the solutions defined the present document, be they NSCE or CAMARA or both, would resolve these shortcomings.

NOTE:	CAPIF exposure of management services (see 7.x.y) addresses the data privacy objective discussed in the present annex.

Next Change

# Annex C: Use cases not pursued

## C.1	General

The use cases and the associated potential requirements in this annex have no associated solutions (clause 7) nor conclusions (clause 8). The potential requirements listed are not to be considered as conclusions of the present document.

## C.2	Business use case: DSO obtains network performance and outage information, current practice

### C.2.1	Description

Motivation

When there is an electrical outage, the electrical service operator strives to restore service as quickly as possible. There are several reasons for the need for rapid recovery from an interruption of electrical service:

-	In some countries, regulations require rapid recovery and penalize an Energy Utility service provider for any time in which services does not operate. For example, at the least, in many countries customers do not have to pay for electrical service when it is not available.

-	Interruptions in electrical service can be expensive, as businesses often require electricity for operations, manufacturing and to properly store valuable products. Thus, electrical outages can translate directly into business losses (of productivity or inventory.)

-	Power outages for hospitals and care facilities can result in harm or even death to patients.

NOTE 1: 	Regulations identify 'critical' electricity customers which are obliged to install and maintain secure sources (e.g. local generators.) Public mobile network operators in some cases are not covered by these regulations. Further, public mobile network operators are generally not considered 'critical' electricity customers by regulation, so Energy Utility service providers cannot prioritize service to these customers.

-	Electrical service outages affect many customers, so failure of service is not comparable in terms of business consequences to outages of mobile telecommunication service to a single customer.

For this reason, there are regulations that make energy service availability the highest priority. In order to achieve this, Smart Energy services such as protection, SCADA and Distribution Automation are used to monitor and adjust distribution equipment to avoid incidents that would reduce energy service availability.

As a point of comparison, a fiber optic access service is often offered with an availability of at least 99.999%. Telecommunication systems may not achieve this level of service availability. Since telecommunications offers an alternative to fixed fiber optic access, additional means to achieve high degrees of availability are essential to the DSO.

Background

To achieve extreme telecommunication service availability, it is currently not feasible to rely on a single telecommunication network. Instead, DSOs networks employ communication access equipment that have multiple USIMs. If one telecommunication service provider is not available, the second can be used. However, this arrangement ('failover') is insufficient, as it requires in practice 2 minutes or more to bring up a secondary USIM and register with a back up network.

NOTE 2: 	An electrical service operator is a more general term than a DSO. For the purpose for the purpose of the use case, the term DSO is more appropriate, while the more general term is applicable to the motivation above.

To prevent an outage that will last an hour or more, Distribution Automation has to be used to intervene in the first minutes, ideally in the first seconds, in which an outage occurs. The following examples show two outages and can be considered characteristic of the prospects of resolution in most situations.

In Figure 1, an incident affecting an underground Medium Voltage (MV) line eliminated service to 4223 customers. The existence of DA in secondary substations along the line allowed the fault to be isolated quickly and then resolved. This dramatically reduced the service outage duration for a substantial number of customers. A few customers that were along a line without DA access required local operation that took more than one hour. (The time scale on the X axis is not to scale.)

Figure 1: Incident Example

Over 50% of the customers could have their service restored in 3 minutes. Another 40% of the customers had their service restored in under 10 minutes. The remaining roughly 10% of the customers required manual intervention in order to have their service restored. The rapid service restoration saved EUR 1000s in saved penalties as well as needing only one service truck to roll.

In Figure 2, another example, another MV line was damaged, again showing a complication of a line that did not offer the possibility of DA intervention.

Figure 2: Incident Example featuring a time consuming recovery for a minority of customers

This incident affected 5292 customers. The outage lasted 7 hours but it can be observed that most of the customers could have their service restored within the first minutes due to remote access to primary and secondary substations and intervention using DA. In this incident, EUR 10000s could be saved.

Considering the importance of rapid response during a power outage, it is important that the communication facility is available at the time of an outage. If communication failure only is ascertained during an incident, the outage duration can be extended significantly for most of the affected customers.

In order to improve availability, some DSOs use dual USIM UE deployments. In practice, it takes on the order of 2 minutes to bring up service on an alternate mobile network. These minutes, if they coincide with an outage, are expensive both in terms of penalties and in terms of problems faced by customers during the outage. This also uses up roughly half of the ‘downtime budget’ of 99.999% availability (5 minutes per year unscheduled downtime maximum.)

Communication links are tested, e.g. every minute by means of ping messages end to end, to identify availability and latency. The effective availability of 3GPP telecommunications networks observed in practice can be more like 98.5% (where availability means ability to achieve communication with the expectations of performance according to the service level agreement. While this is far below the levels we expect from 5G and that we cite in stage 1 requirements, the fact is that observations of performance of past generations indicate that in order to achieve the target availability, information is needed. Please see TS 22.104 [14], Annex A.4.

NOTE 3: 	The actual availability of mobile network services is not exposed to energy utility service provider (customers), nor is the cause of the availability limit, e.g. limited capacity, radio quality issues, etc. This makes it difficult for energy utilities to perform risk assessment and network planning for communication services to carry their smart energy services.

Communication performance failure, based on extensive field experience, can be correlated with network performance events. Network performance can be compared to historic information indicating failures. In this case when communication with a ‘primary’ PLMN shows signs of declining performance, an ‘alternate’ communication channel can be employed, e.g. registration with another PLMN for a multi-USIM device. This action can be performed proactively, so that in the event that the ‘primary’ communication session does fail to deliver required performance, recovery can occur quickly (within seconds) to the ‘alternative’ communication session.

### C.2.2	Details

There is currently no mechanism to convey network or services status from the MNO to the DSO. While the DSO can access real information from their CPEs (radio module or connectivity state, e.g.), no information comes from the MNO. Thus, in case service failure is happening in an area, the DSO can just infer this situation through a qualified guess.

DSOs have alternative telecommunication connectivity possibilities in any given site. This connectivity can be provided by private networks (different to the MNOs’), or can be provided via alternative MNOs (another CPE in the same site; several SIM cards in the same CPE; etc.). Many of the sites where DSOs need connectivity are mission critical; this means that the service has to be highly available. If the DSO is forced to use a standard MNO service, with no specific availability target, and no mechanism to know that service may be failing in an area, the DSO will not rely in MNO services and will develop alternative telecommunications infrastructure that will eventually make MNOs’ services unnecessary.

The MNO has information on the state of its infrastructure. From the Core to the Radio Access Network, and through the different intelligent elements that make the 3GPP standards-based MNO network provide their services, the MNO can tell which network or service parts may be showing a degraded performance or potential unavailability. This information will enhance the MNO service visibility for specific user groups that may benefit from this, while making sure that some of them do not develop alternative solutions they can fully control.

Between the complete lack of information today, and the total control that MNO have of their network infrastructure, and that eventually may reach all the services provided to their customers, there is room to find added value service status information for specific user groups.

Use case actors

DSO network operations center engineer:	The DSO operations engineer is responsible for deploying monitoring and control mechanisms in the network. The DSO operations center engineer determines how to control and configure the network for resiliency, e.g. when and how to switch between different accesses to maximize availability.

DSO electrical system operations center engineer: This actor is responsible for maintaining availability, efficiency and safety of the energy system.

Use case service flow

0.	The energy system is monitored and managed by the electrical system engineer. It relies upon the telecommunications network.

1.	The DSO network operations center engineer deploys monitoring and control mechanisms to determine when a telecommunications network is not available.

2.	There is no standard way to obtain network availability or performance information, so the DSO network operations center engineer has to rely upon 'over the top' monitoring processes. In order to ensure that these monitoring processes do not use a significant amount of the communications capacity, the monitoring relies on periodic probes, e.g. ICMP messages ('pings') every minute. This is a very 'coarse grain' monitoring process, since a failure or performance degradation may take a significant amount of time to detect.

3a.	The DSO network operations center engineer may manually trigger or set up a configuration that will automatically trigger a switch between one telecommunication network to a back-up communication access.

3b. The DSO network operations center engineer is not successful to initiate and complete a telecommumications network switch because insufficient information was available to the network operations center to predict the event so the network fails before fail-over can occur, i.e. for more than 2 minutes. This means that the fail-over occurs during a time in which no communication is possible. This results in a network failure visible to the energy system engineer who requires communication for smart energy services. If a fault or protection problem for example occurs in that time the failure can cause damage to the energy system, inefficiencies and even outages. Especially if control is needed (e.g. critical SCADA) during this time frame, a serious energy system outage could occur.

Service flow result

Since the information that the DSO network operations center engineer relies upon is of coarse granularity, there is a high likelihood that performance degradation or failure will be detected after several seconds from the point when they occurred. This extends the period of time in which communication service will be inadequate or unavailable before a secondary communication access can be made available. This reduces the availability of the energy system because, should an energy outage or instability occur during the time in which there is no communication possible, or inadequate performing communication this will lead to an energy service outage as described above.

### C.2.3	Potential requirements

There are no potential requirements.

## C.3	Business use case: DSO reporting to MNO to resolve problems and incidents, current practice

### C.3.1	Description

Annex A of the present document describes two processes, Problem Management and Incident Management. In current practice, these processes require interaction between the DSO and MNO, but the interaction is specialized, entirely depending on the processes of each individual MNO.

Problems, generally an observed decline or lack of expected service in certain scenarios over a period of time, need to be addressed or they will likely lead to incidents (where service fails completely or degrades so that service levels do not meet agreed key performance indicator objectives). Problem management allows interaction between a service provider and service customer to address longer term issues - such as capacity planning, disaster recovery or, most relevant, analysis of problems to identify their root cause. If the observed communication problem in the energy utility service provider's network is decisively not the MNO's service and network, this will greatly help the energy utility service provider to diagnose and resolve the problem. If, on the other hand, it is the result of the MNO's service and network, this realization can lead to more rapid response.

Incidents, a service failure, are resolved by means of specific procedures defined by service providers. The goal of the customer and service provider is to resolve incidents as quickly as possible.

It is acknowledged that the specifics of the IT processes for Problem Management and Incident Management are out of scope of 3GPP standards. It is however possible that the diversity of interfaces used to report information and the format of those reports can be standardized in 3GPP.

Since an energy utility service provider has to work with many MNOs, especially if the energy utility service provider operates across national borders, the lack of standards in this area for providing information regarding a problem or incident brings complexity and can delay the resolution process.

Energy utilities have vast deployments of communicating devices in hundreds or thousands of static sites. The communication status of each of these devices is constantly monitored, especially their availability, but also latency and other performance metrics. This data, currently collected by energy utility service providers to identify problems and incidents in their own networks, could potentially be very useful to mobile network operators. This information is currently not shared by energy utilities with network operators.

NOTE: 	An energy utility service operator is a more general term than a DSO. For the purpose for the purpose of the use case, the term DSO is more appropriate, as it concerns the operation of energy distribution services, while the more general term is applicable to the motivation above. This use case focusses on energy distribution not generation, transmission or consumption. Though each of these energy system components has relevant aspects for 'smart energy,' we focus on distribution, where communication availability is absolutely critical for stability and recovery of service to customers.

### C.3.2	Details

Use case I: Energy System Service Provider Network Problem Report

Problem Management processes, by which the DSO works with the MNO to address such communication issues as observed erratic or declining performance occurs very infrequently today, with resolution and improvements for processes such as capacity planning and adjustments to service level agreements taking months or years. The opportunities to analyze problems that emerge from time to time in energy utility service provider networks essentially have to be analyzed by the energy utility service providers independently, without cooperation with the MNOs to ascertain the root cause of failure. This network problem discovery, trend analysis and identification of root causes of problems is very labor intensive for energy utility service providers.

Use case actors:

DSO network operations center engineer: The DSO operations center engineer analyzes collected information, creates and operates network monitoring tools including alarms, and seeks to identify areas of potential improvement in service delivery.

MNO technical service / account manager: The MNO operations center service representative responds to queries from the energy utility service provider when needed. Such questions may arise when problems emerge, e.g. as network performance or availability appears to decline in the customer's network.

Use case service flow:

1)	Using the DSO operational tools, the DSO network operations center engineer gathers and analyzes data, seeking to identify trends that appear threatening to the proper function of the energy utility service provider network.

2)	When such trends are identified, the DSO network operations center engineer uses tools to study the details of the potential problem.

3)	The 'root cause' of the problem may need to be investigated before identifying a potential remedy is possible, or determining how high a priority the mitigation has. It is possible at that time that the DSO network operations center engineer contacts the MNO technical service representative with particular questions. The communication between the two organizations is not based on any standard data model - it requires ad hoc consultation which is labor intensive for both the MNO technical service and DSO operations engineers.

Service flow result:

The DSO learns about performance problems in their network. With consultation, it is possible to ascertain whether the issues are also present in the mobile network. This consultation is complicated by the lack of standard ways to exchange management information regarding network problems. The MNO does not benefit from information ascertained by the DSO about network performance issues except through these consultations.

Use case II: Energy System Service Provider Network Incident Report

Incident management is triggered when there is a performance incident - either a service is delivered below the service level agreement parameters or there is a service outage. In order to qualify as an 'incident' some parameters may apply (how long it is sustained, how frequently it occurs, etc.) according to the service level agreement, the details of which are out of scope of 3GPP.

When an incident occurs, this triggers processes to end the service performance failure or service outage as quickly as possible. The DSO needs to diagnose the root cause of the incident and take the necessary steps to recover service. To the extent that the service failure is attributed to the mobile network, or the root cause is unknown and could be due to service problems in the mobile network, the DSO works together with the MNO to diagnose the incident and resolve it.

The term 'service desk' represents the service provider contact that is available when incidents occur. The service desk can take many forms and interactions and interfaces are not standardized in current practice.

While resolution is needed on the order of seconds or minutes, communication and collaboration between the DSO and MNO can take much longer, sometimes hours even days. A significant amount of time is needed to describe the incident in terms that enables the MNO to take action. The lack of standards in this area means that the DSO has to identify the reporting formats of each MNO separately, which is labor intensive especially for DSOs that operate across national borders and therefore have large numbers of subscriptions with each operator in each country they operate.

Use case actors:

DSO network operations center engineer: The DSO operations center engineer analyzes collected information, creates and operates network monitoring tools including alarms, and seeks to identify areas of potential improvement in service delivery.

MNO technical service desk engineer: The MNO is represented by this actor, whose role is to capture the incident information, analyze the MNO's management system to find corresponding relevant information and to work with the customer who reports the incident and MNO operational staff to resolve the incident if it is indeed an issue in the mobile network.

Use case service flow:

1)	The DSO network operational management system indicates a service outage in the DSO network. The DSO operations center engineer analyzes the alarm.

2)	The root cause of the service outage is not clear. The DSO network operations center engineer initiates a service desk procedure with the MNO providing service for the network that has a service outage.

3)	The MNO technical service desk engineer gathers information from the DSO network operations center engineer, partly through tools provided by the MNO service desk (e.g. web based reporting tools), partly through other forms of communication (phone, messages, etc.)

4)	The MNO technical service desk engineer works with the MNO's operational staff and management tools to identify the corresponding network status.

a)	Sure enough, there is a service outage in the mobile network corresponding to the service desk 'ticket' initiated by the DSO.

5a)	The MNO technical service desk engineer informs the DSO network operations center engineer of the incident and the expected time until resolution.

6a)	The DSO network operations center engineer initiates recovery procedures using a back up access system, if possible, or otherwise works to mitigate the impact of the outage.

b)	Alternatively, the MNO network is functioning as expected - there is no service outage in the mobile network corresponding to the service desk 'ticket' initiated by the DSO.

5b)	The MNO technical service desk engineer informs the DSO network operations center engineer that the incident is not related to MNO network performance degradation or service failure.

6b)	The DSO network operations engineer continues to work to identify the root cause of the failure, excluding the mobile network as the cause.

6c)	Eventually the source of the failure is identified and remedied.

Service flow result:

The DSO and MNO work to identify the root cause of the service failure or performance failure incident. If the incident's cause is in the MNO network, MNO operational staff and tools resolve the problem. If the incident's cause is in the DSO network, the DSO's operational staff and tools resolve the problem. Since there is no standard means for communicating the nature of the incident, the 'reporting' process may be complex and varies from operator to operator. This may result in extended resolution times for incidents.

NOTE: 	The 'standard means for communicating the nature of the incident' from the DSO to the MNO is not part of current practice and is not explained further in this use case. The point of this use case is to identify the absence of any such interface to establish a motivation, or 'gap' that can be pursued in additional use cases in this study.

### C.3.3	Potential requirements

NOTE: 	Currently, there is no interface between DSO and MNOs. This means that both systems work based on their own premises. Through the use of non-standard, MNO-specific interactions, the DSO can report problems and incidents and work with the MNO to resolve these.

Since the interaction described above uses non-standard means, this paragraph does not imply any requirement on the 3GPP system.

There are however synergies and potential improvements for building an interface. More investigation and analysis are needed to find appropriate use cases, see TR 22.867 [6] which includes relevant possible requirements to address this.

## C.4	Business use case Energy outage coordination current practice

### C.4.1	Description

Energy outage incidents occur as a result of storms, accidents (e.g. a distribution line is broken due to construction work) and other unforeseen factors beyond the control of the energy utility operator. When an energy outage occurs, it will affect a specific region. Service contracts and regulations make a rapid recovery of energy essential.

If the power grid is faulty, then the energy utility operator localizes the fault to solve the problem and perform necessary recovery actions. Some power grid equipment may also require “manual” actions for equipment that are not capable of remote management and automatic recovery. Other equipment can be remotely managed by means of smart energy services that allow recovery operations without requiring manual intervention.

There are several Smart Energy services that can greatly reduce the recovery time: SCADA for remote control and monitoring of distribution systems and Distribution Automation specifically are quite important. Without these services it is generally necessary to send a technician to affected sites. While Smart Energy services cannot entirely eliminate the possibility of manual intervention (e.g. to restore damaged cables, etc.) in many cases, hours of service interruption can be reduced to minutes or even seconds.

Smart Energy services depend on telecommunications, increasingly. If there is an energy outage that affects telecommunications services in the same area where recovery is required, then there is a 'vicious circle': The energy system requires telecommunications to support smart energy services for a rapid recovery. The telecommunication system requires energy services in order to function at all.

This use case considers how this unfortunate system is addressed today.

NOTE: 	An energy utility operator is a more general term than a DSO. For the purpose of the use case, the term DSO is more appropriate, while the more general term is applicable to the motivation above.

### C.4.2	Details

The ambition of a DSO is to provide an always-on electricity service, with the agreed quality and the costs fixed by Regulators. DSOs understand the criticality of some electricity services they provide (e.g., hospitals), and try to prioritize the availability of these services. This applies not only to business-as-usual situations, but to disaster recovery circumstances. There are plans to provide emergency electricity service through ancillary devices, but also to restore the service in selected areas considering priorities.

MNO services are also considered in these plans. Although in some countries many institutions (blue light services) do not depend on MNO networks for disaster recovery other countries use MNO networks for first responder services, and MNO public service is highly relevant for regular citizens. Thus, energy utility operators find that helping MNOs to recover their normal electricity service is relevant for the society.

A second aspect of the above is to be considered with the MNO service for energy utility operators. DSOs do not rely on MNO service to help recover the grid from a blackout, as they know that there is no certainty that the MNO will work when needed in a blackout.

It is acknowledged that it would be of great value, if upon blackout recovery circumstances, some procedures and supporting management operations existed to allow DSO service recovery through the MNO network.  Unfortunately, these procedures and standards specifically to support management operations coordination for recovery from energy outage events between DSO and MNO networks do not exist. See 3GPP TR 22.867 [6] for more details.

NOTE: It is acknowledged that 3GPP does not specify procedural standards.

Communication between DSOs and MNOs for resolving incidents might be necessary. As there are so many actors (DSOs and MNOs) and each DSO has only information about a limited part in energy sector, resolving the incident could be very complex.

### C.4.3 	Use case actors

DSO response team member:	A representative of the DSO's incident response team who is responsible for communication with service providers, officials and key customers during power cuts.

MNO service desk team member:	A representative of a mobile network operator's incident response team who can be reached by 'enterprise customers.'

NOTE: 	In the present document, the role 'MNO' is to be considered as equivalent to 'CSP+NOP' as defined in TS 28.530  [m] clause 4.8.

### C.4.4 	Use case service flow

1.	An energy system incident occurs in which a power cut results.

2.	A DSO response team member may contact a MNO service desk team member. The DSO informs the MNO of the incident and the expected time until its resolution. It is usually not required by regulations and often not required by service contract that the DSO inform the MNO except as they would 'any energy service customer.'

3.	The MNO may have the capability to continue to operate their network despite the power cut for a limited time, as they may support an uninterruptable power supply capability in the affected region. The DSO will not know this.

4.	The DSO may have the ability to restore energy services rapidly using Smart Energy Services. However, the DSO does not have any means to identify this opportunity nor IT support to coordinate their recovery with the MNO so as to provide communication services for this operation. Without communication service, the DSO cannot use their Smart Energy Services to accelerate recovery and has to send a technician to affect manual operations to restore electrical services.

Service flow result

The power cut is resolved manually. Even if there is the possibility to intervene using Smart Energy Services, lack of telecommunications service during the power cut prevents this. There is no standard based coordination of energy system recovery between the MNO and DSO, so only ad hoc communication regarding energy system recovery is possible.

### C.4.5	Potential requirements

There are no potential requirements.

## C.5	Business use case: MNO exposes network alarm

### C.5.1	Description

Motivation

In C.2, it was explained that DSOs require extremely high availability for communication in order to provide distribution automation and SCADA services to prevent energy service outages.

The sooner the DSO obtains information regarding a communication incident the better. (The term 'incident' is used in the sense described in Annex A, a service outage - either where no communication is possible, or the service levels required cannot be maintained.)

As described in C.2, a DSO can determine an outage by means of their own infrastructure. The DSO has many routers in their network. These provide networking within substation networks and have wireless access interfaces to connect the substation network over a wide area. These routers perform periodic monitoring operations, e.g. sending ICMP echo (ping) messages to ascertain latency and reachability. If the access fails entirely, it is impossible for the DSO management operations system to collect this monitoring information. The failure is only detected after the periodic monitoring operations fail.

It has to be taken into account that DSO could have tens of thousands of services like these. So, automation has to be used to improve availability.

Despite response time (including the identification of the root cause) for these kind of incidents should be less than two hours, DSO has to dedicate more time (8 hour or more) because there is no information from the MNO and it is supposed that all incidents that are not solved by their own in 8 hour, need to be addressed individually. After these 8 hours, DSO generate incidents automatically with the whole information captured automatically. These incidents are managed manually. There are remote or onsite operation to solve the incidents.  If DSO was able to receive information from MNO, this response time could be reduced dramatically to less than 1 hour, an availability of the network could be improved significantly

This use case considers the exposure of alarm messages from the MNO to the DSO to reduce this delay.

Background

See use case C.2.

### C.5.2	Details

Use Case Actors

DSO network operations center engineer:	The DSO network operations engineer is responsible for deploying monitoring and control mechanisms in the network. The DSO network operations center engineer determines how to control and configure the network for resiliency, e.g. when and how to switch between different accesses to maximize availability.

DSO electrical system operations center engineer: This actor is responsible for maintaining availability, efficiency and safety of the energy system.

Use case service flow

1. 	The energy system is monitored and managed by the DSO electrical system operations center engineer. It relies upon a functioning network to communicate with electrical utility equipment in substation networks.

2. 	There are a set of substation networks with routers that include a UE for wide area communications. The DSO network operations center engineer is responsible for this network. The DSO network operations center engineer employs monitoring mechanisms to determine when the telecommunications network is not available. These monitoring mechanisms are exposed by the MNO, so that authorized third parties (including the DSO) are able to receive alarm messages.

NOTE 1: 	It has to be taken into account that the DSO doesn’t need all alarms received in the MNO network. The DSO only needs to receive these alarms which could affect to the availability of their services.

NOTE 2: 	The alarms are provided at the network level, based on the overall statistical performance of the network. The alarms described here do not correspond to the performance of individual UEs or sessions.

3. The DSO network operations center engineer, using the exposed monitoring mechanism interface, can request which alarms, under which conditions, will be provided. The interface provides a means by which the alarms can be configured, including thresholds and parameters to report. These alarms can indicate the start and end of incidents.

Table C.5.2-1: Parameters for alarms

| Parameter | Configuration | Expected Behavior | Notes |
| --- | --- | --- | --- |
| latency | location (cell ID), incident threshold for 'start' in msec, incident threshold for 'stop' in msec. | If average latency for service crosses the thresholds, an alarm is sent. The alarm is provided by network exposure from the MNO. | The time of the alarm is crucial, so this information is included in the alarm delivered to the DSO. |
| throughput | location (cell ID), incident threshold for 'start' in Mbps, incident threshold for 'stop' in Mbps. | If average throughput for service crosses the thresholds, an alarm is sent. The alarm is provided by network exposure from the MNO.. | As above. |
| packet loss | location (cell ID), incident threshold for 'start' in packet loss ratio 'loss per million', incident threshold for 'stop' in packet loss ratio 'loss per million'. | If the average packet loss ratio for service crosses the thresholds, an alarm is sent. The alarm is provided by network exposure from the MNO.. | As above. |
| Cell outage | location (cell ID) | If service cannot be provided in a cell (of interest to the 3rd party) from the MNO. | As above |

4. An alarm is triggered because either

- 	The 3GPP management system determines that service has degraded with some KPIs (e.g. throughput, latency, etc.) above the threshold, or recovered so that they are below a threshold.

- 	The 3GPP management system determines that service is no longer available or has become available after having not been available.

Service flow result

When an incident occurs, an alarm is sent. Once the alarm is received, the DSO network operations center engineer (or an automated management function) may trigger a fail-over to a backup network communications facility. This immediate alarm from the network facilitates a rapid response by the DSO, reducing or even eliminating the interval in which the DSO substation network components are unreachable or only reachable with inadequate quality of service.

### C.5.3	Potential requirements

PR C.5.3-1. The 3GPP management system shall according to mobile network operator policy expose interfaces to third parties that provide the ability to:

a) enumerate alarms;

b) create alarms;

c) remove alarms;

d) configure alarms.

NOTE: 	The intention of this requirement is to enable DSOs to manage the set of alarms that the MNO will expose. That is, the DSO can create, configure, remove and enumerate the alarms that they request that the MNO expose to the DSO.

PR .C.5.3-2. The 3GPP management system shall according to mobile network operator policy expose interfaces to third parties that provide a mechanism for the mobile network operator to send alarms to the third party.

PR C.5.3-3. The 3GPP management system shall support the following alarms and associated configuration:

a) 	Maximum throughput threshold crossed  (where the trigger characteristic corresponds to measurement of [an average for the third party's network traffic]);

b) 	Maximum packet loss threshold crossed (where the trigger characteristic corresponds to measurement of [an average for the third party's network traffic]);

c) 	Cell outage (where the trigger characteristic corresponds to measurement of [where communication service is not possible at all on the third party]).

## C.6	Business use case: DSO provides performance information indicating relevant changes to its network

### C.6.1	Description

Motivation

As described in C.3, DSO reporting to MNO to resolve problems and incidents, current practice, and Annex A, incident reporting is a vital component of service management. The reporting of such problems today occurs very infrequently, e.g. when service agreements are created or renewed. This exchange of information could be very beneficial to an MNO and lead to considerations such as adjustment of coverage (e.g. for optimization), capacity planning (e.g. to react to growth in utilization in some part of the network), disaster recovery planning, and more.

Providing information from the DSO to the MNO serves a broader set of motivations as well. As the DSO's organization and use of their own network changes, their measured performance will change. This information may be important in distinguishing whether differences in network usage arise from performance problems or instead whether the changes are intentional on the part of the DSO.

The DSO is in a unique position to provide information to an MNO regarding their service. The DSO not only has many deployed stationary UEs, but performs significant performance management for these and requires extremely high availability. The DSO therefore has a very detailed set of information gathered over time, across the MNO's network. Specifically, the DSO identifies where there are problems for their own purposes, for the purposes of proactive response to increase availability. (See C.3)

This 'problem' or 'utilization' information is in the interest of the DSO to share, as it could lead the MNO to consider service improvements or to identify issues that they may not have otherwise detected.  Providing problem analysis information is an example of mutual advantage for both the customer and the service provider. In any case, if such information is needed by both sides, a standard interface for providing it will reduce the informal communication needed by both parties.

Background

See C.3.

### C.6.2	Details

Problem Management processes, by which the DSO works with the MNO to address such issues as observed erratic or declining performance, occur very infrequently today. Problem Management results in concrete proposals for improvements, essentially as input for processes such as capacity planning and adjustments to service level agreements. Problem management therefore results in changes that need to be considered over time, decisions that take months or years to conclude.

The opportunities to analyze problems that emerge from time to time in energy utility service provider communication networks can be analyzed by the energy utility service providers in cooperation with the MNOs to ascertain the root cause of unexpectedly low or declining performance levels. This network problem discovery, trend analysis and identification of root causes of problems is, as a result of sharing information and conferring with MNOs, made easier for energy utility service providers.

Use case actors:

DSO network operations center engineer: The DSO network operations center engineer analyzes collected information, creates and operates network monitoring tools including alarms, and seeks to identify areas of potential improvement in service delivery.

MNO technical service / account manager: The MNO technical service / account manager responds to queries from the energy utility service provider when needed. Such questions may arise when problems emerge, e.g. as network performance or avaialbility appears to decline in the DSO's network.

Use case service flow:

1)	Using the DSO operational tools, the DSO o network perations center engineer gathers and analyzes data, seeking to identify trends that appear threatening to the proper function of the energy utility service provider network. An example of such a trend would be a gradual decline in performance parameters including observed increasing latency or jitter, or gradual decline in availability. Note that these parameters' decline even while remaining within the expectations of the service level agreement may merit investigation if the decline is prolonged and pronounced.

2a)	When such trends are identified, the DSO network operations center engineer uses tools to study the details of the potential problem.

2b)	Another goal is to identify where there are not problems, despite data that appears to indicate problems. In some cases, when less or no traffic is sent or received over the mobile telecommunications network this may be a decision of the customer.

3)	The DSO network operations center engineer shares this information with the MNO by means of a standard interface, including the measured performance, physical location, Cell ID and other information.

NOTE:	The physical location, Cell ID and other information is available to the MNO. It is given by the DSO in order to provide sufficient information that the observed performance information provided by the DSO can effectively be correlated and compared to management information collected by the MNO.

4a)	The 'root cause' of the problem may need to be investigated by the DSO before identifying a potential remedy is possible, or determining how high a priority the mitigation has. This process can be accelerated and eased by collaboration and interaction between the DSO network operations center engineer and the MNO technical service representative. The communication between the two organizations is based on any standard data model which eases communication and determining answers to essential questions for both the MNO technical service and DSO operations engineers.

4b)	The apparent problem (reduced or zero network utilization) can be flagged to the MNO as 'intended'.

Service flow result:

The DSO learns about performance problems in their network. By sharing these problems in a standardized manner with the MNO, it is possible to more straightforwardly ascertain whether the issues are also present in the mobile network. The MNO benefits from information acertained by the DSO about network performance issues.

The DSO is able to differentiate in the information it provides to the MNO between reduced performance in the network and intentional reduced utilization of the network. This will ease interpetation of the data by the MNO.

### C.6.3	Potential requirements

PR C.6.3-1. Subject to operator policies, regulatory requirements and contractual obligations, the 3GPP management system may support a means for authorized third parties to provide network performance information to an MNO. The data model of the performance information provided by the DSO to the MNO shall include at least the following elements: {measurement granularity, location of measurement, latency, packet loss, throughput.}

a) Latency between the DSO’s device and the DSO’s server the device is communicating with [an average for the DSO's network traffic];

NOTE 1:	Latency measurement end to end will include delay arising from a network outside the mobile telecommunication system.

b) Throughput [an average for the DSO's network traffic];

c) Packet loss [an average for the DSO's network traffic];

NOTE 2: 	Packet loss can be determined by means of observation of ICMP and TCP control information for traffic, sending probes, etc. This methodology is out of scope of 3GPP.

NOTE 3:	The information elements to include in the incident report will be further clarified during the 'solution definition' stage of the present document.

NOTE 4:	The measurements provided by the DSO can be acquired by means that are outside the scope of 3GPP. Measurements may be acquired from management MIBs of routers that are operated by the DSO.

## C.7	Business use case: DSO reports an incident to MNO

### C.7.1	Description

Motivation

As described in C.3, DSO reporting to MNO to resolve problems and incidents, current practice, and Annex A, incident reporting is a vital component of service management. Currently incident reporting is done through diverse mechanisms specific to each MNO, such as a voice call to a service desk or a web-based form. While such processes are out of scope of 3GPP and are vital to support of mobile subscribers in general, they are not well suited to a DSO. The DSO operates 100s of 1000s of UEs often with dual SIM configurations, in some cases in multiple countries. This results in a high degree of complexity for the DSO in terms of divergent procedures and manual effort required. A specific aspect of this challenge for DSOs is that the terminology, units and other aspects of incident reports differ between operators. Reporting incidents using 'business to consumer' oriented interfaces is not only cumbersome; it does not scale up to the number of concurrent incidents that can arise for a DSO.

Furthermore, the availability requirements of a mobile consumer are different from that of an energy utility service provider. It is essential that the DSO network operations network operations staff be able to ascertain the root cause of an incident as quickly as possible. If the root cause is an incident in the mobile telecommunications network, and this information is readily available, this will help to focus the recovery efforts in the DSO. If on the other hand, there is no known mobile telecommunications network failure, the DSO can focus their recovery efforts on their own network.

The intention of this use case is to identify how standardized interfaces can facilitate large scale, rapid and uniform incident reports from a DSO to an MNO. This use case does not consider how to automate or standardize the incident reporting process since process standardization is out of scope of 3GPP and it is recognized that different operators have unique and specific business processes. As incident reporting mechanisms will vary between operators, only a 'least common subset' of reporting information is identified in the requirement, and a 'least common subset' of responses to these reports.

Background

See C.3.

### C.7.2	Details

Incident management is triggered when there is a performance incident - either a service is delivered below the service level agreement parameters or there is a service outage. In order to qualify as an 'incident' some parameters may apply (how long it is sustained, how frequently it occurs, etc.) according to the service level agreement, the details of which are out of scope of 3GPP.

When an incident occurs, this triggers processes to end the service performance failure or service outage as quickly as possible. The DSO needs to diagnose the root cause of the incident and take the necessary steps to recover service. To the extent that the service failure is attributed to the mobile network, or the root cause is unknown and could be due to service problems in the mobile network, the DSO works together with the MNO to diagnose the incident and resolve it.

The term 'service desk' represents the service provider contact that is available when incidents occur. The service desk can take many forms and interactions and interfaces are not standardized in current practice.

While resolution is needed on the order of seconds or minutes, communication and collaboration between the DSO and MNO can take much longer, sometimes hours even days. A significant amount of time is needed to describe the incident in terms that enables the MNO to take action. The lack of standards in this area means that the DSO has to identify the reporting formats of each MNO separately, which is labor intensive especially for DSOs that operate across national borders and therefore have large numbers of subscriptions with each operator in each country they operate.

Use case actors:

DSO network operations centre engineer: The DSO network operations centre engineer analyses collected information, creates and operates network monitoring tools including alarms, and seeks to identify areas of potential improvement in service delivery.

MNO technical service desk engineer: The MNO is represented by this actor, whose role is to capture the incident information, analyse the MNO's management system to find corresponding relevant information and to work with the customer who reports the incident and MNO operational staff to resolve the incident if it is indeed an issue in the mobile network.

Use case service flow:

1)	The DSO operational management system indicates a service outage in the DSO network. The DSO network operations centre engineer analyses the alarm.

2)	The root cause of the service outage is not clear. The DSO operations center engineer initiates a service desk procedure with the MNO providing service for the network that has a service outage. This service desk procedure includes submission of a standard form for incident reporting that can be provided by the DSO by means of an interface exposed by the MNO.

3)	The MNO technical service desk engineer obtains information from the DSO network operations centre engineer, principally by means of the exposed interface. If needed, other means of communication are also provided by the MNO service desk (e.g. web based reporting tools), partly through other forms of communication (phone, messages, etc.)

4)	The MNO technical service desk engineer works with the MNO's operational staff and management tools to identify the corresponding network status.

a)	Sure enough, there is a service outage in the mobile network corresponding to the service desk 'ticket' initiated by the DSO.

a.1)	The MNO technical service desk engineer informs the DSO network operations centre engineer of the incident and the expected time until resolution, by means of the standard form for incident response.

a.2)	The DSO network operations centre engineer takes the appropriate action, e.g. initiates recovery procedures using a back up access system, if possible, or otherwise works to mitigate the impact of the outage.

b)	Alternatively, the MNO network is functioning as expected - there is no service outage in the mobile netwo

b.1)	The MNO technical service desk engineer informs the DSO network operations centre engineer by means of the standard form for incident response that the incident is not related to MNO network performance degradation or service failure.

b.2)	The DSO network operations engineer continues to work to identify the root cause of the failure, excluding the mobile network as the cause.

b.3)	Eventually the source of the failure is identified and remedied.

A quick and standard exchange of information can provide valuable data to both sides minimising the request for further information during the troubleshooting of the incident.

### C.7.3	Potential requirements

PR C.7.3-1. Subject to operator policies, regulatory requirements and contractual obligations, the 3GPP management system shall support a means to expose a standard interface for incident reporting by authorized DSOs. The process of incident reporting itself is out of scope of this requirement as this is a business process. The information that the present document contains is intended to be a 'common subset' supported by incident reporting procedures and tools today.

NOTE 1:	An example of such information elements includes: {UE Identifier e.g. IMSI, performance characteristics that constitute a service level failure, time of start of incident, location of UE, cell ID serving the UE, ... }

NOTE 2: 	More than one incident report may be submitted, e.g. as more information is learned by the customer.

NOTE 3:	The information elements to include in the incident report will be defined and evaluated during the 'solution description' phase. These information elements are expected to include {UE Identifier e.g. IMSI, performance characteristics that constitute a service level failure, time of start of incident, location of UE, cell ID serving the UE, . . . }

NOTE 4:	This requirement does not imply that the incident reporting interface required will replace existing tools or processes. The 'common subset' of information defined for the incident reporting interface does not limit incident reporting by the customer to the operator to this subset - additional information will be collected by tools and/or interfaces as well.

PR C.7.3-2. Subject to operator policies, regulatory requirements and contractual obligations, the 3GPP management system shall support a means to expose a standard interface for incident response to authorized DSOs. The process of responding to incident reports itself is out of scope of this requirement as this is a business process. The information that this response contains is intended to be a 'common subset' supported by incident responses procedures and tools today.

NOTE 5:	An example of such information elements includes: {incident does or does not correspond to known fault or performance degradation in the mobile network, expected time of restoration of service, . . .}

NOTE 6: 	More than one incident reporting response may be sent to the customer, e.g. as more information is learned by the MNO.

NOTE 7:	The information elements to include in the incident report will be defined and evaluated during the 'solution description' phase. These information elements are expected to include {incident does or does not correspond to known fault or performance degradation in the mobile network, expected time of restoration of service, . . .}

NOTE 8:	This requirement does not imply that the incident response interface required will replace existing tools or processes. The 'common subset' of information defined for the incident response interface does not limit incident response by the operator to the costomer to this subset - additional information will be provided by tools and/or interfaces as well.

NOTE 9: 	All requirements above concerning the MNO apply to the site operator. In some network sharing scenarios the site operator could be an entity other than an MNO. In active network sharing scenarios the DSO can communicate with an MNO that operates the site, but can be distinct from the serving MNO.

Next Change

# Annex D: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 04-2022 | SA5#142e | S5-222671 S5-222672 S5-222019 S5-222020 S5-222016 |  |  |  | Update to implement the agreed pCRs in SA5#142e: 1. S5-222671_Rel-18 TR 28.829-Cover-Sheet 2. S5-222672_TR 28.829-Structure 3. S5-222019 Rel-18 pCR TR 28.829-Scope-02 4. S5-222020 Rel-18 pCR TR 28.829-Overview-05 5. S5-222016 Rel-18 pCR TR 28.829-Annex-B-IT Services Model-03 | 0.1.0 |
| 05-2022 | SA5#143e | S5-223611 S5-223612 S5-223613 |  |  |  | Update to implement the agreed pCRs in SA5#143e: 1. S5-223611 pCR TR 28.829 Business use case - MNO provides performance info 2. S5-223612 pCR TR 28.829 Business use case - Utility provides performance and failure info 3. S5-223613 pCR TR 28.829 Business use case - Energy outage coordination | 0.2.0 |
| 07-2022 | SA5#144e | S5-223613 S5-224434 S5-224434 |  |  |  | Update 0.2.0 version to properly implement S5-223613 from SA5#143e: a change missed in (definitions were added but not implemented), style corrected in 6.1.1, 6.3.1 and 6.3.2. Reference corrected in 6.2.3. Update to implement the agreed pCRs in SA5#144e 1. S5-224434 2. S5-224435 | 0.3.0 |
| 08-2022 | SA5#145e | S5-225865 S5-225866 S5-225867 S5-225868 |  |  |  | Rel-18 pCR 28.829 KI and solution for MNO exposes Network Performance pCR TR 28.829 Business use case - DSO Provides Performance Reporting indicating Problems pCR TR 28.829 Business use case - DSO Provides an Incident Report pCR TR 28.829 Clean up | 0.4.0 |
| 11-2022 | SA5#146 | S5-226808 S5-226809 |  |  |  | Rel-18 pCR 28.829 Corrections Rel-18 pCR 28.829 Use case on Energy System Recovery | 0.5.0 |
| 2023-03 | SA5#147 | S5-232850 S5-232851 S5-232852 S5-232657 S5-232854 S5-232856 S5-232857 S5-232671 S5-232858 S5-232673 S5-232859 S5-232675 S5-232855 S5-232678 S5-232896 S5-232898 S5-232894 S5-232895 S5-233065 |  |  |  | Rel-18 pCR 28.829 Completion of 7.1.2.1 Rel-18 pCR 28.829 Clean Up 6.1, 6.3 Rel-18 pCR 28.829 Update 6.4, 6.5, 6.6 Rel-18 pCR 28.829 Discontinuing work on objective #2 Rel-18 pCR 28.829 – Update 6.7 Rel-18 pCR 28.829 – Update 6.8 Rel-18 pCR 28.829 – Update 6.1 Rel-18 pCR 28.829 – Update 6.2 Rel-18 pCR 28.829 – Update 6.3 Rel-18 pCR 28.829 – Update 6.4 Rel-18 pCR 28.829 – Update 6.5 Rel-18 pCR 28.829 – Update 6.6 Rel-18 pCR 28.829 – Update 6.8 Rel-18 pCR 28.829 – Update 7 Rel-18 pCR 28.829 – Key Issue for Energy utility and telecommunication coordinated rapid recovery of energy service Rel-18 pCR 28.829 – New use case for Rapid Intervention for Outages without Redundant Topology Rel-18 pCR 28.829 – Conclusion and Recommendation for Key Issue 1 - MNO exposes Network Performance Monitoring to DSO Rel-18 pCR 28.829 – Conclusion and Recommendation for Key Issue i (energy utility and telecommunication coordinated recovery of energy service) 28829 pCR Annex on NSOEU Applicability | 0.6.0 |
| 2023-03 | SA#99 | SP-230191 |  |  |  | Presented for information | 1.0.0 |
| 2023-04 | SA5#148e |  |  |  |  |  | 1.1.0 |

Next Change

## 6.1	Business use case: DSO obtains network performance and outage information, current practice

### 6.1.1	Description

Motivation

When there is an electrical outage, the electrical service operator strives to restore service as quickly as possible. There are several reasons for the need for rapid recovery from an interruption of electrical service:

-	In some countries, regulations require rapid recovery and penalize an Energy Utility service provider for any time in which services does not operate. For example, at the least, in many countries customers do not have to pay for electrical service when it is not available.

-	Interruptions in electrical service can be expensive, as businesses often require electricity for operations, manufacturing and to properly store valuable products. Thus, electrical outages can translate directly into business losses (of productivity or inventory.)

-	Power outages for hospitals and care facilities can result in harm or even death to patients.

NOTE 1: 	Regulations identify 'critical' electricity customers which are obliged to install and maintain secure sources (e.g. local generators.) Public mobile network operators in some cases are not covered by these regulations. Further, public mobile network operators are generally not considered 'critical' electricity customers by regulation, so Energy Utility service providers cannot prioritize service to these customers.

-	Electrical service outages affect many customers, so failure of service is not comparable in terms of business consequences to outages of mobile telecommunication service to a single customer.

For this reason, there are regulations that make energy service availability the highest priority. In order to achieve this, Smart Energy services such as protection, SCADA and Distribution Automation are used to monitor and adjust distribution equipment to avoid incidents that would reduce energy service availability.

As a point of comparison, a fiber optic access service is often offered with an availability of at least 99.999%. Telecommunication systems may not achieve this level of service availability. Since telecommunications offers an alternative to fixed fiber optic access, additional means to achieve high degrees of availability are essential to the DSO.

Background

To achieve extreme telecommunication service availability, it is currently not feasible to rely on a single telecommunication network. Instead, DSOs networks employ communication access equipment that have multiple USIMs. If one telecommunication service provider is not available, the second can be used. However, this arrangement ('failover') is insufficient, as it requires in practice 2 minutes or more to bring up a secondary USIM and register with a back up network.

NOTE 2: 	An electrical service operator is a more general term than a DSO. For the purpose for the purpose of the use case, the term DSO is more appropriate, while the more general term is applicable to the motivation above.

To prevent an outage that will last an hour or more, Distribution Automation must be used to intervene in the first minutes, ideally in the first seconds, in which an outage occurs. The following examples show two outages and can be considered characteristic of the prospects of resolution in most situations.

In Figure 1, an incident affecting an underground Medium Voltage (MV) line eliminated service to 4223 customers. The existence of DA in secondary substations along the line allowed the fault to be isolated quickly and then resolved. This dramatically reduced the service outage duration for a substantial number of customers. A few customers that were along a line without DA access required local operation that took more than one hour. (The time scale on the X axis is not to scale.)

Figure 1: Incident Example

Over 50% of the customers could have their service restored in 3 minutes. Another 40% of the customers had their service restored in under 10 minutes. The remaining roughly 10% of the customers required manual intervention in order to have their service restored. The rapid service restoration saved EUR 1000s in saved penalties as well as needing only one service truck to roll.

In Figure 2, another example, another MV line was damaged, again showing a complication of a line that did not offer the possibility of DA intervention.

Figure 2: Incident Example featuring a time consuming recovery for a minority of customers

This incident affected 5292 customers. The outage lasted 7 hours but it can be observed that most of the customers could have their service restored within the first minutes due to remote access to primary and secondary substations and intervention using DA. In this incident, EUR 10000s could be saved.

Considering the importance of rapid response during a power outage, it is important that the communication facility is available at the time of an outage. If communication failure only is ascertained during an incident, the outage duration can be extended significantly for most of the affected customers.

In order to improve availability, some DSOs use dual USIM UE deployments. In practice, it takes on the order of 2 minutes to bring up service on an alternate mobile network. These minutes, if they coincide with an outage, are expensive both in terms of penalties and in terms of problems faced by customers during the outage. This also uses up roughly half of the ‘downtime budget’ of 99.999% availability (5 minutes per year unscheduled downtime maximum.)

Communication links are tested, e.g. every minute by means of ping messages end to end, to identify availability and latency. The effective availability of 3GPP telecommunications networks observed in practice can be more like 98.5% (where availability means ability to achieve communication with the expectations of performance according to the service level agreement. While this is far below the levels we expect from 5G and that we cite in stage 1 requirements, the fact is that observations of performance of past generations indicate that in order to achieve the target availability, information is needed. Please see TS 22.104 [14], Annex A.4.

NOTE 3: 	The actual availability of mobile network services is not exposed to energy utility service provider (customers), nor is the cause of the availability limit, e.g. limited capacity, radio quality issues, etc. This makes it difficult for energy utilities to perform risk assessment and network planning for communication services to carry their smart energy services.

Communication performance failure, based on extensive field experience, can be correlated with network performance events. Network performance can be compared to historic information indicating failures. In this case when communication with a ‘primary’ PLMN shows signs of declining performance, an ‘alternate’ communication channel can be employed, e.g. registration with another PLMN for a multi-USIM device. This action can be performed proactively, so that in the event that the ‘primary’ communication session does fail to deliver required performance, recovery can occur quickly (within seconds) to the ‘alternative’ communication session.

### 6.1.2	Details

There is currently no mechanism to convey network or services status from the MNO to the DSO. While the DSO can access real information from their CPEs (radio module or connectivity state, e.g.), no information comes from the MNO. Thus, in case service failure is happening in an area, the DSO can just infer this situation through a qualified guess.

DSOs have alternative telecommunication connectivity possibilities in any given site. This connectivity can be provided by private networks (different to the MNOs’), or can be provided via alternative MNOs (another CPE in the same site; several SIM cards in the same CPE; etc.). Many of the sites where DSOs need connectivity are mission critical; this means that the service must be highly available. If the DSO is forced to use a standard MNO service, with no specific availability target, and no mechanism to know that service may be failing in an area, the DSO will not rely in MNO services and will develop alternative telecommunications infrastructure that will eventually make MNOs’ services unnecessary.

The MNO has information on the state of its infrastructure. From the Core to the Radio Access Network, and through the different intelligent elements that make the 3GPP standards-based MNO network provide their services, the MNO can tell which network or service parts may be showing a degraded performance or potential unavailability. This information will enhance the MNO service visibility for specific user groups that may benefit from this, while making sure that some of them do not develop alternative solutions they can fully control.

Between the complete lack of information today, and the total control that MNO have of their network infrastructure, and that eventually may reach all the services provided to their customers, there is room to find added value service status information for specific user groups.

Use case actors

DSO network operations center engineer:	The DSO operations engineer is responsible for deploying monitoring and control mechanisms in the network. The DSO operations center engineer determines how to control and configure the network for resiliency, e.g. when and how to switch between different accesses to maximize availability.

DSO electrical system operations center engineer: This actor is responsible for maintaining availability, efficiency and safety of the energy system.

Use case service flow

0.	The energy system is monitored and managed by the electrical system engineer. It relies upon the telecommunications network.

1.	The DSO network operations center engineer deploys monitoring and control mechanisms to determine when a telecommunications network is not available.

2.	There is no standard way to obtain network availability or performance information, so the DSO network operations center engineer must rely upon 'over the top' monitoring processes. In order to ensure that these monitoring processes do not use a significant amount of the communications capacity, the monitoring relies on periodic probes, e.g. ICMP messages ('pings') every minute. This is a very 'coarse grain' monitoring process, since a failure or performance degradation may take a significant amount of time to detect.

3a.	The DSO network operations center engineer may manually trigger or set up a configuration that will automatically trigger a switch between one telecommunication network to a back-up communication access.

3b. The DSO network operations center engineer is not successful to initiate and complete a telecommumications network switch because insufficient information was available to the network operations center to predict the event so the network fails before fail-over can occur, i.e. for more than 2 minutes. This means that the fail-over occurs during a time in which no communication is possible. This results in a network failure visible to the energy system engineer who requires communication for smart energy services. If a fault or protection problem for example occurs in that time the failure can cause damage to the energy system, inefficiencies and even outages. Especially if control is needed (e.g. critical SCADA) during this time frame, a serious energy system outage could occur.

Service flow result

Since the information that the DSO network operations center engineer relies upon is of coarse granularity, there is a high likelihood that performance degradation or failure will be detected after several seconds from the point when they occurred. This extends the period of time in which communication service will be inadequate or unavailable before a secondary communication access can be made available. This reduces the availability of the energy system because, should an energy outage or instability occur during the time in which there is no communication possible, or inadequate performing communication this will lead to an energy service outage as described above.

### 6.1.3	Potential requirements

## There are no potential requirements.

## 6.2	Business use case: DSO reporting to MNO to resolve problems and incidents, current practice

### 6.2.1	Description

Annex A of this document describes two processes, Problem Management and Incident Management. In current practice, these processes require interaction between the DSO and MNO, but the interaction is specialized, entirely depending on the processes of each individual MNO.

Problems, generally an observed decline or lack of expected service in certain scenarios over a period of time, need to be addressed or they will likely lead to incidents (where service fails completely or degrades so that service levels do not meet agreed key performance indicator objectives). Problem management allows interaction between a service provider and service customer to address longer term issues - such as capacity planning, disaster recovery or, most relevant, analysis of problems to identify their root cause. If the observed communication problem in the energy utility service provider's network is decisively not the MNO's service and network, this will greatly help the energy utility service provider to diagnose and resolve the problem. If, on the other hand, it is the result of the MNO's service and network, this realization can lead to more rapid response.

Incidents, a service failure, are resolved by means of specific procedures defined by service providers. The goal of the customer and service provider is to resolve incidents as quickly as possible.

It is acknowledged that the specifics of the IT processes for Problem Management and Incident Management are out of scope of 3GPP standards. It is however possible that the diversity of interfaces used to report information and the format of those reports can be standardized in 3GPP.

Since an energy utility service provider has to work with many MNOs, especially if the energy utility service provider operates across national borders, the lack of standards in this area for providing information regarding a problem or incident brings complexity and can delay the resolution process.

Energy utilities have vast deployments of communicating devices in hundreds or thousands of static sites. The communication status of each of these devices is constantly monitored, especially their availability, but also latency and other performance metrics. This data, currently collected by energy utility service providers to identify problems and incidents in their own networks, could potentially be very useful to mobile network operators. This information is currently not shared by energy utilities with network operators.

NOTE: 	An energy utility service operator is a more general term than a DSO. For the purpose for the purpose of the use case, the term DSO is more appropriate, as it concerns the operation of energy distribution services, while the more general term is applicable to the motivation above. This use case focusses on energy distribution not generation, transmission or consumption. Though each of these energy system components has relevant aspects for 'smart energy,' we focus on distribution, where communication availability is absolutely critical for stability and recovery of service to customers.

### 6.2.2	Details

Use case I: Energy System Service Provider Network Problem Report

Problem Management processes, by which the DSO works with the MNO to address such communication issues as observed erratic or declining performance occurs very infrequently today, with resolution and improvements for processes such as capacity planning and adjustments to service level agreements taking months or years. The opportunities to analyze problems that emerge from time to time in energy utility service provider networks essentially have to be analyzed by the energy utility service providers independently, without cooperation with the MNOs to ascertain the root cause of failure. This network problem discovery, trend analysis and identification of root causes of problems is very labor intensive for energy utility service providers.

Use case actors:

DSO network operations center engineer: The DSO operations center engineer analyzes collected information, creates and operates network monitoring tools including alarms, and seeks to identify areas of potential improvement in service delivery.

MNO technical service / account manager: The MNO operations center service representative responds to queries from the energy utility service provider when needed. Such questions may arise when problems emerge, e.g. as network performance or availability appears to decline in the customer's network.

Use case service flow:

1)	Using the DSO operational tools, the DSO network operations center engineer gathers and analyzes data, seeking to identify trends that appear threatening to the proper function of the energy utility service provider network.

2)	When such trends are identified, the DSO network operations center engineer uses tools to study the details of the potential problem.

3)	The 'root cause' of the problem may need to be investigated before identifying a potential remedy is possible, or determining how high a priority the mitigation has. It is possible at that time that the DSO network operations center engineer contacts the MNO technical service representative with particular questions. The communication between the two organizations is not based on any standard data model - it requires ad hoc consultation which is labor intensive for both the MNO technical service and DSO operations engineers.

Service flow result:

The DSO learns about performance problems in their network. With consultation, it is possible to ascertain whether the issues are also present in the mobile network. This consultation is complicated by the lack of standard ways to exchange management information regarding network problems. The MNO does not benefit from information ascertained by the DSO about network performance issues except through these consultations.

Use case II: Energy System Service Provider Network Incident Report

Incident management is triggered when there is a performance incident - either a service is delivered below the service level agreement parameters or there is a service outage. In order to qualify as an 'incident' some parameters may apply (how long it is sustained, how frequently it occurs, etc.) according to the service level agreement, the details of which are out of scope of 3GPP.

When an incident occurs, this triggers processes to end the service performance failure or service outage as quickly as possible. The DSO needs to diagnose the root cause of the incident and take the necessary steps to recover service. To the extent that the service failure is attributed to the mobile network, or the root cause is unknown and could be due to service problems in the mobile network, the DSO works together with the MNO to diagnose the incident and resolve it.

The term 'service desk' represents the service provider contact that is available when incidents occur. The service desk can take many forms and interactions and interfaces are not standardized in current practice.

While resolution is needed on the order of seconds or minutes, communication and collaboration between the DSO and MNO can take much longer, sometimes hours even days. A significant amount of time is needed to describe the incident in terms that enables the MNO to take action. The lack of standards in this area means that the DSO has to identify the reporting formats of each MNO separately, which is labor intensive especially for DSOs that operate across national borders and therefore have large numbers of subscriptions with each operator in each country they operate.

Use case actors:

DSO network operations center engineer: The DSO operations center engineer analyzes collected information, creates and operates network monitoring tools including alarms, and seeks to identify areas of potential improvement in service delivery.

MNO technical service desk engineer: The MNO is represented by this actor, whose role is to capture the incident information, analyze the MNO's management system to find corresponding relevant information and to work with the customer who reports the incident and MNO operational staff to resolve the incident if it is indeed an issue in the mobile network.

Use case service flow:

1)	The DSO network operational management system indicates a service outage in the DSO network. The DSO operations center engineer analyzes the alarm.

2)	The root cause of the service outage is not clear. The DSO network operations center engineer initiates a service desk procedure with the MNO providing service for the network that has a service outage.

3)	The MNO technical service desk engineer gathers information from the DSO network operations center engineer, partly through tools provided by the MNO service desk (e.g. web based reporting tools), partly through other forms of communication (phone, messages, etc.)

4)	The MNO technical service desk engineer works with the MNO's operational staff and management tools to identify the corresponding network status.

a)	Sure enough, there is a service outage in the mobile network corresponding to the service desk 'ticket' initiated by the DSO.

5a)	The MNO technical service desk engineer informs the DSO network operations center engineer of the incident and the expected time until resolution.

6a)	The DSO network operations center engineer initiates recovery procedures using a back up access system, if possible, or otherwise works to mitigate the impact of the outage.

b)	Alternatively, the MNO network is functioning as expected - there is no service outage in the mobile network corresponding to the service desk 'ticket' initiated by the DSO.

5b)	The MNO technical service desk engineer informs the DSO network operations center engineer that the incident is not related to MNO network performance degradation or service failure.

6b)	The DSO network operations engineer continues to work to identify the root cause of the failure, excluding the mobile network as the cause.

6c)	Eventually the source of the failure is identified and remedied.

Service flow result:

The DSO and MNO work to identify the root cause of the service failure or performance failure incident. If the incident's cause is in the MNO network, MNO operational staff and tools resolve the problem. If the incident's cause is in the DSO network, the DSO's operational staff and tools resolve the problem. Since there is no standard means for communicating the nature of the incident, the 'reporting' process may be complex and varies from operator to operator. This may result in extended resolution times for incidents.

NOTE: 	The 'standard means for communicating the nature of the incident' from the DSO to the MNO is not part of current practice and is not explained further in this use case. The point of this use case is to identify the absence of any such interface to establish a motivation, or 'gap' that can be pursued in additional use cases in this study.

### 6.2.3	Potential requirements

NOTE: 	Currently, there is no interface between DSO and MNOs. This means that both systems work based on their own premises. Through the use of non-standard, MNO-specific interactions, the DSO can report problems and incidents and work with the MNO to resolve these.

Since the interaction described above uses non-standard means, this paragraph does not imply any requirement on the 3GPP system.

There are however synergies and potential improvements for building an interface. More investigation and analysis are needed to find appropriate use cases, see 3GPP TR 22.867 [11] which includes relevant possible requirements to address this

## 6.3	Business use case Energy Outage Coordination current practice

### 6.3.1	Description

Energy outage incidents occur as a result of storms, accidents (e.g. a distribution line is broken due to construction work) and other unforeseen factors beyond the control of the energy utility operator. When an energy outage occurs, it will affect a specific region. Service contracts and regulations make a rapid recovery of energy essential.

If the power grid is faulty, then the energy utility operator localizes the fault to solve the problem and perform necessary recovery actions. Some power grid equipment may also require “manual” actions for equipment that are not capable of remote management and automatic recovery. Other equipment can be remotely managed by means of smart energy services that allow recovery operations without requiring manual intervention.

There are several Smart Energy services that can greatly reduce the recovery time: SCADA for remote control and monitoring of distribution systems and Distribution Automation specifically are quite important. Without these services it is generally necessary to send a technician to affected sites. While Smart Energy services cannot entirely eliminate the possibility of manual intervention (e.g. to restore damaged cables, etc.) in many cases, hours of service interruption can be reduced to minutes or even seconds.

Smart Energy services depend on telecommunications, increasingly. If there is an energy outage that affects telecommunications services in the same area where recovery is required, then there is a 'vicious circle': The energy system requires telecommunications to support smart energy services for a rapid recovery. The telecommunication system requires energy services in order to function at all.

This use case considers how this unfortunate system is addressed today.

NOTE: 	An energy utility operator is a more general term than a DSO. For the purpose of the use case, the term DSO is more appropriate, while the more general term is applicable to the motivation above.

### 6.3.2	Details

The ambition of a DSO is to provide an always-on electricity service, with the agreed quality and the costs fixed by Regulators. DSOs understand the criticality of some electricity services they provide (e.g., hospitals), and try to prioritize the availability of these services. This applies not only to business-as-usual situations, but to disaster recovery circumstances. There are plans to provide emergency electricity service through ancillary devices, but also to restore the service in selected areas considering priorities.

MNO services are also considered in these plans. Although in some countries many institutions (blue light services) do not depend on MNO networks for disaster recovery other countries use MNO networks for first responder services, and MNO public service is highly relevant for regular citizens. Thus, energy utility operators find that helping MNOs to recover their normal electricity service is relevant for the society.

A second aspect of the above is to be considered with the MNO service for energy utility operators. DSOs do not rely on MNO service to help recover the grid from a blackout, as they know that there is no certainty that the MNO will work when needed in a blackout.

It is acknowledged that it would be of great value, if upon blackout recovery circumstances, some procedures and supporting management operations existed to allow DSO service recovery through the MNO network.  Unfortunately, these procedures and standards specifically to support management operations coordination for recovery from energy outage events between DSO and MNO networks do not exist. See 3GPP TR 22.867 [11] for more details.

NOTE: It is acknowledged that 3GPP does not specify procedural standards.

Communication between DSOs and MNOs for resolving incidents might be necessary. As there are so many actors (DSOs and MNOs) and each DSO has only information about a limited part in energy sector, resolving the incident could be very complex.

### 6.3.3 	Use case actors

DSO response team member:	A representative of the DSO's incident response team who is responsible for communication with service providers, officials and key customers during power cuts.

MNO service desk team member:	A representative of a mobile network operator's incident response team who can be reached by 'enterprise customers.'

NOTE: 	In this TR, the role 'MNO' is to be considered as equivalent to 'CSP+NOP' as defined in TS 28.530 clause 4.8.

### 6.3.4 	Use case service flow

1.	An energy system incident occurs in which a power cut results.

2.	A DSO response team member may contact a MNO service desk team member. The DSO informs the MNO of the incident and the expected time until its resolution. It is usually not required by regulations and often not required by service contract that the DSO inform the MNO except as they would 'any energy service customer.'

3.	The MNO may have the capability to continue to operate their network despite the power cut for a limited time, as they may support an uninterruptable power supply capability in the affected region. The DSO will not know this.

4.	The DSO may have the ability to restore energy services rapidly using Smart Energy Services. However, the DSO does not have any means to identify this opportunity nor IT support to coordinate their recovery with the MNO so as to provide communication services for this operation. Without communication service, the DSO cannot use their Smart Energy Services to accelerate recovery and must send a technician to affect manual operations to restore electrical services.

Service flow result

The power cut is resolved manually. Even if there is the possibility to intervene using Smart Energy Services, lack of telecommunications service during the power cut prevents this. There is no standard based coordination of energy system recovery between the MNO and DSO, so only ad hoc communication regarding energy system recovery is possible.

### 6.3.5	Potential requirements

There are no potential requirements.

## 6.4	Business use case: MNO exposes Network Alarm

### 6.4.1	Description

Motivation

In 6.1, it was explained that DSOs require extremely high availability for communication in order to provide distribution automation and SCADA services to prevent energy service outages.

The sooner the DSO obtains information regarding a communication incident the better. (The term 'incident' is used in the sense described in Annex A, a service outage - either where no communication is possible, or the service levels required cannot be maintained.)

As described in 6.1, a DSO can determine an outage by means of their own infrastructure. The DSO has many routers in their network. These provide networking within substation networks and have wireless access interfaces to connect the substation network over a wide area. These routers perform periodic monitoring operations, e.g. sending ICMP echo (ping) messages to ascertain latency and reachability. If the access fails entirely, it is impossible for the DSO management operations system to collect this monitoring information. The failure is only detected after the periodic monitoring operations fail.

It has to be taken into account that DSO could have tens of thousands of services like these. So, automation has to be used to improve availability.

Despite response time (including the identification of the root cause) for these kind of incidents should be less than two hours, DSO has to dedicate more time (8 hour or more) because there is no information from the MNO and it is supposed that all incidents that are not solved by their own in 8 hour, need to be addressed individually. After these 8 hours, DSO generate incidents automatically with the whole information captured automatically. These incidents are managed manually. There are remote or onsite operation to solve the incidents.  If DSO was able to receive information from MNO, this response time could be reduced dramatically to less than 1 hour, an availability of the network could be improved significantly

This use case considers the exposure of alarm messages from the MNO to the DSO to reduce this delay.

Background

See use case 6.1.

### 6.4.2	Details

Use Case Actors

DSO network operations center engineer:	The DSO network operations engineer is responsible for deploying monitoring and control mechanisms in the network. The DSO network operations center engineer determines how to control and configure the network for resiliency, e.g. when and how to switch between different accesses to maximize availability.

DSO electrical system operations center engineer: This actor is responsible for maintaining availability, efficiency and safety of the energy system.

Use case service flow

1. 	The energy system is monitored and managed by the DSO electrical system operations center engineer. It relies upon a functioning network to communicate with electrical utility equipment in substation networks.

2. 	There are a set of substation networks with routers that include a UE for wide area communications. The DSO network operations center engineer is responsible for this network. The DSO network operations center engineer employs monitoring mechanisms to determine when the telecommunications network is not available. These monitoring mechanisms are exposed by the MNO, so that authorized third parties (including the DSO) are able to receive alarm messages.

NOTE 1: 	It has to be taken into account that the DSO doesn’t need all alarms received in the MNO network. The DSO only needs to receive these alarms which could affect to the availability of their services.

NOTE 2: 	The alarms are provided at the network level, based on the overall statistical performance of the network. The alarms described here do not correspond to the performance of individual UEs or sessions.

3. The DSO network operations center engineer, using the exposed monitoring mechanism interface, can request which alarms, under which conditions, will be provided. The interface provides a means by which the alarms can be configured, including thresholds and parameters to report. These alarms can indicate the start and end of incidents.

4. An alarm is triggered because either

- 	The 3GPP management system determines that service has degraded with some KPIs (e.g. throughput, latency, etc.) above the threshold, or recovered so that they are below a threshold.

- 	The 3GPP management system determines that service is no longer available or has become available after having not been available.

Service flow result

When an incident occurs, an alarm is sent. Once the alarm is received, the DSO network operations center engineer (or an automated management function) may trigger a fail-over to a backup network communications facility. This immediate alarm from the network facilitates a rapid response by the DSO, reducing or even eliminating the interval in which the DSO substation network components are unreachable or only reachable with inadequate quality of service.

### 6.4.3	Potential Requirements

PR 6.4.3-1. The 3GPP management system shall according to mobile network operator policy expose interfaces to third parties that provide the ability to:

a) enumerate alarms;

b) create alarms;

c) remove alarms;

d) configure alarms.

NOTE: 	The intention of this requirement is to enable DSOs to manage the set of alarms that the MNO will expose. That is, the DSO can create, configure, remove and enumerate the alarms that they request that the MNO expose to the DSO.

PR 6.4.3-2. The 3GPP management system shall according to mobile network operator policy expose interfaces to third parties that provide a mechanism for the mobile network operator to send alarms to the third party.

PR 6.4.3-3. The 3GPP management system shall support the following alarms and associated configuration:

a) 	Maximum throughput threshold crossed  (where the trigger characteristic corresponds to measurement of [an average for the third party's network traffic]);

b) 	Maximum packet loss threshold crossed (where the trigger characteristic corresponds to measurement of [an average for the third party's network traffic]);

c) 	Cell outage (where the trigger characteristic corresponds to measurement of [where communication service is not possible at all on the third party]).

Next Change

## 6.6	Business use case: DSO Provides Performance Information indicating Relevant Changes to its Network

### 6.6.1	Description

Motivation

As described in 6.2, DSO reporting to MNO to resolve problems and incidents, current practice, and Annex A, incident reporting is a vital component of service management. The reporting of such problems today occurs very infrequently, e.g. when service agreements are created or renewed. This exchange of information could be very beneficial to an MNO and lead to considerations such as adjustment of coverage (e.g. for optimization), capacity planning (e.g. to react to growth in utilization in some part of the network), disaster recovery planning, and more.

Providing information from the DSO to the MNO serves a broader set of motivations as well. As the DSO's organization and use of their own network changes, their measured performance will change. This information may be important in distinguishing whether differences in network usage arise from performance problems or instead whether the changes are intentional on the part of the DSO.

The DSO is in a unique position to provide information to an MNO regarding their service. The DSO not only has many deployed stationary UEs, but performs significant performance management for these and requires extremely high availability. The DSO therefore has a very detailed set of information gathered over time, across the MNO's network. Specifically, the DSO identifies where there are problems for their own purposes, for the purposes of proactive response to increase availability. (See 6.2)

This 'problem' or 'utilization' information is in the interest of the DSO to share, as it could lead the MNO to consider service improvements or to identify issues that they may not have otherwise detected.  Providing problem analysis information is an example of mutual advantage for both the customer and the service provider. In any case, if such information is needed by both sides, a standard interface for providing it will reduce the informal communication needed by both parties.

Background

See 6.2.

### 6.6.2	Details

Problem Management processes, by which the DSO works with the MNO to address such issues as observed erratic or declining performance, occur very infrequently today. Problem Management results in concrete proposals for improvements, essentially as input for processes such as capacity planning and adjustments to service level agreements. Problem management therefore results in changes that need to be considered over time, decisions that take months or years to conclude.

The opportunities to analyze problems that emerge from time to time in energy utility service provider communication networks can be analyzed by the energy utility service providers in cooperation with the MNOs to ascertain the root cause of unexpectedly low or declining performance levels. This network problem discovery, trend analysis and identification of root causes of problems is, as a result of sharing information and conferring with MNOs, made easier for energy utility service providers.

Use case actors:

DSO network operations center engineer: The DSO network operations center engineer analyzes collected information, creates and operates network monitoring tools including alarms, and seeks to identify areas of potential improvement in service delivery.

MNO technical service / account manager: The MNO technical service / account manager responds to queries from the energy utility service provider when needed. Such questions may arise when problems emerge, e.g. as network performance or avaialbility appears to decline in the DSO's network.

Use case service flow:

1)	Using the DSO operational tools, the DSO o network perations center engineer gathers and analyzes data, seeking to identify trends that appear threatening to the proper function of the energy utility service provider network. An example of such a trend would be a gradual decline in performance parameters including observed increasing latency or jitter, or gradual decline in availability. Note that these parameters' decline even while remaining within the expectations of the service level agreement may merit investigation if the decline is prolonged and pronounced.

2a)	When such trends are identified, the DSO network operations center engineer uses tools to study the details of the potential problem.

2b)	Another goal is to identify where there are not problems, despite data that appears to indicate problems. In some cases, when less or no traffic is sent or received over the mobile telecommunications network this may be a decision of the customer.

3)	The DSO network operations center engineer shares this information with the MNO by means of a standard interface, including the measured performance, physical location, Cell ID and other information.

NOTE:	The physical location, Cell ID and other information is available to the MNO. It is given by the DSO in order to provide sufficient information that the observed performance information provided by the DSO can effectively be correlated and compared to management information collected by the MNO.

4a)	The 'root cause' of the problem may need to be investigated by the DSO before identifying a potential remedy is possible, or determining how high a priority the mitigation has. This process can be accelerated and eased by collaboration and interaction between the DSO network operations center engineer and the MNO technical service representative. The communication between the two organizations is based on any standard data model which eases communication and determining answers to essential questions for both the MNO technical service and DSO operations engineers.

4b)	The apparent problem (reduced or zero network utilization) can be flagged to the MNO as 'intended'.

Service flow result:

The DSO learns about performance problems in their network. By sharing these problems in a standardized manner with the MNO, it is possible to more straightforwardly ascertain whether the issues are also present in the mobile network. The MNO benefits from information acertained by the DSO about network performance issues.

The DSO is able to differentiate in the information it provides to the MNO between reduced performance in the network and intentional reduced utilization of the network. This will ease interpetation of the data by the MNO.

### 6.6.3	Potential Requirements

PR 6.6.3-1. Subject to operator policies, regulatory requirements and contractual obligations, the 3GPP management system may support a means for authorized third parties to provide network performance information to an MNO. The data model of the performance information provided by the DSO to the MNO shall include at least the following elements: {measurement granularity, location of measurement, latency, packet loss, throughput.}

a) Latency between the DSO’s device and the DSO’s server the device is communicating with [an average for the DSO's network traffic];

NOTE 1:	Latency measurement end to end will include delay arising from a network outside the mobile telecommunication system.

b) Throughput [an average for the DSO's network traffic];

c) Packet loss [an average for the DSO's network traffic];

NOTE 2: 	Packet loss can be determined by means of observation of ICMP and TCP control information for traffic, sending probes, etc. This methodology is out of scope of 3GPP.

NOTE 3:	The information elements to include in the incident report will be further clarified during the 'solution definition' stage of this study.

NOTE 4:	The measurements provided by the DSO can be acquired by means that are outside the scope of 3GPP. Measurements may be acquired from management MIBs of routers that are operated by the DSO.

## 6.7	Business use case: DSO reports an incident to MNO

### 6.7.1	Description

Motivation

As described in 6.2, DSO reporting to MNO to resolve problems and incidents, current practice, and Annex A, incident reporting is a vital component of service management. Currently incident reporting is done through diverse mechanisms specific to each MNO, such as a voice call to a service desk or a web-based form. While such processes are out of scope of 3GPP and are vital to support of mobile subscribers in general, they are not well suited to a DSO. The DSO operates 100s of 1000s of UEs often with dual SIM configurations, in some cases in multiple countries. This results in a high degree of complexity for the DSO in terms of divergent procedures and manual effort required. A specific aspect of this challenge for DSOs is that the terminology, units and other aspects of incident reports differ between operators. Reporting incidents using 'business to consumer' oriented interfaces is not only cumbersome; it does not scale up to the number of concurrent incidents that can arise for a DSO.

Furthermore, the availability requirements of a mobile consumer are different from that of an energy utility service provider. It is essential that the DSO network operations network operations staff be able to ascertain the root cause of an incident as quickly as possible. If the root cause is an incident in the mobile telecommunications network, and this information is readily available, this will help to focus the recovery efforts in the DSO. If on the other hand, there is no known mobile telecommunications network failure, the DSO can focus their recovery efforts on their own network.

The intention of this use case is to identify how standardized interfaces can facilitate large scale, rapid and uniform incident reports from a DSO to an MNO. This use case does not consider how to automate or standardize the incident reporting process since process standardization is out of scope of 3GPP and it is recognized that different operators have unique and specific business processes. As incident reporting mechanisms will vary between operators, only a 'least common subset' of reporting information is identified in the requirement, and a 'least common subset' of responses to these reports.

Background

See 6.2.

### 6.7.2	Details

Incident management is triggered when there is a performance incident - either a service is delivered below the service level agreement parameters or there is a service outage. In order to qualify as an 'incident' some parameters may apply (how long it is sustained, how frequently it occurs, etc.) according to the service level agreement, the details of which are out of scope of 3GPP.

When an incident occurs, this triggers processes to end the service performance failure or service outage as quickly as possible. The DSO needs to diagnose the root cause of the incident and take the necessary steps to recover service. To the extent that the service failure is attributed to the mobile network, or the root cause is unknown and could be due to service problems in the mobile network, the DSO works together with the MNO to diagnose the incident and resolve it.

The term 'service desk' represents the service provider contact that is available when incidents occur. The service desk can take many forms and interactions and interfaces are not standardized in current practice.

While resolution is needed on the order of seconds or minutes, communication and collaboration between the DSO and MNO can take much longer, sometimes hours even days. A significant amount of time is needed to describe the incident in terms that enables the MNO to take action. The lack of standards in this area means that the DSO has to identify the reporting formats of each MNO separately, which is labor intensive especially for DSOs that operate across national borders and therefore have large numbers of subscriptions with each operator in each country they operate.

Use case actors:

DSO network operations centre engineer: The DSO network operations centre engineer analyses collected information, creates and operates network monitoring tools including alarms, and seeks to identify areas of potential improvement in service delivery.

MNO technical service desk engineer: The MNO is represented by this actor, whose role is to capture the incident information, analyse the MNO's management system to find corresponding relevant information and to work with the customer who reports the incident and MNO operational staff to resolve the incident if it is indeed an issue in the mobile network.

Use case service flow:

1)	The DSO operational management system indicates a service outage in the DSO network. The DSO network operations centre engineer analyses the alarm.

2)	The root cause of the service outage is not clear. The DSO operations center engineer initiates a service desk procedure with the MNO providing service for the network that has a service outage. This service desk procedure includes submission of a standard form for incident reporting that can be provided by the DSO by means of an interface exposed by the MNO.

3)	The MNO technical service desk engineer obtains information from the DSO network operations centre engineer, principally by means of the exposed interface. If needed, other means of communication are also provided by the MNO service desk (e.g. web based reporting tools), partly through other forms of communication (phone, messages, etc.)

4)	The MNO technical service desk engineer works with the MNO's operational staff and management tools to identify the corresponding network status.

a)	Sure enough, there is a service outage in the mobile network corresponding to the service desk 'ticket' initiated by the DSO.

a.1)	The MNO technical service desk engineer informs the DSO network operations centre engineer of the incident and the expected time until resolution, by means of the standard form for incident response.

a.2)	The DSO network operations centre engineer takes the appropriate action, e.g. initiates recovery procedures using a back up access system, if possible, or otherwise works to mitigate the impact of the outage.

b)	Alternatively, the MNO network is functioning as expected - there is no service outage in the mobile netwo

b.1)	The MNO technical service desk engineer informs the DSO network operations centre engineer by means of the standard form for incident response that the incident is not related to MNO network performance degradation or service failure.

b.2)	The DSO network operations engineer continues to work to identify the root cause of the failure, excluding the mobile network as the cause.

b.3)	Eventually the source of the failure is identified and remedied.

A quick and standard exchange of information can provide valuable data to both sides minimising the request for further information during the troubleshooting of the incident.

### 6.7.3	Potential Requirements

PR 6.7.3-1. Subject to operator policies, regulatory requirements and contractual obligations, the 3GPP management system shall support a means to expose a standard interface for incident reporting by authorized DSOs. The process of incident reporting itself is out of scope of this requirement as this is a business process. The information that this report contains is intended to be a 'common subset' supported by incident reporting procedures and tools today.

NOTE 1:	An example of such information elements includes: {UE Identifier e.g. IMSI, performance characteristics that constitute a service level failure, time of start of incident, location of UE, cell ID serving the UE, ... }

NOTE 2: 	More than one incident report may be submitted, e.g. as more information is learned by the customer.

NOTE 3:	The information elements to include in the incident report will be defined and evaluated during the 'solution description' phase. These information elements are expected to include {UE Identifier e.g. IMSI, performance characteristics that constitute a service level failure, time of start of incident, location of UE, cell ID serving the UE, . . . }

NOTE 4:	This requirement does not imply that the incident reporting interface required will replace existing tools or processes. The 'common subset' of information defined for the incident reporting interface does not limit incident reporting by the customer to the operator to this subset - additional information will be collected by tools and/or interfaces as well.

PR 6.7.3-2. Subject to operator policies, regulatory requirements and contractual obligations, the 3GPP management system shall support a means to expose a standard interface for incident response to authorized DSOs. The process of responding to incident reports itself is out of scope of this requirement as this is a business process. The information that this response contains is intended to be a 'common subset' supported by incident responses procedures and tools today.

NOTE 5:	An example of such information elements includes: {incident does or does not correspond to known fault or performance degradation in the mobile network, expected time of restoration of service, . . .}

NOTE 6: 	More than one incident reporting response may be sent to the customer, e.g. as more information is learned by the MNO.

NOTE 7:	The information elements to include in the incident report will be defined and evaluated during the 'solution description' phase. These information elements are expected to include {incident does or does not correspond to known fault or performance degradation in the mobile network, expected time of restoration of service, . . .}

NOTE 8:	This requirement does not imply that the incident response interface required will replace existing tools or processes. The 'common subset' of information defined for the incident response interface does not limit incident response by the operator to the costomer to this subset - additional information will be provided by tools and/or interfaces as well.

NOTE 9: 	All requirements above concerning the MNO apply to the site operator. In some network sharing scenarios the site operator could be an entity other than an MNO. In active network sharing scenarios the DSO can communicate with an MNO that operates the site, but can be distinct from the serving MNO.

End of Changes