3GPP TSG-SA5 Meeting #149 	S5-234628

Berlin, Germany, 22 - 26 May 2023

# Source:	Ericsson

# Title:	Rel-18 pCR 28.839 Correcting exposure solution using NEF

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.839: "Study on charging aspects for Time Sensitive networking"

# 3	Rationale

Correcting solution #1 for exposure of Time Synchronization Service via NEF.

# 4	Detailed proposal

First change

## 7.1		Solution #1: Exposure of Time Synchronization Service via NEF

### 7.1.1	General

The solution #1, addresses the Key Issue #1a and #1b, covering the REQ-CH_ETS-02.

Based on the clause 4.15.9.2 Exposure of UE availability for Time Synchronization service, specified in the TS 23.502 [4], the AF subscribes to notifications and to explicitly cancel a previous subscription for UE availability for time synchronization service, via the following NEF services:

-	Nnef_TimeSynchronization create or update time synchronization configuration as well as to activate and deactivate the time synchronization service for which the NEF uses service operations provided by TSCTSF. Described in TS 23.501 [3] clause 5.27.1.8.

-	Nnef_ASTI service to control the 5G access stratum time distribution configuration, for which the NEF uses service operations provided by TSCTSF. Described in TS 23.501 [3] clause 5.27.1.8

-	Nnef_AFsessionWithQoS to reserve resources for an AF session. Described in TS 23.503 [20] clause 6.1.3.22

### 7.1.2	Architecture description

Figure 7.1.2-1: NEF converged charging architecture

### 7.1.3	Flow description

The figure 7.1.3-1 describes the high-level charging procedure for Time Synchronization service based on NEF.

Figure 7.1.3-1: AF Request via NEF using PEC

1.	NEF receives a Time Synchronization Service Request from an AF, including a list of UE identities (GPSIs) or Groups of UEs identified by an External Group Identifier that further define the subset of the target UEs and AF-Service-Identifier, or DNN and optionally S-NSSAI which used to indicate the TS-SC.

2.	NEF performs the actions needed to fulfil the Time Synchronization Service Request.

3.	If authorized, the NEF continues the Time Synchronization Service Request processing and sends the Time Synchronization Service Response to the AF.

3ch-a. The NEF sends Charging Data Request [Event] to CHF for the received Time Synchronization Service Request.

3ch-b. The CHF creates a CDR for this Time Synchronization Service Request.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the NEF.

Editor’s note: the subscriber id is ffs.

End of changes