3GPP TSG-SA5 Meeting #149 	S5-234664

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0039 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add general concept and requirements for intent report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The intent report has been is discussed and described in clause 4.5 in TR 28.912, the intent report may contain Intent Fulfilment information, Achieved values for targets, Intent conflict information and Intent fulfilment feasibility check information. Therefore, it is necessary to add general concept and requirements for intent report |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add general concept and requirements for intent report based on clause 4.5 in TR 28.912 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The intent reporting capabilities is missing in TS 28.312 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1.S-234664 is the revision of S5-233915 |  |  |  |  |  |  |  |  |

1st Change

5.X	Generic use case for intent driven management

5.X.1 Intent Report

5.X.1.1 Introduction

The Fulfilment information (including the intentFulfilmentInfo, expectationFulfilmentInfo and targetFulfilmentInfo) are defined for the MnS consumer to monitor the intent fulfilment information. The intent report also can contain the current and optional predicted value for performance indicated by corresponding expectation targets (e.g. WeakRSRPRatio for the weakRSRPRatioTarget, Average UL RAN UE Throughput for aveULRANUEThptTarget), which can be used by MnS consumer to validate whether the intent is really fulfilled and to evaluate whether the intent (especially for expectation targets) needs to be updated if needed (improve the target value when corresponding target is fulfilled or reduce the target value when corresponding target is not fulfilled or not fulfilled with the reason of target confliction). Besides, intent conflict information and intent fulfilment feasibility check information which send by MnS producer to MnS consumer is another type of intent report information. So, following are the four types of information needs to be monitored by MnS consumer:

- 	Intent Fulfilment information, which represents the properties of a specific fulfilment information for an aspect of the intent (i.e. either an expectation, a target or the whole intent), including fulfilment status and

achieved values for targets.

- 	Intent conflict information, which represents conflict type (i.e., intent conflict, expectation conflict and target conflict) and possible solution recommendation to address the conflicts.

- 	Intent fulfilment feasibility check information, which indicates that the intent is feasible or infeasible. Intent fulfilment feasibility check information is provided after MnS producer automatically performs feasibility check when receive the intent creation and modification request from MnS consumer.

Different MnS consumer may have different requirements for intent report (e.g. Some MnS consumer may want to have corresponding performance value information while others do not want. Different MnS consumer may want to calculate or monitor the performance value in different period).

5.X.1.2 Requirements

REQ-Intent_Driven_MnS_Report-1: The intent driven MnS shall have the capability to enable the MnS consumer to request intent report information.

REQ-Intent_Driven_MnS_Report-2: The intent driven MnS shall have the capability to enable the MnS consumer to obtain intent report information with intent fulfilment information (including fulfilment status and achieved values for targets).

REQ-Intent_Driven_MnS_Report-3: The intent driven MnS shall have the capability to enable the MnS consumer to obtain intent report information with intent conflict information.

REQ-Intent_Driven_MnS_Report-4: The intent driven MnS shall have the capability to enable the MnS consumer to obtain intent report information with intent fulfilment feasibility check information.

REQ-Intent_Driven_MnS_Report-5: The intent driven MnS shall have capability enabling MnS consumer to specify the content of the report.

REQ-Intent_Driven_MnS_Report-6: The intent driven MnS shall have capability enabling MnS consumer to configure the frequency of the intent reporting.

REQ-Intent_Driven_MnS_Report-7: The intent driven MnS shall have the capability enabling MnS consumers to receive reports, with different content and intervals according to its specified requirements.

End of Changes