3GPP TSG-SA5 Meeting #151	S5-236447

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0465 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add DL/UL PRB Usage per SSB to support AI/ML enabled NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecomunication Corp., Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To fulfil the objectives of the WID in SP-230173 (PM_KPI_5G_Ph3), define the performance measurements of predicted radio resource status required to support AI/ML enabled NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DL/UL PRB Usage per SSB including: - DL PRB Usage per SSB - UL PRB Usage per SSB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AI/ML based procedures and functions cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.X(new), 5.1.1.2.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 5.1.1.2	Radio resource utilization

##### 5.1.1.2.X	DL PRB Usage per SSB

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) per SSB beam of the NR cell on the downlink for any purpose. The measurement is optionally split into subcounter per QoS level (mapped GBR resource type and non-GBR resource type).

b)	SI

c)	This measurement is obtained as: , where is the DL total PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of full physical resource blocks and all PRBs used for DL traffic transmission for the SSB beam per QoS level (mapped GBR and non-GBR resource type) shall be included; is total number of PRBs available for DL traffic transmission during time period ; and is the time period during which the measurement is performed.

d)	A single integer value from 0 to 100. If the optional measurements are perfomed, the number of measurements is equal to the number of QoS levels.

e)	The measurement name has the form RRU.PrbDlSSB, which indicates the DL PRB Usage for all traffic,

or optionally RRU.PrbDlSSB.QoS, where the QoS identifies the mapped mapped GBR resource type and non GBR resource type level.

f)	Beam

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

##### 5.1.1.2.Y	UL PRB Usage per SSB

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) per SSB beam of the NR cell on the uplink for any purpose. The measurement is optionally split into subcounter per QoS level (mapped GBR resource type and non-GBR resource type).

b)	SI

c)	This measurement is obtained as: , where is the UL total PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of full physical resource blocks and all PRBs used for UL traffic transmission for the SSB beam per QoS level (mapped GBR and non-GBR resource type) shall be included; is total number of PRBs available for UL traffic transmission during time period ; and is the time period during which the measurement is performed.

d)	A single integer value from 0 to 100. If the optional measurements are perfomed, the number of measurements is equal to the number of QoS levels.

e)	The measurement name has the form RRU.PrbTotUlSSB, which indicates the UL PRB Usage for all traffic,

or optionally RRU.PrbUlSSB.QoS, where the QoS identifies the mapped mapped GBR resource type and non GBR resource type level.

f)	Beam

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

End of change