# 3GPP TSG-SA5 Meeting #151	S5-236342

# Xiamen, China, October 09-13, 2023

# Source:	Nokia

# Title:	Rel-18 pCR TS 28.318 Availability KPI definition

# Document for:	Approval

Agenda Item:	6.6.4.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[X]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[Y]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[Z]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[A]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[B]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

# 3	Rationale

This pCR builds on the Samsung document (tDoc number S5-235714) which was noted in the SA5#150 meeting that provided some definition of the availability KPI.

# 4	Detailed proposal

It is proposed to agree to the following change to TS 28.318.

Begin Change

##### 7.X.1.3.3	Total cell In-Service duration

a)	This measurement provides the total time duration for which an active cell in a gNB-DU remains In-Service. The gNB-DU reports the Service Status of the Active cells to the gNB-CU. If the Service Status value is “In-Service” it means the active cell is operational, and it is able to serve UEs. The active cells are considered as In-Service until the gNB-DU reports the Service Status as “Out-Of-Service” to the gNB-CU. The gNB-DU reports the Service Status to gNB-CU using the GNB DU CONFIGURATION UPDATE messages.

b)	CC.

c)	This measurement is obtained in two steps. First step involves determining the time interval between the time stamps Tin and Tout. Where Tin is the time stamp when the Service Status is reported as “In-Service” for a cell by gNB-DU to gNB-CU and Tout is the time stamp when Service Status is reported as “Out-Of-Service” by gNB-DU to gNB-CU for the same cell in the GNB DU CONFIGURATION UPDATE messages. Second step involves summing up all these time intervals of a cell which fall within the desired observation time frame Tobv. This gives total in-service duration of a cell within the observation time frame Tobv. Where, Tobv is monitorGranularityPeriod  as defined in TS 28.622[X] clause 4.3.16.

d)	Each measurement is an integer value representing the number of minutes for which the cell was in-Service.

e)	SO.CellInServiceTotal.NCGI, where NCGI is NR Cell Global Identifier as defined in TS 28.541[A]. The number of measurements is equal to the number of NCGIs (cells).

f)	GNBCUCPFunction.

g)	Valid for packet switching.

h)	5GS.

### 7.X.2      KPIs

The KPIs defined in the caluse are in accordance with KPI definitions template described in [Z].

#### 7.X.2.1	Availability KPI

##### 7.X.2.1.1	Cell Availability KPI

a) CellAvail

b) This KPI describes the amount of time (in percentage) that a specific cell served by the gNB-DU is providing services to UEs. Cell availability is measured by the gNB-CU and represents the fraction of time (in percentage) that the active cell is “in-service” over the desired observation time frame Tobv.

c) It is obtained as: CellAvail=SO.CellInServiceTotal.NCGI Tobv*100, where CellAvail is the cell availability KPI, and SO.CellInServiceTotal.NCGI  and Tobv are as defined in clause 7.X.1.3.3.

d) A single integer value from 0 to 100

e) GNBCUCPFunction

##### 7.X.2.1.2 Average Cell Availability KPIgNB-DU

a)	CellAvailAvgDU

b)	This KPI describes the Average  amount of time (in percentage) that the cells served by a specific gNB-DU are providing services to UEs. This KPI is computed by the gNB-CU and provides  the average availability (in percentage) of cells served by a specific gNB-DU.

c)	It is obtained by: CellAvailAvgDU=i=1NcellCellAvailiNcell, where CellAvailAvgDUis the average cell availability per GNB-DU, CellAvaili is the availability of the ith cell (as defined in clause 7.X.2.1.1)  and Ncell  is the total number of active cells (identified by their NR CGIs) present in Cells Status List in GNB-DU CONFIGURATION UPDATE  message as explained in clause 9.2.1.7 of TS 38.473 [B].

d) A single integer value from 0 to 100

e)	GNBCUCPFunction

##### 7.X.2.1.3 Average Cell Availability KPIgNB-CU

a)	CellAvailAvgCU

b)	This KPI describes the Average availability of all the cells (in percentage) served by different gNB-DUs managed and controlled by a the same gNB-CU. Where cell availability is the percentage of time  for which an active cell stays “In-Service” as reported by the serving gNB-DU to the gNB-CU (as defined in clause 7.X.2.1.1). This KPI is computed by the gNB-CU and provides the average availability (in percentage) per cell at the gNB-CU level.

c)	It is obtained by: CellAvailAvgCU=j=1NDUCellAvailAvgDUjNDU, where CellAvailAvgCU is the average cell availability at the gNB-CU level, CellAvailAvgDUj is the average cell availability for the jth gNB-DU (as defined in clause 7.X.2.1.2) and NDU is the total number of gNB-DUs controlled and managed by the same gNB-CU.

d) A single integer value from 0 to 100

e)	GNBCUCPFunction

##### 7.X.2.1.4	Network Availability KPI

a)	NwAvailAvg

b)	This KPI describes the Average availability of a network in terms of cells availability for providing services to UEs. Where Cell availability is the percentage of time  for which an active cell in a network stays “In-Service” (as defined in clause 7.X.2.1.1). This KPI is measured in percentage and provides the average availability (in percentage)  per cell for an entire RAN network.

c)	It is obtained by: NwAvailAvg=i=1NCUCellAvailAvgCUiNCU ,where NwAvailAvg is the average network availability, CellAvailAvgCUi is the average cell availability for the ith gNB-CU (as defined in clause 7.X.2.1.3) and NCU  is the total number of gNB-CUs present in the network.

d) A single integer value from 0 to 100

e)	SubNetwork

End Change