3GPP TSG-SA5 Meeting #151	S5-236376

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0145 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KPI for cell and network availability for energy utilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, EUTC, EDF, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSOEU |  |  |  |  |  | Date: |  |  | 2023-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The energy utility service provider needs performance information regarding “outages/network availability” and performance degradation of the communication network, as it may be possible for the energy utility service provider to reactively or even proactively establish and use an alternative means of communication. There is currently no mechanism to convey network or cell service status from the MNO to the DSO as a performance metric, in a standardized way. The actual availability of mobile network services is not exposed to energy utility service provider (customers), nor is the cause of the availability limit. This makes it difficult for energy utilities to perform risk assessment and network planning for communication services to carry out their smart energy services. And, in case network failure is happening or has happened in an area, the DSO can just infer this situation through a qualified guess. If the DSO is forced to use a standard MNO service and network, with no specific “availability” target, and no mechanism to know that network or cell service status has failed in an area, the DSO will not rely in MNO network and will develop alternative telecommunications infrastructure that will eventually make MNOs’ services unnecessary. Currently there is no KPI defined to represent availability in network in terms of cells service status as required by energy utilities. The KPI focusses on the RAN network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new KPI for calculating availability in network in terms of cells service status as required by energy utilities is defined This KPI is focussed on the RAN network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cell or network availability performances cannot be known to energy utilites from a MNO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	ITU-T Recommendation E.800: "Definitions of terms related to quality of service".

[4]	3GPP TS 24.501: " Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[6]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[7]	3GPP TS 23.501: " System Architecture for the 5G System; Stage 2".

[8]	ETSI ES 203 228 V1.2.1 (2017-04): "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[9]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[10]	ETSI 202 336-12 V1.2.1 (2019-02): "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[11]	ETSI GS NFV-IFA 027 V4.0.2 (2020-11): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[12]	3GPP TS 38.314: "NR; layer 2 measurements".

[13]	3GPP TS 22.261: "Service requirements for the 5G system".

[14]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[X]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

Second change

## 6.X	Availability KPI

### 6.X.1	Cell Availability KPI

a)	CellAvailAvg

b)	This KPI describes the Average availability of cells in a gNB-CU for providing services to UEs. Cell availability is the time duration for which an active cell in a gNB-CU stays “In-Service”. This KPI provides “average availability time duration per cell” in a gNB-CU. It is a time duration value obtained in “seconds”.

c)	It is obtained by summing all measurements of SO.CellInServiceTotal.NCGI (as defined in TS 28.552 clause ) and then dividing the result by Ncell where Ncell  is the total number of active cells (NR CGIs) present in Cells Status List in GNB-DU CONFIGURATION UPDATE  message as explained in TS 38.473 [X] clause 9.2.1.7.

d)	GNBCUCPFunction

### 6.X.2	Radio Access Network Availability KPI

a)	NwAvailAvg

b)	This KPI describes the Average availability of a network in terms of cells availability for providing services to UEs. Cell availability is the time duration for which an active cell in a network stays “In-Service”. This KPI provides “average availability time duration per cell” of a RAN network. It is a time duration value obtained in “seconds”

c)	It is obtained by summing all measurements of CellAvailAvg and then dividing the result by Ncu where Ncu  is the total number of gNB-CUs present in the network.

d)	SubNetwork

End of changes