# 3GPP TSG-SA5 Meeting #151	S5-236379

# Xiamen, China, October 09-13, 2023

# Source:	Samsung, EUTC, BMWK, EDF, Deutsche Telekom

# Title:	Rel-18 pCR 28.318 6.X Coordinated energy recovery procedures

# Document for:	Approval

Agenda Item:	6.6.4.1 (NSOEU_WoP#1) General

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

This pCR is motivated by reference to preceding work relevant to the feature.

[1] 3GPP TR 28.829, "Study on network and service operations for energy utilities".

# 3	Rationale

A DSO can implement a service that makes use of exposed information from the management system by accessing capabilities that are already defined in the specificationss today. A Procedures section provides informative examples of the specific steps a DSO can take to implement the desired functional service. The procedures only provide examples of how to use normative text defined in other SA5 specifications and therefore this text is not suitable to be normative text in this TS nor another TS. It is however well suited to form an informative annex.

The stage 2 definition is defined in clause 6 of the TS.

# 4	Detailed proposal

It is proposed to agree to the following change to TS 28.318, 0.0.0.

Begin Change

## 6.X	Energy utility and telecommunication coordinated rapid recovery of energy service procedure

#### 6.X.1	General

#### Coordinated energy service recovery requirements are given in clause 5.X. The use cases supported are described in Annex Y.

#### The functionality supported by the signalling flow is

#### -	Inititialization (to initialize the rest of the defined operations). As interaction between MnSs involves creation of a MOI and other objects, the listed operations precede the other operations listed below.

#### -	MNO informs DSO of changes (e.g. to update the list of essential sites and their related information)

#### -	DSO updates the information (e.g. to update the expected end of an energy outage)

#### -	DSO retrieves the changes (e.g. to query the UPS available time for a site from the MNO)

#### 6.X.2	Signal flow

1. createMOI (DsoRapidRecovery) Request

DSO

(MnS Consumer)

MNO

(MnS Producer)

3. createMOI Response

2. MOI will be created

5. createMOI(IntfSubscriptionCtrl) Request

4. IOC containing

UPS info instantiated

6. MOI will be created

7. CreateMOI Response

8. notifyMOIAttributeValueChanges

9. modifyMOIAttributes Request

10. modifyMOIAttributesResponse

11. getMOIAttributeRequest

12. getMOIAttributeResponse

Initialization

MNO informs DSO of the changes

DSO updates the information

DSO retrieves the changes

Figure 6.X.2-1: Coordinated Rapid Recovery and strategic outage plan Procedure

1.	In order to be able to provide energy service outage and recovery related information for MNO, DSO sends createMOI request for creating an IOC which contains information attributes for outage and rapid recovery by DSO such as:

a)	Time stamp of DSO's energy distribution service outage

b)	Locations (latitude-longitude pair and Energy Supply Id) where DSO's energy service outage occurs

c)	Time by when DSO expects restoration of its distribution services for MNO.

d)	Time when DSO has restored its energy transmission service and starts expecting rapid intervention by MNO.

e)	Time duration for which DSO expects to require MNO's rapid intervention for being able to use smart energy services to restore its energy distribution services.

f)	Information of locations where for example DSO substations need to restore distribution services on priority.

g)	The time at which MNO will actually be able to provide rapid intervention to DSO.

h)	The time duration for which MNO will actually be able to provide rapid intervention to DS

i)	The information of locations where MNO will actually be able to provide rapid intervention to DSO.

j)	The time stamp at which DSO's energy distribution services are finally restored.

k)	The information of locations where (e.g. DSO substations) distribution energy service has been finally restored.

NOTE 1: Not all of the above will be configured at the time of IOC creation.

2.	MnS producer in MNO creates the MOI that contains these information attribute records for multiple sites.

3.	createMOI response is sent by MNO MnS producer to DSO.

4.	MNO creates the MOI representing the UPS related information of MNO site. It is name-contained by IOC created in step 1. This is required by DSO from MNO to achieve coordinated intelligent outage planning by DSO during its energy outage recovery. UPS related information of MNO site has to be:

4.1	Information on whether the MNO site has UPS installed/available or not.

4.2	Information on the total UPS backup capacity installed in MNO site (suggested granularity: number of in minutes) i.e. the MNO site UPS backup has installed capacity of how many minutes.

4.3	Information on the status of the remaining UPS backup capacity (suggested granularity: number of minutes) available for MNO site at any given time.

4.4	The identity of DSO energy supply meter present in MNO site.

4.5	The identity of base station present in MNO site.

5.	In order to be able to get automatically notified of any changes in the attributes information in the MOI, DSO sends createMOI request for NtfSubscriptionControl IOC to MNO.

NOTE 2: 	In step 5, if done, a subscription is created such that, subject to parameters in the NtfSubscriptionControl IOC, notifications are sent from the MNO to the DSO. This allows, for example, the MNO to notify the DSO of changes in the UPS backup capacity over time. This step is shown as step 8 below.

6.	MNO MnS producer creates the MOI for NtfSubscriptionControl IOC.

7.	createMOI response is sent by MNO MnS producer to DSO.

8.	If there is a change in the attributes, the MNO sends a notifyMOIAttributeValueChanges notification to inform DSO about the changes.

NOTE 3:	In steps 9 and 10 below, there is a change in an attribute of the MOI. This is done by DSO to update MNO on the changes, e.g. information about an expected outage for a specific site.

9.	DSO can modify/update any information like outage start time stamp by sending a modifyMOIAttributes request to MNO. DSO can create/read/update/delete (CRUD operations) the information in the MOI by using provisioning MnS (defined in TS 28.532).

10.	MNO MnS producer provides the modify response to the DSO.

NOTE 4:	In steps 11 and 12 below, the DSO MnS requests and receives respectively the current value of one or more attribute(s).

11.	DSO can also query the UPS related information as and when required by using the getMOIAttributes operation. For example the information could include identity of DSO energy supply meter, MNO base station, remaining UPS backup duration of a particular site, etc.11. DSO requests the required information from the MNO by using getMOIAttributes operation.

Next Change

12.	MNO provides the required information in the response to the DSO.