3GPP TSG-SA5 Meeting #151 	S5-236621

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0453 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add 5G MBS charging information definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_CH |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To support 5G MBS charging, charging information collected by SMF and MB-SMF have been specified in clause 5.1.3 at the last meeting, which should be further detailed in the message contents in clause 6.1.1 and 6.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add MBS session ID to PDU session charging information - Add MBS session ID and MBS delivery method to PDU Container information - Add MBS container information to multiple unit usage IE in Charging Data Request message, carried by MB-SMF. This information includes time of first and last usage, QoS and established connection nodes (NG-RAN and UPF). - Add MBS session charging information to Charging Data Response message, carried by MB-SMF. This information includes MBS session ID, MBS service type, MBS service area, QoS, MBS start time and end time and Serving Network Function ID (AMF ID). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the 5G MBS related charging information can be defined in the message content. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1, 6.1.1.2, 6.1.1.3, 6.2.1.2, 6.2.1.3, 6.2.1.X (new), 6.2.1.Y(new), 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.1.1.1	General

The Charging Data Request and Charging Data Response are specified in TS 32.290 [57] and include charging information. The Charging Data Request can be of type [Initial, Update, Termination].

Table 6.1.1.1.1 describes the use of these messages for converged charging.

Table 6.1.1.1.1: Converged charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | SMF, MB-SMF | CHF |
| Charging Data Response | CHF | SMF, MB-SMF |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the SMF as used for 5G data connectivity converged charging.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element |  | Category for converged charging |  | Category for offline only charging |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Session Identifier |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Subscriber Identifier |  | OM |  | M |  | Described in TS 32.290 [57] In case SUPI is not present (for emergency service), the User Equipment Info in table 6.2.1.2.1. shall be present for identifying the user. |  |
| NF Consumer Identification |  | M |  | M |  | Described in TS 32.290 [57] |  |
| NF Functionality |  | M |  | OC |  | Described in TS 32.290 [57] |  |
| NF Name |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| NF Address |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| NF PLMN ID |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Invocation Timestamp |  | M |  | M |  | Described in TS 32.290 [57] |  |
| Invocation Sequence Number |  | M |  | M |  | Described in TS 32.290 [57] |  |
| Retransmission Indicator |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Notify URI |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Service Specification Information |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Supported Features |  | OC |  | - |  | This field indicates the features supported by the NF consumer. |  |
| Triggers |  | OC |  | OC |  | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |  |
| Multiple Unit Usage |  | OC |  | OC |  | Described in TS 32.290 [57] This field is not applicable to QBC. |  |
| Rating Group |  | M |  | M |  | Described in TS 32.290 [57] |  |
| Requested Unit |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Used Unit Container |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Triggers |  | OC |  | OC |  | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |  |
| PDU Container Information |  | OC |  | OC |  | This field holds the 5G data connectivity PDU session container specific information described in clause 6.2. |  |
| UPF ID |  | OC |  | OC |  | This field holds the UPF identifier used to identify the UPF. These fields shall only be included when either quota is requested per UPF, or used units are reported per UPF |  |
| multi-homed PDU address |  | OC |  | OC |  | This field holds the IPv6 prefix used by UPF. It may only be used for IPv6 multi-homed PDU sessions and then only for reporting used units. |  |
| PDU Session Charging Information |  | OM |  | OM |  | This field holds the 5G data connectivity specific information described in clause 6.2. This field is applicable to FBC and QBC. |  |
| Roaming QBC information |  | OM |  | OM |  | This field holds the roaming QBC specific information defined in clause 6.2.1.4 This field is only applicable to QBC. |  |

Table 6.1.1.2.X illustrates the basic structure of a Charging Data Request message from the MB-SMF as used for 5G data connectivity converged charging for MBS session.

Table 6.1.1.2.X: Charging Data Request message contents

| Information Element | Category for converged charging | Category for offline only charging | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | OC | Described in TS 32.290 [57] |
| NF Consumer Identification | M | M | Described in TS 32.290 [57] |
| NF Functionality | M | OC | Described in TS 32.290 [57] |
| NF Name | OC | OC | Described in TS 32.290 [57] |
| NF Address | OC | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | M | Described in TS 32.290 [57] |
| Triggers | OC | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| Multiple Unit Usage | OC | OC | Described in TS 32.290 [57] This field is not applicable to QBC. |
| Rating Group | M | M | Described in TS 32.290 [57] |
| Used Unit Container | OC | OC | Described in TS 32.290 [57] |
| Triggers | OC | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| MBS Container Information | OC | OC | This field holds the 5G data connectivity MBS session container specific information described in clause 6.2.1.Y. |
| MBS Session Charging Information | OM | OM | This field holds the MBS session specific information described in clause 6.2.1.X. This field is applicable to FBC. |

6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF as used for 5G data connectivity converged charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element |  | Category for converged charging |  | Category for offline only charging |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Session Identifier |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Invocation Timestamp |  | M |  | M |  | Described in TS 32.290 [57] |  |
| Invocation Result |  | OC |  | M |  | Described in TS 32.290 [57] |  |
| Invoation Result Code |  | OC |  | M |  | Described in TS 32.290 [57] |  |
| Failed Parameter |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Failure Handling |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Invocation Sequence Number |  | M |  | M |  | Described in TS 32.290 [57] |  |
| Session Failover |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Supported Features |  | OC |  | - |  | This field indicates the features supported by the NF consumer. |  |
| Multiple Unit Information |  | OC |  | OC |  | Described in TS 32.290 [57] This field is not applicable to QBC. |  |
| Result Code |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Rating Group |  | M |  | M |  | Described in TS 32.290 [57] |  |
| UPF ID |  | OC |  | OC |  | This field holds the UPF identifier used for quota granted per UPF by CHF |  |
| Granted Unit |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Validity Time |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Final Unit Indication |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Time Quota Threshold |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Volume Quota Threshold |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Unit Quota Threshold |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Quota Holding Time |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Triggers |  | OC |  | OC |  | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |  |
| Triggers |  | OC |  | OC |  | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |  |
| PDU Session Charging Information |  | OM |  | OM |  | This field holds the 5G data connectivity specific information described in clause 6.2. |  |
| Roaming QBC Information |  | OM |  | OM |  | This field holds the roaming QBC specific information defined in clause 6.2.1.4 This field is not applicable to FBC. |  |

Table 6.1.1.3.X illustrates the basic structure of a Charging Data Response message from the CHF to MB-SMF as used for 5G data connectivity converged charging for MBS session.

Table 6.1.1.3.X: Charging Data Response message contents

| Information Element | Category for converged charging | Category for offline only charging | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | M | Described in TS 32.290 [57] |
| Invocation Result | OC | M | Described in TS 32.290 [57] |
| Invoation Result Code | OC | M | Described in TS 32.290 [57] |
| Failed Parameter | OC | OC | Described in TS 32.290 [57] |
| Failure Handling | OC | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | M | Described in TS 32.290 [57] |
| Multiple Unit Information | OC | OC | Described in TS 32.290 [57] This field is not applicable to QBC. |
| Result Code | OC | OC | Described in TS 32.290 [57] |
| Rating Group | M | M | Described in TS 32.290 [57] |
| Time Quota Threshold | OC | - | Described in TS 32.290 [57] |
| Triggers | OC | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| Triggers | OC | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| MBS Session Charging Information | OM | OM | This field holds the MBS session specific information described in clause 6.2.1.X. |

6.2.1.2	Definition of PDU session charging information

PDU session specific charging information used for 5G data connectivity charging is provided within the PDU session charging Information.

The detailed structure of the PDU Session Charging Information can be found in table 6.2.1.2.1.

Table 6.2.1.2.1: Structure of PDU Session Charging Information

| Information Element |  | Category |  | Description |
| --- | --- | --- | --- | --- |
| Charging Id |  | OM |  | This field holds the Charging Identifier for PDU session. The value of Charging Id is the same as that of Home Provided Charging Id in V-SMF only in the home routed roaming scenario for EPS to 5GS interworking and inter-PLMN mobility from HPLMN with I-SMF to VPLMN. |
| Home Provided Charging Id |  | OC |  | This field holds the Charging Identifier generated by H-SMF. This field is only applicable in V-SMF in the home routed roaming scenario for EPS to 5GS interworking and inter-PLMN mobility from HPLMN with I-SMF to VPLMN. |
| SMF Charging Id |  | OM |  | This field holds a string that, be provided from the SMF instead of Charging Id, if supported. |
| SMF Home Provided Charging Id |  | OC |  | This field holds a string that, be provided from the H-SMF instead of Home Provided Charging Id, if supported. |
| User Information |  | OM |  | Group of user information. |
| User Identifier |  | OC |  | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info |  | OC |  | This field holds the identification of the terminal (i.e. PEI, MAC Address) It is used for identifying the user in case SUPI is not present during emergency service. The detail identification of the wireline access is specified in clause 4.7.7 of TS 23.316 [203]. |
| unauthenticated Flag |  | OC |  | This field indicates the served SUPI is not authenticated. |
| Roamer In Out |  | OC |  | This field holds an indication if the roamer is in-bound or out-bound. This field is present only if UE is identified as a roamer. |
| User Location Info |  | OC |  | This field indicates details of where the UE is currently located (access-specific user location information). For MA PDU session, this field holds the user location associated to the 3GPP access |
| IMS Session Information |  | OC |  | This field holds the IMS session related information. |
| Caller Information |  | OC |  | This field holds the address(es) of calling party. |
| Callee Information |  | OC |  | This field holds callee information. |
| MA PDU Non 3GPP User Location info |  | OC |  | This field holds the user location associated to the non 3GPP access for MA PDU session. |
| User Location Time |  | OC |  | This field holds the UTC time at which the UE was last known to be in the location. For MA PDU session, this field holds the user location time associated to the 3GPP access. |
| MA PDU Non 3GPP User Location Time |  | OC |  | This field holds the user location time associated to the non 3GPP access for MA PDU session. |
| UE Time Zone |  | OC |  | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |
| Presence Reporting Area Information |  | OC |  | This field contains part of the Presence Reporting Area Information of UE as defined in TS 23.501[200], comprising the Presence Reporting Area identifier(s) and an indication on whether the UE is inside or outside the Presence Reporting Area, if available. |
| PDU Session Information |  | OC |  | Group of PDU session information. |
| PDU Session ID |  | M |  | This field holds identifier of PDU session. |
| Network Slice Instance Identifier |  | OM |  | This field holds network slice information the PDU session belongs to. |
| S-NSSAI | M |  | This field holds network slice S-NSSAI the PDU session belongs to in the serving PLMN. |  |  |
| HPLMN S-NSSAI | OM |  | This field holds the HPLMN S-NSSAI the VPLMN S-NSSAI is mapped to, for the PDU session. This field is only applicable in V-SMF for roaming. |  |  |
| PDU Type |  | OM |  | This field holds the type of PDU session. |
| PDU Address |  | OC |  | Group of UE IP address. |
| PDU Ipv4 Address |  | OC |  | This field holds the IP Address of the served SUPI allocated for PDU session, i.e. IPv4 address. |
| PDU IPv6 Address with Prefix |  | OC |  | This field holds the IP Address of the served SUPI allocated for PDU session, i.e. IPv6 prefix. |
| PDU Address prefix length |  | OC |  | PDP/PDN Address prefix length of an IPv6 typed Served PDU Address. The field needs not available for prefix length of 64 bits. |
| IPv4 Dynamic Address Flag |  | OC |  | This field indicates whether served PDP/PDN address for IPv4 is dynamically allocated. This field is missing if address is static. |
| IPv6 Dynamic Address Flag |  | OC |  | This field indicates whether served PDP/PDN address for IPv6 is dynamically allocated. This field is missing if address is static. |
| Additional PDU IPv6 prefixes |  | OC |  | This field holds a list of additional IPv6 prefix allocated for the PDU session, when applicable. |
| SSC Mode |  | OC |  | This field holds SSC mode of PDU session. |
| MA PDU session information |  | OC |  | This field holds information associated to the MA PDU session. |
| MA PDU session indicator |  | OC |  | This field indicates the PDU session is a MA PDU session requested by the UE or requested by Network modification based ATSSS capabilities provided by the UE and the Network. |
| ATSSS capability |  | OC |  | This field holds the ATSSS capability supported by the MA PDU session |
| SUPI PLMN ID |  | OC |  | This field holds PLMN ID of the SUPI. |
| CP CIoT Optimisation indicator |  | OC |  | This field holds the indicator whether control plane optimization CIoT for 5GS is used during the PDU session, if this feature is enabled. |
| 5GS Control Plane Only indicator |  | OC |  | This field holds the indicator whether the control plane only is used, i.e., the PDU data only transfers to control plane in case of control plane CIoT optimization. |
| Small data rate control indicator |  | OC |  | This field holds the indicator whether the small data rate control for 5GS CIoT is used during the PDU session. |
| Serving Network Function ID |  | OC |  | This field holds the identity of the serving network function. |
| Serving Network Function Information |  | M |  | This field holds the Information of the serving network function: - AMF for the PDU sessions being served by SMF in non-roaming - SMF for the home routed roaming - I-SMF for the PDU session being served by SMF + I-SMF - ePDG for handover between EPC/ePDG and 5GS - SGW for EPC/E-UTRAN interworking. |
| AMF Identifier |  | OC |  | This field holds the AMF identifier. |
| Serving CN PLMN ID |  | OC |  | This field holds the serving Core Network Operator PLMN ID selected by the UE if different from SMF PLMN ID. |
| RAT Type |  | OC |  | This field holds the Radio Access Technology (RAT) currently serving the UE. For MA PDU session, this field holds the Radio Access Technology (RAT) associated to the 3GPP access |
| MA PDU Non 3GPP RAT Type |  | OC |  | This field holds the Radio Access Technology (RAT) serving the UE in non 3GPP access for MA PDU session. |
| Data Network Name Identifier |  | M |  | This field contains the identifier of the DNN the user is connected to. |
| DNN Selection Mode |  | OC |  | This field indicates whether the requested DNN corresponds to an explicitly subscribed DNN or to the usage of a wildcard subscription. |
| Authorized QoS Information |  | OC |  | This field holds the authorized QoS applied to PDU session. |
| Subscribed QoS Information |  | OC |  | This field holds the subscribed default QoS for the PDU session. |
| Authorized Session-AMBR |  | OC |  | This field holds the authorized Session-AMBR for the PDU session. |
| Subscribed Session-AMBR |  | OC |  | This field holds the subscribed Session-AMBR for the PDU session. |
| PDU session start Time |  | OC |  | This field holds the timestamp when PDU session starts. |
| PDU session stop Time |  | OC |  | This field holds the timestamp when PDU session terminates. |
| Diagnostics |  | OC |  | This field holds a detailed reason for the release of the PDU session and complements the "Change Condition" information. |
| Enhanced Diagnostics |  | OC |  | This field holds a more detailed reason for the release of the PDU session, when a set of causes are applicable. |
| Charging Characteristics |  | OC |  | This field holds the Charging Characteristics for this PDU session. |
| Charging Characteristics Selection Mode |  | OC |  | This field holds information about how the "Charging Characteristics" was selected. |
| 3GPP PS Data Off Status |  | OC |  | This field holds the 3GPP Data off Status when UE's 3GPP Data Off status is Activated or Deactivated. |
| Session Stop Indicator |  | OC |  | This field indicates to the CHF that the PDU session has been terminated. |
| Redundant Transmission Type |  | OC |  | This field holds the redundant transmission Type. |
| PDU Session Pair ID |  | OC |  | This field holds an identifier that identify PDU Session that is redundant with this PDU session. This field is only applicable for dual connectivity based end to end redundant user plane paths case. |
| 5G LAN Type Service |  | OC |  | This field holds the 5G LAN Type service information, if present, the PDU session is for 5G LAN group. |
| Internal Group Identifier |  | M |  | This field holds the internal group identifier of the 5G LAN VN group. |
| SNPN Information |  | OC |  | This field holds information associated to SNPN. |
| SNPN ID |  | M |  | This field holds PLMN ID and the NID which identifies the SNPN. The PLMN ID is the same as PLMN ID of the SUPI. |
| Access Type |  | OC |  | This field identifies the type of access network for SNPN. It indicates whether the access is via 3GPP or via non-3GPP. |
| 5G Multicast Service |  | OC |  | This field holds the 5G MBS service information, if present, the UE has joined the multicast service. It may have multiple occurrences. |
| MBS Session Identifier |  | M |  | This field holds the MBS session identifier referring to clause 6.5.1 of TS 23.247 [204]. |
| Unit Count Inactivity Timer |  | OC |  | This field holds the threshold for the time period when no units has been counted by the SMF. It holds either the value configured in SMF, if it is supported, or the value to be used as received from the CHF. A value of zero indicates that this mechanism shall not be used. This field is not applicable to QBC. |
| RAN Secondary RAT Usage Report |  | OC |  | This field holds the secondary RAT usage reported from NG-RAN. |
| NG RAN Secondary RAT Type |  | OM |  | This field holds the value of Secondary RAT Type, as provided by the NG-RAN. |
| Qos Flows Usage Reports |  | OM |  | This field holds a list of containers per QFI with volumes reported, each container is time stamped. |
| QoS Flow Id |  | OM |  | This field holds the QoS flow Identifier (QFI) |
| Start Timestamp |  | OC |  | This field holds the start timestamp of the collected usage. |
| End Timestamp |  | OC |  | This field holds the end timestamp of the collected usage. |
| Downlink Volume |  | OC |  | This field holds the amount of used volume in downlink direction. |
| Uplink Volume |  | OC |  | This field holds the amount of used volume in uplink direction. |

6.2.1.3	Definition of PDU Container information

Used Unit Container, described in table 6.1.1.2.1, specific charging information used for 5G data connectivity charging is provided within the PDU Container Information described in table 6.2.1.3.1.

Table 6.2.1.3.1: Structure of PDU Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Time of First Usage | OC | This field holds the Timestamp when the first transmitted IP packet of the service data flow matching the current used unit container |
| Time of Last Usage | OC | This field holds the Timestamp when the last transmitted IP packet of the service data flow matching the current used unit container |
| QoS Information | OC | This field holds the QoS applied during the service data container interval |
| QoS Characteristics | OC | This field holds the QoS characteristics applied for QoS information. It is only be used when the non-standardized 5QI is present in QoS information. |
| AF Charging Identifier | OC | A Charging Identifier, provided from the AF, may be used to correlate the measurement for the Charging key/Service identifier values in this PCC rule with application level reports. |
| AF Charging Id String | OC | A string that, may be provided from the AF instead of AF Charging Identifier, depending on support. |
| User Location Information | OC | This field holds the user location during the used unit container interval |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, during the used unit container interval. |
| Presence Reporting Area Information | OC | This field holds the Presence Reporting Area Information of UE during the used unit container interval. |
| Serving Network Function ID | OC | Serving Network Function identifier. |
| RAT Type | OC | This field holds the RAT type during the used unit container interval. For MA PDU session, this field holds the RAT type associated to the access which activated the rating group. |
| Sponsor Identity | OC | This field holds the identifier of the sponsor when sponsored data connectivity is used |
| Application Service Provider Identity | OC | This field holds the identifier of the application service provider that is delivering a service to the end user. |
| Charging Rule Base Name | OC | This field holds the reference to group of PCC rules predefined at the SMF |
| 3GPP PS Data Off Status | OC | This field holds the 3GPP Data off Status during the used unit container interval |
| MA PDU Steering functionality | OC | This field holds the Steering functionality used during the used unit container interval when MA PDU session |
| MA PDU Steering mode | OC | This field holds the Steering mode used during the used unit container interval when MA PDU session. |
| Traffic Forwarding Way | OC | This field holds the traffic forwarding way for the 5G VN group communication if present. If the SMF can distinguish the traffic forwarding way, the SMF reports the volume per traffic forwarding way. If the SMF cannot distinguish, the SMF reports the traffic and the corresponding traffic forwarding way (may be multiple). |
| QoS Monitoring Report | OC | This field holds the QoS monitoring result (i.e., average packet delay per QoS flow per UE) for the service data flow. |
| MBS Session ID | OC | This field holds the MBS session identifier for the service data flow referring to clause 6.5.1 of TS 23.247 [204] if present. |
| MBS Delivery Method | OC | This field holds the MBS traffic delivery method (i.e. 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method referring to clause 4.1 of TS 23.247 [204]) for the multicast communication if present. |

Next Change

6.2.1.X	Definition of MBS session charging information

MBS specific charging information used for 5G data connectivity charging is provided within the MBS Session Charging Information.

The detailed structure of the MBS Session Charging Information can be found in table 6.2.1.X.1.

Table 6.2.1.X.1: Structure of MBS Session Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| MBS Charging Id | OC | This field holds the MBS Charging Id generated by MB-SMF for the MBS session. |
| MBS Session Information | OC | Group of MBS session information. |
| MBS Session ID | M | This field holds identifier of MBS session. |
| MBS Service Type | M | This field holds the type of the MBS session. |
| MBS Service Area | OC | This field holds MBS Service Area served by the MBS session. |
| QoS Information | OC | This field holds the QoS applied to MBS session. |
| MBS Session Start Time | OC | This field holds the timestamp when MBS session starts. |
| MBS Session Stop Time | OC | This field holds the timestamp when MBS session terminates. |
| Serving Network Function ID | OC | This field holds the identity of the serving network function. It may have multiple occurrences. |
| Serving Network Function Information | M | This field holds the Information of the serving network function: - AMF for the MBS sessions being served by MB-SMF in non-roaming |
| AMF Identifier | OC | This field holds the AMF identifier. |

6.2.1.Y	Definition of MBS container information

Used Unit Container, described in table 6.1.1.2.1, specific charging information used for 5G data connectivity charging is provided within the MBS Container Information described in table 6.2.1.Y.1.

Table 6.2.1.Y.1: Structure of MBS Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Time of First Usage | OC | This field holds the Timestamp when the first transmitted IP packet of the service data flow matching the current used unit container. |
| Time of Last Usage | OC | This field holds the Timestamp when the last transmitted IP packet of the service data flow matching the current used unit container. |
| QoS Information | OC | This field holds the QoS applied during the service data container interval. |
| Established Connection Information | OC | This field holds a list of NG-RAN nodes establishing connection, or a list of UPFs establishing connection with MB-UPF. |

Next Change

6.2.2	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by SMF for 5G data connectivity converged charging or offline only charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with “-“ (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as “-“.

Table 6.2.2.1 defines the basic structure of the supported fields in the Charging Data Request message for 5G data connectivity converged charging or offline only charging.

Table 6.2.2.1: Supported fields in Charging Data Request message

| Information Element |  | Functionality of SMF | FBC |  | QBC |  | FBC |  | QBC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Charging Service | Converged Charging |  | Converged Charging |  | Offline Only Charging |  | Offline Only Charging |  |
|  |  | Supported Operation Types | I/U/T/E |  | I/U/T/E |  | I/U/T/E |  | I/U/T/E |  |
| Session Identifier |  |  | -UT- |  | -UT- |  | -UT- |  | -UT- |  |
| Subscriber Identifier |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| NF Consumer Identification |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Invocation Timestamp |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Invocation Sequence Number |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Retransmission Indicator |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Notify URI |  |  | IU- |  | IU- |  | IU- |  | IU- |  |
| Supported Features |  |  | IU- |  | IU- |  | - |  | - |  |
| Service Specification Information |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Triggers |  |  | -UT- |  | -UT- |  | -UT- |  | -UT- |  |
| Multiple Unit Usage |  |  | IUT- |  | - |  | IUT- |  | - |  |
| Rating Group |  |  | IUT- |  | - |  | IUT- |  | - |  |
| Requested Unit |  |  | IU-- |  | - |  | - |  | - |  |
| Used Unit Container |  |  | -UT- |  | - |  | -UT- |  | - |  |
| Triggers |  |  | -UT- |  | - |  | -UT- |  | - |  |
| PDU Container Information |  |  | -UT- |  | - |  | -UT- |  | - |  |
| UPF ID |  |  | IUT- |  | - |  | IUT- |  | - |  |
| PDU Session Charging Information |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Charging Id |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| SMF Charging Id |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Home Provided Charging Id |  |  | -UT- |  | -UT- |  | -UT- |  | -UT- |  |
| SMF Home Provided Charging Id |  |  | -UT- |  | -UT- |  | -UT- |  | -UT- |  |
| User Information |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| User Location Info |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| IMS Session Information |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| MA PDU Non 3GPP User Location Info |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| User Location Time |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| MA PDU Non 3GPP User Location Time |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| UE Time Zone |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Presence Reporting Area Information |  |  | -UT- |  | -UT- |  | -UT- |  | -UT- |  |
| PDU Session Information |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| PDU Session ID |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Network Slice Instance Identifier |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| PDU Type |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| PDU Address |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| SSC Mode |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| MA PDU session information |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| SUPI PLMN ID |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Serving Network Function ID |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Serving CN PLMN ID |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| RAT Type |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| MA PDU Non 3GPP RAT Type |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Data Network Name Identifier |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| DNN Selection Mode |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Authorized QoS Information |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Subscribed QoS Information |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Authorized Session-AMBR |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Subscribed Session-AMBR |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| PDU session start Time |  |  | I--- |  | I--- |  | I--- |  | I--- |  |
| PDU session stop Time |  |  | --T- |  | --T- |  | --T- |  | --T- |  |
| Diagnostics |  |  | --T- |  | --T- |  | --T- |  | --T- |  |
| Enhanced Diagnostics |  |  | --T- |  | --T- |  | --T- |  | --T- |  |
| Charging Characteristics |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Charging Characteristics Selection Mode |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| 3GPP PS Data Off Status |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Session Stop Indicator |  |  | --T- |  | --T- |  | --T- |  | --T- |  |
| Redundant Transmission Type |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| PDU Session Pair ID |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| 5G LAN Type Service |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| SNPN Information |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| 5G Multicast Service |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Unit Count Inactivity Timer |  |  | IU-- |  | - |  | IU-- |  | - |  |
| RAN Secondary RAT Usage Report |  |  | -UT- |  |  | -UT- |  | -UT- | -UT- |  |
| Roaming QBC information |  |  | - |  | IUT- |  | - |  | IUT- |  |
| Multiple QFI container |  |  | - |  | IUT- |  | - |  | IUT- |  |
| UPF ID |  |  | - |  | IUT- |  | - |  | IUT- |  |
| Roaming Charging Profile |  |  | - |  | IU-- |  | - |  | IU-- |  |

Table 6.2.2.2 defines the basic structure of the supported fields in the Charging Data Response message for 5G data connectivity converged charging or offline only charging.

Table 6.2.2.2: Supported fields in Charging Data Response message

| Information Element |  | Functionality of SMF | FBC |  | QBC |  | FBC |  | QBC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Charging Service | Converged Charging |  | Converged Charging |  | Offline Only Charging |  | Offline Only Charging |  |
|  |  | Supported Operation Types | I/U/T/E |  | I/U/T/E |  | I/U/T/E |  | I/U/T/E |  |
| Session Identifier |  |  | I--- |  | I--- |  | I--- |  | I--- |  |
| Invocation Timestamp |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Invocation Result |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Invocation Sequence Number |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Session Failover |  |  | IU-- |  | IUT- |  | IU-- |  | IUT- |  |
| Supported Features |  |  | IU-- |  | IU-- |  | - |  | - |  |
| Triggers |  |  | IU-- |  | IU-- |  | IU-- |  | IU-- |  |
| Multiple Unit information |  |  | IU-- |  | - |  | IU-- |  | - |  |
| Result Code |  |  | IU-- |  | - |  | IU-- |  | - |  |
| Rating Group |  |  | IU-- |  | - |  | IU-- |  | - |  |
| UPF ID |  |  | IU-- |  | - |  | IU-- |  | - |  |
| Granted Unit |  |  | IU-- |  | - |  | - |  | - |  |
| Validity Time |  |  | IU-- |  | - |  | - |  | - |  |
| Final Unit Indication |  |  | IU-- |  | - |  | - |  | - |  |
| Time Quota Threshold |  |  | IU-- |  | - |  | - |  | - |  |
| Volume Quota Threshold |  |  | IU-- |  | - |  | - |  | - |  |
| Unit Quota Threshold |  |  | IU-- |  | - |  | - |  | - |  |
| Quota Holding Time |  |  | IU-- |  | - |  | - |  | - |  |
| Triggers |  |  | IU-- |  | - |  | IU-- |  | - |  |
| PDU Session Charging Information |  |  | IU-- |  | IU-- |  | IU-- |  | IU-- |  |
| Charging Id |  |  | - |  | - |  | - |  | - |  |
| SMF Charging Id |  |  | - |  | - |  | - |  | - |  |
| Home Provided Charging Id |  |  | - |  | - |  | - |  | - |  |
| SMF Home Provided Charging Id |  |  | - |  | - |  | - |  | - |  |
| User Information |  |  | - |  | - |  | - |  | - |  |
| User Location Info |  |  | - |  | - |  | - |  | - |  |
| IMS Session Information |  |  | - |  | - |  | - |  | - |  |
| MA PDU Non 3GPP User Location info |  |  | - |  | - |  | - |  | - |  |
| User Location Time |  |  | - |  | - |  | - |  | - |  |
| MA PDU Non 3GPP User Location Time |  |  | - |  | - |  | - |  | - |  |
| UE Time Zone |  |  | - |  | - |  | - |  | - |  |
| Presence Reporting Area Information |  |  | IU-- |  | IU-- |  | IU-- |  | IU-- |  |
| PDU Session Information |  |  | - |  | - |  | - |  | - |  |
| PDU Session ID |  |  | - |  | - |  | - |  | - |  |
| Network Slice Instance Identifier |  |  | - |  | - |  | - |  | - |  |
| PDU Type |  |  | - |  | - |  | - |  | - |  |
| PDU Address |  |  | - |  | - |  | - |  | - |  |
| SSC Mode |  |  | - |  | - |  | - |  | - |  |
| MA PDU session information |  |  | - |  | - |  | - |  | - |  |
| SUPI PLMN ID |  |  | - |  | - |  | - |  | - |  |
| Serving Network Function ID |  |  | - |  | - |  | - |  | - |  |
| Serving CN PLMN ID |  |  | - |  | - |  | - |  | - |  |
| RAT Type |  |  | - |  | - |  | - |  | - |  |
| MA PDU Non 3GPP RAT Type |  |  | - |  | - |  | - |  | - |  |
| Data Network Name Identifier |  |  | - |  | - |  | - |  | - |  |
| DNN Selection Mode |  |  | - |  | - |  | - |  | - |  |
| Authorized QoS Information |  |  | - |  | - |  | - |  | - |  |
| Subscribed QoS Information |  |  | - |  | - |  | - |  | - |  |
| Authorized Session-AMBR |  |  | - |  | - |  | - |  | - |  |
| Subscribed Session-AMBR |  |  | - |  | - |  | - |  | - |  |
| PDU session start Time |  |  | - |  | - |  | - |  | - |  |
| PDU session stop Time |  |  | - |  | - |  | - |  | - |  |
| Diagnostics |  |  | - |  | - |  | - |  | - |  |
| Enhanced Diagnostics |  |  | - |  | - |  | - |  | - |  |
| Charging Characteristics |  |  | - |  | - |  | - |  | - |  |
| Charging Characteristics Selection Mode |  |  | - |  | - |  | - |  | - |  |
| 3GPP PS Data Off Status |  |  | - |  | - |  | - |  | - |  |
| Session Stop Indicator |  |  | - |  | - |  | - |  | - |  |
| Redundant Transmission Type |  |  | - |  | - |  | - |  | - |  |
| PDU Session Pair ID |  |  | - |  | - |  | - |  | - |  |
| 5G LAN Type Service |  |  | - |  | - |  | - |  | - |  |
| SNPN Information |  |  | - |  | - |  | - |  | - |  |
| 5G Multicast Service |  |  | - |  | - |  | - |  | - |  |
| Unit Count Inactivity Timer |  |  | IU-- |  | - |  | - |  | - |  |
| RAN Secondary RAT Usage Report |  |  | - |  | - |  | - |  | - |  |
| Roaming QBC information |  |  | - |  | IU-- |  | - |  | IU-- |  |
| Multiple QFI container |  |  | - |  | - |  | - |  | - |  |
| UPF ID |  |  | - |  | - |  | - |  | - |  |
| Roaming Charging Profile |  |  | - |  | IU-- |  | - |  | IU-- |  |

Table 6.2.2.X defines the basic structure of the supported fields in the Charging Data Request message for 5G data connectivity converged charging for MB-SMF to support 5G MBS charging.

Table 6.2.2.X: Supported fields in Charging Data Request message

| Information Element | Functionality of MB-SMF | FBC |
| --- | --- | --- |
|  | Charging Service | Converged Charging |
|  | Supported Operation Types | I/U/T/E |
| Session Identifier |  | -UT- |
| NF Consumer Identification |  | IUT- |
| Invocation Timestamp |  | IUT- |
| Invocation Sequence Number |  | IUT- |
| Triggers |  | -UT- |
| Multiple Unit Usage |  | IUT- |
| Rating Group |  | IUT- |
| Used Unit Container |  | -UT- |
| Triggers |  | -UT- |
| MBS Container Information |  | -UT- |
| MBS Session Charging Information |  | IUT- |
| MBS Charging Id |  | IUT- |
| MBS Session Information |  |  |
| MBS Service Type |  | IUT- |
| MBS Service Area |  | IUT- |
| QoS Information |  | IUT- |
| MBS Session Start Time |  | I--- |
| MBS Session Stop Time |  | --T- |
| Serving Network Function ID |  | IUT- |

Table 6.2.2.Y defines the basic structure of the supported fields in the Charging Data Response message for 5G data connectivity converged charging for MB-SMF to support 5G MBS charging.

Table 6.2.2.Y: Supported fields in Charging Data Response message

| Information Element | Functionality of MB-SMF | FBC |
| --- | --- | --- |
|  | Charging Service | Converged Charging |
|  | Supported Operation Types | I/U/T/E |
| Session Identifier |  | I--- |
| Invocation Timestamp |  | IUT- |
| Invocation Result |  | IUT- |
| Invocation Sequence Number |  | IUT- |
| Triggers |  | IU-- |
| Multiple Unit information |  | IU-- |
| Result Code |  | IU-- |
| Rating Group |  | IU-- |
| Time Quota Threshold |  | IU-- |
| Triggers |  | IU-- |
| MBS Session Charging Information |  | IU-- |
| MBS Charging Id |  | - |
| MBS Session Information |  |  |
| MBS Service Type |  | - |
| MBS Service Area |  | - |
| QoS Information |  | - |
| MBS Session Start Time |  | - |
| MBS Session Stop Time |  | - |
| Serving Network Function ID |  | - |

End of Change