# 3GPP TSG-SA5 Meeting #152	S5-237488

# 13 - 17 November 2023, Chicago, IL, USA

# Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO, NEC

# Title:		Input to DraftCR 28.105 AI/ML Inference emulation Requirements

# Document for:	Approval

# Agenda Item:	6.4.3.1 - AIML_MGMT_WoP#1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	S5-237244 DraftCR : “Draft CR AIML_MGMT - TS 28.105; Enhancements for AI-ML management”.

# 3	Rationale

The use case of Inference emulation has been discussed and agreed (in TR 28.908). This CR is to normatively propose the use case and requirements for the mentioned use case.

# 4	Detailed proposal

Start of First change

## 6.3	AI/ML emulation phase

### 6.3.1	Description

Development of an ML entity involves three processes - training, validation, and testing. During the training, validation is done after every training cycle to check if the training process is completed successfully or not. If validation shows that training was not successful, re-training, and subsequent re-validation is further executed. If the training is successful, testing may be executed using specific test data. With the successful testing, the ML entity is then declared as, well developed entity.

Before the ML entity is applied in the production network, it may be necessary to execute its inference in one or more environments that emulate (to different extents) the expected inference characteristics, in a process that may be termed as Inference emulation.

### 6.3.2	Use cases

#### 6.3.2.A	AI/ML Inference emulation

After the ML entity is validated and tested during development, it is necessary to check, through inference emulation, if the ML entity or the associated ML inference function is working correctly under certain runtime context. Here inference emulation refers to executing inference in an environment that emulates the expected runtime context, a process needed mainly for ML entities that may have been developed outside the specific production network. The specification of emulation environments e.g., in terms of complexities, characteristics and numbers are outside the scope of standardization. For example, in one case the emulation environment may only be a simulation environment, in another a simulation and a test network, though the case of using the real network is not strictly emulation and is as such addressed through controlled activation of the ML entity which allows activation for only a sub scope of the real network.

It is necessary to support inference emulation, providing means for:

-	the MnS consumer to request for a specific AI/ML inference to be executed in an AI/ML inference emulation environment available at the emulation MnS producer,

-	an MnS consumer to request an MnS producer for AI/ML inference emulation to execute a given AI/ML inference and for a managed function to execute or apply the AI/ML inference in a controlled way.

-	an MnS consumer to manage the process of evaluating the inference outcomes and progressing ML entities or their related inference functions through multiple AI/ML emulation environments.

The network or its management system needs to have the capabilities to provide the services needed to enable the MnS consumer to request inference emulation and receive feedback on the inference emulation of an ML entity.

### 6.3.3	Requirements for Managing AI/ML Inference emulation

Table 6.3.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AI/ML_EMUL-1 | The MnS Producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer to request execution of ML inference emulation for a specific ML entity. | AI/ML Inference emulation (clause 6.3.2.A) |
| REQ-AI/ML_EMUL-4: | The MnS Producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer (e.g., an operator) to manage an ML inference emulation request or ML inference emulation process, e.g., to start, suspend or restart the inference emulation. | AI/ML Inference emulation (clause 6.3.2.A); |
| REQ-AI/ML_EMUL-5: | The MnS producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer to receive reporting about the ML inference emulation. | AI/ML Inference emulation (clause 6.3.2.A); |

End of Changes