# 3GPP TSG SA WG5 Meeting #152                 	          S5-238009

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	Add the charging data request and response massage

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the message contents in TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

3	Rationale

Describe the message contents in Charging Data Request/Response.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

6.1	Data description for time sensitive networking charging

6.1.1	Message contents

6.1.1.1	General

The Charging Data Request and Charging Data Response are specified in subclause 5.1.2.2.1 of 3GPP TS 32.290 [5].

Table 6.1.1.1-1 describes the use of these messages for time sensitive networking charging.

Table 6.1.1.1-1: Time sensitive networking charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | TSCTSF, TSN AF | CHF |
| Charging Data Response | CHF | TSCTSF, TSN AF |

Note: the SMF and NEF as source for TSN charging message is described in the respective specification.

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of 3GPP TS 32.240 [2].

6.1.1.2	Charging Data Request message

Table 6.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the TSN AF or TSCTSF as used for time sensitive networking charging.

Table 6.1.1.2-1: Charging Data Request message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in 3GPP TS 32.290 [5]. |
| Subscriber Identifier | - | This field is not applicable. |
| Tenant Identifier | OM | This fields indicate AF identifier. |
| NF Consumer Identification | M | Described in 3GPP TS 32.290 [5] and holds the identifier of the TSN AF or TSCTSF. |
| NF Functionality | M | Described in 3GPP TS 32.290 [5]. |
| NF Name | OC | Described in 3GPP TS 32.290 [5]. |
| NF Address | OC | Described in 3GPP TS 32.290 [5]. |
| NF PLMN ID | OC | Described in 3GPP TS 32.290 [5]. |
| Charging Identifier | OM | Described in 3GPP TS 32.290 [5]. |
| Invocation Timestamp | M | Described in 3GPP TS 32.290 [5]. |
| Invocation Sequence Number | M | Described in 3GPP TS 32.290 [5]. |
| Retransmission Indicator | OC | Described in 3GPP TS 32.290 [5]. |
| One-time Event | OC | Described in 3GPP TS 32.290 [5]. |
| One-time Event Type | OC | Described in 3GPP TS 32.290 [5]. |
| Notify URI | - | This field is not applicable. |
| Supported Features | OC | Described in 3GPP TS 32.290 [5]. |
| Service Specification Information | OC | Described in 3GPP TS 32.290 [5]. |
| Triggers | - | This field is not applicable. |
| Multiple Unit Usage | - | This field is not applicable. |
| TSN Charging Information | OM | This field holds TSN specific information described in clause 6.2.1.2. |

6.1.1.3	Charging data response message

Table 6.1.1.3-1 illustrates the basic structure of a Charging Data Response message from the CHF to the TSN AF or TSCTSF as used for time sensitive networking charging.

Table 6.1.1.3-1: Charging Data Response message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57]. |
| Invocation Timestamp | M | Described in TS 32.290 [57]. |
| Invocation Result | OC | Described in TS 32.290 [57]. |
| Invocation Sequence Number | M | Described in TS 32.290 [57].s |
| Session Failover | OC | Described in TS 32.290 [57]. |
| Supported Features | OC | Described in TS 32.290 [57]. |
| Triggers | - | This field is not applicable. |
| Multiple Unit Information | - | This field is not applicable. |

6.1.2	Ga message contents

Details of the Ga message contents are specified in TS 32.295 [55].

End of change