3GPP TSG-SA5 Meeting #153 	S5-240135

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0206 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.312 Rapporteur clean up |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2024-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following issues need to be addressed: - Several editorial issues. - Two “REQ-Intent_NetOpt-1” exist in 5.1.6. - In 6.2.1.4, “with order” is redundant, since“ordered” and “with order” have the same meaning. - The attribute recommendedSolutions in IntentConflictReport << dataType >> is missing in stage3 clause 7.2.2 - The sentence “The ExpectationTarget <<dataType>> includes a contextSelectivity used to define how to select among the stated targetContexts.”should be in clause 6.2.1.3.3.1 instead of 6.2.1.3.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clean up of TS 28.312 to address above observed issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Several issues exist in the published TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, 4.5.2, 4.5.3, 4.5.4, 5.1.3.1, 5.1.5.1, 5.1.5.2, 5.1.6.2, 5.1.7.1, 5.1.7.2, 5.1.8.1, 5.1.8.2, 5.3.2.1, 5.3.3, 5.3.4.3, 6.2.1.3.2.1, 6.2.1.3.3.1, 6.2.1.3.5.1, 6.2.1.3.9.1, 6.2.1.4, 7.2.2, 7.2.3, 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR Link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/993 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.2.3	Intent translation

The Intent driven MnS producer is the provider of Intent driven MnS and is responsible for deriving activities for networks and services or other intent(s).

The MnS consumer may consume Intent Driven MnS(s) provided by the Intent driven MnS producer(s) or may have the consumer role for non-intent MnS producers.

The conflict(s) including conflict between the intent and other intent(s) and/or Non-intent requirements needs to be detected and resolved during the intent translation. Figure 4.2.3-1 illustrates the potential way to satisfy intent-CSC.

-	Intent-CSC MnS producer provides intent driven MnS for communication services. Intent-CSC MnS producers receive the expressed intent and translate it to Intent-CSP or network requirements, then may consume Intent-CSP MnS(s) or Non-Intent MnS(s) for network to fulfil the intent-CSC.

-	Intent-CSP MnS producer provides intent driven MnS for network services. Intent-CSP MnS producers receive the intent and translate it to new Intents for NOP or network requirements, then may consume Intent-NOP MnS(s) or Non-Intent MnS(s) for NE to fulfil the intent-CSP.

-	Intent-NOP MnS producer provides intent driven MnS for network equipment. Intent-NOP MnS Producers receive the expressed intent, and translate it to detailed network requirements, then takes some internal actions to fulfil the intent-NOP.

Figure 4.2.3-1: Potential way to satisfy intent-CSC

Next Change

4.5	General concept of Intent Content

4.5.1	Intent Expectation

In the most basic form, a consumer may use an intent to express to the producer the need for:

"an object O with characteristics S".

Where the characteristics S reflect the requirements, goals and contexts for an object.

The object may be a 3GPP managed object like a network slice, subnetwork (e.g. radio network) or other objects like a service. The consumer may desire the same requirements, goals and contexts for multiple objects with the same properties, in which case the intent may be stated for a list of objects as

"objects {O1,O2, …ON} with characteristics S"

However, the consumer may wish to express different requirements, goals and contexts for objects with different properties. It is in that case necessary to distinguish the requirements, goals and contexts to be achieved for each set of objects with the same properties. Correspondingly, the combination of requirements, goals and contexts for each set of objects with the same properties is the Intent Expectation. Also the consumer may wish to distinguish the requirements, goals and contexts for different objects with the same properties, in this case, the combination of requirements, goals and contexts for each object instance may be contained in a separate Intent Expectation or requirements, goals and contexts for the multiple object instances may be combined in a single Intent Expectation.

4.5.2	Expectation Targets

For a given intent expectation, the desired characteristics of the object(s) are the expectation targets to be achieved. The expectation targets may include the metrics that characterize the performance of the object(s) or some abstract index that expresses the behaviour of the object(s). A given intent expectation may include multiple expectation targets on the same object or on different objects with the same properties. A consumer may for example require for the Network Slice object(s) that User throughput > 5Mbps and latency < 1ms. The expectation targets may also be context specific, i.e. the intent may require a specific expectation targets given a specific target context. As such with the characteristics as a combination of expectation targets and target contexts, the intent expectation may be stated as:

"ensure that for

Expectation Object O,

Expectation Target_1 is T_1, Target Context_1 is C_1

….,

Expectation Target_m is T_m, Target Context_k is C_k;

Each expectation target expresses an aspect of the characteristics of the object under consideration, i.e. it expresses desired characteristics on a specific object. Each of the object characteristic may be desired to be equivalent to a specific value or constrained to a value or a range of values, e.g. as listed in Table 4.5.2-1. The combination of the name of characteristic (or simply the targetName), the condition constraining the characteristic and the value or value range for the characteristic is the target, i.e. the Expectation Target is the tuple:

Expectation Target = [targetName, condition, value range]

Table 4.5.2-1: Examples of Expectation Targets for different Objects

| Example of Expectation Targets | ExpectationObject | targetName | Condition | Value range |
| --- | --- | --- | --- | --- |
| example 1 | Network Slice | Coverage area | Is greater than | 40 km radius |
| example 2 | Communication Service | User throughput | Is greater than | 2 Mbps |

4.5.3	Expectation Objects

The object (s) for which a given expectation is addressed can be expressed with the object's identifier. This may, however, not always be adequate (e.g. if the consumer does not have or know the identifiers of the object) or may be cumbersome for some intents.

EXAMPLE 1:	It may be easier to state "all network slices in city ABC" as opposed to listing the individual network slice. As such it may be easier to identify the objects by stating the object context information that filters and identifies the desired objects.

The objectContext is in form of a context list whose entries are each a tuple (attribute, condition, value range).

EXAMPLE 2:	In the case of "all network slices in a city" there is an object context, which is the tuple (location, =, city_ABC) and (objectType, =, network slice).

4.5.4	Context

Each expectation target may be constrained to only be achieved for a very specific set of conditions as context constraints. The context describes a set of conditions to trigger corresponding management tasks to acheive the expectation targets. For example, the consumer may state that: "ensure that handoverFailureRate < 2 % if Load > 80 %", where the expectation target (HandoverFailureRate < 2 %) is to be achieved only in the context (Load > 80 %). In this example, the producer will perform handover tasks to achieve the expectation target "HandoverFailureRate < 2 %" when observe the context "Load > 80 %".

Similar to the target, the context is also a tuple of < attribute, condition, value range > but where the values having a different semantics.

Although contexts and targets have the same structure, to distinguish between what needs to be achieved and the context which is only to be considered as required conditions, the context has to be explicitly stated separate from the target. For example, if the consumer wishes that the Radio Link Failure rate (RLF) is less than 2 % when the load is more than 50 %. If the context (i.e. load > 50 %) is not explicitly stated/modelled as context, the producer could interpret the request to mean (RLF < 2 % and load > 50 %).

For a given expectation, the specific list of expectation targets may be desired to be achieved for given combined contexts, i.e. besides the expectation targets, an expectation may state a list of contexts which apply to all expectation targets within the intent expectation. Similarly, there may be contexts that apply to all expectations within a given intent. Correspondingly, both Intent expectations and intents should be modelled to only contain the contexts that apply to all the contained sub elements.

Multiple contexts may be stated on an intent, on an intentExpecation, on an expectationObject or on an expectationtarget. However, the MnS consumer expressing multiple overlapping contexts may not have interest in simultaneously applying all the overlapping contexts but only in applying any one or more of the contexts. For example, the consumer may have requirements for any one of multiple physical locations at any given time. In such cases, the MnS consumer requires means to express the context to be selected among any one of these. Otherwise, the producer will either deliver for all contexts. Alternatively, the MnS consumer would only be able to express one context at time and wait e.g., for a feasibility check to fail to then resend the intent with revised context.

Next Change

5.1.3	Intent containing an expectation for delivering a service at the edge.

5.1.3.1	Introduction

This use case describes a scenario where the MnS consumer, express the intent containing an expectation for delivering a service at the edge of the network. The intent expectation for a service includes service type (URLLC, eMBB), service requirements (number of concurrent subscribers and number of concurrent sessions), service availability and the target location.

MnS producer notifies MnS consumer about the fulfilment information of the intent containing an expectation for delivering a service at the edge.

5.1.3.2	Requirements

REQ-Intent_Deploy_Net-CON-1 The intent driven MnS shall have capability enabling authorized MnS consumer to express intent containing an expectation for delivering a service at the edge of the network to MnS producer.

REQ-Intent_Deploy_Net-CON-2 The intent driven MnS shall have capability enabling authorized MnS consumer to obtain intent report information (including fulfilment information) for intent containing an expectation for delivering a service at the edge of the network.

Next Change

### 5.1.5	Intent containing an expectation on radio network performance to be assured

#### 5.1.5.1	Introduction

In this scenario, MnS consumer expresses its intent containing an intent expectation on radio network performance (including RAN UE throughput performance, radio network capacity performance) to be assured to MnS producer, which may include area information (e.g. geographical area), RATs (e.g. NR only, EUTRAN only, or all RATs), frequency information (e.g. nRFrequencyBand), radio network performance targets, optional performance scope (e.g. specific service type, specific UE groups).

The radio network performance targets include the targets in the following target categories based on what radio network performance MnS consumer expected to be assured.

-	RAN UE throughput targets, for example, target average UL/DL RAN UE throughput, target percentage of UE with low UL/DL RAN UE throughput (e.g. < 5 Mbps), target percentage of UE with high UL/DL RAN UE throughput (e.g. > 50 Mbps).

-	Radio network capacity target, for example, target percentage of high UL/DL PRB Load (e.g. < 70%), target average UL/DL PRB load (e.g. <85%).

Based on the intent containing expectation on radio network performance for the specified area to be assured received, MnS producer collects and analyses corresponding radio network performance related data (e.g. RAN UE throughput data, number of PRBs used for UL/DL traffic transmission) in the specified areas, identifies the potential radio network performance issues (e.g. low RAN UE throughput for certain areas, high load for certain areas, high UL/DL PRB Load issue), which will impact radio network performance intent satisfaction, analyses the cause, evaluates, decides and adjusts the radio feature configuration parameters for impacted RAN NEs/Cells to address the radio network performance issues in the specified areas. The artificial intelligence or machine learning technologies may be used to select the optimal radio feature configuration parameters set rapidly to satisfy radio network performance targets.

MnS producer continuously monitors the radio network performance (e.g. average UL/DL RAN UE throughput, percentage of UE with low UL/DL RAN UE throughput (e.g. < 5 Mbps), percentage of UE with high UL/DL RAN UE throughput (e.g. > 50 Mbps), percentage of high UL/DL PRB Load (e.g. < 70%)) for the specified area, and decides whether radio network performance targets are satisfied.

MnS producer may notify MnS consumer about the intent fulfilment information and achieved value for radio network targets, including the RAN UE throughput performance (e.g. average UL/DL RAN UE throughput, percentage of UE with low UL/DL RAN UE throughput, percentage of high UL/DL PRB Load) for the specified area which enables MnS consumer to monitor the intent containing an expectation on radio network performance to be assured.

#### 5.1.5.2	Requirements

REQ-Intent_Opt_Thp-CON-1 The intent driven MnS for radio network shall have capability enabling MnS consumer to express intent containing an expectation on RAN UE throughput performance to be assured for specified area to MnS producer.

REQ-Intent_Opt_Thp-CON-2 The intent driven MnS for radio network shall have capability enabling MnS consumer to obtain intent report information (including fulfilment information) for the intent containing an expectation on RAN UE throughput performance to be assured.

REQ-Intent_Opt_Capacity-CON-1: The intent driven MnS for radio network shall have capabilities enabling the MnS consumer to express intent containing an expectation on radio network capacity performance to be assured for the specified area.

REQ-Intent_ Opt _Capacity-CON-2: The intent driven MnS for radio network shall have capabilities enabling the MnS consumer to obtain intent report information (including fulfilment information and achieved value) for intent containing an expectation on radio network capacity performance to be assured.

Note: the example of radio network capacity performance is target percentage of high UL/DL PRB Load for a specified Geographical area.

### 5.1.6	Intent containing an expectation for end-to-end network optimization

#### 5.1.6.1	Introduction

In this scenario, MnS consumer expresses its intent containing an intent expectation with targets on the whole Network including RAN and Core. The intent may for example be for optimization of the network resources, i.e. the intent expectation captures the objectives for an entity that undertakes optimization for the network. The expectation may be termed as network resources expectation. The network resources expectation targets may express the desired performance optimization outcomes. Depending on the stated targets, the MnS producer may as such configure one or more optimization functions to achieve the desired targets. The network optimization expectation targets may for example be end-to-end KPI targets that the optimization is required to achieve.

MnS producer notifies MnS consumer about the fulfilment information of the intent containing an expectation for end-to-end network optimization.

The network optimization expectation may include relative prioritizations of the different targets which indicate the relative interests of the intent MnS consumer on the different network attributes.

#### 5.1.6.2	Requirements

REQ-Intent_NetOpt-1 The intent driven MnS for end-to-end network shall have capability enabling MnS consumer to express intent containing an expectation with targets on the end-to-end Network including the RAN and core.

REQ-Intent_NetOpt-2 The intent driven MnS for end-to-end network shall have capability enabling MnS consumer to express intent containing an expectation with the prioritization of the targets to be achieved.

REQ-Intent_ NetOpt-3 The intent driven MnS for end-to-end network shall have capability enabling MnS consumer to obtain intent report information (including fulfilment information) for the intent containing an expectation with targets on the end-to-end Network.

5.1.7	Intent containing an expectation for RAN energy saving

5.1.7.1	Introduction

Operators are aiming at decreasing power consumption in 5G networks to lower their operational expense with energy saving management solutions. Energy saving is achieved by executing the energy saving actions with suitable parameter configurations, e.g. energy saving state switch, start time and end time, the energy saving thresholds. However, the various combinations of energy saving actions can lead to conflicts. For example, different energy saving actions may be contradictory, or the energy saving actions may conflict with other activities (e.g. network optimization actions). Moreover, it is not straightforward to evaluate the influence on service experience (e.g. UL/DL RAN UE throughput, latency) of energy saving actions beforehand, which makes it difficult to balance the energy saving effect and service experience, for example the energy saving actions may deteriorate the service experience. To avoid affecting the service experience, MnS consumer may express energy saving target with the maximum value of RAN energy consumption in intent expectation, and MnS producer is able to choose an optimal value of RAN energy consumption to save energy as much as possible in the context to satisfy the service experience.

As clause 4.1.1 described, an intent focuses more on describing the "What" needs to be achieved but less on "How" that outcomes should be achieved, which not only relieves the burden of the consumer knowing implementation details but also leaves room to allow the producer to explore alternative options and find optimal solutions. So, introducing the intent approach for energy saving, which can enable the 3GPP management system to analyse and select the optimal balance between the energy saving effect and service experience by utilizing some intelligence mechanisms. In intent driven approach, a MnS consumer expresses intent expectation for RAN energy saving in the specified area (e.g. geographical area) to a MnS producer, which may include the RAN energy saving target (e.g. the maximum value of target RAN energy consumption, reduction radio of energy consumption) and service experience (e.g. RAN UE throughput, latency), as well as the frequencies and RATs to be considered for energy saving. Some contexts for RAN energy saving (e.g. RAN energy saving allowed time (e.g., 1:00 am-5:00 am), RAN energy saving trigger event (e.g. PRB load ratio < 50%)) also can be specified by MnS consumer to provide the conditions to allow corresponding energy saving actions to be triggered to satisfy the energy saving targets. MnS producer analyses and determines the optimal RAN energy saving solution (i.e. a set of energy saving actions) to satisfy MnS consumer's intent expectation for RAN energy saving. MnS producer continuously monitors the RAN energy saving performance (e.g. RAN energy consumption, RAN energy efficiency) and service experience performance (e.g. target average UL/DL RAN UE throughput, target) for the specified area, and decides whether RAN energy saving target is satisfied.

MnS producer may report the intent fulfilment information and achieved value for RAN energy saving targets (e.g. RAN energy consumption, RAN energy efficiency) for the specified area to MnS consumer which enables MnS consumer to monitor the intent containing an expectation for RAN energy saving.

5.1.7.2	Requirements

REQ-Intent_Opt_EnergySaving-1: The intent driven MnS for radio network shall have capabilities enabling MnS consumer to express intent containing an expectation for RAN energy saving for the specified area to MnS producer.

REQ-Intent_Opt_EnergySaving -2: The intent driven MnS for radio network shall have capabilities enabling MnS consumer to obtain intent report information (i.e. fulfilment information, achieved value for corresponding targets) of the intent containing an expectation for RAN energy saving.

5.1.8	Intent containing an expectation for 5GC network

5.1.8.1	Introduction

This use case describes a scenario where a MnS consumer expresses intent containing an expectation related to 5GC network to the intent driven MnS producer. In this scenario, MnS consumer expresses its intent expectation which may include area information (e.g. geographic area, data center), type of the network (e.g. NPN), included 5GC NF list (e.g. NF type, range of NF instance ID), PLMN information, supported APN information, transport related parameters (e.g. list of end point addresses information), and target network capacity information (e.g. number of PDU session, number of registered subscribers, UL/DL throughput).

Based on the received intent containing an expectation for 5GC network, the intent driven MnS producer decides whether to deploy a new 5GC network in the specified area or to re-use and modify an existing 5GC network. If a new 5GC network is to be deployed, the intent driven MnS producer translates the intent 5GC network expectations into appropriate 5GC network provisioning actions, this may include generation of network configuration parameters (including 5GC network/NFs configuration parameters and transport network configuration parameters) and triggering NS/VNF creation procedure by interworking with ETSI NFV MANO. If an existing 5GC network is to be re-used, the intent driven MnS producer identifies the potential 5GC network performance issues (e.g. low performance because of high load) for the existing 5GC network, modifies the 5GC NF configuration parameters if needed to satisfy the performance expectation targets (this may also trigger scaling procedure by interworking with ETSI NFV MANO). Multiple interactions between the Intent driven MnS consumer and the intent driven MnS producer may be needed based on the intent management capabilities (e.g. intent feasibility check) provided by intent driven MnS producer.

The intent driven MnS producer continuously monitors the 5GC network performance (e.g. mean number of registered UE, mean number of created PDU session), and decides whether 5GC network expectation is satisfied. If the 5GC network expectation is not satisfied, the intent driven MnS producer identifies the potential 5GC network performance issues and modifies the 5GC NF configuration parameters if needed to satisfy the performance expectation targets.

On a regular basis, the intent driven MnS producer notifies MnS consumer about the fulfilment information of the intent.

5.1.8.2	Requirements

REQ-Intent_5GC_Net-CON-1 The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation for delivering a 5GC network for the specified area to MnS producer.

REQ-Intent_5GC_Net-CON-2 The intent driven MnS shall have capability enabling MnS consumer to obtain intent report information (i.e. fulfilment information, achieved value for corresponding targets, conflict information and fulfilment feasibility check information) of the intent containing an expectation for delivering a 5GC network.

REQ-Intent_5GC_Net-CON-3 The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation on 5GC network performance to be assured to the intent driven MnS producer.

REQ-Intent_5GC_Net-CON-4 The intent driven MnS shall have capability enabling MnS consumer to obtain intent report information (i.e. fulfilment information, achieved value for corresponding targets, conflict information and fulfilment feasibility check information) of the intent containing an expectation on 5GC network performance to be assured.

Next Change

5.3.2	Intent report

5.3.2.1	Introduction

The intent fulfilment information is defined as one type of intent report information in which the intentFulfilmentInfo, expectationFulfilmentInfo and targetFulfilmentInfo are included for the MnS consumer to monitor the intent fulfilment information. The intent fulfilment information also can contain the current and optional predicted value for performance indicated by corresponding expectation targets (e.g. WeakRSRPRatio for the weakRSRPRatioTarget, Average UL RAN UE Throughput for aveULRANUEThptTarget), which can be used by MnS consumer to validate whether the intent is really fulfilled and to evaluate whether the intent (especially for expectation targets) needs to be updated if needed (improve the target value when corresponding target is fulfilled or reduce the target value when corresponding target is not fulfilled). Besides, intent conflict information and intent fulfilment feasibility check information sent by MnS producer to MnS consumer is another type of intent report information. So, following are the three types of information needs to be monitored by MnS consumer:

-	Intent Fulfilment information, which represents the properties of a specific fulfilment information for an aspect of the intent (i.e. either an expectation, a target or the whole intent), including fulfilment status and achieved values for targets.

-	Intent conflict information, which represents intent with conflict. The information includes conflict type (i.e., intent conflict, expectation conflict and target conflict) and possible solutions (e.g. intent deletion, intent modification).

-	Intent fulfilment feasibility check information, which indicates that the intent is feasible or infeasible. Intent fulfilment feasibility check information is provided after MnS producer automatically performs feasibility check when receiving the intent creation and modification request from MnS consumer.

Different MnS consumer may have different requirements for intent report (e.g. Some MnS consumer may want to have corresponding performance value information while others do not want. Different MnS consumer may want to calculate or monitor the performance value in different period).

MnS consumers may subscribe to get notifications of changes in the intent report.

An intent under fulfilment may go through multiple states, i.e., the life cycle includes multiple alternative transitions. The transitions are triggered either by actions of the MnS consumer or observations at the MnS producer or its intent handler. Generally, intents received at the MnS producer will have one of the states represented in Figure 5.3.2.1-1.

Figure 5.3.2.1-1: State transitions and reporting events for Intents delivered for fulfilment.

Intent reports should be delivered for each of these different states and related state transitions, specifically for:

1.	ACKNOWLEDGED: When an intent is created, its default state is "ACKNOWLEDGED". An intent report should be delivered to indicate that the intent has been created. The transitions from "ACKNOWLEDGED" state can only be to "COMPLIANT", "FULFILLMENT_FAILED" or "TERMINATED " states as described hereafter.

2.	COMPLIANT: When the feasibility check for the intent is successful and the intent is accepted as being compliant, the intent state is changed to "COMPLIANT". An intent report should be delivered to indicate the results of the feasibility check. In the case of an intent delivered for fulfillment, a corresponding intent report should be delivered to indicate the status change. The payload for the intent feasibility report (outcome of the feasibility check) may be one of the following:

1.	an indication for feasible or infeasible;

2.	a detailed report indicating which intent expectations or expectationTargets are feasible or not;

3.	a detailed report indicating the achievable outcomes on the individual intent expectations or expectationTargets.

The transitions from "COMPLIANT" state are either to "ACKNOWLEDGED" state when the intent attributes are modified by the MnS consumer, or to "SUSPENDED", "FULFILLED" or "FULFILLMENT_FAILED" states as described hereafter.

3.	SUSPENDED: If the MnS consumer decides to suspend the intent, the intent state changes to "SUSPENDED". Alternatively, an event may occur while the MnS producer attempts to fulfil the intent, in which case the MnS producer may suspend the intent pending further actions. Such events at the MnS producer may among others include conflicts from another intent and resource constraints. An intent report should be delivered to indicate that the intent has been suspended. The intent report may include a reason for the suspension in the case where the intent was suspended by the MnS producer.

The transitions from "SUSPENDED" state are either to "ACKNOWLEDGED" state when the intent attributes are modified by the MnS consumer, or to "COMPLIANT" state if the suspension is lifted by the entity (MnS consumer or MnS producer) that suspended the intent. Otherwise, the state is "FULFILLMENT_FAILED" if the intent is deleted.

4.	FULFILLED: If the MnS producer considers that the intent, expectation or target have been fulfilled as stated by the MnS consumer, the state changes to "FULFILLED". An intent report should be delivered to indicate that the intent has been fulfilled.

The transitions from "FULFILLED" state are either to "ACKNOWLEDGED" state when the intent attributes are modified by the MnS consumer or to "DEGRADED" as described for the "DEGRADED" state above.

5.	DEGRADED: If an intent that was previously fulfilled but after a period of observation it is found not be meeting the initially stated requirements, the state changes to "DEGRADED". An intent report should be delivered to indicate that the intent has degraded.

The transitions from "DEGRADED" state are either to "ACKNOWLEDGED" state when the intent attributes are modified by the MnS consumer, or to "FULFILLED" state if the MnS producer once again fulfills the intent.

6.	FULFILLMENT_FAILED: If the MnS producer determines that they cannot do anything to fulfil the intent, the state states to "FULFILLMENT_FAILED". An intent report should be delivered to indicate that the fulfillment of the intent has ceased. The intent is not deleted unless the MnS consumer explicitly requests for it to be deleted.

The only transition from "FULFILLMENT_FAILED" state is to "ACKNOWLEDGED" state when the intent attributes are modified by the MnS consumer.

7.	TERMINATED: If the MnS consumer requests to delete the intent, the state changes to "TERMINATED". An intent report should be delivered to indicate that the intent has been terminated.

There is no possible transition from TERMINATED state.

8.	The intent may be modified when in any of the states "ACKNOWLEDGED", "COMPLIANT ", "SUSPENDED " "FULFILLED ", "DEGRADED" or "FULFILLMENT_FAILED". An intent report should be delivered to indicate that the intent has been modified.

9.	The intent may be deleted when in any of the states "ACKNOWLEDGED", "COMPLIANT ", "SUSPENDED " "FULFILLED ", "DEGRADED" or "FULFILLMENT_FAILED". An intent report should be delivered to indicate that the intent has been deleted.

Next Change

5.3.3	Intent fulfilment feasibility check

5.3.3.1	Introduction

This capability can be used to assist MnS consumer to generate the suitable intent information for intent driven MnS producer. When intent driven MnS producer receives the intent instance creation or modification request from MnS consumer, the intent driven MnS producer automatically conducts feasibility check to determine whether the intent instance is feasible (including check the satisfaction of intent fulfilment and potential conflicts between one or more intent instances), and notify MnS consumer the result of feasibility check. If the result of intent fulfillment feasibility check is feasible, the MnS producer performs the service or network management tasks to satisfy the intent instance. In case the result of intent fulfillment feasibility check is infeasible, MnS producer notifies the MnS consumer the reason of infeasibility and corresponding recommendations, then the MnS consumer decides how to handle the issue that intent is infeasible, e.g. update the intent, suspend the intent, delete the intent, etc.

5.3.3.2	Requirements

REQ-Intent_Driven_MnS- Feasibilitycheck-1: The intent-driven MnS producer should have capability to report the authorized MnS consumer the output of automatic feasibility check when receive the intent creation and modification request.

REQ-Intent_Driven_MnS- Feasibilitycheck-2: The intent-driven MnS producer shall have capability to inform the authorized MnS consumer about the result of intent fulfilment feasibility check, including feasible or infeasible.

REQ-Intent_Driven_MnS- Feasibilitycheck-3: The intent-driven MnS producer shall have capability to inform the authorized MnS consumer about the infeasible reason and corresponding recommendations if the result of intent fulfilment feasibility check is infeasible.

Next Change

5.3.4.3	Resolving Intent-related conflicts

To resolve the conflict, the MnS producer may derive required solutions, such as suspension of the whole intent that contains conflicts or has conflicts with another intent. Alternatively, the MnS producer may recommend a new intent in place of the conflict intent (e.g., adding recommended expectations or targets, or termination of part of intent). Also, the MnS producer may suggest alternative intent fulfilment e.g., by updating the execution time of the intent.

The decisions taken by the MnS producer may also be guided by the MnS consumer through explicit conflict handling guidelines. The MnS consumer may provide intent expectations and expectation targets as an ordered list for which the first intent expectations or expectation targets are the most important and should thus be prioritized in case of conflicts. Alternatively, the MnS consumer may assign explicit priorities for such intents or intent expectations or expectation targets so that the MnS producer uses the priorities to choose the intents or intent expectations or expectation targets to be discarded in favour of others. The priority mechanism not only applies to targets, but also to the expected managed entities (listed or scoped by the object context). The MnS consumer may also send the reduced scope of the expected managed entities in intent creation or modification request When the intent cannot be fulfilled in the expected managed entities. Then, actions would be taken by the MnS producer to only ensure the fulfilment of targets on the part of the expected managed entities until the intent conflict is solved.

Next Change

6.2.1.3.2	ExpectationObject <<dataType>>

6.2.1.3.2.1	Definition

The ExpectationObject <<dataType>> represents the Object to which the IntentExpectation should apply.

The ExpectationTarget <<dataType>> includes a contextSelectivity used to define how to select among the stated targetContexts.

6.2.1.3.2.2	Attributes

The ExpectationObject includes the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| objectType | CM | T | T | F | F |
| objectInstance | CM | T | T | F | F |
| objectContexts | O | T | T | F | F |

6.2.1.3.2.3	Attribute constraints

Table 6.2.1.3.2.3-1

| Name | Definition |
| --- | --- |
| objectType Support Qualifier | Condition: The intent expectation is not for a specific object instance or MnS consumer have no knowledge of the DN of this specific object instance. |
| objectInstance Support Qualifier | Condition: The intent expectation is for a specific object instance and MnS consumer have the knowledge of the DN of this specific object instance. |

6.2.1.3.3	ExpectationTarget <<dataType>>

6.2.1.3.3.1	Definition

The ExpectationTarget <<dataType>> represents the target of the IntentExpectation that are required to be achieved.

The ExpectationTarget <<dataType>> includes a contextSelectivity used to define how to select among the stated targetContexts.

6.2.1.3.3.2	Attributes

The ExpectationTarget includes the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetName | M | T | T | F | T |
| targetCondition | M | T | T | F | F |
| targetValueRange | M | T | T | F | F |
| contextSelectivity | O | T | T | F | F |
| targetContexts | O | T | T | F | F |

6.2.1.3.3.3	Attribute constraints

None.

6.2.1.3.3.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

Next Change

##### 6.2.1.3.5	FulfilmentInfo << dataType >>

###### 6.2.1.3.5.1	Definition

This dataType represents the properties of a specific fulfilment information for an aspect of the intent (i.e. either an expectation, a target or the whole intent). The fulfilment information describes the MnS producer's assessment of the degree to which a specific aspect of the intent is being fulfilled. The MnS consumer may however assess the fulfilment differently, e.g. the MnS consumer may evaluate the delivered outcome or network state to compute its fulfilment satisfaction.

The fulfilmentStatus field indicates whether the intent is being fulfilled or not being fulfilled. The possible values of the fulfilment include:

-	NOT_FULFILLED: This is the default status for any aspect of the intent and the fulfilmentStatus remains as "NOT_FULFILLED" until the MnS producer is satisfied that the actions undertaken meet the requirements as stated by the MnS consumer.

-	FULFILLED: This is the status if the MnS producer considers that the intent, expectation or target is being fulfilled as desired by the MnS consumer that created the intent. The consumer may provide a fulfilment satisfaction report that either confirms the fulfilment or describes its evaluation of the fulfilment.

The degree of fulfilment of an intent with the NOT_FULFILLED status may have multiple explanations and related states. These different progress states and conditions are recorded in the notFulfilledState field. The possible values of the notFulfilledState include:

-	ACKNOWLEDGED: this is the default state and is the initial notFulfilledState right after the intent has been received.

-	COMPLIANT: this is the state after the feasibility check has been run for the intent and the intent accepted as being compliant for fulfilment.

-	DEGRADED: this is the state if an intent that was previously fulfilled but after a period of observation it is found not be meeting the initially stated requirements.

-	SUSPENDED: this is the state if the MnS producer decides to suspend the fulfilment of the intent, expectation or target for whatever reason. This notFulfilledState shall be supported by a reason such as the event(s) that were observed when fulfilment was attempted.

-	TERMINATED: This state is registered if the respective aspect of the intent (i.e. either an expectation, a target or the whole intent) shall not be considered for fulfilment e.g. when an authorized MnS consumer sends an indication terminating the specific aspect of the intent. For instance, if the MnS consumer sends an update of the intent in which a particular target is eliminated, then that target shall be marked as cancelled.

-	FULFILMENTFAILED: This is the state when the MnS producer decides that the intent, expectation or target cannot be fulfilled. This state shall be supported by a reason such as the event(s) that were observed when fulfilment was attempted.

For some scenarios (in particular for the notFulfilledState with value "DEGRADED", “TERMINATED", "SUSPENDED" and "FULFILMENTFAILED"), the notFulfilledState should be supported by extra information describing or related to the state. This extra information is recorded into the notFulfilledReasons field.

###### 6.2.1.3.5.2	Attributes

The FulfilmentInfo includes the following attributes.

Table 6.2.1.3.5.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fulfilmentStatus | M | T | F | F | T |
| notFulfilledState | CM | T | F | F | T |
| notFulfilledReasons | CO | T | F | F | T |

###### 6.2.1.3.5.3	Attribute constraints

Table 6.2.1.3.5.3-1

| Name | Definition |
| --- | --- |
| notFulfilledState Support Qualifier | Condition: when FulfilmentInfo is implemented for IntentFulfilmentInfo |
| notFulfilledReasons Support Qualifier | Condition: when FulfilmentInfo is implemented for IntentFulfilmentInfo |

###### 6.2.1.3.5.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

Next Change

6.2.1.3.9	IntentConflictReport << dataType >>

6.2.1.3.9.1	Definition

IntentConflictReport <<dataType>> represents the conflict information for the detected conflict.

When a conflict is detected, the MnS producer will configure the value of attributes of IntentConflictReport and notify the MnS consumer about the conflict, indicating the intent, intent expectation or expectation target which give rise to the conflict. The value of recommendedSolutions may be configured by MnS producer and notified to MnS consumer.

6.2.1.3.9.2	Attributes

The IntentConflictReport includes the following attributes.

Table 6.2.1.3.9.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| conflictId | M | T | F | T | T |
| conflictType | M | T | F | F | T |
| conflictingIntent | CM | T | F | F | T |
| conflictingExpectation | CM | T | F | F | T |
| conflictingTarget | CM | T | F | F | T |
| recommendedSolutions | O | T | F | F | T |

6.2.1.3.9.3	Attribute constraints

| Name | Definition |
| --- | --- |
| conflictingIntent Support Qualifier | Condition: This will be present if the value of conflictType is INTENT_CONFLICT |
| conflictingExpectation Support Qualifier | Condition: This will be present if the value of conflictType is EXPECTATION_CONFLICT |
| conflictingTarget Support Qualifier | Condition: This will be present if the value of conflictType is TARGET_CONFLICT |

Next Change

#### 6.2.1.4	Attribute definition

Table 6.2.1.4-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentExpectations | It describes the expectations including requirements, goals and contexts (including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for expectation object(s). The intentExpectations are arranged in an ordered list such that the most important intentExpectations are on the top of the list. allowedValues: Not Applicable | type: IntentExpectation multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| intentFulfilmentInfo | It describes status of fulfilment of an intent and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationFulfilmentInfo | It describes status of fulfilment of an intentExpectation and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetFulfilmentInfo | It describes status of fulfilment of an expectationTarget and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fulfilmentStatus | It describes the current status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", "NOT_FULFILLED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notFulfilledState | It describes the current progress of or the reason for not achieving fulfilment for the intent, intentExpectation or expectationTarget. It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: "ACKNOWLEDGED", "COMPLIANT", "DEGRADED", "SUSPENDED", "TERMINATED" "FULFILMENTFAILED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notFulfilledReasons | It describes the reasons/observations related to the specific notFulfilledState allowedValues: Not Applicable | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentContexts | It describes the list of IntentContext(s) which represents the constraints and conditions that should apply for the entire intent even if there may be specific contexts defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationId | A unique identifier of the intentExpectation within the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationVerb | It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations. Examples of verbs and their related types of expectation are Deliver: DeliveryIntentExpectation, e.g. Deliver a RAN network, Service, Slice, function Ensure: AssuranceintentExpectation, e.g. Ensure the target performance value. allowedValues: DELIVER, ENSURE Vendor extensions are allowed | type: String multiplicity: 1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationObject | It describes the expectation objects to which the IntentExpectation should apply. allowedValues: Not Applicable | type: ExpectationObject multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectType | It describes the type of expectation object of the IntentExpectation that is required to be applied to. It can be class name of the managed object. allowedValues: see scenario specific IntentExpectation | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | It describes a specific object instance (e.g. instance of managed object) to which the intentExpectation should apply. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectContexts | It describes the list of ObjectContext(s) which represents the constraints and conditions to be used as filter information to identify the object(s) to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent, for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. The concrete ObjectContext depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ObjectContexts follow the common structure of ObjectContext. | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationTargets | It describes the list of ExpectationTarget(s) which represent specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. The concrete ExpectationTarget depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ExpectationTargets follow the common structure of ExpectationTarget. The expectionTargets are arranged in an ordered list such that the most important expectionTargets are on the top of the list. | type: ExpectationTarget multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| expectationContexts | It describes the list of context(s) which represents the constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Expectation Object in the IntentExpectation | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetName | It describes the name of the expectation target which represents specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. allowedValues: depends on ExpectationObject in the IntentExpectation | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| targetCondition | It expresses the limits within which the targetName is allowed/supposed to be. allowedValues: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE", "IS_ONE_OF", " IS_EQUAL_TO_OR_LESS_THAN”, "IS_EQUAL_TO_OR_GREATER_THAN", "IS_NOT_ONE_OF", "IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "IS_EQUAL_TO" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the targetCondition. allowedValues: depends on the targetCondition. The value will be a single value when the targetCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS EQUAL TO OR LESS THAN", "IS EQUAL TO OR GREATER THAN” The value will be a pair of values when the targetCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the targetCondition is "IS_ONE_OF", "IS_NOT_ONE_OF","IS_ALL_OF". See NOTE 1. | type: ValueRangeType multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| targetContexts | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: Not Applicable | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc) to which the expectation should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE, "IS_ONE_OF", "IS_EQUAL_TO_OR LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN", "IS_NOT_ONE_OF","IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: " IS_EQUAL_TO " isNullable: False |
| contextValueRange | It describes the range of values that applicable to the ContextAttribute and the ContextCondition. AllowedValue: depends on the contextCondition The value will be a single value when the contextCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_EQUAL_TO_OR_LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN". The value will be a pair of values when the contextCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the contextCondition is "IS_ONE_OF", "IS_NOT_ONE_OF","IS_ALL_OF". See NOTE 1. | type: ValueRangeType multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| intentPriority | It expresses the priority of the stated intent within an MnS consumer. AllowedValue: values in the range [1-100] where 1 indicates the highest priority and 100 indicates the lowest priority. NOTE: The handing of the priorities across MnS consumers is left to implementation | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 1 isNullable: False |
| geoArea | It describes a geographical area defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: GeoArea multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| pLMNId | It describes the information of a PLMN identification defined in 3GPP 28.658[10] AllowedValue: As defined by the data type | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dateTime | It describes the information of a date time defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| timeWindow | It describes the information of a time window (including startTime, endTime) defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| geoCoordinate | It describes the information of a geoCoordinate defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: GeoCoordinate multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| frequency | It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number) and/or the frequency operating band used for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD. AllowedValue: As defined by the data type | type: Frequency multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| arfcn | It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number). Allowed Value: For NR, see TS 38.104 [8] clause 5.4.2.1. For EUTRAN, see TS 36.104 [14] clause 5.7.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| freqband | It desribes the the frequency operating band. Allowed Value: For NR, see TS 38.104 [8] clause 5.4.2.3. For EUTRAN, see TS 36.104 [14] clause 5.7.3. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uEGroup | It describes the information of a UE Group (represented by specific 5QI, specific S-NSSAI, or a specific combination of S-NSSAI and 5QI). AllowedValue: As defined by the data type | type: UEGroup multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| fiveQI | It describes the information of a 5QI defined in 3GPP TS 28.541[5]. AllowedValue: 0 - 255 | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sNSSAI | It describes the information of a S-NSSAI defined in 3GPP TS 28.541[5]. AllowedValue: As defined by the data type | type: S-NSSAI multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intentAdminState | It describes the intent administrative state, which enables the MnS consumer to suspend an intent or cancel the suspension for a suspended intent. A suspended intent means this intent is not considered for fulfilment allowedValues: "ACTIVATED", "DEACTIVATED" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ACTIVATED" isNullable: False |
| intentReference | It indicates the associated intent instance allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentReportReference | It indicates the associated intent report instance(s) allowedValues: Not Applicable | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| observationPeriod | It represents the observation period of the fulfilmentInfo for corresponding ExpectationTargets, IntentExpectations and Intent. At the end of the observation period, the corresponding fulfilment info is updated in the intent report. The observation period can be assigned by MnS consumer through requesting the MnS producer to set attribute "observationPeriod". MnS producer also can assign the observation period if MnS consumer didn’t assign it. The observation time is expressed in seconds. allowedValues: Not Applicable | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentFulfilmentReport | It describes the fulfillment information which is reported for the associated intent instance. allowedValues: Not Applicable | type: IntentFulfilmentReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentConflictReports | It describes the conflict information which is reported for associated intent instance if needed. allowedValues: Not Applicable | type: IntentConflictReport multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conflictId | It is used to identify the detected conflict within an IntentReport instance. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictType | It describes the type of intent conflict. allowedValues: INTENT_CONFLICT, EXPECTATION_CONFLICT, TARGET_CONFLICT | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingIntent | It describes the DN of the conflicting intent allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingExpectation | It describes the expectationId of the conflicting IntentExpectation within an Intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingTarget | It describes the targetName of the conflicting ExpectationTarget within an IntentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedSolutions | It describes the action recommended by the MnS producer to be undertaken by the MnS consumer to resolve intent conflict. The recommended solution applies only for the specific intent whose intent report contains this attribute. allowedValues: "MODIFY", "DELETE" | type: ENUM multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationFulfilmentResults | It includes the expectationFulfilmentInfo and targetFulfilmentResults for each IntentExpectation. The expectationFulfilmentInfo describes status of fulfilment of an intentExpectation and the related reasons for infeasible status. allowedValues: Not Applicable | type: ExpectationFulfilmentResult multiplicity:1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetFulfilmentResults | It includes targetFulfilmentInfo and targetAchievedValue for each ExpectationTarget. The targetFulfilmentInfo describes status of fulfilment of an expectationTarget and the related reasons for infeasible status. The targetAchieveValue describes current performance value for the ExpectationTarget. allowedValues: Not Applicable | type: TargetFulfilmentResult multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetAchievedValue | It describes the value that has been achieved for the expectation target at the time at which the report is generated. allowedValues: Not Applicable | type: Number multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentFeasibilityCheckReport | It describes the intent feasibility check information which is reported if needed. allowedValues: Not Applicable | type: IntentFeasibilityCheckReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| feasibilityCheckResult | It describes the result of intent fulfilment feasibility check allowedValues: FEASIBLE, INFEASIBLE | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| infeasibilityReasons | It describes the reason (e.g. invalid intent expression, the intent conflict) of the result of intent fulfilment feasibility check is INFEASIBLE Editor’s Note: the ENUM value for infeasibilityReason is FFS. | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentHandlingCapabilityList | It describes the list of expectation object information and expectation target information which can be supported by intent handling function. allowedValues: Not Applicable | type: IntentHandlingCapability multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentHandlingCapabilityId | A unique identifier of property of intent handling capability should be supported by the intent handling function of MnS producer. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedExpectationObjectType | It describes the expectation object type which can be supported by a specific intent handling function of MnS producer. allowedValues: objectType defined in clause 6.2.1.3.2. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedExpectationTargetNames | It describes the supported expectation targets for the supported expectation object type. allowedValues: targetName defined in clause 6.2.1.3.3 | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| lastUpdatedTime | It describes the time for the latest update of the IntentReport Instance. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ContextSelectivity | It expresses the may in which all or a subset of the expectationTargets may be applied. AllowedValue: "ALL_OF", "ONE_OF", "ANY_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ALL_OF" isNullable: False |
| intentPreemptionCapability | It describes the pre-emption capability. The attribute is used by MnS producer to decide the target of intent deletion or intent modification AllowedValue: TRUE,FALSE | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |
| NOTE: For "IS_ALL_OF", the value shall be a match of the entire list. |  |  |

Next Change

### 7.2.2	OpenAPI document "TS28312_IntentNrm.yaml"

<CODE BEGINS>

openapi: 3.0.1

info:

title: Intent NRM

version: 18.2.0

description: >-

OAS 3.0.1 definition of the Intent NRM

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.312; Intent driven management services for mobile networks

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.312/

paths: {}

components:

schemas:

#-------Definition of generic IOCs ----------#

SubNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

IntentHandlingFunction:

$ref: '#/components/schemas/IntentHandlingFunction-Multiple'

Intent-Single:

description: >-

This IOC represents the properties of an Intent driven management information between MnS consumer and MnS producer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

userLabel:

type: string

intentExpectations:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/IntentExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/RadioNetworkExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/EdgeServiceSupportExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/5GCNetworkExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/RadioServiceExpectation"

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

intentContexts:

type: array

items:

$ref: '#/components/schemas/Context'

description: >-

It describes the list of Context(s) which represents the constraints and conditions that should apply

for the entire intent even if there may be specific contexts defined for specific parts of the intent

intentAdminState:

type: string

enum:

- ACTIVATED

- DEACTIVATED

description: >-

It describes the intent administrative state.

This attribute is used when MnS consumer-suspension mechanism is supported

intentPriority:

type: integer

minimum: 1

maximum: 100

description: It expresses the priority of the stated intent within a MnS consumer.

intentPreemptionCapability:

type: string

enum:

- TRUE

- FALSE

observationPeriod:

type: integer

description: >-

It represents the observation period of the fulfilmentInfo for corresponding

ExpectationTargets, IntentExpectations and Intent.

intentReportReference:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

IntentReport-Single:

description: It represents intent report information from MnS producer to MnS consumer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

intentFulfilmentReport:

$ref: '#/components/schemas/IntentFulfilmentReport'

intentConflictReports:

type: array

items:

$ref: '#/components/schemas/IntentConflictReport'

intentFeasibilityCheckReport:

$ref: '#/components/schemas/IntentFeasibilityCheckReport'

lastUpdatedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

intentReference:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

IntentHandlingFunction-Single:

description: >-

It represents the intent handling capabilities can be supported by a specific intent

handling function of MnS producer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

intentHandlingCapabilityList:

type: array

items:

$ref: '#/components/schemas/IntentHandlingCapability'

Intent:

$ref: '#/components/schemas/Intent-Multiple'

IntentReport:

$ref: '#/components/schemas/IntentReport-Multiple'

#-------Definition of generic IOCs ----------#

#-------Definition of the generic IntentExpectation dataType ----------#

IntentExpectation:

description: >-

This data type is the "IntentExpectation" data type without specialisations

It represents MnS consumer's requirements, goals and contexts given to a 3GPP system

type: object

properties:

expectationId:

type: string

description: A unique identifier of the intentExpectation within the intent.

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObject:

$ref: "#/components/schemas/ExpectationObject"

expectationTargets:

type: array

items:

$ref: '#/components/schemas/ExpectationTarget'

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

expectationContexts:

type: array

items:

$ref: '#/components/schemas/Context'

required:

- expectationId

#-------Definition of the generic IntentExpectation dataType ----------#

#-------Definition of the generic ExpectationObject dataType ----------#

ExpectationObject:

description: >-

It represents the Object to which the IntentExpectation should apply.

This data type is the "ExpectationObject" data type without specialisations

type: object

properties:

objectType:

type: string

enum:

- RAN_SubNetwork  #value for Radio Network Expectation--#

- Edge_Service_Support  #value for Edge Service Support Expectation--#

- 5GC_SubNetwork  #value for 5GC Network Expectation--#

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

objectContexts:

type: array

items:

$ref: '#/components/schemas/Context'

description: >-

It describes the list of Context(s) which represents the constraints and conditions to be

used as filter information to identify the object(s) to which a given intentExpectation should apply.

#-------Definition of the generic ExpectationObject dataType ----------#

#-------Definition of the generic dataType --------------#

Condition:

type: string

enum:

- IS_EQUAL_TO

- IS_LESS_THAN

- IS_GREATER_THAN

- IS_WITHIN_RANGE

- IS_OUTSIDE_RANGE

- IS_ONE_OF

- IS_NOT_ONE_OF

- IS_EQUAL_TO_OR_LESS_THAN

- IS_EQUAL_TO_OR_GREATER_THAN

- IS_ALL_OF

Selectivity:

type: string

enum:

- ALL_OF

- ONE_OF

- ANY_OF

FulfilmentStatus:

type: string

readOnly: true

enum:

- FULFILLED

- NOT_FULFILLED

description: It describes the current status of the intent fulfilment result.

NotFulfilledState:

type: string

readOnly: true

enum:

- ACKNOWLEDGED

- COMPLIANT

- DEGRADED

- SUSPENDED

- TERMINATED

- FULFILMENTFAILED

description: It describes the current progress of or the reason for not achieving fulfilment

for the intent, intentExpectation or expectationTarget.

An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

FulfilmentInfo:

description: >-

This dataType represents the properties of a specific fulfilment information for an aspect of

the intent (i.e. either an expectation, a target or the whole intent).

type: object

properties:

fulfilmentStatus:

$ref: '#/components/schemas/FulfilmentStatus'

notFullfilledState:

$ref: "#/components/schemas/NotFulfilledState"

notFulfilledReasons:

type: array

items:

type: string

readOnly: true

description: An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

ExpectationVerb:

type: string

enum:

- DELIVER

- ENSURE

description: It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations.

Frequency:

description: >-

It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number)

and/or the frequency operating band used for a given direction (UL or DL) in FDD or

for both UL and DL directions in TDD.

type: object

properties:

arfcn:

type: integer

description: >-

This attribute shall be supported, when the frequency information represent RF reference frequency.

The allowed values for NR see TS 38.104 subclause 5.4.2.1; The allowed values for EUTRAN see TS 36.104 [X] subclause 5.7.3;

freqband:

type: string

description: >-

This attribute shall be supported, when the frequency information represent frequency operating band.

The allowed values for NR see TS 38.104 subclause 5.4.2.3; The allowed value for EUTRAN see TS 36.104 subclause 5.7.3

ValueRangeType:

oneOf:

- type: number

- type: string

- type: boolean

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoCoordinate'

- $ref: '#/components/schemas/UEGroup'

- $ref: '#/components/schemas/Frequency'

UEGroup:

description: >-

It describes the UE Group, which is

represented by specific 5QI, specific S-NSSAI, or a specific combination

of S-NSSAI and 5QI

type: object

properties:

fiveQI:

type: integer

minimum: 0

maximum: 255

sNssai:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/Snssai'

#-------Definition of the generic dataType --------------#

#-------Definition of the generic ExpectationTarget dataType----------#

ExpectationTarget:

description: >-

This data type represents the target of the IntentExpectation that are required to be achieved.

This data type is the "ExpectationTarget" data type without specialisations

type: object

properties:

targetName:

type: string

targetCondition:

$ref: '#/components/schemas/Condition'

targetValueRange:

oneOf:

- type: array

items:

$ref: "#/components/schemas/ValueRangeType"

- $ref: "#/components/schemas/ValueRangeType"

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

targetContexts:

type: array

items:

$ref: '#/components/schemas/Context'

description: It describes the list of constraints and conditions that should apply for a specific expectationTarget.

#-------Definition of the generic ExpectationTarget  dataType----------#

#-------Definition of the generic Context dataType----------------#

Context:

description: >-

This data type is the "Context" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: '#/components/schemas/Condition'

contextValueRange:

oneOf:

- type: array

items:

$ref: "#/components/schemas/ValueRangeType"

- $ref: "#/components/schemas/ValueRangeType"

#-------Definition of the generic Context dataType----------------#

#-------Definition of the generic IntentFulfilmentReport dataType----------------#

IntentFulfilmentReport:

description: >-

It includes the intentFulfilmentInfo and expectationFulfilmetResult.

This attribute shall be supported when intent fulfilment information is supported by IntentReport

type: object

properties:

intentFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

expectationFulfilmentResult:

type: array

items:

$ref: '#/components/schemas/ExpectationFulfilmentResult'

#-------Definition of the concrete IntentFulfilmentReport dataType----------------#

#-------Definition of the generic ExpectationFulfilmentResult dataType----------------#

ExpectationFulfilmentResult:

description: >-

It includes the expectationFulfilmentInfo and targetFulfilmentResults for each IntentExpectation.

type: object

properties:

expectaitonId:

type: string

expectationFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

targetFulfilmentResult:

type: array

items:

$ref: '#/components/schemas/TargetFulfilmentResult'

#-------Definition of the concrete ExpectationFulfilmentResult dataType----------------#

#-------Definition of the generic TargetFulfilmentResult dataType----------------#

TargetFulfilmentResult:

description: >-

This data type includes targetFulfilmentInfo and targetAchievedValue for each ExpectationTarget.

type: object

properties:

targetName:

type: string

targetFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

targetAchievedValue:

type: number

#-------Definition of the concrete TargetFulfilmentResult dataType----------------#

#-------Definition of the generic IntentConflictReport dataType----------------#

IntentConflictReport:

description: >-

It represents the conflict information for the detected conflict

This attribute shall be supported when intent conflict information is supported by IntentReport

type: object

properties:

conflictId:

type: string

conflictType:

type: string

enum:

- INTENT_CONFLICT

- EXPECTATION_CONFLICT

- TARGET_CONFLICT

conflictingIntent:

description: >-

This will be present if the value of conflictType is INTENT_CONFLICT. It describes the DN of the conflicting intent

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

conflictingExpectation:

description: >-

This will be present if the value of conflictType is EXPECTATION_CONFLICT. It describes the expectationId of the conflicting IntentExpectation with an Intent

type: string

conflictingTarget:

description: >-

This will be present if the value of conflictType is TARGET_CONFLICT. It describes the targetName of the conflicting ExpectationTarget with an IntentExpectation

type: string

recommendedSolutions:

type: string

enum:

- MODIFY

- DELETE

#-------Definition of the concrete IntentConflictReport dataType----------------#

#-------Definition of the generic IntentFeasibilityCheckReport dataType----------------#

IntentFeasibilityCheckReport:

description: >-

It represents the intent feasibility check information

This attribute shall be supported when intent feasibility check information information is supported by IntentReport

type: object

properties:

feasibilityCheckResult:

type: string

enum:

- FEASIBLE

- INFEASIBLE

infeasibilityReason:

type: string

description: An attribute which is used when feasibilityCheckResult is INFEASIBLE

#-------Definition of the concrete IntentFeasibilityCheckReport dataType----------------#

#-------Definition of the generic IntentHandlingCapability dataType----------------#

IntentHandlingCapability:

description: >-

It represents expectation object information and expectation target information

which can be supported by a specific intent handling function of MnS producer.

type: object

properties:

intentHandlingCapabilityId:

type: string

supportedExpectationObjectType:

type: string

enum:

- RAN_SUBNETWORK

- EDGE_SERVICE_SUPPORT

- 5GC_SUBNETWORK

description: It describes the expectation object type which can be supported by a specific intent handling function of MnS producer.

supportedExpectationTargetNames:

type: array

items:

type: string

description: It describes the supported expectation targets for the supported expectation object type.

#-------Definition of the concrete IntentHandlingCapability dataType----------------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

Intent-Multiple:

type: array

items:

$ref: '#/components/schemas/Intent-Single'

IntentReport-Multiple:

type: array

items:

$ref: '#/components/schemas/IntentReport-Single'

IntentHandlingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/IntentHandlingFunction-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

resources-intentNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/IntentHandlingFunction-Single'

- $ref: '#/components/schemas/Intent-Single'

- $ref: '#/components/schemas/IntentReport-Single'

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

<CODE ENDS>

Next Change

### 7.2.3	OpenAPI document "TS28312_IntentExpectations.yaml"

<CODE BEGINS>

openapi: 3.0.1

info:

title: Scenario specific Intent Expectations

version: 18.2.0

description: >-

OAS 3.0.1 definition of scenario specific Intent Expectations

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.312; Intent driven management services for mobile networks

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.312/

paths: {}

components:

schemas:

#-------Definition of the Scenario specific IntentExpectation dataType ----------#

RadioNetworkExpectation:

description: >-

This data type is the "IntentExpectation" data type with specialisations to represent MnS consumer's expectations for radio network delivering and performance assurance

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/ExpectationVerb"

expectationObject:

$ref: "#/components/schemas/RadioNetworkExpectationObject"

expectationTargets:

type: array

items:

type: object

oneOf:

- $ref: '#/components/schemas/WeakRSRPRatioTarget'

- $ref: '#/components/schemas/LowSINRRatioTarget'

- $ref: '#/components/schemas/AveULRANUEThptTarget'

- $ref: '#/components/schemas/AveDLRANUEThptTarget'

- $ref: '#/components/schemas/LowULRANUEThptRatioTarget'

- $ref: '#/components/schemas/LowDLRANUEThptRatioTarget'

- $ref: '#/components/schemas/HighULPrbLoadRatioTarget'

- $ref: '#/components/schemas/HighDLPrbLoadRatioTarget'

- $ref: '#/components/schemas/AveULPrbLoadTarget'

- $ref: '#/components/schemas/AveDLPrbLoadTarget'

- $ref: "#/components/schemas/RANEnergyConsumptionTarget"

- $ref: "#/components/schemas/RANEnergyEfficiencyTarget"

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/ExpectationTarget'

expectationContexts:

type: array

items:

type: object

oneOf:

- $ref: '#/components/schemas/TargetAssuranceTimeContext'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/Context'

required:

- expectationId

RadioServiceExpectation:

description: >-

This data type is the "IntentExpectation" data type with specialisations to represent MnS consumer's expectations for radio service delivering

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/ExpectationVerb"

expectationObject:

$ref: "#/components/schemas/RadioServiceExpectationObject"

expectationTargets:

type: array

items:

type: object

oneOf:

- $ref: '#/components/schemas/DLLatencyTarget'

- $ref: '#/components/schemas/ULLatencyTarget'

- $ref: '#/components/schemas/DLThptPerUETarget'

- $ref: '#/components/schemas/ULThptPerUETarget'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/ExpectationTarget'

expectationContexts:

type: array

items:

$ref: 'TS28312_IntentNrm.yaml#/components/schemas/Context'

required:

- expectationId

EdgeServiceSupportExpectation:

description: >-

This data type is the "IntentExpectation" data type with specialisations to represent MnS consumer's expectations for service deployment

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: 'TS28312_IntentNrm.yaml#/components/schemas/ExpectationVerb'

expectationObject:

$ref: '#/components/schemas/EdgeServiceSupportExpectationObject'

expectationTargets:

type: array

items:

type: object

oneOf:

- $ref: '#/components/schemas/DLThptPerUETarget'

- $ref: '#/components/schemas/ULThptPerUETarget'

- $ref: '#/components/schemas/DLLatencyTarget'

- $ref: '#/components/schemas/ULLatencyTarget'

- $ref: '#/components/schemas/MaxNumberofUEsTarget'

- $ref: '#/components/schemas/ActivityFactorTarget'

- $ref: '#/components/schemas/UESpeedTarget'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/ExpectationTarget'

expectationContexts:

type: array

items:

type: object

oneOf:

- $ref: '#/components/schemas/ServiceStartTimeContext'

- $ref: '#/components/schemas/ServiceEndTimeContext'

- $ref: '#/components/schemas/UEMobilityLevelContext'

- $ref: '#/components/schemas/ResourceSharingLevelContext'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/Context'

required:

- expectationId

5GCNetworkExpectation:

description: >-

This data type is the "IntentExpectation" data type with specialisations to represent MnS consumer's expectations for 5GC network delivering

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/5GCNetworkExpectationObject"

expectationTargets:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/MaxNumberofPDUsessionsTarget"

- $ref: "#/components/schemas/MaxNumberofRegisteredsubscribersTarget"

- $ref: "#/components/schemas/IncomingDataTarget"

- $ref: "#/components/schemas/OutgoingDataTarget"

- $ref: "TS28312_IntentNrm.yaml#/components/schemas/ExpectationTarget"

expectationContexts:

type: array

items:

type: object

oneOf:

- $ref: '#/components/schemas/StartTimeContext'

- $ref: '#/components/schemas/ResourceSharingLevelContext'

- $ref: "TS28312_IntentNrm.yaml#/components/schemas/Context"

required:

- expectationId

#-------Definition of the IntentExpectation dataType ----------#

#-------Definition of the scenario specific ExpectationObject dataType ----------#

RadioNetworkExpectationObject:

description: >-

This data type is the "ExpectationObject" data type with specialisations for RadioNetworkExpectation

type: object

properties:

objectType:

type: string

enum:

- RAN_SubNetwork

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

objectContexts:

type: array

items:

type: object

oneOf:

- $ref: '#/components/schemas/CoverageAreaPolygonContext'

- $ref: '#/components/schemas/CoverageTACContext'

- $ref: '#/components/schemas/PLMNContext'

- $ref: '#/components/schemas/DlFrequencyContext'

- $ref: '#/components/schemas/UlFrequencyContext'

- $ref: '#/components/schemas/RATContext'

- $ref: "#/components/schemas/UEGroupContext"

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/Context'

RadioServiceExpectationObject:

description: >-

This data type is the "ExpectationObject" data type with specialisations for RadioServicekExpectation

type: object

properties:

objectType:

type: string

enum:

- Radio_Service

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

objectContexts:

type: array

items:

type: object

oneOf:

- $ref: '#/components/schemas/CoverageAreaPolygonContext'

- $ref: '#/components/schemas/ServiceTypeContext'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/Context'

EdgeServiceSupportExpectationObject:

description: >-

This data type is the "ExpectationObject" data type with specialisations for EdgeServiceSupportExpectation

type: object

properties:

objectType:

type: string

enum:

- EdgeService_Support #value for Edge Service Support Expectation--#

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

objectContexts:

type: array

items:

type: object

oneOf:

- $ref: '#/components/schemas/EdgeIdentificationIdContext'

- $ref: '#/components/schemas/EdgeIdentificationLocContext'

- $ref: '#/components/schemas/CoverageAreaTAContext'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/Context'

5GCNetworkExpectationObject:

description: >-

This data type is the "ExpectationObject" data type with specialisations for 5GCNetworkExpectation

type: object

properties:

objectType:

type: string

enum:

- 5GC_SubNetwork #value for 5GC Network Expectation--#

objectInstance:

$ref: "TS28623_ComDefs.yaml#/components/schemas/Dn"

objectContexts:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/NfTypeContext"

- $ref: "#/components/schemas/NfInstanceLocationContext"

- $ref: "#/components/schemas/PLMNContext"

- $ref: "#/components/schemas/TaiContext"

- $ref: "#/components/schemas/ServingScopeContext"

- $ref: "#/components/schemas/DnnContext"

- $ref: "TS28312_IntentNrm.yaml#/components/schemas/Context"

#-------Definition of the ExpectationObject dataType ----------#

#-------Definition of the Scenario specific ExpectationTarget dataType----------#

WeakRSRPRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for WeakRSRPRatioTarget

type: object

properties:

targetName:

type: string

enum:

- WeakRSRPRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: '#/components/schemas/WeakRSRPContext'

WeakRSRPContext:

description: >-

This data type is the "TargetContext" data type with specialisations for WeakRSRPContext

type: object

properties:

contextAttribute:

type: string

enum:

- WeakRSRPThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: number

LowSINRRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowSINRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowSINRRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: '#/components/schemas/LowSINRContext'

LowSINRContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowSINRContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowSINRThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: integer

AveULRANUEThptTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for AveULRANUEThptTarget

type: object

properties:

targetName:

type: string

enum:

- AveULRANUEThpt

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

type: integer

AveDLRANUEThptTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for AveDLRANUEThptTarget

type: object

properties:

targetName:

type: string

enum:

- AveDLRANUEThpt

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

type: integer

LowULRANUEThptRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowULRANUEThptRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowULRANUEThptRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: '#/components/schemas/LowULRANUEThptContext'

LowULRANUEThptContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowULRANUEThptContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowULRANUEThptThreshold

contextCondition:

type: string

enum:

- Is_less_than

contextValueRange:

type: number

LowDLRANUEThptRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowDLRANUEThptRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowDLRANUEThptRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: '#/components/schemas/LowDLRANUEThptContext'

LowDLRANUEThptContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowDLRANUEThptContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowDLRANUEThptThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: number

HighULPrbLoadRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for HighULPrbLoadRatioTarget

type: object

properties:

targetName:

type: string

enum:

- HighULPrbLoadRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: '#/components/schemas/HighULPrbLoadContext'

HighULPrbLoadContext:

description: >-

This data type is the "TargetContext" data type with specialisations for HighULPrbLoadContext

type: object

properties:

contextAttribute:

type: string

enum:

- HighULPrbLoadThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: integer

minimum: 0

maximum: 100

HighDLPrbLoadRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for HighDLPrbLoadRatioTarget

type: object

properties:

targetName:

type: string

enum:

- HighDLPrbLoadRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: '#/components/schemas/HighDLPrbLoadContext'

HighDLPrbLoadContext:

description: >-

This data type is the "TargetContext" data type with specialisations for HighDLPrbLoadContext

type: object

properties:

contextAttribute:

type: string

enum:

- HighDLPrbLoadThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: integer

minimum: 0

maximum: 100

AveULPrbLoadTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for AveULPrbLoadTarget

type: object

properties:

targetName:

type: string

enum:

- AveULPrbLoad

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

AveDLPrbLoadTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for AveDLPrbLoadTarget

type: object

properties:

targetName:

type: string

enum:

- AveDLPrbLoad

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

RANEnergyConsumptionTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for RANEnergyConsumptionTarget

type: object

properties:

targetName:

type: string

enum:

- RANEnergyConsumption

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

RANEnergyEfficiencyTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for RANEnergyEfficiencyTarget

type: object

properties:

targetName:

type: string

enum:

- RANEnergyEfficiency

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

type: integer

DLThptPerUETarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for DLThptPerUETarget

type: object

properties:

targetName:

type: string

enum:

- DlThptPerUE

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

$ref: 'TS28541_SliceNrm.yaml#/components/schemas/XLThpt'

ULThptPerUETarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ULThptPerUETarget

type: object

properties:

targetName:

type: string

enum:

- UlThptPerUE

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

$ref: 'TS28541_SliceNrm.yaml#/components/schemas/XLThpt'

DLLatencyTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for DLLatencyTarget

type: object

properties:

targetName:

type: string

enum:

- DlLatency

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

ULLatencyTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ULLatencyTarget

type: object

properties:

targetName:

type: string

enum:

- UlLatency

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

MaxNumberofUEsTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for MaxNumberofUEsTarget

type: object

properties:

targetName:

type: string

enum:

- maxNumberofUEs

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

ActivityFactorTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ActivityFactorTarget

type: object

properties:

targetName:

type: string

enum:

- activityFactor

targetCondition:

type: string

enum:

- IS_EQUAL_TO

targetValueRange:

type: integer

UESpeedTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for UESpeedTarget

type: object

properties:

targetName:

type: string

enum:

- uESpeed

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

MaxNumberofPDUsessionsTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for MaxNumberofPDUsessionsTarget

type: object

properties:

targetName:

type: string

enum:

- MaxNumberofPDUsessions

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

targetContexts:

$ref: '#/components/schemas/5GSessionContext'

5GSessionContext:

description: >-

This data type is the "TargetContext" data type with specialisations for 5GSessionContext

type: object

properties:

contextAttribute:

type: string

enum:

- 5GSession

contextCondition:

type: string

enum:

- IS_less_THAN

contextValueRange:

type: integer

MaxNumberofRegisteredsubscribersTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for MaxNumberofRegisteredsubscribersTarget

type: object

properties:

targetName:

type: string

enum:

- MaxNumberofRegisteredsubscribers

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

IncomingDataTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for IncomingDataTarget

type: object

properties:

targetName:

type: string

enum:

- IncomingData

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

OutgoingDataTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for OutgoingDataTarget

type: object

properties:

targetName:

type: string

enum:

- OutgoingData

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

#-------Definition of the concrete ExpectationTarget  dataType----------#

#-------Definition of the concrete ObjectTarget dataType----------------#

CoverageAreaPolygonContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageAreaPolygonContext

type: object

properties:

contextAttribute:

type: string

enum:

- CoverageAreaPolygon

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

CoverageTACContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageTACContext

type: object

properties:

contextAttribute:

type: string

enum:

- CoverageAreaTac

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "TS28623_ComDefs.yaml#/components/schemas/Tac"

PLMNContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for PLMNContext

type: object

properties:

contextAttribute:

type: string

enum:

- PLMN

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

DlFrequencyContext:

description: >-

This data type is the "Context" data type with specialisations for Object context "DLFrequencyContext".

It describes the downlink frequency information (RF reference frequencies and/ or the

frequency operating band) supported by the RAN SubNetwork that the intent expectation is applied.

type: object

properties:

contextAttribute:

type: string

enum:

- DlFrequency

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: 'TS28312_IntentNrm.yaml#/components/schemas/Frequency'

UlFrequencyContext:

description: >-

This data type is the "Context" data type with specialisations for Object context "ULFrequencyContext".

It describes the uplink frequency information (RF reference frequencies and/ or the frequency

operating band) supported by the RAN SubNetwork that the intent expectation is applied.

type: object

properties:

contextAttribute:

type: string

enum:

- UlFrequency

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: 'TS28312_IntentNrm.yaml#/components/schemas/Frequency'

RATContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for RATContext

type: object

properties:

contextAttribute:

type: string

enum:

- RAT

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

type: string

enum:

- UTRAN

- EUTRAN

- NR

UEGroupContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for UEGroup([5QI, SNSSAI])

type: object

properties:

contextAttribute:

type: string

enum:

- UEGroup

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/UEGroup"

EdgeIdentificationIdContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for EdgeIdentificationIdContext

type: object

properties:

contextAttribute:

type: string

enum:

- edgeIdentificationId

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: string

EdgeIdentificationLocContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for EdgeIdentificationLocContext

type: object

properties:

contextAttribute:

type: string

enum:

- edgeIdentificationTarget

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoCoordinate'

CoverageAreaTAContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageAreaTAContext

type: object

properties:

contextAttribute:

type: string

enum:

- coverageAreaTA

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "TS28623_ComDefs.yaml#/components/schemas/Tac"

NfTypeContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for NfTypeContext

type: object

properties:

contextAttribute:

type: string

enum:

- NfType

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "TS28623_GenericNrm.yaml#/components/schemas/NFType"

NfInstanceLocationContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for NfInstanceLocationContext

type: object

properties:

contextAttribute:

type: string

enum:

- NfInstanceLocation

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

type: string

TaiContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for TaiContext

type: object

properties:

contextAttribute:

type: string

enum:

- Tai

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "TS28623_GenericNrm.yaml#/components/schemas/Tai"

ServingScopeContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for ServingScopeContext

type: object

properties:

contextAttribute:

type: string

enum:

- ServingScope

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

type: string

DnnContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for DnnContext

type: object

properties:

contextAttribute:

type: string

enum:

- Dnn

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

type: string

#-------Definition of the scenario specific  ObjectTarget dataType----------------#

#-------Definition of the concrete ExpectationContext dataType----------------#

TargetAssuranceTimeContext:

description: >-

This data type is the "Expectation Context" data type with specialisations for TargetAssuranceTimeContext

type: object

properties:

contextAttribute:

type: string

enum:

- TargetAssuranceTime

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

ServiceStartTimeContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ServiceStartTimeContext

type: object

properties:

contextAttribute:

type: string

enum:

- ServiceStartTime

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

ServiceEndTimeContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ServiceEndTimeContext

type: object

properties:

contextAttribute:

type: string

enum:

- ServiceEndTime

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

UEMobilityLevelContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for UEMobilityLevelContext

type: object

properties:

contextAttribute:

type: string

enum:

- UEMobilityLevel

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/MobilityLevel"

ResourceSharingLevelContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ResourceSharingLevelContext

type: object

properties:

contextAttribute:

type: string

enum:

- ResourceSharingLevel

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/SharingLevel"

ServiceTypeContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ServiceTypeContext

type: object

properties:

contextAttribute:

type: string

enum:

- ServiceType

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

$ref: "TS28541_NrNrm.yaml#/components/schemas/Sst"

StartTimeContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for StartTimeContext

type: object

properties:

contextAttribute:

type: string

enum:

- StartTime

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

#-------Definition of the concrete ExpectionContext dataType----------------#

<CODE ENDS>

Next Change

8	Guidelines for using scenario specific intent expectation for intent driven use cases

This clause describes guidelines for using scenario specific intent expectation defined in clause 6.2.2 to satisfy the intent driven use cases defined in clause 5.1. Following table provide the information which ObjectContexts and ExpectationTargets defined in clause 6.2.2 are used for corresponding use case.

Table 8-1: Guidelines for using scenario specific intent expectation for intent driven use cases

| Use case | Scenario specific IntentExpectation | ExpectationObject. ObjectContext | ExpectationTarget |
| --- | --- | --- | --- |
| Intent containing an expectation for delivering radio network (clause 5.1.1) | Radio Network Expectation | -coverageAreaPolygonContext - coverageTACContext - pLMNContext - dlFrequencyContext - ulFrequencyContext - rATContext | -weakRSRPRatioTarget - lowSINRRatioTarget - aveULRANUEThptTarget - aveDLRANUEthptTarget |
| Intent containing an expectation for delivering a service at the edge (clause 5.1.3) | Edge Service Support Expectation | - edgeIdentificationIdContext - edgeIdentificationLocContext - coverageAreaTAContext | - dlThptPerUETarget - ulThptPerUETarget - dLLatencyTarget - uLLatencyTarget - maxNumberofUEsTarget - activityFactorTarget - uESpeedTarget |
| Intent containing an expectation on coverage performance to be assured (clause 5.1.4) | Radio Network Expectation | -coverageAreaPolygonContext - dlFrequencyContext - ulFrequencyContext - rATContext | -weakRSRPRatioTarget -lowSINRRatioTarget |
| Intent containing an expectation on RAN UE throughput performance to be assured (clause 5.1.5) | Radio Network Expectation | -coverageAreaPolygonContext - dlFrequencyContext - ulFrequencyContext - rATContext - uEGroupContext | - aveULRANUEThptTarget - aveDLRANUEthptTarget - lowULRANUEThptRatioTarget - lowDLRANUEThptRatioTarget |
| Intent containing an expectation for delivering 5GC network (clause 5.1.8) | 5GC Network Expection | - nfTypeContext - nfInstanceLocationContext - pLMNContext- taiContext - servingScopeContext - dnnContext | - maxNumberofPDUsessionsTarget - maxNumberofRegisteredsubscribersTarget - incomingDataTarget - outgogingDataTarget |
| Intent containing an expectation on RAN capacity performance to be assured (clause 5.1.5) | Radio Network Expectation | -coverageAreaPolygonContext - dlFrequencyContext - ulFrequencyContext - rATContext | - highUlPrbLoadRatioTarget - highDlPrbLoadRatioTarget - aveUlPrbLoadTarget - aveDlPrbLoadTarget |
| Intent containing an expectation on RAN energy saving (clause 5.1.7) | Radio Network Expectation | - coverageAreaPolygonContext - pLMNContext - dlFrequencyContext - ulFrequencyContext - rATContext | - rANEnergyConsumptionTarget -rANEnergyEfficiencyTarget - aveULRANUEThptTarget - aveDLRANUEThptTarget |
| Intent containing an expectation for delivering radio service (clause 5.1.2) | Radio Service Expectation | - coverageAreaPolygonContext - serviceType | - dLLatencyTarget - uLLatencyTarget - dLThptPerUETarget - uLThptPerUETarget |

End of changes