3GPP TSG-SA5 Meeting #153 	S5-240357

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0508 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add performance measurements for NSOEU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSOEU |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To follow SA5 performance management working approach, it is proposed to move the performance measurements for NSOEU which are captured in draft TS 28.318 into TS 28.552. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add performance measurements related to cell service status for NSOEU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of performance measurements for NSOEU in TS 28.552 which specifies 5G performance measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 5.X, new clause A.XX |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.X	Performance measurements for NSOEU

### 5.X.1	Measurements related to cell service status

#### 5.X.1.1	Total cell In-Service duration

a)	This measurement provides the total time duration for which an active cell in a gNB-DU remains In-Service. The gNB-DU reports the Service Status of the Active cells to the gNB-CU. If the Service Status value is “In-Service” it means the active cell is operational, and it is able to serve UEs. The active cells are considered as In-Service until the gNB-DU reports the Service Status as “Out-Of-Service” to the gNB-CU. The gNB-DU reports the Service Status to gNB-CU using the GNB DU CONFIGURATION UPDATE messages.

b)	CC.

c)	This measurement is obtained in two steps. First step involves determining the time interval between the time stamps Tin and Tout. Where Tin is the time stamp when the Service Status is reported as “In-Service” for a cell by gNB-DU to gNB-CU and Tout is the time stamp when Service Status is reported as “Out-Of-Service” by gNB-DU to gNB-CU for the same cell in the GNB DU CONFIGURATION UPDATE messages. Second step involves summing up all these time intervals of a cell which fall within the desired observation time frame Tobv. This gives total in-service duration of a cell within the observation time frame Tobv. Where, Tobv is monitorGranularityPeriod  as defined in TS 28.622[13] clause 4.3.16.

d)	Each measurement is an integer value representing the number of minutes for which the cell was in-Service.

e)	SO.CellInServiceTotal.NCGI, where NCGI is NR Cell Global Identifier as defined in TS 28.541 [16]. The number of measurements is equal to the number of NCGIs (cells).

f)	GNBCUCPFunction.

g)	Valid for packet switching.

h)	5GS.

#### 5.X.1.2	Total cell outage duration

a)	This measurement provides the total time duration for which a cell in a gNB remains out of service. The gNB-DU reports the Service Status of the Active cells to the gNB-CU. If the Service Status value is “Out-Of-Service” it means the active cell is not operational, and it is not able to serve UEs. The active cells are considered as Out-of-Service until the gNB-DU reports the Service Status as In-Service to the gNB-CU. The gNB-DU reports the Service Status to gNB-CU using the GNB DU CONFIGURATION UPDATE message

b)	CC.

c)	This measurement is obtained in two steps. First step involves determining the time interval between the time stamps Tout and Tin. Where Tout is the time stamp when Service Status is reported as “Out-Of-Service” by gNB-DU to gNB-CU for a cell and Tin is the time stamp when the Service Status is reported as “In-Service” for the same cell by gNB-DU to gNB-CU in the GNB DU CONFIGURATION UPDATE messages. Second step involves summing up all these time intervals of a cell which fall within the desired observation time frame Tobv. This gives total outage duration of a cell within the observation time frame Tobv. Where, Tobv is monitorGranularityPeriod  as defined in TS 28.622 [13] clause 4.3.16.

d)	Each measurement is an integer value. It represents the number of seconds for which the cell was Out-Of-Service

e)	SO.CellOOSTimeTotal.NCGI, where NCGI is NR Cell Global Identifier as defined in TS 28.541 [16]. The number of measurements is equal to the number of NCGIs (cells).

f)	GNBCUCPFunction.

g)	Valid for packet switching.

h)	5GS.

#### 5.X.1.3	Average cell outage duration

a)	This measurement provides the average time duration of an outage i.e. average time for which a cell in a gNB-DU remains out of service per outage. The gNB-DU reports the Service Status of the Active cells to the gNB-CU. If the Service Status value is “Out-Of-Service” it means the active cell is not operational, and it is not able to serve UEs. The active cells are considered as Out-of-Service until the gNB-DU reports the Service Status as In-Service to the gNB-CU. The gNB-DU reports the Service Status to gNB-CU using the GNB DU CONFIGURATION UPDATE message.

b)	CC.

c)	This measurement is obtained by dividing the measurement SO.CellOOSTimeTotal.NCGI (clause 5.X.1.2) with Noutage where Noutage  is the number of occurrences of outage for that cell within the desired observation time frame Tobv where, Tobv is monitorGranularityPeriod  as defined in clause 4.3.16 of TS 28.622 [13].This gives an average duration per outage of a cell.

d)	Each measurement is an integer value representing the average number of seconds for which the cell was Out-Of-Service per outage occurrence.

e)	SO.CellOOSTimeAverage.NCGI, where NCGI is NR Cell Global Identifier as defined in TS 28.541 [16]. The number of measurements is equal to the number of NCGIs (cells).

f)	GNBCUCPFunction.

g)	Valid for packet switching.

h)	5GS.

#### 5.X.1.4	Cell AvailabilityNRCellDU

a) This measurement provides the amount of time that the NRCellDU part of the cell is available to provide services to UEs within a given measurement window (Twin).  It is computed by the NRCellDU as the arithmetic sum of the amount of time that the NRCellDU is unlocked and enabled to serve UEs in each measurement window Twin.

b) cc

c) This measurement is obtained as: CellAvailNRCellDU = TOSTNRCellDU=Enabled && T(ASTNRCellDU=Unlocked)Twin*100, where CellAvailNRCellDU is the availability of the NRCellDU, Twin is the measurement window, T(OSTNRCellDU=Enabled) is the total amount of time that the operational state attribute(OST) of the NRCellDU is enabled in a given Twin, and T(ASTNRCellDU=Unlocked) is the total amount of time that the administrative state attribute(AST) of the NRCellDU is unlocked in a given Twin.

d) Two values: The CellAvailNRCellDU from 0 to 100 and the corresponding Twin.

e) CellAvailNRCellDU

f) NRCellDU

g) Valid for packet switched traffic

h) 5GS

#### 5.X.1.5	Cell AvailabilityNRCellCU

a) This measurement provides the amount of time that the NRCellCU part of the cell is available and providing services within a given measurement window (Twin).  It is computed by the NRCellCU as the arithmetic sum of the amount of time that the NRCellCU is unlocked and enabled in each measurement window Twin.

b) cc

c) This measurement is obtained as: CellAvailNRCellCU = TOSTNRCellCU=Enabled && T(ASTNRCellCU=Unlocked)Twin*100, where CellAvailNRCellCU is the availability of the NRCellCU, Twin is the measurement window, T(OSTNRCellCU=Enabled) is the total amount of time that the operational state attribute(OST) of the NRCellCU is enabled in a given Twin, and T(ASTNRCellCU=Unlocked) is the total amount of time that the administrative state attribute(AST) of the NRCellCU is unlocked in a given Twin.

d) Two values: The CellAvailNRCellDU value from 0 to 100 and the corresponding Twin.

e) CellAvailNRCellCU

f) NRCellCU

g) Valid for packet switched traffic

h) 5GS

Next change

# A.XX	Monitoring for Network and Service Operations for Energy Utilities (NSOEU)

To monitor and control its distribution grid, the DSO uses thousands of 3GPP compatible UEs. These UEs are spread across a wide geographical area, just like the distribution grid the UEs support. The DSO uses the UEs to provide connectivity to the monitor and control infrastructure of the distribution grid. This infrastructure has very high availability service requirements. To fulfil these requirements, highly available communication is required. To achieve this highly available communication, the DSO monitors performance of communications services they use. If and when the DSO deems it necessary, the DSO proactively activates additional communication services.

The 3GPP management system shall, subject to mobile network operator policy, regulatory requirements and contractual obligations, allow the DSO to request collection and reporting of NSOEU related cell service status PM data.

End of changes