# 3GPP TSG-SA5 Meeting #153	S5-240649

# Seville, Spain, 29th January-2nd February, 2024

# Source:	Samsung

# Title:	Rel-18 pCR TS 28.318 NRM Definitions for coordinated rapid recovery Stage 3

# Document for:	Approval

Agenda Item:	6.6.4.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	Rationale

This pCR proposes the stage 3 changes for Coordinated Rapid Recovery

Forge Link: https://forge.3gpp.org/rep/sa5/MnS/-/tree/Rel_18_pCR_TS_28.318_NRM_definitions_Coordinated_Rapid_Recovery

# 3	Detailed proposal

First Change

## 8	Stage 3 definitions

## 8.1	General

This annex contains the OpenAPI definition of the DSORecovery NRM in YAML format.

The Information Service (IS) of the DSORecovery NRM is defined in clause x.

Mapping rules to produce the OpenAPI definition based on the IS are defined in TS 32.160 [10].

## 8.2	Solution Set (SS) definitions

### 8.2.1	OpenAPI document "TS28318_ DSORecovery.yaml"

openapi: 3.0.1

info:

title: 3GPP NRM

version: 18.5.0

description: >-

OAS 3.0.1 specification of the DSORecovery NRM

© 2023, 3GPP Organizational DSORecovery Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.318; DSORecovery NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.318/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

DsoInterventionInfo:

type: object

properties:

dsoFeederRestoreTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

dsoRequiredInterventionDuration:

type: integer

isDsoPriorityRecoveryLocation:

type: boolean

ServiceRestoreInfo:

type: object

properties:

dsoRestoreTimeActual:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

isDsoRestoreComplete:

type: boolean

ServiceLocationIdentifier:

type: object

properties:

geoCoordinate:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoCoordinate'

energySupplyId:

type: string

UpsInfo:

type: object

properties:

isUpsAvailable:

type: boolean

totalUpsBackupDuration:

type: integer

remainingUpsBackupDuration:

type: integer

baseStationId:

type: string

#-------- Definition of concrete IOCs --------------------------------------------

MnS:

oneOf:

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

SubNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-Attr'

- type: object

properties:

Subnetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

DsoRapidRecovery:

$ref: '#/components/schemas/DsoRapidRecovery-Multiple'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-ncO'

DsoRapidRecovery-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

dsoOutStartTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

dsoRestoreTimeExpected:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

dsoRapidInterventionInfo:

$ref: '#/components/schemas/DsoInterventionInfo'

dsoServiceRestoreInfo:

$ref: '#/components/schemas/ServiceRestoreInfo'

dsoRapidRecoveryLocation:

$ref: '#/components/schemas/ServiceLocationIdentifier'

upsInfo:

$ref: '#/components/schemas/UpsInfo'

- type: object

properties:

MnoRapidInterventionInfo:

$ref: '#/components/schemas/MnoRapidInterventionInfo-Multiple'

MnoRapidInterventionInfo-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

mnoInterventionTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

mnoInterventionDuration:

type: integer

isMnoInterventionPossible:

type: boolean

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

DsoRapidRecovery-Multiple:

type: array

items:

$ref: '#/components/schemas/DsoRapidRecovery-Single'

MnoRapidInterventionInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/MnoRapidInterventionInfo-Single'

#--------------------------------- Definition ------------------------------------

resources-DSORecovery:

oneOf:

- $ref: '#/components/schemas/MnS'

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/DsoRapidRecovery-Single'

- $ref: '#/components/schemas/MnoRapidInterventionInfo-Single'

End of Changes