3GPP TSG-SA5 Meeting #155 	S5-242417

Jeju, South Korea, 27 – 31 May 2024

# Source:	Ericsson

# Title:	pCR TR 28.873 Job identity generator between producer and consumer

# Document for:	Approval

Agenda Item:	6.19.11

# 1	Decision/action requested

For agreement and approval

# 2	References

[1]	TR 28.873 " Study on data management, subscriptions and reporting"

[2]	SP-231732 "Study on data management regarding subscriptions and reporting"

# 3	Rationale

As discussed and agreed in pCR S5-242096, the identity used in a measurement job shall be globally unique.

# 4	Detailed proposals

*** START OF NEXT CHANGE ***

# 6 Potential solutions

## 6.X	Potential solution #X: Job identity generator between producer and consumer

### 6.X.1 Description

As discussed and agreed in subclause 5.1 that the identity used in a measurement job shall be globally unique between consumers and producers.

This solution introduces a new function, named ID generator, in the network which has the responsibility to generate a global unique identity for use case 1 defined in section 5.1.

At PM/Trace/MDT/QoE activation, all requests are going to be sent to the ID generator without a global unique identity.  Upon receiving this request, the ID generator

- generates a globally unique identity,

- inserts it into the PM/Trace/MDT/QoE activation request,

- sends to the producer, and

- responses to the consumer with the generated globally unique identity

In any subsequent PM/Trace/MDT/QoE message, the globally unique identity is going to be used as Job ID, Trace Reference, or QoE Reference.

Figure 6.x.1-1 PM/Trace/MDT/QoE with ID generator

*** END OF CHANGE ***