3GPP TSG-SA5 Meeting #155 	S5-242882

Jeju, South Korea, 27 - 31 May 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Add potential solution for discovering management services through the CCF

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR proposes adding a new potential solution for discovering management services to clause 5.1 of TR 28.879 [1].

# 4	Detailed proposal

It is proposed that the following changes be made in clause 5.1 of TR 28.879 [1].

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]            	3GPP TS 28.533: "Management and orchestration; Architecture Framework".

[3]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[4]           	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[5]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2"

[6]	SP-231669: "LS on collaboration and alignment of 3GPP defined application enablers with GSMA Open Gateway".

[7]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[8]	3GPP TS 23.255: "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows".

[9]	3GPP TS 23.286: "Application layer support for Uncrewed Aerial Systems (UAS) services; Functional architecture and information flows".

[10]	3GPP TS 23.545: "Application layer support for Factories of the Future (FF) ".

[11]	3GPP TS 23.542: "Application layer support for Personal IoT Networks".

[12] 	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[13]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3".

[14]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP Northbound APIs".

[15]	 "The Ecosystem for Open Gateway NaaS API Development", white paper, June 2023 [link]

[16]	"GSMA Operator Platform Group – Requirements and Architecture", version 5.0, July 2023 [link]

[17]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[18]	3GPP TS 28.531: "Management and orchestration; Provisioning"

[19]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service"

[X]	3GPP TS 23.222: "Functional architecture and information flow to support Common API Framework for 3GPP Northbound APIs; stage 2".

[Z]                       3GPP TS 28.319: "Management and orchestration; Access control for management services".

[Y]                      3GPP TR28.871: " Study on Service Based Management Architecture (SBMA) enhancement phase 3".

Second Change

## 5.1 Exposure of management services

#### 5.1.Z.3	Potential solutions

##### 5.1.Z.3.X	Potential solution #X: Discovery of management services through CCF

###### 5.1.Z.3.X.1	Introduction

This potential solution describes how an external MnS consumer can discover the published management services and associated management capabilities through the CCF.

###### 5.1.Z.3.X.2	Description

The external MnS consumer needs to initially perform the onboarding procedure to the CCF to discover management services. To do this, the external MnS consumer should be provided with the onboarding information (e.g., the end-point URL of the CCF and the authentication and authorization information). Accordingly, the external MnS consumer and the CCF should be configured with the authentication and authorization information to enable successful onboarding. The details on how this is to be achieved are for further study under MSAC work (TR 28.319[Z]).

The external MnS consumer can discover the management services upon successful onboarding. Moreover, every external MnS consumer should be able to discover only the management services and supported capabilities they are authorized to discover and later consume on the MnS producer. The details on how this is to be achieved are for further study under MSAC work (TR 28.319[Z]).

To enable the external MnS consumer to discover the available management services and the associated management capabilities at the CCF, there is a need to enhance the query information element of the service API discovery request  (see clause 8.7 TS 23.222[X]), as shown in Table 5.1.Y.3.X.2-1.

Table 5.1.Y.3.X.2-1: Proposed enhancement to the CAPIF discovery query information element to support the discovery of management services (extract from Table 8.1.2.2.3.1-1 of TS 29.222[13]).

| Name | Data type | P | Cardinality | Description | Comments |
| --- | --- | --- | --- | --- | --- |
| api-invoker-id | string | M | 1 | It represents the identifier (assigned by the CCF) ofthe API invoker that is sending the request. It may also represent the identifier of the CCF that is sending the request if the request is sent over the CAPIF-6/6e reference point. (NOTE) |  |
| api-name | string | O | 0..1 | Contains the API name as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |
| api-version | string | O | 0..1 | Contains the API major version conveyed in the URI (e.g. v1). |  |
| comm-type | CommunicationType | O | 0..1 | Communication type used by the API (e.g. REQUEST_RESPONSE). |  |
| protocol | Protocol | O | 0..1 | Protocol used by the API. |  |
| aef-id | string | O | 0..1 | AEF identifier. |  |
| data-format | DataFormat | O | 0..1 | Data format used by the API (e.g. serialization protocol JSON). |  |
| api-cat | string | O | 0..1 | The service API category to which the service API belongs. |  |
| preferred-aef-loc | AefLocation | O | 0..1 | The preferred AEF location. If this parameter is present, the CCF shall try to discover a matched AEF location the service API supports. This parameter is ignored by the CCF if there is no matching record found. |  |
| supported-features | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |  |
| api-supported-features | SupportedFeatures | C | 0..1 | Features supported by the discovered service API indicated by api-name parameter. This may only be present if the api-name query parameter is present. |  |
| mnsType (clause 4.3.42 TS 28.622[3]) | ENUM | O | 1 | Type of management service. | New enhancements to the Service API Query information element are needed to enable the discovery of MnSs. |
| mnsCapability (clause 5.3.3 TR 28.871[Y]) | ENUM | O | 0..* | It describes the types of management capabilities provided by MnS instance that the MnSType is ProvMnS The allowed value needs further discussion | New enhancements to the Service API Query information element are needed to enable the discovery of MnSs. |
| NOTE: This parameter is not part of API filter criteria so that it is not used in matching APIs published in the CCF. |  |  |  |  |  |

It is to be determined if the management scope (i.e., mnsScope attribute of MnSInfo IOC clause 4.3.42 in 28.622[3]) of the management service should be included in the discovery request sent from the external MnS consumers to the CCF. The management scope could also be limited so that the external MnS consumer can only discover the management object instances (MOIs) associated with their service (e.g., for a vertical, this could be the MOIs serving their user equipment). If and how this can be done remains for further study under MSAC (TR 28.319 [Z]).

#### 5.1.Z.4	Evaluation of potential solutions

##### 5.1.Z.4.X Evaluation of Potential Solution #X: Discovery of management services and supported capabilities through CAPIF

The proposed solution still has the following gaps that need further discussion:

- How does the external MnS consumer obtain the onboarding information (i.e., which entity in the management system/Management services API provider domain is responsible for this?) to enable the onboarding to the CCF?

  - Requires further study under MSAC (TR 28.319 [Z]).

- How to configure the CCF with management access control information to enable the successful onboarding of the external MnS consumer?

  - Requires further study under MSAC (TR 28.319 [Z]).

- How to configure the CCF with the discovery policies for the management services? The configuration should be such that the external MnS consumer can only discover those management services and associated capabilities they can consume on the MnS producer. This requires some means to mirror the access control information present in the management system at the CCF authorization function. Otherwise, it would be inefficient for an external MnS consumer to discover management services that they cannot consume on the MnS producers.

  - Requires further study under MSAC (TR 28.319 [Z]).

- How to communicate to SA6 to add the required enhancements to the query information element (shown in Table 5.1.Y.3.X.2-1) to enable the discovery of management services with the associated management capabilities?

  - New requirement to SA6

- Do we want to include the management scope of the management service as part of the query parameters for the service API discovery request?

End of Changes