3GPP TSG-SA5 Meeting #155 	S5-243252

Jeju, South Korea, 27 - 31 May 2024

# Source:	Nokia, Ericsson

# Title:	Rel-19 pCR TR 28.879 Add use case for publishing management services to the CCF

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR proposes to add a new use case, i.e., the publishing of management services in clause 5.1 of TR 28.879 [1].

# 4	Detailed proposal

It is proposed that the following changes be made in clause 5.1 of TR 28.879 [1].

First Change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC		5G Core

CAPIF 	Common API Framework

CCF 	CAPIF Core Function

CSP	Communication Service Provider

EAS	Edge Application Server

ECS	Edge Configuration Server

EDN	Edge Data Network

EEC	Edge Enabler Client

EES	Edge Enabler Server

FF	Factories of the Future

GSMA	GSM Association

MnS	Management Service (see TS28.533[2])

NaaS	Network as a Service

NEF	Network Exposure Function

NOP                    Network Operator

NSACF	Network Slice Access Control Function

NSCE 	Network Slice Capability Enablement

NWDAF	Network Data Analytics Function

OAM	Operation, Administration and Maintenance

OPAG	Operator Platform API Group

OPG	Operator Platform Group

SEAL		Service Enabler Abstraction Layer

UAS	Uncrewed Aerial Systems

V2X		Vehicle-to-Everything

VAE		Vertical App Enabler

WAS	Whole Agreement Services

WG	Working Group

APF                     API publishing function

Second Change

## 5.1 Exposure of management services

### 5.1.Y Use case #<Y>: Publishing of management services to the CCF

#### 5.1.Y.1	Description

Upon registration of MnS producer into the CAPIF, then the MnS producer can be enabled to decide what management service information will be made available for external consumption (i.e., the entire MnS, or a subset of it).

Once this decision is made, then this information needs to be published into CCF, so that it can later be used by the external MnS consumers to discover and subsequently invoke the MnS. To this end, the following should occur:

First, the management services are mapped into service APIs that can be published to the CCF. This is due to the fact that the information published into CAPIF is information related to service APIs. This information is represented with “ServiceAPIDescription” (see clause 8.2.4.2.2 in TS 29.222[13]). However, in the 3GPP management system, the IOC that describes the information related to an MnS’s is MnSInfo IOC (see clause 4.3.42 in TS 28.622[3]). The MnS producer posts and manage MnSInfo instances into MnSRegistry IOC (see clause 4.3.41 in TS 28.622[3]). In this regard, there is a need to map MnSInfo elements into serviceAPIDescription and to determine who in the 3GPP management system does the map.

Secondly, the mapped management services (now service APIs) will be published in the CCF. In CAPIF, the API Publishing Function (APF) is the API provider domain’s entity responsible for publishing and managing service API information (see clause 6.3.5 in TS 23.222[5]). The APF supports the CAPIF-4 interface (see clause 6.4.7 in TS 23.222[5]). However, in the 3GPP management system, there is a need to determine who will play the APF role, for example:

- MnS registry (represented by the  MnSRegistry IOC): The issue is that MnS registry is a managed object and therefore as a managed object, it is not capable of either the mapping of management service information into service API information or the actual publishing of the mapped management service information to the CCF. Accordingly, the MnSRegistry IOC can not play the role of the CAPIF APF. There is an option of defining a new management function (MnF) that consumes the MnSInfo IOCs data from MnSRegistry IOC and, in turn, takes on the role of APF and publishes the management services to the CCF via the CAPIF-4 interface. This option has the advantage of simultaneously publishing all the management services to the CCF. However, considering SBMA principles, the definition of a new MnF would mean going back to IRP definitions.

- MnS producer: the MnS producers are already capable of providing the functionalities offered by the CAPIF APFs. Accordingly, the MnS producers can be enhanced to support the CAPIF-4 interface.

#### 5.1.Y.2	Potential requirements

PREQ-FS_MExpo-01 The 3GPP management system shall have the capability to map management services information (described by the MnSInfo IOC) into service API information (described by the CAPIF ServiceAPIDescription information element).

PREQ-FS_MExpo-02 The 3GPP management system shall have the capability to publish the mapped management service information to the CCF.

End of Changes