3GPP TSG-SA5 Meeting #155 	S5-243255

Jeju, South Korea, 27 - 31 May 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Add potential solution for discovering management services through the CCF

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR proposes adding a new potential solution for discovering management services to clause 5.1 of TR 28.879 [1].

# 4	Detailed proposal

It is proposed that the following changes be made in clause 5.1 of TR 28.879 [1].

Begin Change

## 5.1 Exposure of management services

#### 5.1.Z.3	Potential solutions

##### 5.1.Z.3.X	Potential solution #X: Discovery of management services through CCF

###### 5.1.Z.3.X.1	Introduction

This potential solution describes how an external MnS consumer can discover the published management services through the CCF.

###### 5.1.Z.3.X.2	Description

The external MnS consumer needs to initially perform the onboarding procedure to the CCF to discover management services. To do this, the external MnS consumer should be provided with the onboarding information (e.g., the end-point URL of the CCF and the authentication and authorization information). Accordingly, the external MnS consumer and the CCF should be configured with the authentication and authorization information to enable successful onboarding.

The external MnS consumer can discover the management services upon successful onboarding.

For the external MnS consumer to discover the available management services through the CCF, the external MnS consumer sends an HTTP GET request to the CCF. This HTTP GET request supports optional query parameters of type "query information element," as shown in Table 5.1.Y.3.X.2-1(see clause 8.7 TS 23.222[5] and clause 8.1 TS 29.222[13]).

Table 5.1.Y.3.X.2-1: CAPIF discovery query information element to support the discovery of management services (extract from Table 8.1.2.2.3.1-1 of TS 29.222[13]).

| Name | Data type | P | Cardinality | Description | Equivalent MnS Info IOC attribute/Comments |
| --- | --- | --- | --- | --- | --- |
| api-invoker-id | string | M | 1 | It represents the identifier (assigned by the CCF) ofthe API invoker that is sending the request. It may also represent the identifier of the CCF that is sending the request if the request is sent over the CAPIF-6/6e reference point. (NOTE) | Identifier for the external MnS consumer inside the CCF. |
| api-name | string | O | 0..1 | Contains the API name as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. | mnsLabel |
| api-version | string | O | 0..1 | Contains the API major version conveyed in the URI (e.g. v1). | mnsVersion |
| comm-type | CommunicationType | O | 0..1 | Communication type used by the API (e.g. REQUEST_RESPONSE). | Shall be either REQUEST_RESPONSE or SUBSCRIBE_NOTIFY |
| protocol | Protocol | O | 0..1 | Protocol used by the API. |  |
| aef-id | string | O | 0..1 | AEF identifier. | Identifier of the MnS producer inside the CCF. |
| data-format | DataFormat | O | 0..1 | Data format used by the API (e.g. serialization protocol JSON). |  |
| api-cat | string | O | 0..1 | The service API category to which the service API belongs. |  |
| preferred-aef-loc | AefLocation | O | 0..1 | The preferred AEF location. If this parameter is present, the CCF shall try to discover a matched AEF location the service API supports. This parameter is ignored by the CCF if there is no matching record found. |  |
| supported-features | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |  |
| api-supported-features | SupportedFeatures | C | 0..1 | Features supported by the discovered service API indicated by api-name parameter. This may only be present if the api-name query parameter is present. |  |
| NOTE: This parameter is not part of API filter criteria so that it is not used in matching APIs published in the CCF. |  |  |  |  |  |

End Change