3GPP TSG-SA5 Meeting #155 	S5-243318

Jeju, South Korea, 27 - 31 May 2024

# Source:	China Telecom Corporation Ltd., ZTE Corporation，Huawei Technologies Co., Ltd, China Unicom

# Title:	Add use case and requirements on resource load of RedCap service

# Document for:	Approval

Agenda Item:	6.19.17

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

# 3	Rationale

This contribution proposes to add use case and requirements on resource load of RedCap service.

Network resource load performance evaluation is essential for network management. However, the current method of combining RedCap and non-RedCap UEs for resource utilization may lead to inaccurate representation of RedCap performance. By specifically addressing the resource load of RedCap service, we can ensure more accurate performance evaluations, which are critical for optimizing network management and enhancing user experience for RedCap service.

This is related to WT-2.1 investigate the metrics for resource load of RedCap service.

RedCap UEs typically have reduced capabilities and different performance characteristics compared to standard UEs.

Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) related to eMBB (enhanced mobile broadband) devices. By distinguishing between the two types of UEs, network operators can better identify and troubleshoot specific network resource issues related to RedCap UEs and non-RedCap UEs, thus can allocate resources more efficiently and ensure that RedCap UEs receive adequate resource.

# 4	Detailed proposal

1st Change

## 5.X	Use case #X:  Performance evaluation on resource load of RedCap network service

### 5.X.1	Description

The radio resource utilization measurements could provide operators the load information of the radio network during the measurement time period. Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) relative to eMBB (enhanced mobile broadband) devices due to their reduced capabilities and lower data rate requirements. PRBs related measurements may include total PRB usage, PDSCH PRB Usage, PUSCH PRB Usage etc.

In TS 28.552 [10], radio resource utilization metrics are used to evaluate the performance of the entire network, but the current approach combines the resource utilization of RedCap UEs and non-RedCap UEs, which may not accurately reflect the performance of the RedCap service.

RedCap UEs typically have reduced capabilities and different performance characteristics compared to standard UEs.

Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) related to eMBB (enhanced mobile broadband) devices. By distinguishing between the two types of UEs, network operators can better identify and troubleshoot specific network resource issues related to RedCap UEs and non-RedCap UEs, thus can allocate resources more efficiently and ensure that RedCap UEs receive adequate resource.

Therefore, it is essential to investigate an approach for separately calculating the resource load of RedCap UEs and defining the resource load performance of networks that deliver RedCap service. This will ensure a more precise evaluation of the user experience for RedCap UEs, thus make the operator to be aware of whether a cell has ever experienced high load or not in the monitoring period, and is a key input to network capacity planning and load balancing for RecCap service.

Similarly, to distinguish RedCap UEs and non-RedCap UEs for metric of the number of the active UEs in each cell is helpful for operators to know how many DRBs are running with buffered data per cell for different kinds of UEs.

Through the metrics of radio resource utilization and active users of RedCap UEs and non-RedCap UEs, operators can better evaluate the overall consumption of network resources of RedCap UEs to facilitate adjustment of related network policies.

### 5.X.2	Potential requirements

REQ-RedCap-Perf-Resource: The 3GPP management system should have capability to provide measurements related to resource load of RedCap service.

End of change